3GPP TSG RAN meeting #108                                                 RP-251735

Prague, Czech Republic, June 9-13, 2025

Title: 	ZTE's views on candidate RF/OTA topics for Rel-20

Source: 	ZTE Corporation,Sanechips

Agenda item:	15.4.2

Document for:	Discussion

Introduction

In the RAN#107 meeting, there was some initial discussion on Rel-20 RAN4 package and the discussion outcome is summarized in moderator’s summary [1]. In this contribution, we would like to share our further views from both BS RF and UE RF perspective.

Discussion

- 2.1. BS RF enhancement

- 2.1.1 ZTE’s view

The high level views on the companies’ Rel-20 BS RF proposals so far are summarized in the following table.

| Numbering | Objective | ZTE’s Preference |
| --- | --- | --- |
| 1 | Workable and testable co-location requirements for NR BS operating in FR1 high band(s) | High priority |
| 2 | Additional requirement set(s) for AAS BS with more OTA requirements than the existing BS type 1-H | High priority |
| 4 | NB-IoT introduction to AAS BS specifications | [High priority] |
| 3 | OTA requirements enhancement for receiver in-band blocking for FR1 high band(s) | Low priority |
| 5 | OBUE requirements structure simplification | Low priority |
| 6 (Rel-19 leftover) | SBFD coexisting with SBFD in FR1 | High priority |

- 2.1.2 Justification

Objective 1:

The existing BS co-location requirement in FR1 relies on a Maximum Coupling Loss (MCL) assumption of 30 dBc, which was adopted from FR1 low bands dating back to the UTRA phase. However, during the Release 19 phase, it was identified that the 30 dBc MCL assumption was no longer valid for FR1 high bands. In the Release 20 phase, it is crucial to resolve these issues, particularly for FR1 BSs with massive MIMO operation. Appropriate requirement definitions can effectively guide and support the design of front-end filters.

Objective 2a:

For the legacy FR1 NR BS deployed so far, it is very typical to implement the full digital beamforming architecture with fixed tilt setting within each sub-array. However for FR1 upper mid bands (e.g.U6GHz), hybrid beamforming architecture will be a more important form of implementation to balance the trade off between the number of transmitters and the large number of antenna elements (e.g. up to 1k or 1.5k antenna elements). For the hybrid beamforming architecture of FR1 BS type 1-H, due to the active components within Radio Distribution Network (RDN), it is important to study and specify the appropriate requirement for BS type 1-H with hybrid beamforming.

Objective 2b:

For the existing BS type 1-H, there are only two radiated requirements specified e.g.EIRP and EIS. However the rest of conducted requirement might not reflect the overall radiated performance of the real operation of NR BS. In addition, for some bidding test of the operators, BS type 1-H also include some other radiated requirements e.g. EVM, ACLR, spurious emission, ACS, blocking.

[Objective 4:

NB-IoT In-band operation, NB-IoT guard band operation, NB-IoT standalone operation and NB-IoT operation in NR in-band, NB-IoT operation in NR guard band have not been supported for AAS MSR BS or NR BS type 1-H/1-O. The primary reason for this type of restriction is that NB-IoT is expected to be deployed in operating bands below 2 GHz. However, for these specific frequency bands, BS type 1-C was the most typical BS implementation previously. Recently, for FR1 low bands (e.g. lower than 2GHz), massive MIMO is also supported to further improve the network capacity, therefore it is necessary to specify the NB-IoT operation under the BS type 1-H/1-O architecture in Rel-20.]

Objective 6:

SBFD is one important BS evolution technique in 5G-A phase and SBFD coexisting with SBFD BS is also one important scenario to make sure that SBFD BS could be well deployed especially for some bands without any legacy TDD BS deployed so far. During the RAN4#115 meeting, it is agreed to postpone the study to the future release. It is proposed to consider this SBFD coexisting with SBFD scenario in Rel-20 5G-A phase.

- 2.1.3 WID objective

- 1) Workable and testable co-location requirements for NR BS operating in FR1 high band(s)

  - Necessity of this objective depends on the progress in Rel-19 and it will be checked in September 2025.

    - Both intra-band co-location and inter-band co-location should be considered.

  - NOTE: refinement of wording on this bullet is needed.

- 2) Additional requirement set(s) for AAS BS with more OTA requirements than the existing BS type 1-H

  - Applicable specifications: AAS BS, NR BS

  - Sub-objective 1: Specify the applicable conducted reference points of BS type 1-H for hybrid beamforming BS architecture in FR1

  - Sub-objective 2: additional radiated requirement for BS type 1-H e.g. EVM, ACLR, spurious emission, ACS, co-located blocking requirement;

- 6) SBFD coexisting with SBFD

  - Applicable specifications: FR1 WA and MR BS;

  - To define the additional emission requirement and in-band blocking requirement;

- 2.1.4 Proposed TU budget

Proposed Tu budget (no RAN1/RAN2 impacts expected):

- 2.2. UE RF enhancement

- 2.2.1 ZTE’s view

The high level views on the companies’ Rel-20 UE RF proposals so far are summarized in the following table.

| Numbering | Objective | ZTE’s Preference |
| --- | --- | --- |
| 1 | FR1 HPUE enhancement (PC2/PC1.5/PC1) for single carrier and EN-DC/NR-CA - This item receives common interest, but needs discussion on the feasibility of some use cases and also needs down-selection. | High priority |
| 2 | Power enhancement with relaxed UE Tx EVM and/or uplink throughput improvement with advanced BS receiver (AI based receiver is not precluded as the starting point) - TBD on objectives should include Tx EVM relaxation, uplink throughput improvement, or both. - Study phase is needed if it is considered and FFS whether to treat it in UE RF or demod package - There are still concern on whether Tx EVM should be part of Rel-20 package for 5G-A. | Have the interest and propose to further discuss it under the 6G SI phase given the workload for this objective. - Need some field test to emulate the UE actual PA non-linearity performance to check the performance gain instead of PA modelling and AI based simulation; - Performance comparison between the legacy estimation/compensation method and AI based estimation/compensation method; - Baseband implementation complexity for the legacy method and AI based method. |
| 3 | Band/band combination requirements simplification and MSD simplification/optimization - TBD on whether MSD work is for PC3 and/or HPUE as Rel-19 leftover work. | In Rel-19, the MSD rules of 1 band 1 UL CC and 2 band 2 UL CC for HPUE band combination were agreed. However, the MSD rules of 1 band 2 UL CC and 2 band 3UL CC are out of the scope of Rel-19. In other words, RAN4 only complete part of work for HPUE band combination MSD rules in Rel-19. In Rel-20, we prefer to focus on HPUE band combination leftover works. |
| 4 | Tx requirements enhancement for U6GHz | Neutral |
| 5 | Power classes framework | Neutral, RAN4 had discussed it for about two years with little consensus, the discussion was stopped due to it is very difficult to achieve the agreements after two years of discussion. |
| 6 | Optimize MPR/A-MPR | Have the interest and propose to discuss it under the 6G SI phase since the new UE Tx modelling requires lots of UE implementation inputs and also lots of debates are expected to minimize the RF implementation margin. |
| 7 | Requirement enhancement to support coherent UL MIMO | High priority |
| 8 | UL Tx switching enhancement | Neutral, pending on the Rel-19 discussion. |
| 9 | 256QAM evolution | Neutral |
| 10 | SRS enhancement | Neutral |
| 11 | SL-U enhancement | Low priority |
| 12 | MPR reduction by leveraging asymmetric ACLR | Neutral Asymmetric ACLR is valid observation due to the memory effects of PA. However this might be random from different PA vendors or even different PA chipset for the same vendor, and it might be difficult to quantify the effects of asymmetric ACLR. |

- 2.2.2 Justification

Objective 1:

For 4Tx on a single carrier for CPE/FWA/vehicle/industrial device, only PC1.5 was defined in Rel-18. In order to further improve UL performance by increasing the UL power for FWA device, a higher power class for PC1 4Tx FWA with 4x26dBm PA configuration is a possible way.

For PC1.5 on a single carrier, it is only for TDD via 2Tx as of today, and only 26dBm+26dBm PA power configuration is used for PC1.5 2Tx inter-band CA and PC1.5 intra-band CA band combination. Due to the RFFE development and increasing of the demand, PC1.5 for TDD via 1Tx could be possible to further increase the UL performance. Accordingly, PC1.5 intra-band contiguous/non-contiguous CA and PC1.5 2Tx/3Tx inter-band CA band combination for the constitute NR TDD band are also possible to further increase the UL performance.

Objective 7:

Coherent UL MIMO has been introduced from Rel-15 specification, however it is rare to see the commercial UE supporting this feature on the market due to the potential following assumptions:

- RF chains within RFIC could well synchronized (e.g. via shared PLL and shared Quality gen circuit)

- Additional timing misalignment caused by PA, front-end filter/combiner etc.

In Rel-20, it is proposed to study how to make the UL-MIMO to be commercially available e.g. with certain uplink transmission periodicity by introducing certain mechanism by leveraging the current DPD feedback link to do the calibration between RF chains or re-sync with DL downlink reference signal to conduct the phase calibration.

- 2.2.3 WID objective

- 1) FR1 HPUE enhancement ([PC2]/PC1.5/PC1) for single carrier and EN-DC/NR-CA

  - Specify the requirements for PC1 with 4Tx for FWA single carrier (i.e. 4*26dBm)

  - Investigate and specify the requirement PC1.5 with 1Tx, including:

    - Single carrier operation. Example band: [n41, n77/n78, n79, n104]

    - Intra-band contiguous and non-contiguous CA operation:

      - Example band combination: [n41C, n77C/n78C, n79C, n104C] for intra-band contiguous CA, and [n41(2A), n77(2A)/n78(2A)]

  - Investigate and specify the requirement for HPUE 2Tx/3Tx inter-band EN-DC/NR-CA with 1Tx PC1.5 for the constitute NR band.

- 7) Requirement enhancement to support coherent UL MIMO

  - Investigate and specify the mechanism to calibrate the transmitter chains to fulfill the UL coherent requirements;

  - The existing UL coherent MIMO requirement should be the baseline.

- 2.2.4 Proposed TU budget

Proposed Tu budget (potential some UE capability or signalling definition in RAN2):

Conclusions

In this contribution, we shared some further views on Rel-20 RAN4 package based on RAN#107 moderator’s summary and Rel-19 leftover issues.

References

- Moderator's summary for RAN4 led REL-20 topics [RP-250817]