3GPP TSG RAN Meeting #108	RP-251742

Prague, Czech Republic, June 9-13, 2025

Source:	ZTE Corporation, Sanechips

Title:	Study on 6G scenarios and requirements - ISAC

Agenda Item:	16.1

Document for:	Discussion and Decision

Introduction

In RAN#107 meeting, the new 6G RAN SID was updated, where new services and the corresponding scenarios and requirements should be a crucial part of study. The agreed objective is listed below [1].

This study item aims to - investigate a candidate set of items for minimum TPRs based on the Recommendation ITU-R M.2160, and, where applicable, the associated target values and key assumptions for the identified minimum TPRs. - The outcome is expected to be shared by LS with ITU-R WP5D, as suitable, and used as a baseline for the subsequent study 6G in RAN - Identify typical and practical deployment scenarios defined by attributes such as carrier frequency, inter-site distance, user density, maximum mobility speed, and other relevant factors. - Develop 3GPP requirements for 6G Radio for improvement of existing services and for new services. - Determine the applicability of legacy services to 6G Radio, and define radio requirements for these, as appropriate. - Develop 3GPP requirements for 6G Radio for these practical deployment scenarios to ensure substantial gains in all relevant bands: overall performance, user experience, TCO reduction including at least: - Ensure appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting - Energy efficiency and energy saving: both for network and device - Enhanced spectral efficiency - Enhanced overall coverage, focus on cell-edge performance and UL coverage - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band - Target scalable and forward compatible design for diverse device types - Improved spectrum utilization and operations taking into account diverse spectrum allocations - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs - Aim at a harmonized 6G Radio design for TN and NTN, including their integration - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc. - Define a time plan and steer work as appropriate for the RAN WGs during the 6G WG SI to deliver high-level decisions at least on the following areas: - Fundamental 6G radio design aspects: waveform, numerology, channel coding, etc… - Overall high-level aspects of 5G to 6G migration - RAN architecture and interfaces, including RAN-Core interface - Coordination of 6G AI/ML framework Note: The detailed time plan for the high-level decisions above will be decided at RAN#108 at the approval of the 6G RAN WG SI and is to be aligned with the RAN SI progress.

As widely known, ISAC is expected to play an important role in 6G as a new service. In this contribution, we provide our views on promising use cases, scenarios and requirements for 6G ISAC.

ISAC use cases

  - 2.0 General

In this section, we provide some ISAC use cases which are very promising to commercialize. Most of them have been well tested in the field trial.

Specifically, the first four ISAC use cases have been included in SA1 5G-A requirements [2]. Also, the Rel-19 ISAC channel model has supported these four use cases. The last three use cases are also promising and very useful especially for public safety, and these new use cases are supported by a number of companies including many operators as shown in our companion contribution [18]. Some commercialized or field trial information are also listed in the corresponding subsections.

Proposal 1: For 6G ISAC, the applicable use cases should at least include

- Detection and tracking of UAV, Pedestrian, Vehicle, AGV, and Ship

- Infrastructure collapse monitoring

- Micro-deformation detection

UAV detection and tracking

Low altitude economy is a comprehensive economic form based on manned and unmanned aerial vehicles. It has the characteristics of a long industrial chain, wide radiation, strong growth and driving force, etc. The development of the low-altitude economy helps to expand the market; it contributes to promoting innovation and enriching supply; it has the capacity to break down barriers and promote integration.

UAV is one of the main drivers of low altitude economy. The integration of UAVs with the low-altitude economy is ushering in a new chapter for future airspace, driving transformations across multiple sectors including transportation, logistics, agriculture, and emergency rescue.

There are numerous applications for UAVs in low altitude economy, including but not limited to power line inspections, UAV delivery, intrusion detection, agricultural crop protection, and more. Some use cases are as shown in Figure 2-1.

- Power line inspection         (b) UAV delivery               (c) UAV intrusion detection

Figure 2-1 Use cases for UAV sensing

The flight safety of UAVs and their regulation thus becomes a pressing issue that needs to be addressed. Solving flight safety relying on the UAV itself has issues such as limited observation range and blind spots. Therefore, networks providing sensing is becoming a trend. In short, sensing will play a crucial role in these applications, and sensing can be used to monitor illegal intrusions by low, slow, and small UAVs. Sensing can also be used for trajectory tracking and for collision avoidance.

In [3], some field test results for UAV intrusion detection and tracking were provided. It shows good sensing accuracy with implementation based on base station sensing.

Pedestrian detection and tracking

According to data from the World Health Organization, road traffic accidents cause nearly 1.3 million deaths globally each year, with pedestrians accounting for approximately one quarter of all global road traffic fatalities annually. Ensuring pedestrian safety becomes an urgent issue that needs to be addressed.

With the trend of digitization and intelligence in the transportation field, 6G system is expected to provide real-time data communication for massive traffic terminals, and at the same time efficiently sense the real-time status of roads, vehicles, and people to enable the transportation system to have the capabilities of sensing, interconnection, analysis, prediction and control in a larger space-time range.

Hence, pedestrian detection and tracking becomes one of the critical scenarios in smart transportation. Use cases for pedestrian detection include but are not limited to railway pedestrian monitoring, highway pedestrian monitoring, and pedestrian monitoring in urban district roads.

- Highway intrusion detection         (b) Railway intrusion detection     (c) Pedestrian monitoring in urban

Figure 2-2 Use cases for pedestrian detection

Vehicle detection and tracking

The global automotive industry size is calculated at USD 4,359.98 billion in 2024 and is expected to reach around USD 6,678.28 billion by 2032[3]. And the global intelligent driving solutions market was worth around USD 29 billion in 2023 and is predicted to grow to around USD 78.44 billion by 2033[4].

Currently, technologies such as in-vehicle radars and lidars are thriving, but these technologies do have limitations.

- Blind Spots in Coverage: The coverage area of single-vehicle sensing is fan-shaped, leading to blind spot regions.

- Large Momentary Errors: Due to high speeds, single vehicles only have access to instantaneous sensing information, resulting in a high probability of misidentification or missed detection of targets.

- Significant Impact of Obstructions: When obstructed by preceding vehicles, it's unable to detect obscured areas.

- Limited Sensing Distance: The effective range of sensing is restricted.

- Inability to Sense All-Weather: Perception range is drastically reduced under adverse weather conditions, and sensing efficacy is also limited without proper lighting.

Therefore, the sensing capabilities provided by cellular systems will be a beneficial complement. The vehicle monitoring and tracking provided by cellular networks not only aids in traffic management but also supports autonomous driving illustrated in the figures below.

- Vehicle information statistics                      (b) Automotive maneuvering and navigation

Figure 2-3 Use cases for vehicle detection and tracking

AGV detection and tracking

Based on market forecasts, the global industrial automation and control systems market size was estimated at USD 206.33 billion in 2024. The market is growing rapidly, driven by the rising demand for efficiency, precision, and safety across manufacturing industries[5].

Automated Guided Vehicles (AGVs) are key components of the new smart factories, used for a variety of tasks such as heavy or hazardous materials transportation and distribution. Based on market forecasts, the global automated guided vehicle market size was valued at USD 2.51 billion in 2024. The market is projected to grow from USD 2.75 billion in 2025 to USD 5.44 billion by 2032[6].

Simultaneous presence of AGVs and human workers at the industrial side creates safety challenges and calls for stringent safety requirements [7]. For example, the driverless, automated guided industrial vehicles ANSI/ITSDF B56.5 [8] safety standard requires that “the AGV shall detect and avoid both static and dynamic obstacles appearing in the path of travel direction”. Reliable detection of AGV/human presence or proximity is therefore an important safety criterion.

6G system can be deployed in a factory which uses RAN entities and/or UEs to measure sensing signal and achieve sensing data, that are made available to sensing management entities in order to derive sensing results such as the detection of the presence or proximity of AGVs and humans and the surrounding environment, as shown in Figure below.

- AGV detection and tracking in factories           (b) AMR collision avoidance in smart factories

Figure 2-4 Use cases for AGV detection and tracking

Ship detection and tracking

Maritime and inland waterway transportation are the primary means of cargo transport worldwide. Over two-thirds of global international trade volume and approximately 90% of import and export freight are transported by sea and river routes. Ship detection plays an important role in port management, ship traffic, maritime rescue, cargo transportation and so on. Usually, ship detection in nearshore waters and rivers is a crucial scenario. Since base stations are primarily land-based, large sensing coverage is required to ensure effective ship detecting at nearshore areas.

Figure 2-5 Ship detection in ocean

By allowing 6G network to detect and track vessels in real time, ISAC can dramatically improve maritime safety, efficiency, and environmental monitoring. For example, ISAC-enabled networks can support collision avoidance, search and rescue, or the detection of unauthorized vessels even in challenging conditions such as fog or rough seas, the corresponding capabilities are highlighted in the 3GPP TR 22.837 [10].

From a commercial perspective, the prospects are highly promising. The maritime industry is undergoing rapid digital transformation, driven by the need for better security, smarter port operations, and stricter environmental regulations. Market research predicts that the global maritime surveillance sector will grow to over $35 billion by 2029, fueled in part by demand for real-time, network-based sensing and tracking [11] . ISAC offers a cost-effective solution by combining communication and sensing infrastructure, enabling operators and authorities to extend surveillance coverage without the high costs of traditional radar or satellite systems.

Moreover, as noted in [12], ISAC’s ability to leverage existing and future mobile networks makes it particularly attractive for ports and coastal regions. This positions ISAC as a key enabler for smarter, safer, and more sustainable maritime operations, with clear and growing business opportunities for network operators, equipment vendors, and maritime stakeholders alike.

Table 2-1 Estimation of market of ISAC enabled ship detection and maritime surveillance

| Market Segment | Estimated Value / Growth | Timeframe |
| --- | --- | --- |
| Maritime Surveillance | USD 35.7 billion (market size) | By 2029 [11] |
| Smart Ports & Shipping | USD 9.8 billion (market size) | By 2027 [12] |

Some field tests for ship intrusion detection and tracking have been done with operators [17]. The good sensing accuracy with implementation based on base station sensing was observed.

Infrastructure collapse monitoring

ISAC has the potential to revolutionize how we respond to sudden infrastructure collapse such as subsidence and landslides, especially in critical environments like highways, farmlands, and so on. In these scenarios, the rapid detection and precise localization of a disaster event are crucial for saving lives and minimizing property loss. Traditional monitoring systems, such as cameras or standalone sensors, often suffer from coverage gaps and delayed reporting. In contrast, ISAC-enabled base stations can simultaneously monitor ground movement while providing real-time communication services.

Figure 2-6 An illustration for infrastructure collapse

For example, if a landslide or sudden road collapse occurs on a highway, nearby ISAC-capable base stations can immediately sense the structural anomaly using their radio signals - such as by detecting abnormal changes in signal reflection or propagation. This information can then be instantly communicated to vehicles and personnel in the vicinity, triggering automated warnings and rerouting instructions. Such rapid response is critical, as drivers may not have enough time to react if they only notice a hazard visually, especially at high speeds. In rural or agricultural zones, ISAC can also help protect equipment and workers from unexpected ground failures.

Figure 2-7 Infrastructure collapse monitoring

The commercial outlook for this use case is highly positive. With growing awareness of climate change and the increasing frequency of extreme weather events, there is a rising demand for intelligent, network-based disaster monitoring systems. Industry analyses predict robust growth in the smart disaster management market, with ISAC technologies expected to play a key role in enhancing early warning capabilities and reducing the costs associated with manual monitoring and dedicated sensor infrastructure. According to a MarketsandMarkets report, the global disaster management market is projected to reach USD 222.2 billion by 2027 [13], with a significant portion driven by rapid sensing and alerting solutions.

Table 2-2 Estimation of market of infrastructure collapse monitoring

| Market Segment | Estimated Value / Growth | Timeframe |
| --- | --- | --- |
| Disaster Management | USD 222.2 billion (market size) | By 2027 [13] |
| Smart Sensing for Geological Hazards | Multi-billion USD (market size) | By early 2030s [12] |

Micro-deformation detection

As urbanization accelerates and infrastructure ages, monitoring the subtle movements of critical structures, such as bridges, flyovers, and skyscrapers has become increasingly important for public safety and asset management. These structures are constantly exposed to environmental factors like temperature changes, wind, rain, and even minor seismic activity, all of which can cause micro-deformations or displacements at the millimeter or centimeter scale. Although such movements are small, they can indicate early signs of fatigue, stress, or potential failure, making their detection crucial for preventive maintenance and extending infrastructure lifespan.

ISAC deployed over wide-area networks like 6G, offers a transformative solution. By leveraging the high-resolution sensing capabilities of 6G base stations, it becomes possible to detect and track these micro-deformations in real time across large and complex urban environments. ISAC-enabled base stations can monitor changes in the reflection and propagation of their radio signals caused by even slight shifts in structural position.

A typical implementation might see 6G base stations installed around or within major bridges and high-rise buildings, continuously scanning for abnormal movements. When micro-deformations are detected, maintenance teams can be promptly alerted, enabling targeted inspections and repairs before problems escalate. This approach is particularly valuable for megacities with extensive infrastructure networks, where manual monitoring is costly and inefficient. According to a [14], the integration of ISAC for structural health monitoring is regarded as a game-changer for smart city management, offering both operational and financial benefits.

Figure 2-8  Micro-deformation monitoring on a bridge or a construction

The commercial outlook for ISAC-driven micro-deformation detection is robust. The global market for structural health monitoring (SHM) systems is expected to reach over USD 4.1 billion by 2029, driven by increasing demand for real-time, automated monitoring solutions in both developed and emerging economies. The unique advantage of ISAC lies in its ability to combine communication and sensing on the same infrastructure, reducing deployment costs while expanding coverage. Industry studies and 3GPP discussions identify this use case as a key vertical for next-generation networks, particularly as cities and private operators move toward smarter, more proactive infrastructure management.

Table 2-3 Estimation of market of micro-deformation monitoring

| Market Segment | Estimated Value / Growth | Timeframe |
| --- | --- | --- |
| Structural Health Monitoring (SHM) | Was USD 2.5 billion | In 2024 [15] |
|  | USD 4.1 billion | By 2029 [15] |

ISAC deployment scenarios

  - 3.0 Mapping between use cases and scenarios

Deployment scenarios for IMT-2030 are described in this section. The mapping between sensing use cases and deployment scenarios are shown in the following table. As we can see, most deployment scenarios for sensing and communication can be shared. Hence, it is natural to integrate the deployment scenarios of sensing and communication in the new TR 38.914. The suggested TR skeleton is shown in our companion contribution [19].

Table 3-0: Mapping between ISAC use cases and deployment scenarios

| Use cases Scenarios | UAV | Human | Automotive vehicle | AGV | Ship | Geological disaster (road) | Micro-deformation (wall, bridge) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Indoor hotspot |  |  |  |  |  |  |  |
| Indoor factory |  |  |  |  |  |  |  |
| UMi |  |  |  |  |  |  |  |
| UMa |  |  |  |  |  |  |  |
| RMa |  |  |  |  |  |  |  |
| Highway |  |  |  |  |  |  |  |
| Urban Grid |  |  |  |  |  |  |  |
| High speed train |  |  |  |  |  |  |  |
| Nearshore waters |  |  |  |  |  |  |  |

High-level descriptions on deployment scenarios including carrier frequency, aggregated bandwidth, BS/UE antenna elements can be updated based on potential new requirement for IMT-2030. In addition, ISAC related descriptions including sensing target characteristics, sensing transmitter and sensing receiver are proposed in the following subsections.

In addition, to achieve good sensing accuracy in terms of angle estimation, 6G should consider large antenna array size, such as 1536 or 2048 antenna elements.

Proposal 2: For ISAC, support deployment scenarios including Indoor hotspot, Indoor factory, UMi, UMa, RMa, Highway, Urban grid, High speed train and Nearshore waters.

- Integrate the deployment scenarios of sensing and communication in the new TR 38.914

- Support 1536 or 2048 antenna elements

Indoor hotspot

The indoor hotspot deployment scenario focuses on small coverage per site/TRxP (transmission and reception point) and high user throughput or user density in buildings. The key characteristics of this deployment scenario are high capacity, high user density and consistent user experience indoor.

The indoor hotspot deployment scenario refers to section 6.1.1 of 38.913 [9]. Some of its attributes are listed in Table 3-1.

Table 3-1: Attributes for indoor hotspot

| Attributes | Values or assumptions |  |
| --- | --- | --- |
| Carrier Frequency | Around 30 GHz or Around 4 GHz or 7 GHz |  |
| Aggregated system bandwidth | Around 30GHz: Up to 1 GHz (DL+UL) Around 4GHz: Up to 200MHz (DL+UL) 7GHz: Up to 400MHz (DL+UL) |  |
| Layout | Single layer: - Indoor floor (Open office) |  |
| ISD | 20m (Equivalent to 12TRxPs per 120m x 50m) |  |
| BS antenna elements | Around 30GHz: Up to 256 Tx and Rx antenna elements Around 4GHz: Up to 256 Tx and Rx antenna elements 7GHz: Up to 1536/2048 Tx and Rx antenna elements |  |
| UE antenna elements | Around 30GHz: Up to 32 Tx and Rx antenna elements Around 4GHz: Up to 8 Tx and Rx antenna elements 7GHz: Up to 16 Tx and Rx antenna elements |  |
| User distribution and UE speed | 100% Indoor, 3km/h, 10 users per TRxP |  |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. For certain KPIs, full buffer traffic is desirable to enable comparison with IMT-Advanced values. |  |
| Sensing transmitters and sensing receivers | Rx/Tx location are selected among the TRPs and UEs location in the corresponding communication scenario |  |
| Sensing targets | Outdoor/indoor | Indoor |
|  | Mobility | Uniform distribution between 0km/h and 3km/h |
|  | Distribution | Uniformly distributed over the horizontal area of the convex hull of the TRP deployment |
|  | Orientation | Random over the horizontal area |
|  | Physical characteristics | Size (Length x Width x Height): - Child: 0.2m x 0.3m x 1m - Adult Pedestrian: 0.5m x 0.5m x 1.75m |

Indoor factory

Improving safety and work conditions in factories and industrial environments is a critical component for industry 4.0. The indoor factory deployment scenario can refer to TR 38.901 and TR 38.857. Some of its attributes are listed in Table 3-2.

ISAC related descriptions including sensing target characteristics, sensing transmitter and sensing receiver are proposed in the following subsections.

Table 3-2: Attributes for indoor factory

| Attributes | Values or assumptions |  |
| --- | --- | --- |
| Carrier Frequency | Around 30 GHz or Around 4 GHz or 7 GHz |  |
| Aggregated system bandwidth | Around 30GHz: Up to 1 GHz (DL+UL) Around 4GHz: Up to 200MHz (DL+UL) 7GHz: Up to 400MHz (DL+UL) |  |
| Layout | Single layer: - Indoor floor |  |
| ISD | 50m (Equivalent to 18TRxPs per 300m x 150m) |  |
| BS antenna elements | Around 30GHz: Up to 256 Tx and Rx antenna elements Around 4GHz: Up to 256 Tx and Rx antenna elements 7GHz: Up to 1536/2048 Tx and Rx antenna elements |  |
| UE antenna elements | round 30GHz: Up to 32 Tx and Rx antenna elements Around 4GHz: Up to 8 Tx and Rx antenna elements 7GHz: Up to 16 Tx and Rx antenna elements |  |
| User distribution and UE speed | 100% Indoor, 3km/h, 10 users per TRxP |  |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. For certain KPIs, full buffer traffic is desirable to enable comparison with IMT-Advanced values. |  |
| Sensing transmitters and sensing receivers | Rx/Tx location are selected among the TRPs and UEs location in the corresponding communication scenario |  |
| Sensing targets | Outdoor/indoor | Indoor |
|  | Mobility | For human as sensing target: Uniform distribution between 0km/h and 3km/h For AGV as sensing target: Uniform distribution between 0km/h and 30km/h |
|  | Distribution | Uniformly distributed over the horizontal area of the convex hull of the TRP deployment |
|  | Orientation | Random over the horizontal area |
|  | Physical characteristics | Size (Length x Width x Height): - Child: 0.2m x 0.3m x 1m - Adult Pedestrian: 0.5m x 0.5m x 1.75m - AGV: 1.5 m x 3.0m x 1.5 m |

UMi

According to TR38.913 section 6.1.2, the dense urban micro-cellular deployment scenario focuses on macro TRxPs with or without micro TRxPs and high user densities and traffic loads in city centres and dense urban areas. The key characteristics of this deployment scenario are high traffic loads, outdoor and outdoor-to-indoor coverage. This scenario will be interference-limited, using macro TRxPs with or without micro TRxPs. A continuous cellular layout and the associated interference shall be assumed.

On top of that, ISAC related descriptions including sensing target characteristics, sensing transmitter and sensing receiver are proposed, some of its attributes are listed in Table 3-3.

Table 3-3: Attributes for dense urban

| Attributes | Values or assumptions |  |
| --- | --- | --- |
| Carrier Frequency | 7GHz/Around 4GHz + Around 30GHz (two layers) |  |
| Aggregated system bandwidth | Around 30GHz: Up to 1 GHz (DL+UL) Around 4GHz: Up to 200MHz (DL+UL) 7GHz: Up to 400MHz (DL+UL) |  |
| Layout | Two layers: - Macro layer: Hex. Grid - Micro layer: Random drop Step 1: 7GHz/Around 4GHz in Macro layer Step 2: Both 7GHz/Around 4GHz & Around 30GHz may be available in Macro & Micro layers (including 1 macro layer, macro cell only) |  |
| ISD | Macro layer: 200m Micro layer: 3micro TRxPs per macro TRxP All micro TRxPs are all outdoor |  |
| BS antenna elements | Around 30GHz: Up to 256 Tx and Rx antenna elements Around 4GHz: Up to 256 Tx and Rx antenna elements 7GHz: Up to 1536/2048 Tx and Rx antenna elements |  |
| UE antenna elements | Around 30GHz: Up to 32 Tx and Rx antenna elements Around 4GHz: Up to 8 Tx and Rx antenna elements 7GHz: Up to 16 Tx and Rx antenna elements |  |
| User distribution and UE speed | Step1: Uniform/macro TRxP, 10 users per TRxP Step2: Uniform/macro TRxP + Clustered/micro TRxP, 10 users per TRxP, 80% indoor (3km/h), 20% outdoor (30km/h) |  |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. For certain KPIs, full buffer traffic is desirable to enable comparison with IMT-Advanced values. |  |
| Sensing targets | Outdoor/indoor | Outdoor |
|  | Mobility | For human as sensing target: Uniform distribution between 0km/h and 3km/h For UAV as sensing target: Uniform distribution between 0km/h and 180km/h |
|  | Distribution | Horizontal: Uniformly distributed with a sensing area (e.g., center cell) Vertical for UAV: Option A: Uniform between 1.5m and 300m. Option B: Fixed height value chosen from {25, 50, 100, 200, 300} m |
|  | Orientation | Random in the horizontal domain |
|  | Physical characteristics | Size (Length x Width x Height): - Child: 0.2m x 0.3m x 1m - Adult Pedestrian: 0.5m x 0.5m x 1.75m - UAV 1: 1.6m x 1.5m x 0.7m - UAV 2: 0.3m x 0.4m x 0.2m |

UMa

According to TR38.913 section 6.1.4, the urban macro deployment scenario focuses on large cells and continuous coverage. The key characteristics of this scenario are continuous and ubiquitous coverage in urban areas. This scenario will be interference-limited, using macro TRxPs (i.e. radio access points above rooftop level).

On top of that, ISAC related descriptions including sensing target characteristics, sensing transmitter and sensing receiver are proposed, some of its attributes are listed in Table 3-4.

Table 3-4: Attributes for urban macro

| Attributes | Values or assumptions |  |
| --- | --- | --- |
| Carrier Frequency | Around 4 GHz or 7 GHz or Around 30 GHz |  |
| Aggregated system bandwidth | Around 30GHz: Up to 1 GHz (DL+UL) Around 4GHz: Up to 200MHz (DL+UL) 7GHz: Up to 400MHz (DL+UL) |  |
| Layout | Single layer: - Hex. Grid |  |
| ISD | 500m |  |
| BS antenna elements | Around 30GHz: Up to 256 Tx and Rx antenna elements Around 4GHz: Up to 256 Tx and Rx antenna elements 7GHz: Up to 1536/2048 Tx and Rx antenna elements |  |
| UE antenna elements | Around 30GHz: Up to 32 Tx and Rx antenna elements Around 4GHz: Up to 8 Tx and Rx antenna elements 7GHz: Up to 16 Tx and Rx antenna elements |  |
| User distribution and UE speed | 20% Outdoor in cars: 30km/h, 80% Indoor in houses: 3km/h 10 users per TRxP |  |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. For certain KPIs, full buffer traffic is desirable to enable comparison with IMT-Advanced values. |  |
| Sensing targets | Outdoor/indoor | Outdoor |
|  | Mobility | For human as sensing target: Uniform distribution between 0km/h and 3km/h For UAV as sensing target: Uniform distribution between 0km/h and 180km/h |
|  | Distribution | Horizontal for human and UAV: Uniformly distributed with a sensing area (e.g., center cell) Vertical for UAV: Option A: Uniform between 1.5m and 300m. Option B: Fixed height value chosen from {25, 50, 100, 200, 300} m |
|  | Orientation | Random in the horizontal domain |
|  | Physical characteristics | Size (Length x Width x Height): - Child: 0.2m x 0.3m x 1m - Adult Pedestrian: 0.5m x 0.5m x 1.75m - UAV 1: 1.6m x 1.5m x 0.7m - UAV 2: 0.3m x 0.4m x 0.2m |

RMa

According to TR38.913 section 6.1.3, the rural deployment scenario focuses on larger and continuous coverage. The key characteristics of this scenario are continuous wide area coverage supporting high speed vehicles. This scenario will be noise-limited and/or interference-limited, using macro TRxPs.

On top of that, ISAC related descriptions including sensing target characteristics, sensing transmitter and sensing receiver are proposed, some of its attributes are listed in Table 3-5.

Table 3-5: Attributes for rural scenario

| Attributes | Values or assumptions |  |
| --- | --- | --- |
| Carrier Frequency | Around 700MHz or Around 4GHz (for ISD 1) Around 700 MHz and Around 2 GHz combined (for ISD 2) |  |
| Aggregated system bandwidth | Around 700MHz: Up to 20MHz(DL+UL) Around 4GHz: Up to 200MHz (DL+UL) |  |
| Layout | Single layer: - Hex. Grid |  |
| ISD | ISD 1: 1732m ISD 2: 5000m |  |
| BS antenna elements | Around 4GHz: Up to 256 Tx and Rx antenna elements Around 700MHz: Up to 64 Tx and Rx antenna elements |  |
| UE antenna elements | Around 4GHz: Up to 8 Tx and Rx antenna elements Around 700MHz: Up to 4 Tx and Rx antenna elements |  |
| User distribution and UE speed | 50% outdoor vehicles (120km/h) and 50% indoor (3km/h), 10 users per TRxP |  |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. For certain KPIs, full buffer traffic is desirable to enable comparison with IMT-Advanced values. |  |
| Sensing targets | Outdoor/indoor | Outdoor |
|  | Mobility | For human as sensing target: Uniform distribution between 0km/h and 3km/h For UAV as sensing target: Uniform distribution between 0km/h and 180km/h |
|  | Distribution | Horizontal for human and UAV: Uniformly distributed with a sensing area (e.g., center cell) Vertical for UAV: Option A: Uniform between 1.5m and 300m. Option B: Fixed height value chosen from {25, 50, 100, 200, 300} m |
|  | Orientation | Random in the horizontal domain |
|  | Physical characteristics | Size (Length x Width x Height): - Child: 0.2m x 0.3m x 1m - Adult Pedestrian: 0.5m x 0.5m x 1.75m - UAV 1: 1.6m x 1.5m x 0.7m - UAV 2: 0.3m x 0.4m x 0.2m |

Highway

According to TR38.913 section 6.1.8, the highway deployment scenario focuses on scenario of vehicles placed in highways with high speeds. The main KPIs evaluated under this scenario would be reliability/availability under high speeds/mobility (and thus frequent handover operations).

On top of that, ISAC related descriptions including sensing target characteristics, sensing transmitter and sensing receiver are proposed, some of its attributes are listed in Table 3-6.

Table 3-6: Attributes of Highway

| Attributes | Values or assumptions |  |
| --- | --- | --- |
| Carrier Frequency | Macro only: 7 GHz Macro + RSUs: 1) For BS to RSU: 7 GHz 2) RSU to vehicles or among vehicles: 7 GHz |  |
| Aggregated system bandwidth | Up to 400MHz (DL+UL) |  |
| Layout | Option 1: Macro only Option 2: Macro + RSUs |  |
| ISD | Macro cell: ISD = 1732m, 500m(Optional) Inter-RSU distance = 50m or 100m |  |
| BS antenna elements | Tx: Up to 1536/2048 Tx Rx: Up to 1536/2048 Rx |  |
| UE antenna elements | RSU Tx: Up to 16 Tx RSU Rx: Up to 16 Rx Vehicle Tx: Up to 16 Tx Vehicle Rx: Up to 16 Rx |  |
| User distribution and UE speed | 100% in vehicles Average inter-vehicle distance (between two vehicles’ center) in the same lane is 0.5sec or 1sec * average vehicle speed (average speed: 100-300km/h) |  |
| Traffic model | 50 messages per 1 second with absolute average speed of either - 100-250 km/h (relative speed: 200 – 500km/h), or - 30 km/h |  |
| Sensing targets | Outdoor/indoor | Outdoor |
|  | Mobility | For vehicle as sensing target: Option A: Vehicle speed is 140 km/h in all the lanes as baseline and 70 km/h in all the lanes optionally. Option B: - Speed in Lane 1: 80km/h - Speed in Lane 2: 100km/h - Speed in Lane 3: 140km/h - Speed in Lane 4: 40km/h - Speed in Lane 5: 30km/h - Speed in Lane 6: 20km/h For road as sensing target: 0km/h |
|  | Distribution | For vehicle as sensing target: The distance between the rear bumper of a vehicle and the front bumper of the following vehicle in the same lane is max {2 meter, an exponential random variable with the average of the speed * 2 sec}. For road as sensing target: all lanes |
|  | Orientation | For vehicle as sensing target: Lane direction in horizontal plane |
|  | Physical characteristics | For vehicle as sensing target: Size (Length x Width x Height): - Type 1/2 (passenger vehicle): 5m x 2m x 1.6m - Type 3 (truck/bus): 13m x 2.6m x 3m |

Illustrative diagram of freeway mode is as follows

Figure 3-1: Road configuration for highway scenario

Urban grid

According to TR38.913 section 6.1.9, the urban macro deployment scenario focuses on scenario of highly densely deployed vehicles placed in urban area. It could cover a scenario where freeways lead through an urban grid. The main KPI evaluated under this scenario are reliability/availability/latency in high network load and high UE density scenarios.

On top of that, ISAC related descriptions including sensing target characteristics, sensing transmitter and sensing receiver are proposed, some of its attributes are listed in Table 3-7.

Table 3-7: Attributes of urban grid for vehicle

| Attributes | Values or assumptions |  |
| --- | --- | --- |
| Carrier Frequency | Macro only: 7 GHz Macro + RSUs: 1) For BS to RSU: 7 GHz 2) RSU to vehicles or among vehicles/pedestrians: 7 GHz |  |
| Aggregated system bandwidth | Up to 400 MHz (DL+UL) |  |
| Layout | Option 1: Macro only Option 2: Macro + RSUs |  |
| ISD | Macro cell: ISD = 500m RSU at each intersection for Option 2. Other values (50m and 100m) should also be considered for option 2 |  |
| BS antenna elements | Tx: Up to 1536/2048 Tx Rx: Up to 1536/2048 Rx |  |
| UE antenna elements | RSU Tx: Up to 16 Tx RSU Rx: Up to 16 Rx Vehicle Tx: Up to 16 Tx Vehicle Rx: Up to 16 Rx Pedestrian/bicycle Tx: Up to 16 Tx Pedestrian/bicycle Rx: Up to16 Rx |  |
| User distribution and UE speed | Urban grid model (car lanes and pedestrian/bicycle sidewalks are placed around a road block. 2 lanes in each direction, 4 lanes in total, 1 sidewalk, one block size: 433m x 250m) Average inter-vehicle distance (between two vehicles’ center) in the same lane is 1sec * average vehicle speed (average speed 15 – 120km/h) Pedestrian/bicycle dropping: average distance between UEs is 20m |  |
| Traffic model | 50 messages per 1 second with 60km/h, 10 messages per 1 second with 15km/h |  |
| Sensing targets | Outdoor/indoor | Outdoor |
|  | Mobility | For vehicle as sensing target: Option A: Vehicle speed is 60 km/h in all the lanes Option B: - Speed in Lane 1: 60km/h - Speed in Lane 2: 50km/h - Speed in Lane 3: 25km/h - Speed in Lane 4: 15km/h For road as sensing target: 0km/h |
|  | Distribution | For vehicle as sensing target: The distance between the rear bumper of a vehicle and the front bumper of the following vehicle in the same lane is max {2 meter, an exponential random variable with the average of the speed * 2 sec}. For road as sensing target: all lanes |
|  | Orientation | For vehicle as sensing target: Lane direction in horizontal plane |
|  | Physical characteristics | For vehicle as sensing target: Size (Length x Width x Height): - Type 1/2 (passenger vehicle): 5m x 2m x 1.6m - Type 3 (truck/bus): 13m x 2.6m x 3m |

Illustrative diagram of urban grid model with vehicle distribution is as follows:.

Table 3-8: Details of vehicle drop and mobility model

| Parameter | Urban case | Freeway case |
| --- | --- | --- |
| Number of lanes | 2 in each direction (4 lanes in total in each street) | 3 in each direction (6 lanes in total in the freeway) |
| Lane width | 3.5 m | 4 m |
| Road grid size by the distance between intersections | 433 m * 250 m. NOTE1 | N/A |
| Simulation area size | Minimum 1299 m * 750 m NOTE2 | Freeway length >= 2000 m. Wrap around should be applied to the simulation area. |
| Vehicle density | Average inter-vehicle distance in the same lane is 2.5 sec * absolute vehicle speed. Baseline: The same density/speed in all the lanes in one simulation. |  |
| Absolute vehicle speed | 15 km/h, 60 km/h, 120 km/h | 250 km/h, 140 km/h, 70 km/h |

Figure 3-2: Road configuration for urban grid

High speed train

According to TR38.913 section 6.1.5, the high speed deployment scenario focuses on continuous coverage along track in high speed trains. The key characteristics of this scenario are consistent passenger user experience and critical train communication reliability with very high mobility. In this deployment scenario, dedicated linear deployment along railway line and the deployments including SFN scenarios captured in Section 6.2 of 3GPP TR 36.878 [6] are considered, and passenger UEs are located in train carriages. For the passenger UEs, if the antenna of relay node for eNB-to-Relay is located at top of one carriage of the train, the antenna of relay node for Relay-to-UE could be distributed to all carriages.

On top of that, ISAC related descriptions including sensing target characteristics, sensing transmitter and sensing receiver are proposed, some of its attributes are listed in Table below.

Table 3-9: High Speed

| Attributes | Values or assumptions |  |
| --- | --- | --- |
| Carrier Frequency | Macro only: 7GHz/Around 4GHz Macro + relay nodes: 1) For BS to relay: 7GHz/Around 4 GHz For relay to UE: Around 30 GHz or 7GHz/Around 4 GHz 2) For BS to relay: Around 30 GHz For relay to UE: Around 30 GHz or 7GHz/Around 4 GHz |  |
| Aggregated system bandwidth | Around 4GHz: Up to 200 MHz (DL+UL) Around 30GHz: Up to 1GHz (DL+UL) 7GHz: Up to 400 MHz (DL+UL) |  |
| Layout | Macro only: - 7GHz/Around 4GHz: Dedicated linear deployment along the railway line. RRH site to railway track distance: 100m Macro + relay nodes: - 7GHz/Around 4GHz: Dedicated linear deployment along the railway line. RRH site to railway track distance: 100m - Around 30GHz: Dedicated linear deployment along the railway line. RRH site to railway track distance: 5m. |  |
| ISD | - 7GHz/Around 4GHz: ISD 1732m between RRH sites, two TRxPs per RRH site. - Around 30GHz: 1732m between BBU sites, 3 RRH sites connected to 1 BBU, one TRxP per RRH site, inter RRH site distance (580m, 580m, 572m). - Small cell within carriages: ISD = 25m. |  |
| BS antenna elements | Around 30GHz: Up to 256 Tx and Rx antenna elements Around 4GHz: Up to 256 Tx and Rx antenna elements 7GHz: Up to 1536/2048 Tx and Rx antenna elements |  |
| UE antenna elements | Relay Tx: Up to 256 antenna elements Relay Rx: Up to 256 antenna elements Around 30GHz: Up to 32 Tx and Rx antenna elements Around 4GHz: Up to 8 Tx and Rx antenna elements 7GHz: Up to 16 Tx and Rx antenna elements |  |
| User distribution and UE speed | 100% of users in train For non-full buffer, 300 UEs per macro cell (assuming 1000 passengers per high-speed train and at least 10% activity ratio) Maximum mobility speed: 500km/h |  |
| Service profile | Alt 1: Full buffer Alt 2: FTP model 1/2/3 with packet size 0.5 Mbytes, 0.1 Mbytes (other value is not precluded) Other traffic models are not precluded, e.g., for critical train communications. |  |
| Sensing targets | Outdoor/indoor | Outdoor |
|  | Mobility | For human as sensing target: Uniform distribution between 0km/h and 3km/h For animal (sheep/deer) as sensing target: Uniform distribution between 0km/h and 5km/h For trains as sensing target: Uniform distribution between 100km/h and 350km/h |
|  | Distribution | Horizontal: Uniformly distributed with a sensing area |
|  | Orientation | Random in the horizontal domain |
|  | Physical characteristics | Size (Length x Width x Height): - Child: 0.2m x 0.3m x 1m - Adult Pedestrian: 0.5m x 0.5m x 1.75m - Animal: 1.5m x 0.5m x 1m - Trains: 24m x 3.5m x 3 m |

Figure 3-3: 4 GHz/7GHz deployment

Figure 3-4: 30 GHz deployment

Nearshore waters

The extreme Long Range deployment scenario is defined to allow for the Provision of services for very large areas.

2-20km

2-20km

Figure 3-5 Deployment scenario of nearshore waters

ISAC related descriptions including sensing target characteristics, sensing transmitter and sensing receiver are proposed, some of its attributes are listed in Table 3-10.

Table 3-10: Attributes of Nearshore waters

| Attributes | Values or assumptions |  |
| --- | --- | --- |
| Carrier Frequency | 7 GHz, 30GHz |  |
| Aggregated system bandwidth | Up to 400MHz (DL+UL) |  |
| Layout | Macro only |  |
| ISD | Macro cell: ISD = at least 500m |  |
| BS antenna elements | Tx: Up to 1536/2048 Tx Rx: Up to 1536/2048 Rx |  |
| Sensing targets | Outdoor/indoor | Outdoor |
|  | Mobility | Uniform distribution between 10km/h and 60km/h |
|  | Distribution | Uniformly distributed with a sensing area |
|  | Orientation | Random in the horizontal domain |
|  | Physical characteristics | Size (Length x Width x Height): - Ship 1: 270m x 30m x 30m - Ship 2: 100m x 15m x 15m |

ISAC Key performance indicators

Considering SA1 requirements and contributions [2][16], and the reality situation, the following requirements are suggested for all use cases suggested in section 2.1. Similar as performance indicators of other metrics for communications, the described KPIs can refer to the outcome of ITU TPRs. Since more use cases may be included in 6G RAN-level study compared with those for ITU evaluation, ITU-TPRs could be be a subset of the 6G RAN-level ones, e.g. performance requirements for UAV, vehicle, and human detection and tracking.

Proposal 3: For 6G ISAC use cases expecting to be captured in TR 38.914, the following KPIs can be considered where a subset can be for ITU-TPRs

| Use cases | Sensing service category | Accuracy of positioning estimate by sensing (for 90% targets) |  | Accuracy of velocity estimate by sensing (for 90% targets) |  | Max sensing service latency [ms] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] |  |  |  |
| UAV, human, vehicle, AGV | 1 | 10 | 10 | N/A | N/A | 1000 | 5 | 2 |
|  | 2 | 2 | 5 | 1 | N/A | 1000 | 2 | 5 |
|  | 3 | 1 | 1 | 1 | 1 | 100 | 2 | 2 |
| Ship detection | 1 | ≤100 | N/A | N/A | N/A | 5000 | 5 | 5 |
|  | 2 | ≤10 | N/A | N/A | N/A | 5000 | 5 | 5 |
| Infrastructure collapse monitoring |  | 4 | N/A | N/A | N/A | 100 | 1~2 | 1~2 |
| Micro-Deformation |  | 0.01 | 0.01 | N/A | N/A | 5000 | 5 | 5 |

Conclusion

In this contribution, we provide our views on support of 6G ISAC use cases, deployment scenarios, and corresponding key performance requirements.

Proposal 1: For 6G ISAC, the applicable use cases should at least include

- Detection and tracking of UAV, Pedestrian, Vehicle, AGV, and Ship

- Infrastructure collapse monitoring

- Micro-deformation detection

Proposal 2: For ISAC, support deployment scenarios including Indoor hotspot, Indoor factory, UMi, UMa, RMa, Highway, Urban grid, High speed train and Nearshore waters.

- Integrate the deployment scenarios of sensing and communication in the new TR 38.914

- Support 1536 or 2048 antenna elements

Proposal 3: For 6G ISAC use cases expecting to be captured in TR 38.914, the following KPIs can be considered where a subset can be for ITU-TPRs

| Use cases | Sensing service category | Accuracy of positioning estimate by sensing (for 90% targets) |  | Accuracy of velocity estimate by sensing (for 90% targets) |  | Max sensing service latency [ms] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] |  |  |  |
| UAV, human, vehicle, AGV | 1 | 10 | 10 | N/A | N/A | 1000 | 5 | 2 |
|  | 2 | 2 | 5 | 1 | N/A | 1000 | 2 | 5 |
|  | 3 | 1 | 1 | 1 | 1 | 100 | 2 | 2 |
| Ship detection | 1 | ≤100 | N/A | N/A | N/A | 5000 | 5 | 5 |
|  | 2 | ≤10 | N/A | N/A | N/A | 5000 | 5 | 5 |
| Infrastructure collapse monitoring |  | 4 | N/A | N/A | N/A | 100 | 1~2 | 1~2 |
| Micro-Deformation |  | 0.01 | 0.01 | N/A | N/A | 5000 | 5 | 5 |

References

- RP-250810 Revised SID: Study on 6G Scenarios and requirements CMCC, Verizon, NTT DOCOMO, INC., 	Deutsche Telekom

- TS 22.137, Service requirement for Integrated Sensing and Communication; Stage 1 (Release 19)

- https://www.fintechfutures.com/press-releases/automotive-industry-size-expected-to-reach-usd-6-678-28-bn-by-2032

- https://www.sphericalinsights.com/reports/intelligent-driving-solutions-market

- https://www.grandviewresearch.com/industry-analysis/industrial-automation-market

- https://www.fortunebusinessinsights.com/automated-guided-vehicle-agv-market-101966

- Javed MA, Muram FU, Hansson H, Punnekkat S, Thane H. Towards dynamic safety assurance for Industry 4.0. Journal of Systems Architecture. 2021 Mar 1; 114:101914.

- American National Standards Institute/Industrial Truck Safety Development Foundation, Safety standard for driverless, automatic guided industrial vehicles and automated functions of manned industrial vehicles, December 2019, 2019, [Online] http://www.itsdf.org.

- TS 38.913, Study on Scenarios and Requirements for Next Generation Access Technologies (Release 18)

- TR 22.837, Feasibility Study on Integrated Sensing and Communication (Release 19)

- Maritime Surveillance Market Size & Share Analysis - Growth Trends & Forecasts (2025 - 2030) Source: https://www.mordorintelligence.com/industry-reports/maritime-surveillance-market

- Liu, Fan, et al. "Integrated sensing and communications: Toward dual-functional wireless networks for 6G and beyond." IEEE journal on selected areas in communications 40.6 (2022): 1728-1767.

- Disaster Recovery as a Service (DRaaS) Market by Service Type (Backup & Restore, Real-Time Replication, Data Protection), Deployment Mode (Public Cloud, Private Cloud), Organization Size, Vertical and Region - Global Forecast to 2028, https://www.marketsandmarkets.com/Market-Reports/recovery-as-a-service-market-962.html

- Shao, Zelong, Xiangkun Zhang, and Yingsong Li. "Analysis and validation of super-resolution micro-deformation monitoring radar." Progress In Electromagnetics Research M 62 (2017): 41-50.

- Structural Health Monitoring Market Size, https://www.marketsandmarkets.com/ResearchInsight/structural-health-monitoring-market-size.asp

- TR 22.870, Study on 6G Use Cases and Service Requirements, stage 1 (Release 20)

- Türk Telekom and ZTE launch Europe's first millimeter-wave supported 5G-A ISAC maritime management solution

- RP-251746 Views on 6G ISAC use cases and scenarios  ZTE Corporation, Turk Telekom, Verizon, China Telecom, China Unicom, Telecom Italia, CAICT, ETRI, Queens University Belfast, Pengcheng Laboratory, Shanghai Jiao Tong University, Sanechips

- RP-251739 High level views on 6G RAN general study  ZTE Corporation, Sanechips