3GPP TSG RAN Meeting #108                                             RP-2501744

Prague, Czech Republic, June 9-13, 2025

Source:	ZTE Corporation, Sanechips

Title:	Views on 6G NTN

Agenda Item:	16.1

Document for:	Discussion and Decision

Introduction

As listed in the objectives [1] for 6G RAN SID in RAN#107 meeting, the following is considered

- “Aim at a harmonized 6G Radio design for TN and NTN, including their integration”.

In this contribution, some preliminary views on the corresponding typical scenarios for NTN are elaborated.

NTN use cases and scenarios

High-level considerations

As captured in [2], the typical use cases for 6G NTN have been provided and mainly focus on the following two aspects:

- Communication-oriented: Ubiquitous and Resilient Network, Global mobile video, enhanced user experience with sparse LEO satellite deployment, service continuity for wearable mobile devices, Disaster relief, low-altitude logistics supported by NTN, etc

- Service-oriented: resilient positioning in satellite networks, low-energy positioning in satellite networks, etc.

Among all these use cases, to align with the ITU-R requirement, at least the communication-oriented aspects should be considered as part of 6G to ensure the ubiquitous service with harmonized design for TN and NTN. In addition, among all use cases, various UE types, e.g., IoT device, Mobile phone, vehicle-mounted terminal, CPE, and intermediate node can also be considered.

Proposal 1: For 6G NTN, the various UE types should be supported for communication service.

The consideration for NTN scenario

For NTN deployment, considering the restriction on capability, regulation and other factors, in addition to the scenario defined in Table 6.1.12-1 for 5G [3], more typical scenarios should be defined considering the limitation of satellite system. For example:

- Regarding the constellation type, in 5G, the single-orbit system (e.g., pure GEO or LEO) is considered as baseline from Rel-17. However, for some deployments, the multi-orbits system (e.g., combination between GEO and LEO) is preferred to enable the fast deployment along with refarming of the space-borne resource.

- Regarding the payload type, similar as 5G, both regenerative and transparent payload can be considered for future study. However, an additional case with “limited on-board processing capability” to enable the controlling of the satellite beam directly by gNB on the ground for transparent payload is also preferred. In this scenario, e.g., transparent payload with processing unit, e.g., MT onboard, can guarantee the jointly operation between satellite controlling and scheduling.

- Regarding the typical assumption of satellite, in 5G, the main assumption is for the RF related parameters, e.g., carrier frequency, antenna/beam gain, noise figure, which are used for the “link level” performance evaluation. However, to ensure the systematic assessment of satellite system, additional assumptions, e.g., number of beams/ratio between beam and planned footprint, should be considered.

- Regarding the interaction between TN and NTN, typically, in 5G, the case with “complementary” coverage is considered for both technical discussion and co-existence evaluation. For 6G, the overlapped/joint service may need to be studied according to the needs and regulation.

Additionally, the scenarios, in which the combination of the space-borne system with other nodes, e.g., repeater, to further extend the service to the case, e.g., indoor or in-vehicle, can be considered to boost the benefits of NTN deployment.

Proposal 2: For 6G NTN, the following aspects should be considered to capture typical NTN scenarios in TR 38.914:

- Orbit types, i.e., single and multi-orbits.

- Payload type, i.e., regenerative, transparent and transparent with limited on-board processing.

- Satellite assumption, e.g., beam hopping related parameter.

- TN-NTN relationship, e.g., overlapped/joint coverage.

Conclusion

In this contribution, the views on the definition of NTN scenarios are provided with following proposals:

Proposal 1: For 6G NTN, the various UE types should be supported for communication service.

Proposal 2: For 6G NTN, the following aspects should be considered to capture the typical NTN scenarios in TR 38.914:

- Orbit types, i.e., single and multi-orbits.

- Payload type, i.e., regenerative, transparent and transparent with limited on-board processing.

- Satellite assumption, e.g., beam hopping related parameter.

- TN-NTN relationship, e.g., overlapped/joint coverage.

References

- RP-250810 Revised SID: Study on 6G Scenarios and requirements CMCC, Verizon, NTT DOCOMO, INC., 	Deutsche Telekom

- TR 22.870 Study on 6G Use Cases and Service Requirements, stage 1 (Release 20)

- TR 38.913 Study on Scenarios and Requirements for Next Generation Access Technologies (Release 15)