3GPP TSG RAN #108		RP-251749

Prague, June 9th-14th, 2025

Agenda Item:	15.1

Source:	T-Mobile USA

Title:	Operators views for the AI_ML model delivery options and the need for a WID objective in Release 20.

Document for:	Discussion/Decision

# Introduction

This contribution outlines a set of requirements from mobile network operators (MNOs) regarding the transfer of AI/ML models, and it proposes a corresponding objective for Release 20 of 5G Advanced (5GA), grounded in these operator-driven needs.

User Equipment (UE) devices are inherently limited in their ability to store multiple trained AI/ML models due to constraints such as RAM, processing capabilities, and the absence of generalized models. Furthermore, the models used by UEs for inference must be updated regularly to reflect changing environmental conditions, incorporate new training data, and replace underperforming models.

To address these challenges, MNOs require a standardized mechanism to manage the transfer of trained models—residing within the MNO’s trusted network—to and from the UE. This is essential to ensure efficient, secure, and scalable deployment of AI/ML functionality across the network.

RAN2 agreements:

Agreements 1 Dataset/model parameter transfer solution from NW to UE training entity shall follow below principles: - A1 - Size: From RAN2 point of view, aim to define a unified solution (e.g. OTA or non-OTA) to support various sizes of dataset/model parameter transfer (dataset and/or parameter sharing size could range from tens of KBs to hundreds of MBs, but in average around hundreds of MBs); - A2 - Continuity: Service continuity of dataset and/or parameter transfer/delivery during UE mobility needs to be supported; - A3 - Controllability: NW decides on if and when to transfer/delivery the dataset and/or model parameter from NW to UE or UE training entity (a server inside MNO or an OTT server); - A4 - Latency: Relaxed latency requirement and infrequent update; - A5 - Visibility: dataset and model parameter to be understandable by UE/UE-side training entity (a server inside MNO or an OTT server). - A6: Proprietary information of the network and UE should be respected and not disclosed. 2 The following approaches for dataset/model parameter transfer Direction A are agreed to be included in the TR: Alternative 1 (non-OTA approach): gNB -> NW dataset/model parameters collection entity -> UE training entity (a server inside MNO or an OTT server) data RAN2 analyzed area dataset/model parameter transfer NW-side dataset/model parameters collection entity (gNB/CN/OAM) for two-sided UE-part model training CSI compression data collection at gNB UE-side training entity for two-sided UE part model training. Alternative 2 (OTA approach): gNB -> NW dataset/model parameters collection entity (if needed) -> gNB -> UE -> UE training entity (a server inside MNO or an OTT server) RAN2 analyzed area CSI compression data collection at gNB data dataset/model parameter transfer NW-side dataset/model parameters collection entity (gNB/CN/OAM) for two-sided UE-part model training UE-side training entity for two-sided UE part model training dataset/model parameter for training. NOTE: The data transfer (up to RAN1 further details on what to transfer) between gNB and NW dataset/model parameters collection entity (OAM/CN) in Alternative 1/2, if needed, is up to RAN3/SA2/SA5.. 3 For non-OTA solutions ‘NW dataset/model parameters collection entity -> UE training entity (a server inside MNO or an OTT server)’, RAN2 identified the following candidates. From RAN2 point of view it is assumed they can be supported within Rel-19 existing architecture framework. RAN3, SA2, and SA5 can confirm this assumption (i.e. we can send an LS). It does not preclude RAN3/SA2/SA5 to identify other candidate solutions beyond options listed below. Option; Impacted WG; Specification impact/Implementation impact. - OAM -> UE-side training entity (a server inside MNO or an OTT server), where OAM is NW-side dataset/model parameter collection entity; SA5, SA3; Up to SA5 (any intermediate node, if any, between OAM and UE-side OTT server is up to SA5; CN involvement if needed is up to SA2/SA5 discussion). - CN -> UE-side training entity (a server inside MNO or an OTT server), where CN is NW-side dataset/model parameter collection entity; SA2, SA3; Up to SA2 (any intermediate node, if any, between CN and UE-side OTT server is up to SA2). - gNB -> OAM/CN -> UE-side training entity (a server inside MNO or an OTT server), where gNB is NW-side dataset/model parameter collection entity; RAN3, SA2, SA5, SA3; Up to RAN3, SA2, SA5 (any intermediate node, if any, between gNB/OAM, OAM/UE-side OTT server, CN/UE-side OTT server is up to RAN3/SA2/SA5). 4 From RAN2 point of view, when gNB is the NW dataset/model parameter collection entity, solution 1b (i.e. gNB -> UE via UP) is not considered as a candidate solution in 5GA Rel-19/20 5 In OTA approach, UE transfers the received dataset/model parameter to UE training entity (an OTT server) transparently to 3GPP network. 6 For OTA solutions ‘gNB -> NW dataset/model parameters collection entity (if needed) -> gNB -> UE -> UE training entity (a server inside MNO or an OTT server)’, RAN2 identified following candidate solutions but there’s no consensus on feasibility: - OTA solution 1a: gNB -> UE via CP - OTA solution 2: CN -> UE via gNB - OTA solution 3: OAM -> UE via gNB 7 The following table is captured in the TR Table 2. OTA candidate solutions gNB -> UE via CP; Challenges; - UE is only required to support 45kB RRC buffer size, according to TS 38.306. - Significant specification impact: - Other segmentation beyond RRC layer requires a new SRB protocol stack to perform segmentation, including functions such as handling segmentation, retransmission, etc - UE selection - Challenges to support E2E reliability, considering dataset/model parameter transfer is shared by different gNB/vendors during UE mobility and different RRC state transition - Uu overhead for data collection from UE and dataset/model parameter transfer to UE - Overloading CP with large datasets would disrupt core control message transmission (e.g. service degradation, reliability, etc). Potential suitable scenario; - Small dataset/model parameter size. However, the maximum RRC segment needs to be further studied - Split large dataset/model parameter into small pieces, and potentially send to multiple UEs, then gather by UE training entity. RAN2 has not study the feasibility of split dataset/model parameter to multiple UEs.. CN -> UE via gNB; Challenges; - Same challenges as OTA solution ‘gNB->UE via CP’, if OTA solution ‘CN -> UE via gNB’ via CP - No benefit over non-OTA solution, as dataset/model parameter needs to transmit to CN, then transmit back to gNB. Relaying dataset/model parameter via gNB to UE then back to UE training entity is not desirable. - Unclear how to guarantee E2E reliability across multiple hops - A risk of proprietary information exposure if gNB and CN are not from the same NW vendor. Potential suitable scenario; - Feasibility analysis of OTA solution ‘CN -> UE via gNB’ via CP is the same as OTA solution ‘gNB -> UE via CP’ - OTA solution ‘CN -> UE via gNB’ and its feasibility is required to be evaluated by RAN3 and SA2.. OAM -> UE via gNB; Challenges; - Same challenges as OTA solution ‘gNB -> UE via CP’, if OTA solution ‘OAM -> UE via gNB’ via CP - No benefit over non-OTA solution, as dataset/model parameter needs to transmit to OAM, then transmit back to gNB. - Unclear how to guarantee E2E reliability across multiple hops - A risk of proprietary information exposure that OAM may share to a second NW vendor. Potential suitable scenario; - Feasibility analysis of OTA solution ‘OAM -> UE via gNB’ via CP is the same as OTA solution ‘gNB -> UE via CP’ - OTA solution ‘OAM -> UE via gNB’ and its feasibility is required to be evaluated by RAN3 and SA5..

In this TDoc, remaining issues will be discussed.

# Discussion

## 3GPP solution for model transfer

The solutions summarized on Table 1 in TR 38.843 [1] can be categorized into three different categories.

- RAN

- Core and LMF

- OAM

Table 1 Need to consider standardization solutions for transferring/delivering AI/ML models.

Whether there is a need to consider standardised solutions for transferring/delivering AI/ML model(s) is unclear from the outcome of the present study

- Solution 1a: gNB can transfer/deliver AI/ML model(s) to UE via RRC signalling. - Solution 2a: Core Network (except LMF) can transfer/deliver AI/ML model(s) to UE via NAS signalling. - Solution 3a: LMF can transfer/deliver AI/ML model(s) to UE via LPP signalling. - Solution 1b: gNB can transfer/deliver AI/ML model(s) to UE via UP data. - Solution 2b: Core Network (except LMF) can transfer/deliver AI/ML model(s) to UE via User Plane (UP) data. - Solution 3b: LMF can transfer/deliver AI/ML model(s) to UE via UP data. - Solution 4a: OTT server can transfer/deliver AI/ML model(s) to UE (e.g., transparent to 3GPP). - Solution 4b: OAM can transfer/deliver AI/ML model(s) to UE. Note: The relationships between model transfer/delivery solutions and use cases can be derived from what is captured in clauses 7.3.2, 7.3.3, and 7.3.4.

Since the OTT solution has no impact on 3GPP specifications, we consider there is no need to discuss this option RAN nor the need to develop features that support this option e.g. CSI configuration.

Proposal 1: Solution 4a, OTT without 3GPP impact, captured on [1] is not discussed in R20 and supporting features are outside the scope of 3GPP.

## Dataset/model parameter transfer solution for one-sided model

For network operators, having multiple solutions for a single problem is never ideal. It leads to market fragmentation and complicates implementation. 3GPP has witnessed several cases where fragmentation hindered the successful adoption of solutions. More importantly, there is no technical justification for differentiating the principles agreed upon in RAN2#129bis for the two-sided model from those that should apply to the one-sided model.

Therefore, for the one-sided UE model, it is essential that the network adheres to the same principles established in RAN2#129bis for two-sided models.

Proposal 2: One-sided dataset/model transfer solution shall follow below principles:

- A1 - Size: Support various dataset/model sizes (final model size to be defined).

- A2 - Continuity: Ensure service continuity during dataset/model transfer, particularly during UE mobility.

- A3 - Controllability: NW decides on if and when to transfer/delivery over the air interface;

- A4 - Latency: Relaxed latency requirements and support for infrequent updates

- A5 - Visibility: dataset and model parameters to be visible and understandable by MNO;

## Principles to transfer/delivery a model

Earlier sections outlined the approach to transferring dataset/model parameters for the one-sided model. This section discusses the core principles for delivering the model once it has been trained. The specific location of the training entities and who controls the training resources is outside the scope of this TDoc.

AI/ML functionality offers the potential to optimize network procedures, improving key areas such as energy efficiency and signaling overhead. Achieving these benefits will require the replacement of well-established and tested procedures. For instance, conventional measurement-based approaches may be bypassed, along with the corresponding work conducted by RAN4 and RAN5 to ensure expected performance. Instead, AI/ML-based actions will rely on predictions, which inherently include margins of error. With well-trained models, these error margins can be minimized to achieve performance levels comparable to current standards. However, it is important to recognize that this represents an ideal scenario.

Model training depends on data availability and time, both of which are critical to improving prediction accuracy. Consequently, initial AI/ML models—or models deployed for new UEs—may lack maturity, potentially causing network performance issues. From a 3GPP perspective, until RAN4 and RAN5 conduct dedicated work on AI/ML applications for the air interface, there can be no assurance regarding the performance of these models.

Observation 1: Currently there are no standardized tests to validate AI/ML models

It is expected that infrastructure vendors, device vendors and chipset vendors create their own AI/ML models. Hence, without individuals SLAs, MNOs have no control or visibility over the AI/ML model creation.

Observation 2: AI/ML models are expected to be created by infrastructure vendors, device vendors and chipset vendors

Given the absence of standardized testing and the limited control that mobile network operators (MNOs) have over AI/ML models, it is clear that MNOs must have the ability to activate and deactivate these models as needed. MNOs are uniquely positioned to assess and manage end-to-end network performance, which is not always visible to other stakeholders.

While an AI/ML model may perform well under specific conditions, it is challenging to train models to manage the full range of real-world scenarios. As a result, overall performance may deviate significantly from expectations. To address such cases, it is essential that MNOs have mechanisms in place to enable or disable AI/ML models as necessary to maintain service quality and reliability.

Observation 2: AI/ML models may require to be deactivated if the end-to-end performance is impacted

In [2], a list of requirements was captured in proposal 2. Considering the challenges, it is important to adopt the following requirements to ensure the end-to-end performance in the network.

Proposal 3: Adopt additional list of requirements for one-sided and two-sided model transfer/delivery.

- Activation: once the AI/ML one-sided or two-sided model is transferred to NW and/or to UE, MNO should be capable to activate and deactivate them at any point in time

- Model transfer/delivery: traffic should be transferred at a different priority, e.g., lower than user traffic.

- Differentiability: model transfer/delivery traffic should be differentiated from other user traffic.

- Security: there should be a guarantee that models are transferred securely, in a NW-aware manner, such that untrusted models cannot be downloaded.

- Addressability: Models need to be addressable such that the UE can request the transfer/delivery of a specific one

- Controllability: the NW is in control of if and when to transfer/deliver a model

Proposal 4 - For Release 20 it is necessary to add an objective to the AI/ML WID that states

- Develop the ability to transfer, activate, deactivate trained models that meet the following MNO requirements.

One-sided and two-sided dataset/model transfer and delivery :

- A1 - Size: Support various dataset/model sizes (final model size to be defined).

- A2 - Continuity: Ensure service continuity during dataset/model transfer, particularly during UE mobility.

- A3 - Controllability: NW decides on if and when to transfer/delivery over the air interface.

- A4 - Latency: Relaxed latency requirements and support for infrequent updates

- A5 - Visibility: dataset and model parameters to be visible and understandable by MNO;

- A6 - Activation: once the AI/ML one-sided or two-sided model is transferred to NW and/or to UE, MNO should be capable to activate and deactivate them at any point in time

- A7- Model transfer/delivery: traffic should be transferred at a different priority, e.g., lower than user traffic.

- A8- Differentiability: model transfer/delivery traffic should be differentiated from other user traffic.

- A9 - Security: there should be a guarantee that models are transferred securely, in a NW-aware manner, such that untrusted models cannot be downloaded.

- A10 - Addressability: Models need to be addressable such that the UE can request the transfer/delivery of a specific one

- A11 - Controllability: the NW is in control of if and when to transfer/deliver a model

# Conclusions

There is a strong need/requirement for greatly improved MNO control over trained AI/ML models. The following proposals capture MNO requirements and contain a proposed objective a R20 WID objective.

Observation 1: There are no standardized test to validate AI/ML models

Observation 2: AI/ML models are expected to be created by infraestructe vendors, device vendors and chipset vendors

Observation 2: AI/ML models may require to be deactivated if the end-to-end performance is impacted

For those reasons, it is proposed

Proposal 1: Solution 4a, OTT without 3GPP impact, captured on [1] is not discussed in R20 and any supporting features are outside the scope of 3GPP

Proposal 2: One-sided dataset/model transfer solution shall follow below principles:

- A1 - Size: Support various dataset/model sizes (final model size to be defined).

- A2 - Continuity: Ensure service continuity during dataset/model transfer, particularly during UE mobility.

- A3 - Controllability: NW decides on if and when to transfer/delivery over the air interface;

- A4 - Latency: Relaxed latency requirements and support for infrequent updates

- A5 - Visibility: dataset and model parameters to be visible and understandable by MNO;

Proposal 3: Adopt additional list of requirements for one-sided and two-sided model transfer/delivery.

- Activation: once the AI/ML one-sided or two-sided model is transferred to NW and/or to UE, MNO should be capable to activate and deactivate them at any point in time

- Model transfer/delivery: traffic should be transferred at a different priority, e.g., lower than user traffic.

- Differentiability: model transfer/delivery traffic should be differentiated from other user traffic.

- Security: there should be a guarantee that models are transferred securely, in a NW-aware manner, such that untrusted models cannot be downloaded.

- Addressability: Models need to be addressable such that the UE can request the transfer/delivery of a specific one

- Controllability: the NW is in control of if and when to transfer/deliver a model

Proposal 4 - For Release 20 it is necessary to add an objective to the AI/ML WID that states

- Develop the capability to transfer, activate, and deactivate trained AI/ML models within the trusted network, that meet the following MNO requirements.

One-sided and two-sided dataset/model transfer and delivery :

- A1 - Size: Support various dataset/model sizes (final model size to be defined).

- A2 - Continuity: Ensure service continuity during dataset/model transfer, particularly during UE mobility.

- A3 - Controllability: NW decides on if and when to transfer/delivery over the air interface.

- A4 - Latency: Relaxed latency requirements and support for infrequent updates

- A5 - Visibility: dataset and model parameters to be visible and understandable by MNO;

- A6 - Activation: once the AI/ML one-sided or two-sided model is transferred to NW and/or to UE, MNO should be capable to activate and deactivate them at any point in time

- A7- Model transfer/delivery: traffic should be transferred at a different priority, e.g., lower than user traffic.

- A8- Differentiability: model transfer/delivery traffic should be differentiated from other user traffic.

- A9 - Security: there should be a guarantee that models are transferred securely, in a NW-aware manner, such that untrusted models cannot be downloaded.

- A10 - Addressability: Models need to be addressable such that the UE can request the transfer/delivery of a specific one

- A11 - Controllability: the NW is in control of if and when to transfer/deliver a model

# References

- TR 38.843 - Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

- R2-2500949 - Requirements for Model Transfer/Delivery - T-Mobile USA, Boost Mobile Network, Deutsche Telekom, Orange, Charter Communications Inc., Nokia Corporation