3GPP TSG RAN Meeting #108					        RP-251776

Prague, Czech Republic, June 9-13, 2025

Source:	AT&T, Ericsson, FirstNet, KT Corp., MediaTek Inc., Nokia, Nordic Semi, NTT DOCOMO, Qualcomm, Rakuten, Sequans, Spark NZ, SONY, T-Mobile USA, Verizon

Title:	On 3GPP Support of LPWA in 6G

Agenda Item:	16.1

Document for: 	Discussion/Decision

# 1. Introduction

At the TSG RAN #107 meeting in Incheon, Korea a revised SID was approved to study 6G scenarios and requirements [1]. One of the objectives is for TSG RAN to develop 3GPP requirements that target a scalable and forward compatible design for diverse device types.

In this document, additional views are provided on 3GPP support of Low-Power Wide-Area (LPWA) use cases in 6G. Text proposals are provided for inclusion into 3GPP TR 38.914, “Study on 6G Scenarios and Requirements” [2].

# 2. Discussion

3GPP has been successfully addressing the Internet-of-Things (IoT) segment since LTE Release 13 with the introduction of eMTC and NB-IoT. These technologies have subsequently been evolved as evident in Table 1. Additionally, each release has also improved latency, coverage, device complexity, power consumption, mobility, and spectral efficiency.

For 5G massive Machine Type Communications (mMTC), one of the vertices of the IMT-2020 triangle also comprising URLLC and eMBB, 3GPP TSG RAN agreed that “no NR based solution is specified for Low-Power Wide-Area (LPWA) use cases” and that “LPWA use cases will continue to be addressed by evolving eMTC and NB-IoT” which can coexist with NR in overlapping spectrum [3].

Table 1: Evolution of eMTC and NB-IoT in 3GPP

Since the introduction of eMTC and NB-IoT, together referred to as LPWA technologies by 3GPP [3], 3GPP has introduced additional IoT technologies as shown in Figure 1. IoT-NTN comprises the non-terrestrial network (NTN) complements of eMTC and NB-IoT and was introduced in Rel. 17.  Unlike eMTC and NB-IoT, which are low power wide area technologies, Ambient IoT was specified in Rel. 19 as low power short-range 3GPP technology. And lastly, RedCap introduced reduced capability devices with reduced UE complexity and capability constraints in Rel. 17. All of these are 5G technologies, however, as depicted in Figure 1, some are 5G NR technologies (38 series) while others are 5G LTE technologies (36 series), a consequence of the TSG RAN #79 agreement  that “no NR based solution is specified for Low-Power Wide-Area (LPWA) use cases.”

In this context, it is worth pointing out that many carriers no longer maintain 2G or 3G services to support the IoT segment meaning absent a 5G NR solution, the LPWA segment is entirely served by LTE based technologies. Any strategic planning for LTE maintenance and sunset considerations is thus intimately intertwined with the technological evolution of LPWA in 3GPP.

Figure 1: Overview of 3GPP IoT Technologies

3GPP introduced LPWA very late in the 4G life cycle and early eMTC/NB-IoT devices could not reliably be upgraded over-the-air to connect to 5GC. 3GPP also specified multiple technologies for LPWA, namely, eMTC, which shares some common channels and signals with 4G eMBB, and NB-IoT as an independent RAT. Because 3GPP never specified a 5G NR based technology for LPWA, eMTC and NB-IoT can coexist with 5G NR using dynamic spectrum sharing (DSS) or guard-band deployments (NB-IoT only).

With the introduction of RedCap, history repeated itself. Both 4G eMTC/NB-IoT and 5G (e)RedCap were designed long after the initial release of the respective G. Both 4G eMTC and 5G (e)RedCap inherited the minimum bandwidth from eMBB. The minimum bandwidth in 4G was 1.4 MHz and the minimum bandwidth in 5G was 5 MHz.

Both 4G and 5G introduced bandwidths smaller than the minimum bandwidth for eMBB in later releases. NB-IoT with 200 kHz (occupying 180 kHz) can operate standalone in GSM channels or eMBB guard bands. NB-IoT can also coexist with eMBB in overlapping spectrum, however, with much lower efficiency (2x common channel overhead). In the 5G era, NR added support for dedicated spectrum with less than 5MHz bandwidth in Release 18.

Beyond 3GPP, there are additional considerations of utmost importance to mobile network operators. Absent a 5G NR based solution, the longevity of eMTC/NB-IoT impacts the strategic planning of LTE maintenance and sunset. Concurrently, LPWA design-ins can span 10+ years meaning 2030 and beyond is well within today’s time horizon of day-to-day business decisions. In fact, publicly available RFPs have benefit periods of more than 20 years. The lack of a 5G NR based solution and the multi-decade longevity of LPWA devices mean LPWA solutions for 2030 and beyond significantly impact business decisions TODAY!

The above informs the following 6G LPWA “must haves”:

- Long-term visibility is imperative TODAY both from a technological (e.g., LTE sunset) and business (e.g., 10+ years of benefit period) perspective

- Unlike in 4G and 5G, LPWA requires a single global solution provided by a single 6G RAT

- Because crucial design questions can most efficiently be decided in the first release of 6G, such as the minimum bandwidth for a common design framework for all verticals, 6G LPWA must be specified in the first release of 6G

- Lastly, solutions are needed that can last across technology generations in the core network, the RAN, and in overlapping spectrum with significantly improved efficiency compared to DSS between 5G and eMTC

Areas where LPWA can be improved include, but are not limited to:

- Enhanced accessibility with larger number of simultaneous active connections (number of RRC connected users) compared to eMTC

- Significantly better latency in addition to higher throughputs

- Power efficiency enhancements with support for advanced discontinuous reception (DRX) cycles (e.g., eDRX > 20 minutes) and power saving mode (PSM) extensions > 1 week

- Extended coverage enhancement mechanisms beyond MaxCL of [154] dB

- Improved mobility handling with lightweight handover or RACH-less mobility support for LPWA UEs moving slowly (e.g., <10 km/h)

- Support of optional multicast or group messaging capability for firmware updates and public safety

- Security optimizations

- UE RF requirements without a need for band specific SAW filters enabling global and low cost IoT solutions

All previous G’s have been designed top down starting with the most demanding use cases (eMBB and URLLC) which were then scaled down to ever lower complexities, powers, capabilities, bandwidths …

6G networks should be designed bottom-up, meaning a single RAT is scaled from a minimum set of KPIs, 
and resources are allocated upwards to the most demanding ones. In addition to enabling vertical industries with long life cycles, this bottom-up scalability establishes a platform and toolbox approach with one set of specifications that support all use cases with minimum customization and increased economy of scale.

The departure from a top-down design paradigm with all the aforementioned benefits for verticals is additionally expected to deliver network energy savings and overhead reductions for eMBB operation. Instead of reducing network energy consumption later in the G via dedicated work items (Rel. 12 vs. Rel. 8 in LTE and Rel. 18 vs. Rel. 15 in 5G NR), the 6G RAT is designed scalable from the onset, starting from a minimum set of KPIs up to the most demanding ones.

Figure 2: 6G LPWA Design Paradigms

# 3. Text Proposal

The following text is proposed for inclusion into 3GPP TR 38.914, “Study on 6G Scenarios and Requirements” [2]. The wording relies on previous agreements by 3GPP TSG RAN #79, for continuity from 4G, to 5G, and now 6G and to align common understanding within 3GP TSG RAN.

# 5.x​ 6G RAN support for diverse device types and massive machine-type communications (mMTC)

The 6G RAN and 6G Radio should be designed bottom-up, meaning a single RAT is scaled from a minimum set of KPIs, and resources are allocated upwards to the most demanding ones. The 6G RAN and 6G Radio should enable a platform comprising one set of specifications that support all use cases with minimum customization and large degrees of economy of scale. Because crucial design questions can most efficiently be decided in the first release of 6G, 6G LPWA must be specified in the first release of 6G. 6G LPWA here refers to the evolution of 5G eMTC as specified in 3GPP, and KPIs should meaningfully differ from (e)RedCap. Solutions should be designed to last across technology generations in the core network, the RAN, and in overlapping spectrum with significantly improved efficiency compared to DSS between 5G and eMTC.

# 4. Conclusion

In this document, additional views were provided on 3GPP support of Low-Power Wide-Area (LPWA) use cases in 6G. Text proposals are provided for inclusion into 3GPP TR 38.914, “Study on 6G Scenarios and Requirements”.

# References

- RP-250810, Revised SID: Study on 6G Scenarios and requirements, CMCC/Verizon/NTT DOCOMO, INC./Deutsche Telekom

- 3GPP TR 38.914, Study on 6G Scenarios and requirements, Release 20 (draft)

- RP-181300, Report of 3GPP TSG RAN meeting #79, ETSI MCC