3GPP TSG RAN Meeting #108								RP-251787

Prague, Czech Republic, June 9-13, 2025

Agenda item:	15.1.2

Source: 	Samsung

Title: 	Moderator's summary for NR MIMO Phase 6

Document for:	Discussion and Decision

## 1	Introduction

Following up on the previous moderator summary [1], this contribution provides a summary based on the contributions submitted in RAN#108 [2-22] and the subsequent discussions in RAN#108 on the Rel-20 NR MIMO Ph6 work scope.

## 2	Summary of submitted contributions

A summary of the proposed topics in RAN#108 along with their supports and concerns are given in Table 1.

- The summary is strictly based on the submitted contributions [2]-[22].

- Only proposals with at least 5 supporting companies are listed.

- Note that the first four topics are in accordance to Chair’s guidance for selecting maximum three topics [23, slide 7]

Table 1. Topics and summary of companies’ views

|  | Topic (see Appendix for objective wording) | Companies’ views |
| --- | --- | --- |
| 1 | SRS capacity/coverage: a) cross-slot repetition (U and S slots) in an SRS resource set, b) frequency hopping (different frequency-domain starting positions for SRS repetition symbols within each frequency hop) enhancement for RB-level partial sounding Note: - Objective formulation per [3] - One company demonstrates phase continuity is not an issue in real implementation [17]. | Support (46): Ericsson, Samsung, AT&T, Boost Mobile Network, BT, CATT, China Telecom, China Unicom, ETRI, Fraunhofer HHI, Fraunhofer IIS, Fujitsu, Futurewei, Google, Huawei, HiSilicon, Intel Corporation, Interdigital, KDDI, KT Corporation, Kyocera, LG Electronics, LG Uplus, MediaTek, NEC, NICT, NTT DOCOMO, NTT CORPORATION, OPPO, Orange, Panasonic, Rakuten Mobile, Sharp, SK Telecom, Sony, Spark, TCL, Telia Company, Verizon, Xiaomi, ZTE, Sanechips, Vodafone, Deutsche Telekom, Bouygues Telecom, Telecom Italia |
| 2 | Asymmetric DL sTRP/UL mTRP: PDCCH order PRACH beam indication, same Tx beam constraint across SRS occasions (per given SRS resource(s)) for beam sweeping, targeting FR2 Note: - Objective formulation per [3] | Support (41): Samsung, AT&T, Boost Mobile Network, BT, CATT, China Telecom, China Unicom, ETRI, Fraunhofer HHI, Fraunhofer IIS, Fujitsu, Futurewei, Google, Huawei, HiSilicon, Intel Corporation, Interdigital, KDDI, KT Corporation, Kyocera, LG Electronics, LG Uplus, MediaTek, NEC, NICT, NTT DOCOMO, NTT CORPORATION, OPPO, Orange, Panasonic, Rakuten Mobile, Sharp, SK Telecom, Sony, Spark, TCL, Telia Company, Verizon, Xiaomi, ZTE, Sanechips |
| 3 | Early SRS/CSI/CSI-RS triggering for UEs transitioning from IDLE/INACTIVE to CONNECTED mode via PRACH MSG4, as well as activation (Scell, out of dormancy) in CONNECTED mode; based on legacy signaling, RS, CSI formats Note: - Objective formulation per [3] - Two companies show large throughput gains [2], [21]. | Support (48): Ericsson, Samsung, AT&T, Boost Mobile Network, BT, CATT, China Telecom, China Unicom, ETRI, Fraunhofer HHI, Fraunhofer IIS, Fujitsu, Futurewei, Google, Huawei, HiSilicon, Intel Corporation, Interdigital, KDDI, KT Corporation, Kyocera, LG Electronics, LG Uplus, MediaTek, NEC, NICT, NTT DOCOMO, NTT CORPORATION, OPPO, Orange, Panasonic, Rakuten Mobile, Sharp, SK Telecom, Sony, Spark, TCL, Telia Company, Verizon, Xiaomi, ZTE, Sanechips, Apple, Lenovo, Vodafone, Deutsche Telekom, Bouygues Telecom, Telecom Italia |
| 4 | CSI-RS overhead reduction in frequency-domain (to ¼, 1/3, 1/6, and 1/8 RE/RB/port) targeting 48, 64, 128 ports, without impacting legacy rate matching, reusing legacy CDM group Note: - Objective formulation per [3] - One company shows significant throughput gain [20] | Support (45): Samsung, AT&T, Boost Mobile Network, BT, CATT, China Telecom, China Unicom, ETRI, Fraunhofer HHI, Fraunhofer IIS, Fujitsu, Futurewei, Google, Huawei, HiSilicon, Intel Corporation, Interdigital, KDDI, KT Corporation, Kyocera, LG Electronics, LG Uplus, MediaTek, NEC, NICT, NTT DOCOMO, NTT CORPORATION, OPPO, Orange, Panasonic, Rakuten Mobile, Sharp, SK Telecom, Sony, Spark, TCL, Telia Company, Verizon, Xiaomi, ZTE, Sanechips, Vodafone, Deutsche Telekom, Bouygues Telecom, Telecom Italia |
| 5 | Additional DMRS port configuration for FWA optimization for rank-3/4 + enhancement on CSI framework & CSI-RS resource design to support frequency/port/resource-domain CSI-RS partial sounding Note: - Objective formulation per [4] - One company shows significant throughput gain [4]. But another company shows lack of benefit in throughput [2]. | Support (9): Tejas Networks, Reliance Jio, China Unicom, CEWiT, Lekha Wireless, IISc Bangalore, IIT Madras, IIT Kanpur, IIT Bhilai Not support/Concern (9): Ericsson (no benefit), OPPO (scope/workload, gNB/UE product impact), Samsung (scope/workload, gNB/UE product impact, but open for 6G), MediaTek (scope/workload, product impact), NTT DOCOMO, NTT CORPORATION (no benefit, gNB/UE product impact), CATT (scope, product impact), ZTE, Sanechips (scope/workload, product impact, open for 6G) |
| 6 | Open-loop pre-coder cycling for 2TX UL non-coherent codebook-based transmission (non-full-power, 2 PRGs), no change in legacy RS Note: - Objective formulation per [5] - One company shows BLER gain [5] with the above limitation (2 PRGs, 2TX non-coherent). Another company shows throughput gain [18] with >2 PRGs and coherent precoders. But another company shows throughput loss [2]. | Support (7): vivo, CHTTL, Verizon, Spreadtrum, China Unicom, ETRI, UNISOC Not support/Concern (8): Ericsson (no benefit), OPPO (scope, spec impact), Samsung (scope, but open for 6G), MediaTek (no benefit), NTT DOCOMO, NTT CORPORATION (interference fluctuation), ZTE, Sanechips (scope, in 6G with CL) |
| 7 | Long-term covariance matrix feedback, including procedures for calculation, quantization, and reporting Note: - Objective formulation per [15] - One company shows significant throughput gain [17]. | Support (15): Huawei, HiSilicon, Vodafone, Deutsche Telekom, Bouygues Telecom, BT, Orange, Telecom Italia, China Unicom, LGU+, China Telecom, SK Telecom, CAICT, Google, Futurewei Not support/Concern (7): OPPO (scope/workload, open for 6G), Samsung (scope/workload, open for 6G), MediaTek (scope/workload, open for 6G), NTT DOCOMO, NTT CORPORATION (scope/workload), ZTE, Sanechips (scope/workload) |

In relation to the topics listed in Table 1:

- Two companies ([2], [14]) noted that Rel-20 NR MIMO Ph6 should only include at most 2 topics. In particular:

  - Ericsson proposes to include only topics #1 and #3 in Table 1 [2]

  - Nokia proposes to select 1-2 topics from topics #1, #2, #3, and #4 in Table 1 [14]

- Qualcomm [19] opines that

  - There shouldn’t be coherence requirement for topic #1. Benefit of topics #2 and #5 is unclear.

  - With topic #3 (the most supported one), there “will be very limited time for anything else”

  - Supportive of studying improvement on SRS channel estimation without limiting to topic #7

From the above summary, the following is observed:

- Out of the 7 topics listed in Table 1, topics #1, 2, 3, 4 receive much more support compared to topics #5, 6, 7.

  - Topics #1, 2, 3, and 4 have been identified in the Chair’s guidance as the 4 candidates to be down-selected to maximum 3 [23, slide 7]

- Significant number of companies (relative to the number of supporters) have expressed concerns on topics #5, 6, 7.

  - The reasons include scope/workload, impact on the current NR implementation, and/or lack of benefits.

  - Some of the concerned express openness for 6G, but not for Rel-20 NR due to the 1-TU constraint.

## 3	Further inputs from companies (drafts folder discussion)

From [23, slide 7], the following is the Chair’s guidance on MIMO Ph6:

- Focus on the following for further down-selection (at most 3 items from this list):

  - Cross-slot SRS, SRS frequency hopping for partial sounding

  - Asymmetric DL sTRP/UL mTRP beam management enhancements

  - Early SRS/CSI/CSI-RS triggering for Ues in IDLE/INACTIVE to CONNECTED; and SCell activation/out-of-dormancy in CONNECTED

  - CSI-RS frequency domain overhead reduction for larger number of ports

As noted, the above four candidate topics correspond to topics #1, 2, 3, and 4 in Table 1.

Please share your views on the following questions in Table 2 - Question 1: Given 1TU in RAN1 for 6 quarters (and 0.5TU in RAN2 for 3 quarters and 0.5TU in RAN4 Core for 6 quarters) [23, slides 9/12/16], will topics #1, 2, 3, and 4 in Table 1 fit in the allocated TUs? - Question 2: Regardless of your response to Question 1, if RAN were to choose 3 out of the 4 topics (per Chair’s guidance), what would be your priorities?

Based on the contribution summary in Section 2 and following the Chair’s guidance to down select from the 4 topics to 3, the subsequent discussion focused on the following two questions:

- Question 1: Given 1TU in RAN1 for 6 quarters (and 0.5TU in RAN2 for 3 quarters and 0.5TU in RAN4 Core for 6 quarters) [23, slides 9/12/16], will topics #1, 2, 3, and 4 in Table 1 fit in the allocated TUs?

- Question 2: Regardless of your response to Question 1, if RAN were to choose 3 out of the 4 topics (per Chair’s guidance), what would be your priorities?

On question 1, while the majority of companies (19) answered yes, the number of companies preferring to strictly follow the Chair’s guidance (select 3 out of the 4 topics) is significant (10).

As an alternative to the Chair’s guidance on down selecting from the four topics to 3 (while striving to maintain the spirit), the following was suggested by a few companies:

- Keeping the 4 topics while trimming the scope for each

- Selecting 3 topics out of 4, and introduce a check-point at e.g. RAN#112 to assess whether the left-out topic can be added

- Two-phase work plan

From moderator’s perspective, the second suggestion can still be aligned (in spirit) with the Chair’s suggestion.

On question 2, the most prioritized topic is topic-3 (early triggering, 31 companies), followed by topic-1 (SRS, 27 companies), and topic-4 (CSI-RS, 27 companies). The least prioritized topic is topic-2 (asymmetric, 11 companies).

A few companies mentioned support for topics other than the four and/or increasing the RAN1 TU allocation from 1 to 1.5. From moderator’s perspective, this is not be aligned with the Chair’s guidance.

Table 2. Companies’ inputs

| Company | Input |
| --- | --- |
| DOCOMO | Q1: Yes, each topic would occupy ~0.2TU and there is enough margin to fit into 1TU for all 4 topics. Q2: We don’t think choosing 3 out of the 4 topics is heathy exercise considering huge supporting companies in each topic. If we really need some down scoping, we would propose to narrow down the scope in each topic. For example, topic#1 has following two sub-topics, and OK to keep a) only a) cross-slot repetition (U and S slots) in an SRS resource set b) frequency hopping (different frequency-domain starting positions for SRS repetition symbols within each frequency hop) enhancement for RB-level partial sounding Similarly, topic#2 (even though this is our highest preference) has following two sub-topics, and we would accept to keep a) only a) PDCCH order PRACH beam indication b) Same Tx beam constraint across SRS occasions (per given SRS resource(s)) for beam sweeping |
| vivo | I would like to first comment on the topic#6, also supported by Tejas Network Limited. Since the proposal is to use non-coherent precoders, e.g. [1 0] and [0 1] for 2Tx rank=1 transmission, and up to 2 PRGs. On the interference fluctuation mentioned by DOCOMO, since only 1 antenna is used for transmission of a PRG, there should be no concern on interference fluctuation since it is omni directional and the PRBs are randomly scheduled in the cells including neighboring cells hence there is no difference to interference fluctuation in current system. On performance, non-coherent UE is assumed and especially for non-full power UE (Rel-15 UE) there is power gain (and to some extent antenna diversity gain), hence I fail to understand the concern performance benefit. For a UE with 2Tx, non-coherent, non-fullpower, 3dB power gain is there is for rank=1 transmission. On the scope and spec impact, the draft objective provided in RP-251204 is very specific and amounts to maintenance level for work required in RAN1. Of course there will be performance requirement work in RAN4. Overall, topic#6 is a very small item which is about maintenance level of work with benefits for the devices supporting UL MIMO as well as the network from higher transmit power at the UE. |
| Apple | We prefer to follow the Chair’s guidance to perform the down-selection to at most 3 topics, regardless of the TU. In terms of priority of topics to be included in WID 1st priority: Topic 3 (early CSI/SRS) 2nd priority: Topic 4 (CSI-RS overhead reduction) |
| Ericsson | Q1: No Q2: At least Topic 1 and 3. We are open to Topic 2 or 4 as the third topic if we necessarily must select a third topic. |
| SK telecom | Q2: If we must choose 3 out of the 4 topics as per Chair’s guidance, we prefer to have Topic 4 (CSI-RS overhead reduction) and Topic 3 (early CSI/SRS) at least. We are open to Topic 1 or 2 as the third topic. |
| Tejas Networks | Q2: We consider #1,#2 and #4 should be prioritized based on the available TU Furthermore, we disagree on the restrictive nature of the CSI-RS overhead reduction to FD. CSI-RS overhead reduction should be inclusive towards “Frequency domain”, “Spatial Domain” and “Hybrid Domain” approaches and the domain approach should be concluded during the WG phase. There are companies who have supported the CSI-RS SD domain in their proposals hence has to be discussed. We would prefer to rephrase the CSI-RS overhead reduction to be “CSI-RS overhead reduction for larger number of ports with minimized specification and UE implementation impact” #Note to Moderator #5 Tejas Proposals embeds 2 different proposals - Additional DMRS port configuration for FWA optimization for rank-3/4 + enhancement on CSI framework (for FWA) - CSI-RS resource design to support frequency/port/resource-domain CSI-RS partial sounding (proposal for CSI-RS overhead reduction) |
| Xiaomi | Although we co-sourced RP-251022, we prefer to follow the guidance from Mr. Chair. Among the four objectives, we would like to prioritize Topic #3 and Topic #4 - Early SRS/CSI/CSI-RS triggering for Ues in IDLE/INACTIVE to CONNECTED; and SCell activation/out-of-dormancy in CONNECTED - CSI-RS frequency domain overhead reduction for larger number of ports For the remaining two objectives, we think cross-slot SRS can be prioritized considering the limited spec impact and the requirement from real deployment. - Cross-slot SRS, SRS frequency hopping for partial sounding - Asymmetric DL sTRP/UL mTRP beam management enhancements |
| CATT | Q1: Yes. The 4 topics are clearly defined and with limited scope. They can fit in the allocated 1TU. Q2: If we have to choose 3 out of the 4 topics, topic 1, 3, 4 would be our preference. |
| OPPO | Q1: We prefer to follow Chair’s guidance Q2: Out of the 4 topics, we slightly prefer #2, #3, #4 |
| IDC | Q1: Given the clear and restricted definition of objectives 1,2, 3 and 4, we believe their completion within 1 TU would be possible. If we are to select only 3 topics, we are not supportive of any topics beyond 1-4 for this phase. Q2: Priority 1: Asymmetric DL sTRP/UL mTRP: PDCCH order PRACH beam indication, same Tx beam constraint across SRS occasions (per given SRS resource(s)) for beam sweeping, targeting FR2 Priority 2: Early SRS/CSI/CSI-RS triggering for Ues transitioning from IDLE/INACTIVE to CONNECTED mode via PRACH MSG4, as well as activation (Scell, out of dormancy) in CONNECTED mode; based on legacy signaling, RS, CSI formats Priority 3: CSI-RS overhead reduction in frequency-domain (to ¼, 1/3, 1/6, and 1/8 RE/RB/port) targeting 48, 64, 128 ports, without impacting legacy rate matching, reusing legacy CDM grou |
| Nokia | Q1: No Q2: Even though not originally proposed in our tdoc, we acknowledge the interest and relevance behind topics 5 (DM-RS) and 7 (covariance feedback), and we support work on those two objectives. Given the Chair’s guidance for (up to?) 3 topics, we support also topic 1 (SRS) given it is addressing UL improvements and it is small enough to fit together with the other two above into 1 TU. That is assuming topic 1 does not require extra RAN4 work, and that the overall sum of objectives is not going beyond what is reasonably expected for 1TU in RAN1. |
| Verizon | Q1: Yes. The 4 topics are clearly defined and with limited scope. They can fit in the allocated 1TU. Q2: #3 originated from a field problem currently happening in commercial network – slow CSI availability resulting in overly conservative scheduling for MU-MIMO small packet many devices. Other than #3, we also support #4 and #1. |
| China Telecom | Q1: Yes. We think 4 topics can be accommodated in 1 TU. Q2: We prefer #1, #3, #4 |
| MediaTek | Q1: We prefer to follow Chair’s guidance. Q2: Out of the 4 topics, we prioritize #3, then #4, #2. On #1, SRS Frequency Hopping phase continuity would need RAN4 effort to identify the impact and any new constraints for the FH operation, due to new assumptions for phase continuity vs Rel-17. |
| Futurewei | Q1: Yes. The 4 objectives are very specific and clearly defined. Q2: if down selection is really needed, our priority is #1 (SRS enhancement) and #3 (early CSI). |
| Huawei, HiSilicon | Within 1 TU discussion, for Q1, we are fine for #1,2,3,4 in the allocated 1 TU. For Q2, if necessary, we are priority on #1, 3,4, since FR1 enhancements is more urgently required for commercial deployments. SRS enhancements in #1 is most important among the 4 items in the real deployment, since the coverage and channel estimation accuracy of SRS is impact on TDD systems performance a lot, which also shown in our Tdoc. Require additional 0.5 TU, for #7 long term covariance feedback. Since the inter-cell interference for SRS in both single-TRP and multi-TRP scenarios is an urgent issue need to be addressed in 5G-A, which is the same view as many companies in RP-251596. The long-term covariance could provide single space to filter out interference and noise, which is beneficial interference mitigation and coverage enhancement. So, we require additional 0.5TU (is sufficient as listed all of the spec impacts in our Tdoc) to specify the long-term covariance feedback in R20. |
| LG Electronics | Q1: Yes Q2: #1 is the lowest priority among the four for us. |
| China Unicom | Q1:Yes, 1 TU is enough. Q2: 1st priority is #1 SRS enhancement for us. #7 long term covariance feedback is the important solution to solve the cell-edge interference from multiple Ues scenario. The limited additional 0.5 TU is needed to support #7 in R20. |
| LG Uplus | Q1: Yes. Qw: We prioritize #1 and #3. |
| Spreadtrum | Q1: We prefer to follow Mr. Chair’s guidance on further down selection. Q2: For downlink enhancement, we are fine to support #3 and #4. For uplink enhancement, we prefer #6 rather than #1 and #2. CSI acquisition in RRC IDLE/INACTIVE mode, CSI-RS overhead reduction and open-loop pre-coder cycling with limited scope have clear benefit and meet the practical requirements, and the design principles can be good reference for 6G design as well. For further enhancement on SRS and Asymmetric DL sTRP/UL mTRP, however, we fail to see the necessity at the current stage. |
| NEC | Q1: Yes. We think the first 4 topics can fit well in the 1TU. Q2: Actually, we still think support all the 4 topics can fit well in ~1TU, there is no need of down-selection. For example, topic #2 or even topic #4 only requires very small TU. But if we have to down-select (per Chair’s guidance), we at least support topics #1, #3, and we also prefer #4 for CSI-RS overhead reduction ONLY in frequency domain. If CSI-RS overhead reduction extended in spatial domain or hybrid domain, we are concerned the scope will be too large in 5GA. We can have more discussion for CSI-RS overhead reduction in other domains in 6G. |
| ETRI | Q1: Yes (possible) Q2: Our 1st priority is topic #3 (early triggering), followed by topic #1 (SRS) and #4 (CSIRS). Regarding the proposals to increase the scope for topic 4 (e.g., by adding spatial domain from Tejas), we believe spatial domain should be discussed in 6G, not 5G. |
| Mod V23 | Added LG Uplus input back (missing in V22, perhaps due to encrypted file issue) |
| Lenovo/ MotM | In our opinion, 1 TU can fit up to two topics for adequate evaluation and specification as well as avoiding incomplete/broken specification. In order to fit 3 agendas, we provide our order of preference as well as some scope limitation, is as follows: - Early SRS/CSI-RS/CSI triggering. (prioritizing Ues in idle to connected) - CSI-RS frequency overhead reduction. (scope limited to one new freq. density value per CSI-RS port configuration, no RS design or sequence/port-domain reduction enhancement in Rel-20 5GA) - SRS enhancements. (prioritizing cross-slot SRS for different slot types in TDD mode) Given the limited 1 TU for MIMO, we propose We also prefer to treat no more than two sub-agendas at a time, as follows: - Early SRS/CSI-RS/CSI triggering Specification spans 2025Q3 to 2026Q4 - CSI-RS frequency overhead reduction Specification spans 2025Q3 to 2025Q4 - SRS enhancements Specification spans 2026Q1 to 2026Q4 |
| ZTE, Sanechips | Q1: Yes. Instead of counting the number of items, their exactly occupied TU/WG-efforts should be assessed. In our views, we believe that the 4 items, i.e., #1, 2, 3, 4, can well fit that. Q2: As mentioned in our contribution RP-251740, we prefer the following two topics by order: - #4 CSI-RS frequency domain overhead reduction for larger number of ports - #2 Asymmetric DL sTRP/UL mTRP beam management enhancements Again, while considering huge supporting companies in each topic, further down-selection at most 3 out of 4 is NOT a heathy exercise. Consequently, we are open for the potential solution of further trimming from current scope from #1 to #4, as NTT DOCOMO suggested at the beginning. For instance, topic#2, we are fine for keeping a) only - a) PDCCH order PRACH beam indication - b) Same Tx beam constraint across SRS occasions (per given SRS resource(s)) for beam sweeping In short, after offline, for progress, we can live with the outcome per majority, i.e., a trimming solution for {#1, #2, #3, #4} under condition/check-point, or {#1, #3, #4}. |
| LGE2 | Fixed my mistake to save this file labeled as ‘LGE internal only’ ^^;; Re Q1, my RAN4 colleagues confirm that Topic #2 and #4 have no or very small RAN4 impact so RAN4 TU seems not an issue. |
| Samsung | Question 1: We understand and respect the guidance from the Chair. But to illustrate, if Coverage Enhancement with 0.5TU for 3 quarters can handle 3 small topics (which we agree), we see no strong reason why MIMO with 1TU for 6 quarters cannot handle 4 small topics – at least in terms of RAN1 workload. Question 2. While we don’t support down-selecting to 3 topics, in terms of priority, as also mentioned in our RP-251020 and our previous contributions: - Our 1st priority is topic #3 (early triggering) as this addresses urgent deployment needs as operators such as Verizon and other already explained - Our 2nd priority includes topic #1 (SRS enhancement – benefiting TDD) and #4 (CSI-RS overhead reduction – even for 64 ports, some energy saving gain is present) - Our 3rd priority is topic #2 to complete Rel-19 asymmetric DL/UL which can improve UL coverage for FR2 Regarding the alternative from DOCOMO to reduce the scope for each topic while keeping all the 4 topics, we also see this as a viable alternative before we try the remove 1 out of 4 topics. However, this could generate very long discussion on how to further trim the other topics (which have been made very focused and specific). Another alternative is to introduce a check-point, e.g. RAN#112 (June 2026), to assess the progress of the 3 selected topics. If the progress is good, the 4th topic can be added. |
| Intel | Q1: Strictly speaking, yes. It may be feasible to complete the work within 1TU. However, we acknowledge that this is only possible with lots of offline discussion efforts that is not visible by official TU counting. In the end, more effort on the 1 TU Rel-20 5G Adv work means less amount time available for 6G MIMO discussions. Also, if we assume less amount of offline time spent for MIMO discussion, then we risk less quality specification and output while being able to state “work was complete within 1 TU”. This is not ideally desirable. We ok to accept the four objectives, but would be open for further reduction in order to improve the workload of selected objectives and increase the time that could be spent for 6G MIMO offline discussions. Q2: Among the four objectives, our 1st preference is SRS enhancement. All other objective, while interesting are secondary preferences for us. |
| Ofinno | Q1: Yes. In our view all the 4 topics can fit well in the allocated 1TU. Q2: If RAN were to choose 3 out of 4 topics, then our priority is that topics #1, #2, and #3, are included in the WID. Among topics #1, #2, and #3, topics #2 and # 3 are of highest priority for us. |
| CMCC | Q1: We prefer to follow the guidance from Chair to further down-selection at most 3 items. Q2: Regarding the 4 items, we prioritize #4 (1st priority), #1 (2nd priority) and #3 (3rd priority). Since #2 is targeting FR2 only, #2 is the lowest priority among the four for us. |
| Google | Q2: Our preference is at least following two high priority topics in the four items: - Cross-slot SRS, SRS frequency hopping for partial sounding - Early SRS/CSI/CSI-RS triggering for Ues in IDLE/INACTIVE to CONNECTED; and SCell activation/out-of-dormancy in CONNECTED |
| Mod V31 | Merged Google and CMCC (crossed) |
| ZTE2 | One more clarification on our views on #4 CSI-RS overhead reduction, we support to focus on frequency-domain overhead reduction for now. For port domain reduction, even though we identify the clear benefits on that, e.g., NW energy saving, while considering that TU, it should be better to have that for 6G. |
| KDDI | Q1: Yes, each topic would occupy ~0.2TU and there is enough margin to fit into 1TU for all 4 topics. Q2: If we really need some down scoping, we would propose to rule out “Cross-slot SRS, SRS frequency hopping for partial sounding” |
| BT | Q1: Yes, our view is that it will be up to the proponents of all 4 topics to ensure sufficient consensus for timely completion. Q2: 1,3,4 and 7. Please remove our support for topic 2 as FR2 should be a lower priority in R20 5GA. |
| Vodafone | We support to include SRS coverage/capacity improvements as first priority from the Moderator’s selected topics i.e. Topic 1, and DL CSI acquisition enhancements as second priority. Given the importance of improving SRS performance, we would still be interested in having Topic 7 specified in Rel-20 with potential TU extension to handle the concern of companies regarding workload and/or TU limitation. |
| Mod V35 | Merged BT and Vodafone versions (crossed) |
| DT | Vodafone: On behalf of DT (due to IT Issues). Deutsche Telekom would like to support Vodafone position. |
| Mod V37 | Thanks for all the inputs! As I have now received good number of inputs, the potential outcome becomes clearer. Based on the submitted contributions and the additional inputs from companies on the two questions (per Chair’s guidance), the moderator’s proposal is as follows: Proposal-A: The Rel-20 NR MIMO Ph6 comprises specification of topic-1 (SRS), 3 (early triggering), and 4 (CSI-RS) in the scope. In addition: - Assess (based on the progress of topic-1, 3, and 4) in RAN#112 whether topic-2 (asymmetric) can be added in the scope. If so, the normative work will start in August (3Q) 2026 The objective text can be found in Table 3 of the Appendix. I’d appreciate some comments: - If the bullet point on RAN#112 assessment for topic-2 is agreeable or not. |
| ZTE | Thanks for Moderator’s great effort. Indeed, the above is not our first choice. But, for progress, we tend to agree with the above assessment from the moderator, and then compromise Proposal-A is okay for us. |
| Samsung | Although we maintain that the TU allocation can handle the four topics, given the Chair’s guidance, we are fine with the moderator’s compromise proposal-A |

## 4	Observation and proposal

Based on the submitted contributions (cf. Section 2) and the subsequent discussion per Chair’s guidance (cf. Section 3), it is proposed that RAN decide between proposal-A and proposal-B:

- Proposal-B is strictly based on the top-3 prioritized topics (topic-1, 3, and 4).

- Proposal-A also considers the notable level of support of topic-2

Proposal-A: The Rel-20 NR MIMO Ph6 comprises specification of topic-1 (SRS), 3 (early triggering), and 4 (CSI-RS) in the scope. In addition:

- Assess (based on the progress of topic-1, 3, and 4) in RAN#112 whether topic-2 (asymmetric) can be added in the scope. If so, the normative work will start in August (3Q) 2026

The objective text is given as follows:

- On enhancing UL capacity and coverage, specify the following enhancements:

  - For SRS, reusing the legacy port numbering for SRS resource, comb design, SRS sequence, SRS power control, symbol-level dropping rules (when SRS collides with another UL signal), TCI/UL-spatial-relation frameworks, and SRS bandwidth configuration for SRS frequency hopping, with the maximum number of SRS resources per set following the legacy specification:

    - Multiple frequency-domain starting positions for SRS repetition symbols within each SRS frequency hop for RB-level partial frequency sounding

      - Note: On phase continuity, the applicable conditions and requirements from the legacy RAN4 spec for DMRS bundling should be retained as much as possible.

    - Cross-slot SRS between one U slot and one adjacent S slot within a single SRS resource set

      - When used for one SRS with repetition, cross-slot SRS symbol mapping is limited to within one SRS resource, with a common timing advance (TA), a common UL spatial filter, and common transmit power for the SRS resource across the two consecutive slots

- On enhancing DL CSI acquisition, targeting FR1, specify the following enhancements:

  - Early SRS/CSI/CSI-RS triggering for UE transitioning from IDLE/INACTIVE to CONNECTED mode via MSG4 of 4-step RACH, as well as for SCell activation and switching out of SCell dormancy in CONNECTED mode via the legacy signaling mechanisms (based on legacy SCell activation MAC CE, and legacy switching DCI out of SCell dormancy, respectively)

    - Reusing the legacy design, limit the scope only to SRS for antenna switching, aperiodic wideband CSI with Rel-15 Type-I Single-Panel codebook (targeting FDD) and PMI-free report (targeting TDD) via the legacy UCI, as well as CSI-RS for CSI and (only for IDLE/INACTIVE to CONNECTED mode) CSI-RS for tracking

    - For IDLE mode, UE capability for early triggering and resource/reporting configuration are signaled via MSG3 and System Information (SI), respectively

  - For 48, 64, and 128 CSI-RS ports aggregated over multiple CSI-RS resources per legacy specification: CSI-RS density of 1/3, 1/4, 1/6, and 1/8 RE/RB/port while fully reusing legacy CSI-RS RE mapping per RB for CDM group generation (including OCC length, CDM group type, number of CDM groups), without impacting the legacy specification for rate matching on CSI-RS REs

    - Density of 1/3 and 1/6 RE/RB/port are intended only for 48 ports

- On enhancing UL capacity and coverage, assess in RAN#112, based on the progress of the above two objectives, whether to specify the following enhancements:

  - For asymmetric DL sTRP/UL mTRP, targeting FR2 assuming the Rel-17/18 unified TCI framework:

    - RRC configuration in an SRS resource set for beam management (BM) to indicate whether to apply a same spatial filter and pathloss offset across all the configured SRS resources in the SRS resource set for the case when UL TCI state is not provided to SRS, where more than one SRS resource set for BM can be configured for UL beam sweeping

    - PDCCH-order-triggered PRACH beam indication via the indicated UL TCI state or one of the two indicated UL TCI states for the PRACH transmission to UL TRP without DL RS transmission

If RAN decides to specify, the associated normative work starts from August (3Q) 2026.

Proposal-B: The Rel-20 NR MIMO Ph6 comprises specification of topic-1 (SRS), 3 (early triggering), and 4 (CSI-RS) in the scope. The objective text is given as follows:

- On enhancing UL capacity and coverage, specify the following enhancements:

  - For SRS, reusing the legacy port numbering for SRS resource, comb design, SRS sequence, SRS power control, symbol-level dropping rules (when SRS collides with another UL signal), TCI/UL-spatial-relation frameworks, and SRS bandwidth configuration for SRS frequency hopping, with the maximum number of SRS resources per set following the legacy specification:

    - Multiple frequency-domain starting positions for SRS repetition symbols within each SRS frequency hop for RB-level partial frequency sounding

      - Note: On phase continuity, the applicable conditions and requirements from the legacy RAN4 spec for DMRS bundling should be retained as much as possible.

    - Cross-slot SRS between one U slot and one adjacent S slot within a single SRS resource set

      - When used for one SRS with repetition, cross-slot SRS symbol mapping is limited to within one SRS resource, with a common timing advance (TA), a common UL spatial filter, and common transmit power for the SRS resource across the two consecutive slots

- On enhancing DL CSI acquisition, targeting FR1, specify the following enhancements:

  - Early SRS/CSI/CSI-RS triggering for UE transitioning from IDLE/INACTIVE to CONNECTED mode via MSG4 of 4-step RACH, as well as for SCell activation and switching out of SCell dormancy in CONNECTED mode via the legacy signaling mechanisms (based on legacy SCell activation MAC CE, and legacy switching DCI out of SCell dormancy, respectively)

    - Reusing the legacy design, limit the scope only to SRS for antenna switching, aperiodic wideband CSI with Rel-15 Type-I Single-Panel codebook (targeting FDD) and PMI-free report (targeting TDD) via the legacy UCI, as well as CSI-RS for CSI and (only for IDLE/INACTIVE to CONNECTED mode) CSI-RS for tracking

    - For IDLE mode, UE capability for early triggering and resource/reporting configuration are signaled via MSG3 and System Information (SI), respectively

  - For 48, 64, and 128 CSI-RS ports aggregated over multiple CSI-RS resources per legacy specification: CSI-RS density of 1/3, 1/4, 1/6, and 1/8 RE/RB/port while fully reusing legacy CSI-RS RE mapping per RB for CDM group generation (including OCC length, CDM group type, number of CDM groups), without impacting the legacy specification for rate matching on CSI-RS REs

    - Density of 1/3 and 1/6 RE/RB/port are intended only for 48 ports

## References

| 1 | RP-250802 | Moderator’s summary for RAN1-led REL-20 topics | RAN1 Chair (Samsung) |
| --- | --- | --- | --- |
| 2 | RP-250874 | Discussion on Rel-20 NR MIMO Phase 6 | Ericsson |
| 3 | RP-251022 | Proposal for Rel-20 NR MIMO Ph6 Objectives | Samsung, AT&T, Boost Mobile Network, BT, CATT, China Telecom, China Unicom, ETRI, Fraunhofer HHI, Fraunhofer IIS, Fujitsu, Futurewei, Google, Huawei, HiSilicon, Intel Corporation, Interdigital, KDDI, KT Corporation, Kyocera, LG Electronics, LG Uplus, MediaTek, NEC, NICT, NTT DOCOMO, NTT CORPORATION, OPPO, Orange, Panasonic, Rakuten Mobile, Sharp, SK Telecom, Sony, Spark, TCL, Telia Company, Verizon, Xiaomi, ZTE, Sanechips |
| 4 | RP-251197 | MIMO Phase-6 | Tejas Networks Limited |
| 5 | RP-251204 | WF on open loop precoding for PUSCH in Rel-20 MIMO Phase 6 | vivo, CHTTL, Verizon, Spreadtrum, China Unicom |
| 6 | RP-251221 | Views on NR MIMO Phase 6 | Spreadtrum, UNISOC |
| 7 | RP-251235 | Views on MIMO in Rel-20 5GA | Fujitsu Limited |
| 8 | RP-251253 | Discussion on NR MIMO Phase 6 work scope in Rel-20 | OPPO |
| 9 | RP-251296 | Rel-20 MIMO Scope | MediaTek Inc. |
| 10 | RP-251351 | Views on Rel-20 5G-Advanced MIMO Phase 6 | Apple |
| 11 | RP-251412 | Views on Rel-20 5G-A MIMO enhancements | NTT DOCOMO, INC., NTT CORPORATION |
| 12 | RP-251426 | Views on NR MIMO Phase 6 | Lenovo |
| 13 | RP-251475 | Support of additional MIMO enhancements | Vodafone, Deutsche Telekom, Bouygues Telecom, BT, Orange, Telecom Italia |
| 14 | RP-251545 | On MIMO Phase 6 enhancements | Nokia |
| 15 | RP-251596 | WF on Long Term Covariance Feedback in Rel-20 | China Unicom, LGU+, Deutsche Telekom, China Telecom, Vodafone,Google, SK Telecom, Huawei, HiSilicon, CAICT, Orange |
| 16 | RP-251640 | Views on Rel-20 NR MIMO | CATT |
| 17 | RP-251647 | Rel-20 MIMO scope | Huawei, HiSilicon |
| 18 | RP-251654 | Scope for MIMO for Rel-20 5G-Advanced | InterDigital, Inc. |
| 19 | RP-251708 | Qualcomm views on MIMO WI in Rel-20 | Qualcomm Incorporated |
| 20 | RP-251720 | Views on MIMO evolution for 5G-A in Rel-20 | ZTE Corporation, Sanechips |
| 21 | RP-251020 | General views on 5G-A Rel-20 | Samsung |
| 22 | RP-251699 | AT&T Views on 5G-Advanced in Rel-20 | AT&T |
| 23 | RP-250859 | Release 20 5G-Adv RAN1/RAN2/RAN3/RAN4 package | RAN Chair |

## Appendix: Objective wording

Table 3. Objectives (four topics)

| Topic | Ref | Objective wording |
| --- | --- | --- |
| 1 | [3] | - On enhancing UL capacity and coverage, specify the following enhancements: - For SRS, reusing the legacy port numbering for SRS resource, comb design, SRS sequence, SRS power control, symbol-level dropping rules (when SRS collides with another UL signal), TCI/UL-spatial-relation frameworks, and SRS bandwidth configuration for SRS frequency hopping, with the maximum number of SRS resources per set following the legacy specification: - Multiple frequency-domain starting positions for SRS repetition symbols within each SRS frequency hop for RB-level partial frequency sounding - Note: On phase continuity, the applicable conditions and requirements from the legacy RAN4 spec for DMRS bundling should be retained as much as possible. - Cross-slot SRS between one U slot and one adjacent S slot within a single SRS resource set - When used for one SRS with repetition, cross-slot SRS symbol mapping is limited to within one SRS resource, with a common timing advance (TA), a common UL spatial filter, and common transmit power for the SRS resource across the two consecutive slots |
| 2 | [3] | - On enhancing UL capacity and coverage, specify the following enhancements: - For asymmetric DL sTRP/UL mTRP, targeting FR2 assuming the Rel-17/18 unified TCI framework: - RRC configuration in an SRS resource set for beam management (BM) to indicate whether to apply a same spatial filter and pathloss offset across all the configured SRS resources in the SRS resource set for the case when UL TCI state is not provided to SRS, where more than one SRS resource set for BM can be configured for UL beam sweeping - PDCCH-order-triggered PRACH beam indication via the indicated UL TCI state or one of the two indicated UL TCI states for the PRACH transmission to UL TRP without DL RS transmission |
| 3 | [3] | - On enhancing DL CSI acquisition, targeting FR1, specify the following enhancements: - Early SRS/CSI/CSI-RS triggering for UE transitioning from IDLE/INACTIVE to CONNECTED mode via MSG4 of 4-step RACH, as well as for SCell activation and switching out of SCell dormancy in CONNECTED mode via the legacy signaling mechanisms (based on legacy SCell activation MAC CE, and legacy switching DCI out of SCell dormancy, respectively) - Reusing the legacy design, limit the scope only to SRS for antenna switching, aperiodic wideband CSI with Rel-15 Type-I Single-Panel codebook (targeting FDD) and PMI-free report (targeting TDD) via the legacy UCI, as well as CSI-RS for CSI and (only for IDLE/INACTIVE to CONNECTED mode) CSI-RS for tracking - For IDLE mode, UE capability for early triggering and resource/reporting configuration are signaled via MSG3 and System Information (SI), respectively |
| 4 | [3] | - On enhancing DL CSI acquisition, targeting FR1, specify the following enhancements: - For 48, 64, and 128 CSI-RS ports aggregated over multiple CSI-RS resources per legacy specification: CSI-RS density of 1/3, 1/4, 1/6, and 1/8 RE/RB/port while fully reusing legacy CSI-RS RE mapping per RB for CDM group generation (including OCC length, CDM group type, number of CDM groups), without impacting the legacy specification for rate matching on CSI-RS REs - Density of 1/3 and 1/6 RE/RB/port are intended only for 48 ports |

Other topics (not among the four):

Table 4. Objectives (other topics)

| Topic | Ref | Objective wording |
| --- | --- | --- |
| 5 | [4] | - On enhancing DL & UL capacity, specify the following enhancement, specify additional Demodulation Reference Signal (DMRS) antenna port configurations with particular emphasis on Fixed Wireless Access (FWA) deployments involving BS antenna arrays exceeding 8T8R: - Investigate and define alternative DMRS antenna port design principles to enable more efficient port allocation strategies. Update the antenna port mapping tables in TS 38.212 by specifying new port combinations for SU-MIMO Rank-3 and Rank-4. - Introduce enhancements to the co-scheduling mechanisms defined in TS 38.214 to support improved MIMO performance due to updated port mapping tables. - Define additional UE capability signalling to maintain backward compatibility, ensuring minimal impact for legacy and non-supporting devices. - On CSI-RS overhead reduction, specify the following enhancement, specify CSI-RS overhead reduction for up to 128 CSI-RS ports in frequency and port (resource) domains, without presenting UE receiver enhancements for CSI acquisition. - Modify NR CSI framework to enable partial frequency CSI-RS sounding and measurement to support frequency domain overhead reduction. - Modify NR CSI framework to enable occasional sounding of full CSI-RS ports (resources) and frequent sounding of a subset of CSI-RS ports (resources) to support port domain overhead reduction. - Enhance the CSI-RS resource and report configurations and settings to enable CSI-RS overhead reduction. |
| 6 | [5] | For codebook-based UL transmission, frequency-selective open-loop precoder cycling for dynamic-grant PUSCH using the legacy UL 2TX non-coherent 1-layer precoders (for 1-layer transmission), assuming network-configured cycling pattern (via RRC) and UL PRG size, fully reusing the legacy PUSCH-DMRS, SRS, and PTRS resource design - When configured, the number of PRGs is 2 - Targeting non-UL-full-power mode |
| 7 | [15] |  |

(Detailed) summary of companies’ views from Section 3

Question 1:

- Yes (19): NTT DOCOMO, CATT, IDC, Verizon, China Telecom, Futurewei, Huawei, HiSilicon, LG, China Unicom, LG Uplus, NEC, ETRI, ZTE, Sanechips, Samsung, Intel, Ofinno, BT,

- No (4): Ericsson, Nokia, Lenovo, MotM,

- Follow chair’s guidance regardless (select 3 from the 4) (10): Ericsson, Apple, Xiaomi, OPPO, MediaTek, Spreadtrum, Lenovo, MotM, Intel (open), CMCC,

Question 2 (priorities):

- Topic-1 (27): NTT DOCOMO, Ericsson, Tejas, Xiaomi (cross-slot), CATT, Nokia (no additional RAN4 impact), Verizon, China Telecom, Futurewei, Huawei, HiSilicon, China Unicom, LG Uplus, NEC, ETRI, Lenovo, MotM, ZTE, Sanechips, Samsung, Intel, Ofinno, CMCC, Google, BT, Vodafone, DT,

- Topic-2 (11): NTT DOCOMO, Tejas, OPPO, IDC, MediaTek, LG, ZTE, Sanechips, Samsung, Ofinno, KDDI,

- Topic-3 (31): NTT DOCOMO, Apple, Ericsson, SK Telecom, Xiaomi, CATT, OPPO, IDC, Verizon, China Telecom, MediaTek, Futurewei, Huawei, HiSilicon, LG, LG Uplus, Spreadtrum, NEC, ETRI, Lenovo, MotM, ZTE, Sanechips, Samsung, Ofinno, CMCC, Google, KDDI, BT, Vodafone, DT,

- Topic-4 (27): NTT DOCOMO, Apple, SK Telecom, Tejas (add port, resource), Xiaomi, CATT, OPPO, IDC, Verizon, China Telecom, MediaTek, Huawei, HiSilicon, LG, Spreadtrum, NEC, ETRI, Lenovo, MotM, ZTE, Sanechips, Samsung, CMCC, KDDI, BT, Vodafone, DT,