3GPP TSG RAN Meeting #108										RP-250847

Prague, Czech Republic, June 9-13, 2025

SA WG2 Meeting #169	S2-2506087

19 - 23 May, 2025, Fukuoka, Japan	rev of S2-2506083

Title:	Response to Reply LS on paging capability loss issue

Response to:	R2-2503172=S2-2504513

Release:	Rel-19

Work Item:	NR_LPWUS-Core, TEI19

Source:	SA2

To:	RAN2, RAN3

Cc:	TSG RAN, CT1

Contact Person:

#### Name:	Chris Pudney

E-mail Address:	chris dot pudney at vodafone dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

SA2 thanks RAN2 for this LS and their earlier LS in R2-2501388=S2-2504576.

In the latest LS, RAN 2 state that:

In particular for Rel-19 LP-WUS feature, if it is deployed, all the gNBs belonging to a certain area need to support the Rel-19 “container”.

The 5GS design means that the UE Radio Capability for Paging is loaded into the AMF only in the cell where the UE attaches (does “initial registration”) or first moves from LTE to NR. After that the UE can move throughout the whole PLMN and normally there is no event that triggers any update of the UE Radio Capability for Paging information stored in the AMF. Hence the “certain area” mentioned by RAN 2 is the whole NR coverage of that PLMN.

It is essential that operators are able to use RAN equipment from different vendors and that competition between those RAN vendors is not inhibited by RAN 2 requiring all RAN vendors to upgrade their equipment before new UE functionality can be used with the first RAN vendor to develop a solution.

Hence this RAN2 solution is highly undesirable compared to what was proposed in RAN2’s earlier LS in R2-2501388=S2-2504576.

SA2 are investigating whether a solution that builds on the current process in TS 23.501 clause 5.4.4.3 for handling UEs that support both “LTE connected to 5GC” and NR can be used to solve the problem.

For example, when (as already specified) the UE Radio Capability for Paging is sent in the NGAP Initial UE Context Setup Request to the gNB, the gNB could use the UE Radio Access Capabilities (received in the same message) to check that the UE Radio Capability for Paging contains the full information on paging related features. If the gNB detects an omission, then the gNB should ensure that an update of the UE Radio Capability for Paging Information is sent to the AMF.

2. Actions:

To RAN3, RAN2

SA2 asks RAN3 and RAN2 to provide views on the potential solution described above.

3. Date of Next TSG SA2 Meetings:

SA2#170		25-29 August 2025 	Goteborg, SE

SA2#171		13-17 October 2025	Wuhan, CN