3GPP TSG RAN Meeting #109																		 RP-252023
Beijing, China, September 15-18, 2025

Source:       	ZTE Corporation, T-Mobile USA, Softbank, CBN, China Broadnet, Telstra, Verizon, Qualcomm Inc., Sanechips, Charter, China Telecom, Turkcell, Vestel, Kyocera Corporation, DeepSig Inc, CableLabs, UNISOC, Spreadtrum, Türk Telekom, ABS, Khalifa University, Shanghai Jiao Tong University, NERC-DTV Co. Ltd., Queens University Belfast, Ruijie Networks, TCL, Pengcheng Laboratory, Continental Automotive Technologies GmbH

Title:	Views on IMT-2030 TPRs

Agenda Item:	8.2.1.2

Document for:   Discussion and Decision

- Introduction

In RAN#103 meeting, the following RAN plenary work plan was agreed for IMT-2030 aspects [1] where the RAN study includes two phases corresponding to ITU IMT-2030 TPR and 6G general aspects respectively.

RAN plenary work is split into an ITU focused SI (study item) and General RAN SI - IMT-2030 discussion is expected in RAN from 09/24 to 12/24 - RP SI Rel-20 focusing on ITU– IMT-2030: approval 12/24 until 06/25 - RP SI Rel-20 focusing on 6G General: approval 03/25 (after WS), until 06/26

In RAN#106 meeting, the new SID was endorsed for Study on 6G scenarios/use cases and requirements where the first phase on ITU TPR part is included.

This study item aims to - investigate a candidate set of items for minimum TPRs based on the Recommendation ITU-R M.2160, and, where applicable, the associated target values and key assumptions for the identified minimum TPRs. - The outcome is expected to be shared by LS with ITU-R WP5D, as suitable, and used as a baseline for the subsequent study 6G in RAN

In RAN#107, the moderator submitted a document comprising of all the proposals received from the meeting, as well as a moderator’s proposal to move forward by selecting the middle values within the whole range. The moderator proposal is captured in RP-250819 [6].

In RAN#108 meeting, some preliminary agreements have been made regarding the TPRs and their values, as captured in the reply LS to ITU-R WP 5D RP-251825 [9]. In this contribution, we provide our views on some TPR values that are still TBD.

- TPR candidates and values

In ITU-R WP5D#48 in February 2025, a preliminary list of Minimum Technical Performance Requirements (MTPRs) [4] was agreed and sent to SWG Evaluation from SWG radio aspects as shown below:

Preliminary list of Minimum Technical Performance Requirements

(Note: Square brackets indicate further discussion required)

| Minimum Technical Performance Requirement | Method |
| --- | --- |
| AI-related capabilities | (to be determined) |
| Area traffic capacity | (Quantitative) (Note 1) |
| Average Spectral efficiency | (Quantitative) |
| Peak spectral efficiency | (Quantitative) |
| 5th percentile user Spectral efficiency | (Quantitative) |
| Bandwidth | (Quantitative) |
| Connection density | (Quantitative) |
| Control plane latency | (Quantitative) |
| User plane latency | (Quantitative) |
| Energy Efficiency | (Quantitative) |
| Mobility | (Quantitative) |
| Mobility interruption time | (Quantitative) |
| Peak data rate | (Quantitative) |
| User Experienced Data Rate | (Quantitative) |
| Sensing-related capabilities | (Quantitative) |
| Positioning | (Quantitative) |
| Reliability | (Quantitative) (Note 2) |
| [Joint requirement on data rate, latency, reliability and capacity] | To be determined |
| [Coverage] | To be determined |
| [Security] | To be determined |
| [Resilience] | To be determined |
| [Sustainability] | To be determined |
| [Interoperability] | To be determined |

Note 1: The evaluation technique for quantitative requirements is determined by SWG Eval (e.g., analysis, simulation, etc.).

Note 2: The reliability requirement may be replaced by the “Joint Requirement” (if agreed).

In ITU-R WP5D#49 (June ’25), it was further agreed ([10]) that

- AI and Resilience (Note 3) will be Qualitative;

- Sustainability will be represented by Energy Efficiency;

- Rest of the TPR are still TBD (in []).

Note 3: While the current agreement is to define Resilience as a qualitative TPR, an alternative approach is also being considered to define "Resilience and Extended Connectivity" as a qualitative TPR.

Discussion on quantitative target values is planned to start in WP5D#50.

Based on the views from the co-sourcing members (comprising of network infrastructure vendors, chipset vendors, UE vendors, network operators, research institutes and vertical industries around the globe), and past RAN discussion, the table below captures our proposal on target values for some TPR that are still TBD in RAN, largely re-using the moderator proposal in RP-250819 [6] as baseline, which is considered a reasonable compromise among different RAN inputs and proposals.

Proposal 1: Define the target values for TPRs and RAN KPIs as in the table below.

Target values for TPRs toward IMT-2030

| ITU candidate TPR | Target value of IMT-2030 | Notes | Value of IMT-2020 |
| --- | --- | --- | --- |
| Peak data rate | DL: 100, UL: 50 Gbit/s. | 5x IMT-2020 (Related to the increase of bandwidth, number of antennas, modulation order, etc) | – Downlink peak data rate is 20 Gbit/s. – Uplink peak data rate is 10 Gbit/s. |
| Peak spectral efficiency | DL: 75, UL: 37.5 bit/s/Hz | 2.5x IMT-2020 (Related to the increase of number of antennas and modulation order) | – Downlink peak spectral efficiency is 30 bit/s/Hz. – Uplink peak spectral efficiency is 15 bit/s/Hz. |
| 5th percentile user data rate | DL: 500 Mbit/s | 5x IMT-2020 (for DL) | – Downlink user experienced data rate is 100 Mbit/s. – Uplink user experienced data rate is 50 Mbit/s. |
| 5th percentile user spectral efficiency | Indoor Hotspot 0.9(DL), 0.63 (UL) Dense Urban 0.675(DL), 0.45(UL) Rural 0.36(DL), 0.135(UL) bit/s/Hz | 3x IMT-2020 (all scenarios, both DL and UL) | (bit/s/Hz) Indoor Hotspot – eMBB 0.3(DL) 0.21(UL) Dense Urban – eMBB (NOTE 1) 0.225 0.15 Rural – eMBB 0.12 0.045 |
| Average spectral efficiency | Indoor Hotspot 27(DL) 20.25(UL) Dense Urban 23.4(DL) 16.2(UL) Rural 9.9(DL) 4.8(UL) bit/s/Hz/TRxP | 3x IMT-2020 (all scenarios, both DL and UL) | (bit/s/Hz/TRxP) Indoor Hotspot – eMBB 9 (DL) 6.75(UL) Dense Urban – eMBB (Note 1) 7.8 5.4 Rural – eMBB 3.3 1.6 |
| Area traffic capacity | Downlink 50 Mbit/s/m2 in the Indoor Hotspot-IC | 5x IMT-2020 | Downlink 10 Mbit/s/m2 in the Indoor Hotspot – eMBB |
| Mobility interruption time | 0ms | Same as IMT-2020 | 0ms |

In our view, the above targets represent reasonable advanced TPR values for IMT-2030, compared to IMT-2020.

It can also be noted that 3GPP NR technologies passed the ITU-R IMT-2020 requirements with huge margins for some TPRs, for instance, DL peak data rate can be as high as 171.2 Gbps (in FR2), which is more than 8 times higher than IMT-2020 TPR on DL peak data rate. NR can also achieve 64.6 Gbps (in FR2) UL peak data rate, which is already 6 times higher than IMT-2020 TPR. Thus, even if we propose to set the TPR of peak data rate to be 5 times higher than IMT-2020, it can be seen as a moderately advanced goal as NR can already satisfy this goal.

Observation 1: According to TR 37.910 “Study on self evaluation towards IMT-2020 submission”, many of the proposed IMT-2020 TPR target values are met by 3GPP NR Rel-15 technologies with a large margin.

- Summary

Proposal 1: Define the values for the list of TPRs and RAN KPIs as in the table above (sec. 2).

Observation 1: According to TR 37.910 “Study on self evaluation towards IMT-2020 submission”, many of the proposed IMT-2020 TPR target values are met by 3GPP NR technologies with a large margin.

- References

- RP-240823 Additional Considerations for 6G Timeline Source: TSG Chairs

- RP-250810 “Revised SID: Study on 6G Scenarios and requirements”, CMCC, Verizon, NTT DOCOMO, INC., Deutsche Telekom

- RP-243202	Liaison Statement to External Organizations Minimum requirements related to technical performance for IMT-2030 radio interface(s), source ITU-R WP5D, 3GPP TSG#106, December 2024

- ITU-R WP5D 563 Report on the 48th meeting of Working Party 5D (Geneva, 4-13 February 2025) Chair, WP 5D

- 3GPP RAN, TR 37.910, “Study on self evaluation towards IMT-2020 submission”

- RP-250819 Moderator's summary for IMT-2030 TPRs, Moderator (CMCC)

- Report ITU-R M.2412-0 “Guidelines for evaluation of radio interface technologies for IMT-2020”

- 3GPP SA1, TR 22.870, “Study on 6G Use Cases and Service Requirements”

- RP-251825 Reply LS to ITU-R WP5D =RP-243202 on Minimum requirements related to technical performance for IMT-2030 radio interface(s), RAN

- ITU-R Document 5D/TEMP/345-R1: Working Document Towards A Preliminary Draft New Report ITU-R M.[IMT-2030.TECH PERF REQ]