3GPP TSG RAN Meeting #109								   									RP-252029

Beijing, China, September 15-18, 2025

Title: 	Issues from 6G Radio Study

Source: 	ZTE Corporation, Sanechips

Agenda item:	8.2.2

Document for:	Discussion and decision

- Introduction

In RAN#108, the 6G SID “New SID: Study on 6G Radio” was approved [1]. The preliminary progress has been reached in RAN1#122 meeting with RANP related issues[2], e.g., device type, numerology and available spectrum for 6GR study, etc.  Issues related to diverse device types are discussed in our companion contribution[3].  In this contribution, our views on the aspects of numerology, spectrum and hardware refresh are provided.

- Issues from 6G Radio Study in RAN1

  - Discussion on numerology for 6GR

The following agreements have been achieved with two aspects subject to RAN decision. The first aspect is whether to consider 30 kHz or 7.5 kHz SCS for FDD band in addition to 15 kHz SCS.

Agreement - 6GR takes the following SCS as start point for discussion for all the signals/channels except PRACH. - For sub 6GHz - The following subcarrier spacing is at least supported - 15kHz SCS for FDD, 30kHz SCS for TDD - FFS: 30kHz SCS for FDD for around e.g., 1-2.5GHz - FFS: 7.5kHz SCS for sub1GHz (FDD) - Whether to discuss the FFS will be subject to RANP decision. - For around 7GHz - The following subcarrier spacing options can be studied - 30kHz, 60kHz - FFS: For around 15GHz - The following subcarrier spacing options can be studied - 30kHz, 60kHz, 120kHz - Whether to discuss it will be subject to RANP decision - For between 24.25GHz - 52.6GHz - Subcarrier spacing 120kHz is supported - FFS whether to allow using additional subcarrier spacing for SSB - FFS subcarrier spacing for PRACH and up to initial access discussion.

Regarding the first aspect, some companies argued whether second possible SCS for sub 6GHz can be considered in the study scope due to the following objective description in [1]:

d)	Channel Bandwidth (at least minimum and maximum), Numerology, avoiding multiple numerologies for the same band / sub-range (e.g., enabling synergies among frequency bands in the ~7GHz range)

Based on our understanding, the objective only gives a general guidance on avoiding multiple numerologies but the details (including the granularity) on how to avoid it is up to the study.  From our view, study can be allowed but strong justification is needed if companies would like to support multiple numerogies in a certain frequency range.  Our preference is to support 15 kHz SCS for FDD due to its compatibility with 5G networks and its coverage benefits. With 15 kHz SCS for FDD in place, the justification on introducing 30 kHz and 7.5 kHz additionally for low frequency FDD band is not strong.  Therefore, we are fine not to proceed with the FFS in RAN1

Observation 1: With 15 kHz SCS for FDD in place, the justification on introducing 30 kHz and 7.5 kHz additionally for low frequency FDD band is insufficient.

Proposal 1: For sub-6GHz, only 15kHz SCS for FDD and 30kHz SCS for TDD are supported.

For the frequency around 7GHz, RAN1 will further study the comparison of SCS between 30 kHz and 60 kHz. In [4], the field test result of 7GHz indicates that around 8.08 % ~ 11.48% of the channel delay spread samples are out of the range of CP for 60kHz subcarrier spacing. Thus, we propose to adopt 30 kHz SCS for frequency around 7GHz but this can be decided in RAN1.

  - Discussion on spectrum for 6GR study

As mentioned above, additional RANP guidance on the potential target spectrum in 6GR study is expected. However, as a general issue, the identification of the available spectrum for IMT usage should be confirmed by regulation firstly, e.g., WRC-27 or regional regulation. Otherwise, controversial views will still be expected among different stakeholders, e.g., [5][6]. To avoid inapprorpiate discussion on spectrum in 3GPP, it is not necessary to define the exact frequencies for technical discussion. Only defining around a frequency point is sufficient for study.  For example, in the agreed deployment scenarios in [7], only “Around” 700MHz, 2GHz, 4GHz, 7GHz, 30GHz are defined for study.  For each of these frequency points, one can find at least one confirmed IMT band around that.  It is expected that the design for the frequency bands around the same frequency point should be the same.  The study would not be wasted regardless of the results of the candidate bands around these frequency points i.e. even if candidate bands around these freqency points are not confirmed in regulation bodies in the future. Therefore, it is recommended to just focus on these frequency points with clear regulatory requirements for frequency bands around them.

Proposal 2: For 6GR design, it’s recommended to focus on spectrum with clear regulatory requirements.

  - Hardware refresh vs software upgrade

In RAN1#122, some companies requested to add “software upgrade” or “hardware commonality between 5G and 6G” as a design requirement during the discussion in some of the topics in 6G Radio Study. The proponents of software upgrade also proposed in [8] to capture “6G with Needs-based Hardware Refresh” as a RAN requirement in TR38.914.  The idea of “needs-based hardware refresh” requires 6G radio standards to support a major subset which does not require hardware refresh (i.e. requires software upgrade only).  While it is understood that cost/complexity has been always design consideration in general, a considerable amount of time was unnecessarily used to discuss whether it is needed to explicitly capture “5G-6G hardware commonlity” or “software upgrade” even before the actual study on technical proposals.  From our understanding in RAN1 discussion, it has been decided not to capture this as a design requirement in RAN1 due to the following reasons:

- Whether/how much a technical proposal has hardware impact highly depends on implementation. For example, “software-friendly” network equipment certainly allows more possibility of software upgrade but typically it has some drawback e.g. low energy efficiency.  In most of the cases, companies is not willing to disclose the details of implementation in the discussion.

- The understandings of software upgrade/hardware reuse varies.  While some proponents only mentioned the reuse of a certain network component like radio units for the exisiting 5G bands, some others also require the reuse of 5G baseband units for 6G.  Some even require that 5G backhaul should be compatible with 6G air interface.  It is difficult to align the understanding on different proponents’ requirements on the possibility of software upgrade.

- Depending on the understandings of softwar upgrade, this would possibly put a lot of restrictions on technical proposals and hinder possible innovation/gain.  With these restrictions, achievable gains for 6G can be serisouly impacted.

With these reasons, it is not meaningful to spend too much time for RAN WGs and RAN to discuss the requirement of “software upgrade” or “needs-based hardware refresh” even before the study.  Proponents should not use this to block the start of the study on technical proposals.  Instead, proponents should describe this as one of their complexity consideration together with other complexity consideration to assess the overall complexity-performance tradeoff of technical proposals in general.

Observation 2: It is not meaningful to spend too much time for RAN WGs and RAN to discuss the requirement of “software upgrade” or “needs-based hardware refresh” even before the actual performance study on technical proposals.

- Summary

In this contribution, our views on the identified issues are provided with following observations and proposals:

Observation 1: With 15 kHz SCS for FDD in place, the justification on introducing 30 kHz and 7.5 kHz additionally for low frequency FDD band is insufficient.

Observation 2: It is not meaningful to spend too much time for RAN WGs and RAN to discuss the requirement of “software upgrade” or “needs-based hardware refresh” even before the actual performance study on technical proposals.

Proposal 1: For sub 6GHz, only 15kHz SCS for FDD and 30kHz SCS for TDD are supported.

Proposal 2: For 6GR design, it’s recommended to focus on spectrum with clear regulatory requirements.

- Reference

- RP-251881, New SID: Study on 6G Radio, NTT DOCOMO (Moderator)

- Chair notes RAN1#122

- RP-252044, Discussion on diverse device types, ZTE Corporation, Sanechips

- R1-2505494, Discussion on 6G frame structure, ZTE Corporation, Sanechips, RAN1#122

- RP-252159, 6G study around 7GHz, Thales, Airbus, SES, Hispasat

- RP-252207, TP for 38.914 on Spectrum, Deutsche Telekom, Vodafone, Orange, Telecom Italia Turkcell, Spark NZ, Odido, BT, Bouygues Telecom, Telefonica, Telia Company, SK Telecom, KPN, Rakuten Mobile, CK Hutchison, Telstra, Telenor, KT Corp., KDDI.

- RP-251873, TR 38.914 V0.1.0 Study on 6G Scenarios and Requirements

- RP-252325, Premise and Promise of 6G with Needs-based Hardware Refresh, AT&T, Ericsson, KT Corp., Nokia, Rakuten, Spark NZ