3GPP TSG RAN Meeting #109								   									RP-252044

Beijing, China, September 15-18, 2025

Title: 	Discussion on diverse device types

Source: 	ZTE Corporation, Sanechips

Agenda item:	8.2.2

Document for:	Discussion and decision

- Introduction

In RAN#108, the 6G SID “New SID: Study on 6G Radio” was approved [1]. The preliminary progress has been reached in RAN1#122 meeting with RANP related issues[2], e.g., device types, numerology and available spectrum for 6GR study, etc.  For device types, there was discussion in RAN1#122 whether/how to define device types is under RAN1 scope.      In this contribution, our views on diverse device types are provided. Other issues are discussed in our companion contribution [4].

- Discussion on the Diverse device types

General categorization on the device types

According to the discussion in RAN1#122 and the progress achieved as shown below, it’s beneficial to reach consensus on the potential device types supported by 6GR. The consensus will faciliate well-organized discussion on relevant aspects, e.g., on how to support scalable design.  In the second agreement, it can be understood that RAN1 can study the device types from phyiscal layer perspective but it would need RAN for confirmation and further discussion on other perspectives.

Agreement Study a scalable 6GR design for diverse device types, considering aspects: - What should be commonly applicable to all 6G device types - FFS: add-on features dedicated to specific device types, if any Agreement - Study the device types from physical layer perspective to be supported by 6GR, subject to further discussion and confirmation in RAN

For device types in 5G and potential device type in 6G, as shown in Figure 2-1, various devices (e.g., Redcap, XR) are considered with different use cases and hardware capabilities.

Figure 2-1. Device types and use cases in deployed in 5G and suggested for 6G

For each type of device, the flexible UE capability framework has been introduced in 5G. However, in the real commercialization, due to the complexity and challenges of device development, network planning and various roadmaps from different stakeholders, a large number of candidate capabilities were not implemented. For 6G, as mentioned in [1], 6GR is aimed to support the agreed existing and new services, e.g., immersive communication, HRLLC and massive connection, along with the consideration on scalable and forward-compatible design for diverse device types. However, regarding device types, more factors, e.g., hardware restriction, market scale/fragmentation and impacts on technical design (e.g., initial access and common channel) should be well considered.

However, it would be challenging to define many different types with fine granularity. It’s preferred to focus on some typical categories with balanced consideration.

- Since eMBB service is still with a high priority, normal UEs, e.g. smartphone UEs, should be definitely considered as one direction.

- Additionally, in robotics applications such as industrial automation, autonomous systems and collaborative robotics, the requirements for data rate, latency and reliability become increasingly higher than normal UEs. Furthermore, CPE and FWA are essential components in communication networks, primarily used to provide wireless broadband connectivity. In these cases, an advanced UE type (other than normal UE) should be considered to meet more advanced requirements e.g. joint requirements of high data rate and low latency. For this advanced device type, the form factor is not a limiting factor, allowing for inclusion of advanced features such higher modulation orders, more Tx/Rx chains to enhance performance. The motivation of defining an advanced UE type is to setup a common set of advanced features to create a good size of economy scale.

- For other cases with lower targets, e.g., IoT, Redcap-like application, the one with scalable design is considered, e.g. scalable UE, aiming to lower complexity and cost.

It should be noted that the basic capability sets for different UE types, e.g., scalable UEs and normal UEs, should be clearly differentiated to avoid the market fragmentation.

Proposal 1: For 6GR, at least the following three type of devices are recommended as the target.

- Scalable UE: Designed for the reduced capability scenarios, offering lower complexity and cost.

- Normal UE: Providing improved performance compared to 5G eMBB.

- Advanced UE: Designed for the demand of high data rate, low latency scenario, with relaxed restriction of form factor and higher capability than normal UE.

Initial consideration on the requirement for scalable design

Based on the device types mentioned above, additional consideration on the corresponding requirements including the impacts on the system design should be further considered based on the agreement achieved in RAN1 #122 as follows.

Agreement - Study the following smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework from physical layer perspective, subject to further discussion and confirmation in RAN - Opt1: 3MHz - Opt2: 5MHz - Opt3: 10MHz - Opt4: 20MHz - FFS: the UL bandwidth may be different to the DL bandwidth - FFS: the bandwidth value may be different for different SCS, duplex modes, and bands. - FFS: whether RF and BB UE BW are same or different

Regarding the smallest maximum bandwidth, reduction of the bandwidth from 20MHz to 5MHz achieves more than 10% complexity reduction according to the evaluation in TR 38.865. For scalable UE targeting low‑tier use cases, adopting a smallest maximum bandwidth below 20MHz is beneficial to extend more use cases and reduce cost/complexity. Therefore, option 4 with 20MHz should be precluded as the smallest maximum UE bandwidth.

For 5MHz and 10MHz bandwidth, the detailed comparison is listed in the following table.

Table 1. Comparison between 5MHz and 10MHz assuming SCS=15KHz

|  | Option 2: 5MHz | Option 3: 10MHz |
| --- | --- | --- |
| Common signal design | SSB: Reused or enhanced SIB1 PDCCH: adapted to 5MHz SIB1 PDSCH: adapted to 5MHz Msg1: Reused or enhanced Msg2: Adapted to 5MHz Msg3: Reused or enhanced Msg4: Adapted to 5MHz | SSB: Reused or enhanced SIB1 PDCCH: adapted to 10MHz SIB1 PDSCH: adapted to 10MHz Msg1: Reused or enhanced Msg2: Adapted to 10MHz Msg3: Reused or enhanced Msg4: Adapted to 10MHz |
| Use case | 5MHz with QPSK modulation can achieve around 4Mbps data rate, which can cover the following use cases, e.g., - industrial wireless sensors: the reference bit rate is less than 2 Mbps - Video Surveillance: reference economic video bitrate would be 2-4 Mbps | 10MHz with QPSK modulation can achieve around 8Mbps data rate, which exceeds the peak data rate for some sensors or economic video. |
| Cost/complexity reduction | Compared with 20MHz, more than 10% reduction | Compared with 20MHz, more than 5% reduction. |

Given the analysis above, it is observed that option 2 and option 3 have similar impact on common signal design and impose fewer restrictions on normal UE. However, option 2 can better support diverse use cases and achieve larger cost/complexity reduction. To expand the 6G market and address more use cases, 5MHz is suggested as starting point for study assuming 15KHz.

Although, some deployment scenario described in [3] may require spectrum allocation smaller than 5MHz (i.e., 3MHz), the system design based on 5MHz can be still applied with minor adaptation for 3MHz scenarios. Specially, PSS and SSS in NR occupy 127 REs which does not exceed 3MHz. For PBCH, various solutions, e.g., puncturing as NR Rel-18 or optimized resource mapping can be considered instead of introduction of additional symbols.

Observation 1: For spectrum allocation smaller than 5MHz,

- PSS/SSS bandwidth does not exceed 3MHz

- PBCH design can apply puncturing as specified in NR Rel-18 or optimized resource mapping can be considered.

Furthermore, with 3MHz bandwidth in option 1, the capacities of SSB, SIB, PDCCH, and PDSCH would be limited, and the coverage performance will also be degraded. To compensate coverage performance, more time-domain resources are needed, which makes the capacity limitation more severe. Therefore, option 1 with 3MHz will seriously affect system efficiency and also degrade the performance of normal UE (e.g., smartphone UEs) due to these limitations.

Observation 2: Adopting 3 MHz as smallest maximum bandwidth, the system efficiency and performance of normal UEs will be degraded.

The 3MHz spectrum allocation is a minority scenario, and the associated performance issues can be solved via Alt1 or Alt2. Therefore, 3MHz should not be adopted as the smallest UE RF/BB bandwidth of diverse device types, as it may degrade the performance of normal UEs under normal system bandwidth.

And, based on the discussion above, the corresponding TP is also recommended to be captured in TR 38.914.

Proposal 2: For 6GR design, the smallest UE RF/BB bandwidth of diverse device types is recommended as 5MHz with 15KHz.

With the above discussion, we have the following TP for this meeting:Proposal 3: It’s recommended to capture the following TP in TR 38.914.

< Unchanged text omitted > - 5.3.2 Diverse device types - 6G RAT shall define a limited set of device type, e.g., Scalable UE, normal UE and advanced UE. - 6G UE shall support 5 MHz as the minimum supported bandwidth (i.e., by assuming SCS = 15kHz). < Unchanged text omitted >

In addition to a scalable UE with the smallest bandwidth, it should be considered how the scalable UE type can be scaled up from this lowest capability.  From our view, a sufficient gap should be considered between the normal UE type and the scalable UE type (with scaled-up capability).  A normal 6G UE should have higher capabilities than a normal 5G handheld UE. The gap between the scalable UE type and the normal UE type should be ensured to prevent the abuse of using a scalable UE to avoid the implementation of mandatory features of a normal UE.  Similar to XR device type discussed in NR, a forward-compatible mechanism should be designed to make sure a certain lower capability is only allowed in devices with form factor limitation.

Proposal 4: Study the following aspects and decide how to capture these aspects in TR 38.914:

- The normal 6G UE type should have higher capabilities than normal 5G handheld UE.

- Sufficient gap should be considered between the normal UE type and the scalable UE type.

- A forward-compatible mechanism should be designed to distinguish different UE types (including UEs with  form factor limitation)

- Summary

In this contribution, our views on diverse device types are provided along with the following observations and proposals:

Observation 1: For spectrum allocation smaller than 5MHz,

- PSS/SSS bandwidth does not exceed 3MHz

- PBCH design can apply puncturing as specified in NR Rel-18 or optimized resource mapping.

Observation 2: Adopting 3 MHz as smallest maximum bandwidth, the system efficiency and performance of normal UEs will be degraded.

Observation 3: With 15 kHz SCS for FDD in place, the justification on introducing 30 kHz and 7.5 kHz additionally for low frequency FDD band is insufficient.

Proposal 1: For 6GR, at least the following three type of devices are recommended as the target.

- Scalable UE: Designed for the reduced capability scenarios, offering lower complexity and cost.

- Normal UE: Providing improved performance compared to 5G eMBB.

- Advanced UE: Designed for the demand of high data rate, low latency scenario, with relaxed restriction of form factor and higher capability than normal UE.

Proposal 2: For 6GR design, the smallest UE RF/BB bandwidth of diverse device types is recommended as 5MHz with 15KHz.

Proposal 3: It’s recommended to capture the following TP in TR 38.914.

< Unchanged text omitted > - 5.3.2 Diverse device types - 6G RAT shall define a limited set of device type, e.g., Scalable UE, normal UE and advanced UE. - 6G UE shall support 5 MHz as the minimum supported bandwidth (i.e., by assuming SCS = 15kHz). < Unchanged text omitted >

Proposal 4: Study the following aspects and decide how to capture these aspects in TR 38.914:

- The normal 6G UE type should have higher capabilities than normal 5G handheld UE.

- Sufficient gap should be considered between the normal UE type and the scalable UE type.

- A forward-compatible mechanism should be designed to distinguish different UE types (including UEs with  form factor limitation)

- Reference

- RP-251881, New SID: Study on 6G Radio, NTT DOCOMO (Moderator)

- Chair notes RAN1#122

- RP-222645,NR support for dedicated spectrum less than 5MHz for FR1

- RP-252029, Issues from 6G Radio Study, ZTE Corporation, Sanechips