3GPP TSG-RAN Meeting #109	RP-252058

Beijing, China, 15 – 18 September 2025

Agenda item:		9.2.3

Title:		Views on enhancements for Ambient IoT in NR

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

In RAN#108, discussion on the scope of Ambient IoT(A-IoT) was endorsed in RP-251884[1] with the following summary.

- Study necessary and feasible changes to the Rel-19 A-IoT specifications to support A-IoT in outdoor scenarios under the following conditions [RAN1-led]: - Deployment scenario 4 with topology 1 with no external carrier wave. - Traffic types DO-DTT, DT, DO-A. - Representative use cases rUC5 (outdoor inventory), rUC6 (outdoor sensor), and rUC8 (outdoor command). - Assumption that A-IoT BS readers are deployed on the same sites as existing outdoor NR macro BSs. - FR1 licensed spectrum in FDD in-band to NR and in standalone bands, with R2D in DL spectrum and D2R in UL spectrum. - Assumption of the same air interface for Device 2b and Device C. - Take into account the aspects reported in Clause 6.10 “DO-A assessment” and Clause 6.1.1.7 “CFO calibration signal for device 2b” of TR 38.769 - Positioning for outdoor scenarios is under objective 2.

In this paper, we give our views on details of the scope of A-IoT.

# 2. Discussion

2.1	Device Types

In RP-251884[1], detailed description of device 2b and device C is given as following.A-IoT Devices These are the A-IoT Devices referred to in the objectives: - Device 1: As standardized in the Rel-19 WI Ambient_IoT_Solutions. ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither R2D nor D2R amplification in the device. The device’s D2R transmission is backscattered on a carrier wave provided externally. - Device 2b: ≤ a few hundred µW peak power consumption, has energy storage, IF envelope detector receiver or ZIF receiver, initial sampling frequency offset (SFO) up to 10Y ppm, R2D and/or D2R amplification in the device. The device’s D2R transmission is generated internally by the device. - Device C: ≤ 1 mW to ≤ 10 mW peak power consumption, has energy storage, IF envelope detector receiver or ZIF receiver, initial sampling frequency offset (SFO) up to 10Y ppm, R2D and/or D2R amplification in the device. The device’s D2R transmission is generated internally by the device. - SFO value 10Y is assumed to be better than any SFO value considered for Device 1 standardized in Rel-19.

From the wording of SID RP-251884[1], the only difference between device 2b and device C is the peak power consumption, which is “≤ a few hundred µW” for device 2b and “≤ 1 mW to ≤ 10 mW” for device C. And both device 2b and device C share the same level of SFO, i.e., “up to 10Y ppm”.

In RAN1#122[2], evaluation assumption of active device was discussed as below.

Agreement For Rel-20 study, Device noise figure assumption in link budget for evaluation purpose: - Device 2b: 15 dB - FFS: 40dB as additional assumption for Device 2b with IF-ED - Device C: 13 dB

When discussing peak power consumption, normally we think it is related to Tx power consumption. However, the difference in noise figure is related to Rx power consumption. Such difference in noise figure means Device C may have blocks of LNA, BB amplifier or BB LPF with better performance than those of Device 2b. To ensure such difference is within the scope, SID may need to capture it.

Proposal 1: For Rel-20 A-IoT, the scope should capture the following note:

- Note: Device C may have receiver blocks with better performance than those of Device 2b.

Furthermore, in RAN1#122[2], details related SFO and CFO are given as:

Agreement For Rel-20 study, SFO and CFO in LLS assumption for evaluation purpose is from the followings: - Device C is smaller than Device 2b Device 2b; Device C. Initial SFO; Randomly select a value from the range of 10^3~10^4 ppm; [50] ppm. SFO after clock sync/calibration; 10^2 (O), 10^3 (M) ppm; [10] ppm. Initial CFO; Randomly select a value from the range of 10^3~10^4 ppm; [50] ppm. CFO after clock sync/calibration; 100 (M), 200 (O), 50 (O) ppm; [10] ppm.

Though the exact values of “initial SFO” and “SFO after clock sync/calibration” have not been settled, the concept of “Device C is smaller than Device 2b” has been agreed, which may not be fully aligned with the shared level of SFO, i.e., “up to 10Y ppm” for both Device 2b and Device C in RP-251884[1].

Furthermore, the temporary value of “initial SFO” for Device C is [50] ppm, while the corresponding value set for Device 2b is in range of 103~104 ppm, which is 20~200 times worse. And the temporary value of “SFO after clock sync/calibration” for Device C is [10] ppm, while the corresponding value set of Device 2b is 102(O) or 103(M) ppm, which is 10~100 times worse.

Considering the above potential contradiction, guidance of setting the SFO values for Device C should be given. Similar analysis can also be derived for CFO assumption.

Proposal 2: For evaluation assumption of SFO or CFO, guidance should be given for setting the assumed values for Device C.

2.2	Device (un)availability

During RAN1#122 meeting, some submitted contributions from companies would like to discuss the aspects related to device (un)availability during the SI phase. And there are some ambiguities about whether the device (un)availability is within the scope of SI. From our perspective, Rel-20 SI shall focus on the necessary and feasible enhancements on Device 2b/C to support outdoor scenario, but device (un)availability aspect has already been adequately studied during Rel-19 study phase. Moreover, this aspect is not an outdoor specific issue but a common issue for both indoor and outdoor. Based on the above analysis, to avoid duplicated study in Rel-20 and save the workload, we prefer to start the specification work on device (un)availability during Rel-20 A-IoT WI phase directly, without duplicating the study in SI phase.

Proposal 3: For Rel-20 A-IoT, device (un)availability should be directly specified in Rel-20 WI phase without the duplicated study in SI phase, i.e., confirm device (un)availability is out of the SI scope.

# 3. Conclusion

In this contribution, we shared our further considerations on Rel-20 A-IoT and the following proposals were drawn from the analysis.

Proposal 1: For Rel-20 A-IoT, the scope should capture the following note:

- Note: Device C may have receiver blocks with better performance than those of Device 2b.

Proposal 2: For evaluation assumption of SFO or CFO, guidance should be given for setting the assumed values for Device C.

Proposal 3: For Rel-20 A-IoT, device (un)availability should be directly specified in Rel-20 WI phase without the duplicated study in SI phase, i.e., confirm device (un)availability is out of the SI scope.

# References

[1] RP-251884, Study on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices, June 9-13, 2025

[2] R1-2506615, FL summary #6 for Ambient IoT: “10.3.1 Evaluations”, Huawei, 25-29 Aug, 2025.