3GPP TSG-RAN Meeting #109	RP-252059

Beijing, China, 15 – 18 September 2025

Agenda item:		9.3.1.4

Title:		Views on solutions for Ambient IoT in NR Phase 2

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

In RAN#108, discussion on the scope of Ambient IoT(A-IoT) concluded with an approved new WID in RP-251885 [1] with the following summary.

- Specify support for Deployment Scenario 2 with Topology 2 with intermediate UE as Reader under the following conditions [RAN2-led]: - According to D2T2-B for Device 1 with carrier-wave transmission for waveform 1 only, per TR 38.769. - No specification support for CW control - According to D2T2-C for active Device(s). - Start after March 2026 - Traffic types DO-DTT and DT, and DO-A (only for active Device(s)). - RAN2 to start on DO-A for D2T2 when sufficient progress has been made on DO-A for D1T1, after a checkpoint in RAN plenary in September 2026 - Representative use cases rUC1 (indoor inventory) and rUC4 (indoor command). - FR1 licensed spectrum in FDD, with R2D in UL spectrum, and D2R and CW in UL spectrum. - Specify a single architecture by down-selection between RRC-based architecture and UP-based architecture, in coordination with SA2. - The following are not supported: - Coordination between UEs operating as UE readers - New Uu protocol layer(s) - Simultaneous (time overlapping) operation of Uu and A-IoT at the intermediate UE in the same band This objective begins after RAN#109 (September 2025), and after RAN1#111 (March 2026) for RAN1. The work is subject to the following constraints: RAN1 - The objective includes specification of necessary power control and collision handling procedures between intermediate UE and BS. - No physical layer impact for signaling A-IoT resources to the intermediate UE. - No additional specification change on R2D and D2R for Device 1 in Rel-20 RAN2 - The objective includes specification of necessary Uu procedures to enable the A-IoT operations. - RRC CONNECTED UE reader only, including RLF, without specific enhancement for other temporary out-of-connection scenarios. - A-IoT radio resource of the UE reader is only valid within the serving cell. - Resource allocation to the UE Reader shall be via Uu RRC signaling. RAN3 - Specify signaling for Topology 2 - The objective includes specification of UE reader authorization, including F1AP support, and UE reader selection. - NOTE: F1AP/XnAP/NGAP impact is expected to be minimized. - Inter-gNB resource coordination is not specified. - Specify the necessary signaling support for inter-gNB RRC-connected UE Reader Mobility

In this paper, we give our views on scope of A-IoT phase 2.

# 2. Discussion

2.1	Topology 2 architecture down-selection

In RP- 251885[1], the objectives for supporting Topology 2 include specifying the necessary Uu interface procedures to enable A-IoT operations:

- The objective includes specification of necessary Uu procedures to enable the A-IoT operations.

In TR 38.769[2], there are three candidate solutions for Topology 2 architecture.

To support Topology 2, the following solutions are to be studied for conveying A-IoT upper layer information: - RRC based solution. With this solution, A-IoT CN applies A-IoT upper layer information explicitly over XXAP signaling. A-IoT upper layer information is then relayed explicitly to/from the A-IoT-enabled UE via NR Uu RRC. - NAS based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted over A-IoT enabled UE's NAS. - UP based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted as A-IoT-enabled UE's user plane data. NOTE 1: The protocol stack for each solution option does not illustrate A-IoT CN internal architecture and how A-IoT upper layer information is transported, if any.

And the following WI description was captured in the RANP#108 meeting:

- Specify a single architecture by down-selection between RRC-based architecture and UP-based architecture, in coordination with SA2

Where the NAS-based solution is excluded but RRC-based solution and UP-based solution are still on the table.

Additionally, Work Task of support of A-IoT services in Topology 2 is given by a new SID from SA [3].

As noted in the SID, SA has decided the RRC-based option from the aforementioned three solutions, and the User Plane option is excluded from the SA SID. Therefore, to align with SA’s decision, it is necessary for the RAN side to conduct a down-selection to the RRC-based solution.

Proposal 1: Down-select to RRC-based solution in Topology 2 objectives, and update the WID description as

- Specify necessary Uu procedures to enable the A-IoT operations under RRC-based solution.

2.2	Clarification on out-of-connection scenarios

In TR 38.769[2], candidate options for radio resource validity are provided for the temporary out-of-connection scenario outlined below. From the relevant context, it can be seen that the out-of-connection scenario here includes both RLF and handover cases.

Scenario 2: In RRC_CONNECTED, the UE reader can perform A-IoT operations using the allocated A-IoT radio resource for A-IoT radio interface, upon its temporary out of connection scenarios (e.g., RLF and handover cases): -Option 1: The UE reader considers the resources as temporarily invalid during the temporary out of connection condition. The resource may become valid again after the UE recovers from the condition. -Option 2: The UE reader considers that the resources remain valid for a time period. It needs to be further discussed on whether this time period belongs to the one in Option 2 for Scenario 1 or is a different time period. It needs to be further discussed on whether the duration of this time period is related to the ongoing A-IoT operations.

In RP- 251885[1], to support Topology 2, the following objective has been captured:

- RRC CONNECTED UE reader only, including RLF, without specific enhancement for other temporary out-of-connection scenarios.

From the above objective, it is noted that only one out-of-connection case—RLF, falls within the current scope. This implies that temporary out-of-connection caused by handover is excluded.

However, the WID from the RAN3 perspective still includes the specification of handover-related signaling:

- Specify the necessary signaling support for inter-gNB RRC-connected UE Reader Mobility

The inter-gNB RRC-connected UE Reader Mobility includes the handover between gNBs. Therefore, it seems there is some mis-alignment between RAN2 and RAN3 draft objectives. Whether handover is considered as an out-of-connection scenario and being excluded, or handover is included in the scope of the WID, needs to be clarified.

From a technical perspective, since the UE remains in the RRC_CONNECTED state during handover, it is still in the RRC_CONNECTED state and is not considered out of connection. Therefore, the wording "RRC CONNECTED UE reader only" in the WID should include handover case.

Observation 1: Handover case is excluded according to the description of out-of-connection in TR but inter-gNB UE mobility is already included in the current WI scope.

Proposal 2: Clarify whether the handover of RRC_CONNECTED UE Reader is included in the WI scope.

Proposal 3: The behaviour of the UE reader when a handover occurs during A-IoT operation should be specified.

Proposal 4: We propose updating the relevant content in the WID as follows:

- RRC CONNECTED UE reader only, including RLF, without specific enhancement for other temporary out-of-connection scenarios.

  - NOTE: Specify the UE reader behaviour if handover happens during the A-IoT operation.

# 3. Conclusion

In this contribution, we shared our further considerations on Rel-20 A-IoT and the following observation and proposals were drawn from the analysis.

Observation 1: Handover case is excluded according to the description of out-of-connection in TR but inter-gNB UE mobility is already included in the current WI scope.

Proposal 1: Down-select to RRC-based solution in Topology 2 objectives, and update the WID description as

- Specify necessary Uu procedures to enable the A-IoT operations under RRC-based solution.

Proposal 2: Clarify whether the handover of RRC_CONNECTED UE Reader is included in the WI scope.

Proposal 3: The wording "RRC CONNECTED UE reader only" in the WID should include handover case.

Proposal 3: The behaviour of the UE reader when a handover occurs during A-IoT operation should be specified.

Proposal 4: We propose updating the relevant content in the WID as follows:

- RRC CONNECTED UE reader only, including RLF, without specific enhancement for other temporary out-of-connection scenarios.

  - NOTE: Specify the UE reader behaviour if handover happens during the A-IoT operation.

# References

[1] RP-251885, New WID on Solutions for Ambient IoT (Internet of Things) in NR Phase 2, June 9-13, 2025.

[2] TR 38.769, Study on solutions for ambient IoT (Internet of Things).

[3] SP-250834, NEW SID on Architecture support of Ambient power-enabled Internet of Things - Phase 2,10 -13 June, 2025