3GPP TSG-RAN Meeting #109	RP-252064

Beijing, China, 15–18 September, 2025

Agenda item:	9.3.2.1

Source:	Samsung

Title:	Scope of AI/ML for mobility in Rel-20

Document for:	Discussion and Decision

# Introduction

RAN approved Rel-20 WI: AI/ML for mobility in RP-251864 [1] last June, but it still contains several checkpoints to capture the conclusion of its SI in Rel-19.

The objectives are as the following: - Specify support for RRM measurement prediction for both UE and network sided models [RAN2]: - Specify signalling and protocol aspect to enable reporting inference outcome of UE sided model for RRM measurement prediction - Specify signalling and protocol aspect to enable reporting information from UE for inference operation of network sided model for RRM measurement prediction - At least the use cases studied so far in RAN2 SI are supported: Temporal domain prediction case A and case B, and Frequency domain prediction for collocated cases. Cell level predication is supported Note 1: For UE sided model, spatial domain is not supported. Can check in RAN#109 whether for network sided model spatial domain is supported or not, based on the final outcome of the SI. Note 2: Check in RAN#109 whether L3 beam level prediction is also supported, based on the final outcome of the SI. - Specify support for measurement event prediction for UE sided models [RAN2]: - Specify signaling and protocol aspect to enable reporting inference outcome of UE sided model for measurement event prediction - Measurement event A1~A6 are in the scope in this release - For both RRM measurement prediction and measurement event prediction, specify signalling and protocol aspects to enable LCM functionality management for UE sided model including [RAN2]: - UE capability request and response procedure - Applicability report for both full and partial configuration approach (considering further progress for AI/ML PHY) - Inference configuration - Performance monitoring performed in UE or network side, based on which network can manage UE sided functionality - UE sided data collection, for which AI/ML PHY request/configuration framework is taken as baseline Note 3: Data transfer and model delivery for UE sided model are out of scope of this WID. And model related choices can be up to UE’s implementation. - For RRM measurement prediction, specify signalling and protocol aspects to enable LCM functionality management for network sided model including [RAN2]: - Network sided data collection, at least - UE is configured to log RRM measurement results. - UE supports periodical logging and L3 event triggered logging. - UE can indicate logged data availability. Note 4: The above objectives to be based on the LCM framework for Rel-19 AI/ML for NR air interface WI as much as possible. - For both RRM measurement prediction and measurement event prediction [RAN4]: - Specify core requirements for the use cases and scenarios supported by the previous objectives - Specify LCM-related core requirements to support the LCM functionality management for both UE and network sided functionalities, if any Note 5: Can check in RAN#109 whether RAN2-related objectives need further updates, based on RAN2’s further discussions. Note 6: Can check in RAN#109 whether RAN4-related objectives need further updates, based on RAN4’s further discussions.

# Discussion

## Whether spatial domain is supported for network-sided model

The latest WID contains Note 1 regarding whether spatial domain is supported for network-sided model:

Note 1: For UE sided model, spatial domain is not supported. Can check in RAN#109 whether for network sided model spatial domain is supported or not, based on the final outcome of the SI.

RAN2 also discussed this aspect last August, and made several agreements on this:

Agreements (for NW-sided model for inference): 1 For cell-level temporal domain Case A, sub-case 2 the following enhancement is considered UE can report “cell-level RRM measurement results at multiple time instances in one measurement report. 2 For other cases there is no specification impact. Can be discussed in WI phase whether any additional enhancements are needed and justified (i.e. multi-instances reporting of beam) 3 Study item can conclude that all scenarios and sub-cases are feasible. Agreements (for NW-sided model for inference): Based on what is summarized above, we recommend RRM and measurement event prediction are recommended for normative work .And the following scenarios and/or sub-cases are recommended for normative work: - For UE sided model (RRM and measurement event prediction), intra-frequency temporal domain case A, intra-frequency temporal domain case B and inter-frequency domain prediction for co-located case, - For network sided model (RRM prediction), at least RRM sub-case 2 of intra-frequency temporal domain case A for inference input report and all scenarios and sub-cases for data collection.

As discussed in RAN2, we understand that network can perform spatial domain prediction without having specification impact, as UE can send L1/L3 measurement report for the serving and neighbour cells. Given the workload in RAN2, the scope should be narrowed down as much as possible.

Proposal 1	No standardization work is expected for the spatial domain use case for network sided model.

In addition, for the network sided model, RAN2 discussed RRM sub-cases 1–3, but both sub-cases 1 and 3 require substantial reporting overhead for the beam-level measurement report without any gain from the RAN2 evaluation. Hence, the WI should prioritize the sub-case 2.

Proposal 2	For network sided model (RRM prediction), RRM sub-case 2 of intra-frequency temporal domain case A is only supported.

## Whether L3 beam level prediction is also supported

The latest WID contains Note 2 regarding whether L3 beam level prediction is also supported:

Note 2: Check in RAN#109 whether L3 beam level prediction is also supported, based on the final outcome of the SI.

RAN2 also discussed this aspect last August, and made several agreements on this:

Agreements on L3 beam-level prediction - For RRM measurement prediction, L3 beam-level prediction is feasible for network sided model. CB after spec impact/overhead etc on the different cases. - For RRM measurement prediction, L3 beam-level prediction is feasible for UE sided model. However, there are concerns on complexity and other WGs.

We understand that the primary objective of L3 beam-level prediction is mainly to optimize dedicated resources, rather than mobility performance. Specifically, it aims to optimize dedicated RACH (CFRA) resource allocation during handovers by utilizing beam prediction, rather than allocating dedicated RACH resources for more beams. However, this "optimization" seems not essential as network can already allocate CFRA resource effectively based on the beam-level measurement report currently provided by the UE.

It should also be noted that for network-sided model, this requires additional signalling overhead in a consecutive manner. For UE-sided model, as discussed in RAN2, this requires additional RAN4 work which we may need to avoid. Hence, L3 beam level prediction should not be supported for both UE-sided and network-sided models.

Proposal 3:	L3 beam level prediction is not supported for both UE-sided and network-sided model.

## How to capture the proposals to the WID

Looking at the first objective, it can be divided into two separate objectives for UE-side model and network-sided model, respectively, to reflect the conclusions from RAN2 accurately. If proposals 1–3 above are agreeable, the first objective and Notes 1 & 2 can be revised as follows, and the existing Note 5 can be removed:

- Specify support for RRM measurement prediction for UE sided model [RAN2]: - Specify signalling and protocol aspect to enable reporting inference outcome of UE sided model for RRM measurement prediction - At least the use cases studied so far in RAN2 SI are supported: Temporal domain prediction case A and case B, and Frequency domain prediction for collocated cases. Cell level predication is supported Note 1: For UE sided model, spatial domain is not supported. Note 2: For UE sided model, L3 beam level prediction is not supported. - Specify support for RRM measurement prediction for network sided model [RAN2]: - Specify signalling and protocol aspect to enable reporting information from UE for inference operation of network sided model for RRM measurement prediction - The enhancement on RRM sub-case 2 of intra-frequency temporal domain case A is only supported. Note 3: Network sided model can support the spatial domain without requiring additional standardization. No standardization work is expected for the spatial domain use case. Note 4: For network sided model, L3 beam level prediction is not supported.

# Detailed objectives

Proposal 1	No standardization work is expected for the spatial domain use case for network sided model.

Proposal 2	For network sided model (RRM prediction), RRM sub-case 2 of intra-frequency temporal domain case A is only supported.

Proposal 3:	L3 beam level prediction is not supported for both UE-sided and network-sided model.

Proposal 4: Revise the latest WID as follows:

- Specify support for RRM measurement prediction for UE sided model [RAN2]: - Specify signalling and protocol aspect to enable reporting inference outcome of UE sided model for RRM measurement prediction - At least the use cases studied so far in RAN2 SI are supported: Temporal domain prediction case A and case B, and Frequency domain prediction for collocated cases. Cell level predication is supported Note 1: For UE sided model, spatial domain is not supported. Note 2: For UE sided model, L3 beam level prediction is not supported. - Specify support for RRM measurement prediction for network sided model [RAN2]: - Specify signalling and protocol aspect to enable reporting information from UE for inference operation of network sided model for RRM measurement prediction - The enhancement on RRM sub-case 2 of intra-frequency temporal domain case A is only supported. Note 3: Network sided model can support the spatial domain without requiring additional standardization. No standardization work is expected for the spatial domain use case. Note 4: For network sided model, L3 beam level prediction is not supported.

# Reference

RP-251864	New WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR	CATT (Moderator)