3GPP TSG-RAN5 Meeting #108	R5-255038

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-2 | CR | 1502 | rev | 1 | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of PICS for Further measurement gap enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MG_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to TS 36.306 and TS 36.331, below UE capabilities are needed for Further measurement gap enhancements WI: interRAT-NeedForGapsNR-r16 interRAT-NeedForInterruptionNR-r18 with value no-gap-with-interruption interRAT-NeedForInterruptionNR-r18 with value no-gap-no-interruption simultaneousRxDataSSB-DiffNumerology-FR1-r18 Therefore, the corresponding PICS items are needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Four new PICS were added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of PICS for Further measurement gap enhancements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is final version of R5-253753. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

## A.4.4	Additional information

Table A.4.4-1: Additional information

| Item | Additional information | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Support of USIM removal without power down |  | Rel-8 | pc_USIM_Removal |  |
| … |  |  |  |  |  |
| 267 | Support of flexible Tx-Rx frequency separation for NTN bands | 36.102, 5.4B.3 | Rel-18 | pc_NB_ntn_Flexible_TxRx_Separation |  |
| xx1 | Support of measurement gaps required to perform SSB based inter-RAT measurements on each supported NR band for each supported E-UTRA band or band combination | 36.306, 4.3.6.38 | Rel-16 | pc_interRAT_NeedForGapsNR_r16 |  |
| xx2 | Support of interruption required to perform SSB based inter-RAT measurements without gap but with interruption on each supported NR band for each supported E-UTRA band or band combination | 36.306, 4.3.6.53 | Rel-18 | pc_interRAT_NeedForInterruptionNR_NGWI_r18 | This PICS is set to True only if UE indicates measurement gap is not required (pc_interRAT_NeedForGapsNR = False). |
| xx3 | Support of interruption required to perform SSB based inter-RAT measurements without gap nor interruptionon on each supported NR band for each supported E-UTRA band or band combination | 36.306, 4.3.6.53 | Rel-18 | pc_interRAT_NeedForInterruptionNR_NGNI_r18 | This PICS is set to True only if UE indicates measurement gap is not required (pc_interRAT_NeedForGapsNR = False). |
| xx4 | Support of concurrent SSB-based inter-RAT measurement on NR FR1 cell and PDCCH or PDSCH reception from the serving cell with a different numerology | 36.306, 4.3.6.54 | Rel-18 | pc_simultaneousRxDataSSB_DiffNumerology_FR1_r18 | This PICS is set to True only if interRAT-NeedForInterruptionNR-r18 is reported for the target band in the band combination. (pc_interRAT_NeedForInterruptionNR_NGWI OR pc_interRAT_NeedForInterruptionNR_NGNI = True) |
| Note 1: A UE supporting this PICS shall set pc_StandaloneGNSS_Location to true. Note 2: A UE supporting this PICS shall set pc_NB_ntn_Connectivity_EPC to true. Note 3: A UE supporting this PICS shall set pc_ntn_Connectivity_EPC_CE_ModeA to true. |  |  |  |  |  |

[End of change]