3GPP TSG-RAN5 Meeting #108	R5-255314

# Bengaluru, India, Aug.25th – 29rd 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 4090 | rev | 1 | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating message contents and test procedures for FeMG test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MG_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - To Align with latest version of 38.133. - To add missing test procedures and message contents for Rel-18 MG enhancement TCs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Test parameters and requirements are updated. - Editor’s notes are updated. - Test procedures and message contents are added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work plan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.22.1, 6.6.22.2, 6.6.23.1, 6.6.23.2, 6.6.23.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from: R5-254561 |  |  |  |  |  |  |  |  |

<Start of Change 1>

6.6.22.1	NR SA FR1 and FR1-FR1 event-triggered reporting with concurrent Type-2 measurement gap and Pre-MG

Editor’s Notes: This test case is incomplete in following aspects:

-	TT analysis is still missing;

-	Applicability in TS 38.522 needs to be updated.

6.6.22.1.1	Test purpose

The purpose of this test is to verify that the concurrent gap with Pre-MG capable UE makes correct reporting of events. This test will partly verify the SA intra-frequency and inter-frequency NR cell search requirements in TS 38.133 [6] clauses 9.2.6, 9.3.4 and 9.3.5, respectively. Also, this test will also jointly verify pre-configured measurement gap autonomous activation/deactivation delay in TS 38.133 [6] clause 8.19.2.

6.6.22.1.2	Test applicability

This test applies to all types of NR UE release 18 and forward supporting concurrentMeasGapsPreMG-r18, BWP adaptation of at least 2 BWPs and BWP operation without bandwidth restriction.

6.6.22.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.22.0.

The normative reference for this requirement is TS 38.133 [6] clauses A.6.6.22.1.

6.6.22.1.4	Test description

6.6.22.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.22.1.4.1-1.

Table 6.6.22.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 6.6.22.1-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.22.1-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.22.1-3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. Note 2: Target NR cells have the same SCS, BW and duplex mode as NR serving cells |  |

Configure the test requirement and the DUT according to the parameters in Table 6.6.22.1.4.1-2.

Table 6.6.22.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.22.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. |  |  |

1. The general test parameter settings are set up according to Table 6.6.22.1.4.1-3.

2. Message contents are defined in clause 6.6.22.1.4.3.

3. The test scenario comprises of 2 NR carriers and 3 cells. NR Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 6.6.22.1.4.1-3: General test parameters

| Parameter | Unit | Test config | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 | Two FR1 NR carrier frequencies are used. |
| Active cell |  | Config 1,2,3 | NR Cell 1 (PCell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cells |  | Config 1,2,3 | NR Cells 2 and 3 | NR Cell 2 is on NR RF channel number 1. NR Cell 3 is on the NR RF channel 2. |
| Gap Pattern Id |  | Config 1,2,3 | 1 for MeasGapId #1 (80 ms MGRP) 0 for MeasGapId #2 (40 ms MGRP) | As specified in TS 38.133 [6] clause 9.1.2-1. |
| Measurement gap offset | ms | Config 1,2,3 | 79 for MeasGapId #1 4 for MeasGapId #2 |  |
| A3-Offset | dB | Config 1,2,3 | -6 |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |
| CP length |  | Config 1,2,3 | Normal |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |
| Filter coefficient |  | Config 1,2,3 | 0 | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF | DRX is not used |
| Time offset between serving and neighbour cell |  | Config 1,2,3 | 3 s | The timing of Cell 2 is 3s later than the timing of Cell 1. |
|  |  | Config 1,2,3 | 5 ms | The timing of Cell 3 is 5 ms later than the timing of Cell 1. |
| T1 | s | Config 1,2,3 | 2 |  |
| T2 | s | Config 1,2,3 | 2 |  |

6.6.22.1.4.2	Test procedure

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. SS shall transmit an RRCReconfiguration message to provide measurement configuration, BWP configurations for two dedicated BWP (BWP-1 and BWP-2) and firstActiveDownlinkBWP-Id indicating BWP-1 as the first active BWP.

3. The UE shall transmit an RRCReconfigurationComplete message.

4. The SS starts to continuously schedule DL reception on Cell 1.

5. The SS sets the parameters according to T1 in Table 6.6.22.1.5-1. T1 starts.

6. If pc_dynamicCollision_r18 = true,

-	the UE shall transmit an MeasurementReport message triggered by event A3 for Cell 3 and the overall reporting delay measured from the beginning of time period T1 is less than 902 ms, and

-	The UE shall report corresponding valid ACK/NACK for every PDSCHs scheduled in the slots that are not overlapped with the gap occasions associated to MeasGapId#2.

Otherwise,

-	the UE shall transmit a MeasurementReport message triggered by event A3 for Cell 3 and the overall reporting delay measured from the beginning of time period T1 is less than 1282 ms, and

-	The UE shall report corresponding valid ACK/NACK for every PDSCHs scheduled in the slots that are not overlapped with the non-dropped gap occasions associated to MeasGapId#2.

Then the number of successful tests is increased by one. If the UE fails to report the event within the overall delay requirement or fails to send valid ACK/NACK in the required slots, then the number of failure tests is increased by one.

7. After the SS receive the MeasurementReport message in step 6) or when T2 expires, the SS shall set the parameters according to T2 in Table 6.6.22.1.5-1.

8. The SS sends a DCI format 1_1 to trigger BWP switching from BWP-1 to BWP 2. T2 starts.

9. The UE shall,

-	transmit a MeasurementReport message triggered by event A3 for Cell 2 and the overall reporting delay measured from the beginning of time period T2 is less than 1082 ms, and

-	The UE shall report ACK/NACK for every PDSCHs scheduled in the slots that are not overlapped with the gap occasions associated to MeasGapId #1 or non-dropped gap occasions associated to MeasGapId #2 after MeasGapId #1 is activated, i.e. starting from the first complete gap occasion associated to MeasGapId #1 after the beginning of PCell’s DL slot i+TBWPswitchDelay+5NR slot length, where slot #i is the slot in which the DCI triggering BWP switching is received.

Then the number of successful tests is increased by one. If the UE fails to report the event within the overall delay requirement or fails to send valid ACK/NACK in the required slots, then the number of failure tests is increased by one.

10. After the SS receive the MeasurementReport message in step 9) or when T2 expires, the SS shall

-	sends an RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources, or

-	switch the UE off.

11. Set NR Cell 2 physical cell identity = ((current NR Cell 2 physical cell identity + 3) mod 1008) and NR Cell 3 physical cell identity = ((current NR Cell 3 physical cell identity + 3) mod 1008) for next iteration of the test procedure loop.

12. Depending on the choice in Step 10, the SS:

-	if the RRCRelease message has been sent, transmits in NR Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5), or

-	if the UE has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

13. Repeat step 2-12 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.22.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.6.22.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-4 with A3-offset = -4.5dB |

Table 6.6.22.1.4.3-2: MeasConfig (Step 2, test procedure)

| Derivation path: Table H.3.1-2 with condition INTER-FREQ and GAP NEEDED |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { | 2 entries |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-DEFAULT specified in Table H.3.1-3 with condition INTRA-FREQ MO except that associatedMeasGapSSB-r17 = 1 | MO for intra-frequency carrier is associated to MeasGapId #1 |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod[2] SEQUENCE { |  | entry 2 |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-DEFAULT specified in Table H.3.1-3 with condition INTER-FREQ MO except that associatedMeasGapSSB-r17 = 2 | MO for inter-frequency carrier is associated to MeasGapId #2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 2 entries |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 1 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| MeasIdToAddMod[2] SEQUENCE { |  | entry 2 |  |
| measId | 2 |  |  |
| measObjectId | 2 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| measGapConfig | MeasGapConfig-ConPreMG | Table 6.6.22.1.4.3-3 |  |
| } |  |  |  |

Table 6.6.22.1.4.3-3: MeasGapConfig-ConPreMG (Table 6.6.22.1.4.3-2)

| Derivation Path: Table H.3.1-6 with condition MGe |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasGapConfig ::= SEQUENCE { |  |  |  |
| gapToAddModList-r17 SEQUENCE (SIZE (1..maxNrofGapId-r17)) OF GapConfig-r17 SEQUENCE{ | 2 entries |  |  |
| GapConfig-r17[1] SEQUENCE{ |  | entry 1 |  |
| measGapId-r17 | 1 |  |  |
| gapType-r17 | perUE |  |  |
| gapOffset-r17 | 79 |  |  |
| mgl-r17 | ms6 |  |  |
| mgrp-r17 | ms80 | gap pattern 1 |  |
| mgta-r17 | ms0 |  |  |
| refServCellIndicator-r17 | Not present |  |  |
| refFR2-ServCellAsyncCA-r17 | Not present |  |  |
| preConfigInd-r17 | true | Pre-MG |  |
| ncsgInd-r17 | Not present |  |  |
| gapAssociationPRS-r17 | Not present |  |  |
| gapSharing-r17 | Not present |  |  |
| gapPriority-r17 | 1 | High priority |  |
| } |  |  |  |
| GapConfig-r17[2] SEQUENCE{ |  | entry 2 |  |
| measGapId-r17 | 2 |  |  |
| gapType-r17 | perUE |  |  |
| gapOffset-r17 | 4 |  |  |
| mgl-r17 | ms6 |  |  |
| mgrp-r17 | ms40 | gap pattern 0 |  |
| mgta-r17 | ms0 |  |  |
| refServCellIndicator-r17 | Not present |  |  |
| refFR2-ServCellAsyncCA-r17 | Not present |  |  |
| preConfigInd-r17 | Not present | Type-2 MG |  |
| ncsgInd-r17 | Not present |  |  |
| gapAssociationPRS-r17 | Not present |  |  |
| gapSharing-r17 | Not present |  |  |
| gapPriority-r17 | 2 | Low priority |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.6.22.1.4.3-4: ServingCellConfig (Step 2, Test procedure)

| Derivation Path: TS 38.508-1 [14] Table 4.6.3-167 with condition MEAS, DLBWP_add and ULBWP_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| downlinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { | 2 entries |  |  |
| BWP-Downlink[1] SEQUENCE { | BWP-Downlink specified in TS 38.508-1 [14] Table 4.6.3-9 except that: - bwp-Id = 1, - locationAndBandwidth is set according to DLBWP.1.3 | entry 1 BWP-1 |  |
| BWP-Downlink[2] SEQUENCE { | BWP-Downlink specified in TS 38.508-1 [14] Table 4.6.3-9 except that: - bwp-Id = 2, - locationAndBandwidth is set according to DLBWP.1.2 | entry 2 BWP-2 |  |
| } |  |  |  |
| firstActiveDownlinkBWP-Id | 1 | BWP-1 is activated at first |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP | Not present |  |  |
| uplinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { | 2 entries |  |  |
| BWP-Uplink[1] | BWP-Uplink specified in TS 38.508-1 [14] Table 4.6.3-13 except that: - bwp-Id = 1, - locationAndBandwidth is set according to ULBWP.1.3 | entry 1 BWP-1 |  |
| BWP-Uplink[2] | BWP-Uplink specified in TS 38.508-1 [14] Table 4.6.3-13 except that: - bwp-Id = 2, - locationAndBandwidth is set according to ULBWP.1.2 | entry 2 BWP-2 |  |
| } |  |  |  |
| firstActiveUplinkBWP-Id | 1 | BWP-1 is activated at first |  |
| } |  |  |  |
| } |  |  |  |

Table 6.6.22.1.4.3-5: MeasResults (Step 6 and Step 9, test procedure)

| Derivation Path: Table H.3.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| measResults SEQUENCE { |  |  |  |
| measId | 2 |  | Step 6 |
|  | 1 |  | Step 9 |
| measResultServingMOList | Not Checked |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListNR SEQUENCE (SIZE(1..maxCellReport)) OF MeasResultNR { | 1 entry |  |  |
| MeasResultNR[1] { |  | entry 1 |  |
| physCellId | PhysCellId of NR neighbour Cell 3 |  | Step 6 |
|  | PhysCellId of NR neighbour Cell 2 |  | Step 9 |
| measResult SEQUENCE { |  |  |  |
| cellResults | Checked |  |  |
| rsIndexResults | Not Checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.6.22.1.5	Test requirements

Table 6.6.22.1.4.1-3 and Table 6.6.22.1.5-1 define the primary level settings including test tolerances for the test.

Table 6.6.22.1.5-1: NR Cell specific test parameters

| Parameter | Unit | Test | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Config. | T1 | T2 | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  | 1,2,3 | 1 |  | 1 |  | 2 |  |
| Duplex mode |  | 1 | FDD |  |  |  |  |  |
|  |  | 2,3 | TDD |  |  |  |  |  |
| TDD configuration |  | 1 | N/A |  |  |  |  |  |
|  |  | 2 | TDDConf.1.1 |  |  |  |  |  |
|  |  | 3 | TDDConf.2.1 |  |  |  |  |  |
| BWchannel | MHz | 1,2 | 10: NPRB,c = 52 |  |  |  |  |  |
|  |  | 3 | 40: NPRB,c = 106 |  |  |  |  |  |
| BWP BW | MHz | 1,2 | 10: NPRB,c = 52 |  |  |  |  |  |
|  |  | 3 | 40: NPRB,c = 106 |  |  |  |  |  |
| Initial BWP Configuration |  | 1,2,3 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  | N/A |  |
| BWP-1 Configuration |  | 1,2,3 | DLBWP.1.3 ULBWP.1.3 |  | N/A |  | N/A |  |
| BWP-2 Configuration |  | 1,2,3 | DLBWP.1.2 ULBWP.1.2 |  | N/A |  | N/A |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  | N/A |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  | N/A |  | N/A |  |
|  |  | 3 | TRS.1.2 TDD |  | N/A |  | N/A |  |
| OCNG Patterns |  | 1,2,3 | OP.1 |  | OP.1 |  | OP.1 |  |
| PDSCH RMC |  | 1 | SR.1.1 FDD |  | N/A |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  | N/A |  | N/A |  |
|  |  | 3 | SR.2.1 TDD |  | N/A |  | N/A |  |
| RMSI CORESET RMC |  | 1 | CR.1.1 FDD |  | N/A |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  | N/A |  | N/A |  |
|  |  | 3 | CR.2.1 TDD |  | N/A |  | N/A |  |
| Dedicated CORESET RMC |  | 1 | CCR.1.1 FDD |  | N/A |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  | N/A |  | N/A |  |
|  |  | 3 | CCR.2.1 TDD |  | N/A |  | N/A |  |
| SSB parameters |  | 1,2 | SSB.1 FR1 |  |  |  |  |  |
|  |  | 3 | SSB.2 FR1 |  |  |  |  |  |
| SMTC configuration |  | 1,2,3 | SMTC.2 |  | SMTC.2 |  | SMTC.7 |  |
| PDSCH/PDCCH subcarrier | kHz | 1,2 | 15 |  |  |  |  |  |
| spacing |  | 3 | 30 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | 1,2,3 | 0 |  | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |  |
| NocNote2 | dBm/ 15 kHz |  | -98 |  | -98 |  | -98 |  |
| NocNote2 | dBm/SCS | 1,2 | -98 |  | -98 |  | -98 |  |
|  |  | 3 | -95 |  | -95 |  | -95 |  |
| SS-RSRP Note 3 | dBm/SCS | 1,2 | -94+TT | -94+TT | -Infinity | -94+TT | -91+TT | -Infinity |
|  |  | 3 | -91+TT | -91+TT | -Infinity | -91+TT | -88+TT | -Infinity |
| EsIot | dB | 1,2,3 | 4+TT | -1.46+TT | -Infinity | -1.46+TT | 7+TT | -Infinity |
| EsNoc | dB | 1,2,3 | 4+TT | 4+TT | -Infinity | 4+TT | 7+TT | -Infinity |
| IoNote3 | dBm/9.36 MHz | 1,2 | -64.59 +TT | -62.25 +TT | -64.59 +TT | -62.26 +TT | -62.26 +TT | -70.05 +TT |
|  | dBm/38.16 MHz | 3 | -58.49 +TT | -56.16 +TT | -58.49 +TT | -56.15 +TT | -56.15 +TT | -63.94 +TT |
| Propagation Condition |  | 1,2,3 | AWGN |  | AWGN |  | AWGN |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |

During T1,

-	If pc_dynamicCollision_r18 = true,

-	The UE shall report corresponding valid ACK/NACK for those PDSCHs scheduled in the slots that are not overlapped with the non-dropped MeasGapId #2 occasions;

-	The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 902 ms for NR Cell 3 from the beginning of time period T1.

-	If pc_dynamicCollision_r18 = false

-	The UE shall report corresponding valid ACK/NACK for those PDSCHs scheduled in the slots that are not overlapped with the non-dropped MeasGapId #2 occasions.

-	The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1282 ms for NR Cell 3 from the beginning of time period T1.

During T2,

-	The UE shall report ACK/NACK for PDSCHs scheduled in the slots that are not overlapped with the MeasGapId #1 occasions or non-dropped MeasGapId #2 occasions after MeasGapId #1 is activated, i.e. starting from the first complete MeasGapId #1 occasion after the beginning of PCell’s DL slot i+TBWPswitchDelay+5NR slot length

-	The UE shall send one Event A3 triggered measurement report for each neighboring cell, with a measurement reporting delay less than 1082 ms for Cell 2 from the beginning of time period T2. The measurement reporting delay is derived based on the requirements for intra-frequency measurement in TS 38.133 [6] clause 9.2.6 plus 80 ms, considering that the frist MeasGapId #1 occasion in T2 may collide with the pre-configured gap activation delay.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%. UE is not required to report SSB time index.

NOTE 1:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

6.6.22.2	NR SA FR1 event-triggered reporting with two concurrent Pre-MGs

Editor’s Notes: This test case is incomplete in following aspects:

-	TT analysis is still missing;

-	Applicability in TS 38.522 needs to be updated.

6.6.22.2.1	Test purpose

The purpose of this test is to verify that the UE correctly activates and deactivates the pre-MGs and makes correct measurement and reporting of an event with activated and deactivated pre-MG. This test will partly verify the multiple Pre-MG activation/deactivation delay in TS38.133 [6] clause 8.19.5.2, the intra-frequency cell search requirements in TS38.133 [6] clause 9.2.6 and inter-frequency cell search requirements in TS38.133 [6] clause 9.3.4 and 9.3.5.

6.6.22.2.2	Test applicability

This test applies to all types of NR UE release 18 and forward supporting concurrentMeasGapsPreMG-r18 and BWP adaptation of at least 2 BWPs.

6.6.22.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.22.0.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.22.2.

6.6.22.2.4	Test description

6.6.22.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.22.2.4.1-1.

Table 6.6.22.2.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 6.6.22.2-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.22.2-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.22.2-3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test requirement and the DUT according to the parameters in Table 6.6.22.2.4.1-2.

Table 6.6.22.2.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.22.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. |  |  |

1. The test parameters for PCell and neighbour cell are given in Table 6.6.22.2.4.1-3.

2. Message contents are defined in clause 6.6.22.2.4.3.

3. The test scenario comprises of 2 NR carriers and 3 cells. NR Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 6.6.22.2.4.1-3: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1,2,3 | Cell 1 |  |
| Neighbour cell |  | 1,2,3 | Cell 2, Cell 3 | Cell to be identified. |
| RF Channel Number |  | 1,2,3 | 1: Cell 1 and Cell 2, 2: Cell 3 |  |
| Pre-MG Gap Pattern Id |  | 1,2,3 | 0 for MeasGapId #1 1 for MeasGapId #2 |  |
| Measurement gap offset |  | 1,2,3 | 39 for MeasGapId #1 19 for MeasGapId #2 |  |
| SSB configuration |  | 1,2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 | Same configuration for MO1 and MO2 |
|  |  | 2,3 | SMTC.1 | Same configuration for MO1 and MO2 |
| CSI-RS parameters |  | 1 | CSI-RS.1.2 FDD resource #0 |  |
|  |  | 2 | CSI-RS.1.2 TDD resource #0 |  |
|  |  | 3 | CSI-RS.2.2 TDD resource #0 |  |
| A3-Offset | dB | 1,2,3 | -4.5 |  |
| CP length |  | 1,2,3 | Normal |  |
| Hysteresis | dB | 1,2,3 | 0 |  |
| Time To Trigger | s | 1,2,3 | 0 |  |
| Filter coefficient |  | 1,2,3 | 0 | L3 filtering is not used |
| DRX |  | 1,2,3 | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 2 and Cell 3 is 3 ms later than the timing of Cell 1. |
|  |  | 2,3 | 3 s | Synchronous cells |
| T1 | s | 1,2,3 | 5 |  |
| T2 | s | 1,2,3 | 0.1 |  |
| T3 | s | 1,2,3 | 5 |  |

6.6.22.2.4.2	Test procedure

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. SS shall transmit an RRCReconfiguration message to provide measurement configuration, BWP configurations for two dedicated BWP (BWP-1 and BWP-2) and firstActiveDownlinkBWP-Id indicating BWP-1 as the first active BWP.

3. The UE shall transmit an RRCReconfigurationComplete message.

4. The SS starts to continuously schedule DL reception on Cell 1.

5. The SS sets the parameters according to T1 in Table 6.6.22.2.5-1. T1 starts.

6. The UE shall

-	transmit a MeasurementReport message triggered by event A3 for Cell 3 and the overall reporting delay measured from the beginning of time period T1 is less than 1282 ms, and

-	The UE shall report corresponding valid ACK/NACK for every PDSCHs scheduled in the slots that are not overlapped with the Pre-MG occasions associated to MeasGapId#2.

Then the number of successful tests is increased by one. If the UE fails to report the event within the overall delay requirement or fails to send valid ACK/NACK in the required slots, then the number of failure tests is increased by one.

7. After the SS receive the MeasurementReport message in step 6) or when T1 expires, the SS shall set the parameters according to T2 in Table 6.6.22.2.5-1.

8. The SS sends a DCI format 1_1 to trigger BWP switching from BWP-1 to BWP 2. T2 starts.

9. The UE shall report ACK/NACK for every PDSCHs scheduled in the slots that are not overlapped with the Pre-MG occasions associated to MeasGapId #1 after MeasGapId #1 is activated, i.e. starting from the first complete gap occasion associated to MeasGapId #1 after the beginning of PCell’s DL slot i+TBWPswitchDelay+7NR slot length, where slot #i is the slot in which the DCI triggering BWP switching in step 8 is received.

Then the number of successful tests is increased by one. If the UE fails to send valid ACK/NACK in the required slots, then the number of failure tests is increased by one.

10. When T2 expires, the SS shall set the parameters according to T3 in Table 6.6.22.2.5-1. T3 starts.

11. The UE shall,

-	transmit a MeasurementReport message triggered by event A3 for Cell 2 and the overall reporting delay measured from the beginning of time period T2 is less than 802 ms, and

-	The UE shall report corresponding valid ACK/NACK for every PDSCHs scheduled in the slots that are not overlapped with the Pre-MG occasions associated to MeasGapId#1.

Then the number of successful tests is increased by one. If the UE fails to report the event within the overall delay requirement or fails to send valid ACK/NACK in the required slots, then the number of failure tests is increased by one.

12. After the SS receive the MeasurementReport message in step 11) or when T3 expires, the SS shall

-	sends an RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources, or

-	switch the UE off.

13. Set NR Cell 2 physical cell identity = ((current NR Cell 2 physical cell identity + 3) mod 1008) and NR Cell 3 physical cell identity = ((current NR Cell 3 physical cell identity + 3) mod 1008) for next iteration of the test procedure loop.

14. Depending on the choice in Step 12, the SS:

-	if the RRCRelease message has been sent, transmits in NR Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5), or

-	if the UE has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

15. Repeat step 2-14 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.22.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.6.22.2.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-4 with A3-offset = -4.5dB |

Table 6.6.22.2.4.3-2: MeasConfig (Step 2, test procedure)

| Derivation path: Table H.3.1-2 with condition INTER-FREQ and GAP NEEDED |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { | 2 entries |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-DEFAULT specified in Table H.3.1-3 with condition INTRA-FREQ MO except that associatedMeasGapSSB-r17 = 1 | MO for intra-frequency carrier is associated to MeasGapId #1 |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod[2] SEQUENCE { |  | entry 2 |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-DEFAULT specified in Table H.3.1-3 with condition INTER-FREQ MO except that associatedMeasGapSSB-r17 = 2 | MO for inter-frequency carrier is associated to MeasGapId #2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 2 entries |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 1 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| MeasIdToAddMod[2] SEQUENCE { |  | entry 2 |  |
| measId | 2 |  |  |
| measObjectId | 2 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| measGapConfig | MeasGapConfig-ConPreMG | Table 6.6.22.2.4.3-3 |  |
| } |  |  |  |

Table 6.6.22.2.4.3-3: MeasGapConfig-ConPreMG (Table 6.6.22.2.4.3-2)

| Derivation Path: Table H.3.1-6 with condition MGe |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasGapConfig ::= SEQUENCE { |  |  |  |
| gapToAddModList-r17 SEQUENCE (SIZE (1..maxNrofGapId-r17)) OF GapConfig-r17 SEQUENCE{ | 2 entries |  |  |
| GapConfig-r17[1] SEQUENCE{ |  | entry 1 |  |
| measGapId-r17 | 1 |  |  |
| gapType-r17 | perUE |  |  |
| gapOffset-r17 | 39 |  |  |
| mgl-r17 | ms6 |  |  |
| mgrp-r17 | ms40 | gap pattern 0 |  |
| mgta-r17 | ms0 |  |  |
| refServCellIndicator-r17 | Not present |  |  |
| refFR2-ServCellAsyncCA-r17 | Not present |  |  |
| preConfigInd-r17 | true | Pre-MG |  |
| ncsgInd-r17 | Not present |  |  |
| gapAssociationPRS-r17 | Not present |  |  |
| gapSharing-r17 | Not present |  |  |
| gapPriority-r17 | 2 | Low priority |  |
| } |  |  |  |
| GapConfig-r17[2] SEQUENCE{ |  | entry 2 |  |
| measGapId-r17 | 2 |  |  |
| gapType-r17 | perUE |  |  |
| gapOffset-r17 | 19 |  |  |
| mgl-r17 | ms6 |  |  |
| mgrp-r17 | ms80 | gap pattern 1 |  |
| mgta-r17 | ms0 |  |  |
| refServCellIndicator-r17 | Not present |  |  |
| refFR2-ServCellAsyncCA-r17 | Not present |  |  |
| preConfigInd-r17 | true | Pre-MG |  |
| ncsgInd-r17 | Not present |  |  |
| gapAssociationPRS-r17 | Not present |  |  |
| gapSharing-r17 | Not present |  |  |
| gapPriority-r17 | 1 | High priority |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.6.22.2.4.3-4: ServingCellConfig (Step 2, Test procedure)

| Derivation Path: TS 38.508-1 [14] Table 4.6.3-167 with condition MEAS, DLBWP_add and ULBWP_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| downlinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { | 2 entries |  |  |
| BWP-Downlink[1] SEQUENCE { | BWP-Downlink specified in TS 38.508-1 [14] Table 4.6.3-9 except that: - bwp-Id = 1, - locationAndBandwidth is set according to DLBWP.1.3 - preConfGapStatus = “01” | entry 1 BWP-1 |  |
| BWP-Downlink[2] SEQUENCE { | BWP-Downlink specified in TS 38.508-1 [14] Table 4.6.3-9 except that: - bwp-Id = 2, - locationAndBandwidth is set according to DLBWP.1.2 - preConfGapStatus = “10” | entry 2 BWP-2 |  |
| } |  |  |  |
| firstActiveDownlinkBWP-Id | 1 | BWP-1 is activated at first |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP | Not present |  |  |
| uplinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { | 2 entries |  |  |
| BWP-Uplink[1] | BWP-Uplink specified in TS 38.508-1 [14] Table 4.6.3-13 except that: - bwp-Id = 1, - locationAndBandwidth is set according to ULBWP.1.3 | entry 1 BWP-1 |  |
| BWP-Uplink[2] | BWP-Uplink specified in TS 38.508-1 [14] Table 4.6.3-13 except that: - bwp-Id = 2, - locationAndBandwidth is set according to ULBWP.1.2 | entry 2 BWP-2 |  |
| } |  |  |  |
| firstActiveUplinkBWP-Id | 1 | BWP-1 is activated at first |  |
| } |  |  |  |
| } |  |  |  |

Table 6.6.22.2.4.3-5: MeasResults (Step 6 and Step 11, test procedure)

| Derivation Path: Table H.3.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| measResults SEQUENCE { |  |  |  |
| measId | 2 |  | Step 6 |
|  | 1 |  | Step 11 |
| measResultServingMOList | Not Checked |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListNR SEQUENCE (SIZE(1..maxCellReport)) OF MeasResultNR { | 1 entry |  |  |
| MeasResultNR[1] { |  | entry 1 |  |
| physCellId | PhysCellId of NR neighbour Cell 3 |  | Step 6 |
|  | PhysCellId of NR neighbour Cell 2 |  | Step 11 |
| measResult SEQUENCE { |  |  |  |
| cellResults | Checked |  |  |
| rsIndexResults | Not Checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.6.22.2.5	Test requirement

Table 6.6.22.2.4.1-3 and Table 6.6.22.2.5-1 define the primary level settings including test tolerances for the test.

Table 6.6.22.2.5-1: NR Cell specific test parameters

| Parameter | Unit | Test | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | config. | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| TDD configuration |  | 1 | N/A |  |  | N/A |  |  | N/A |  |  |
|  |  | 2 | TDDConf.1.1 |  |  | TDDConf.1.1 |  |  | TDDConf.1.1 |  |  |
|  |  | 3 | TDDConf.2.1 |  |  | TDDConf.2.1 |  |  | TDDConf.2.1 |  |  |
| PDSCH RMC |  | 1 | SR.1.1 FDD |  |  |  |  |  |  |  |  |
|  |  | 2 | SR.1.1 TDD |  |  | N/A |  |  | N/A |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |  |  |  |  |  |
| RMSI CORESET |  | 1 | CR.1.1 FDD |  |  | N/A |  |  | N/A |  |  |
| RMC |  | 2 | CR.1.1 TDD |  |  | N/A |  |  | N/A |  |  |
|  |  | 3 | CR.2.1 TDD |  |  | N/A |  |  | N/A |  |  |
| Dedicated |  | 1 | CCR.1.2 FDD |  |  | N/A |  |  | N/A |  |  |
| CORESET RMC |  | 2 | CCR.1.2 TDD |  |  | N/A |  |  | N/A |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  | N/A |  |  | N/A |  |  |
| OCNG Patterns |  | 1,2,3 | OP.1 |  |  | OP.1 |  |  | OP.1 |  |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  |  | N/A |  |  | N/A |  |  |
|  |  | 2 | TRS.1.1 TDD |  |  | N/A |  |  | N/A |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  | N/A |  |  | N/A |  |  |
| Initial BWP configuration |  | 1,2,3 | DLBWP.0.1 ULBWP.0.1 |  |  | N/A |  |  | N/A |  |  |
| BWP-1 Configuration |  | 1,2,3 | DLBWP.1.3 ULBWP.1.3 |  |  | N/A |  |  | N/A |  |  |
| BWP-2 Configuration |  | 1,2,3 | DLBWP.1.2 ULBWP.1.2 |  |  | N/A |  |  | N/A |  |  |
| RLM-RS |  | 1,2,3 | CSI-RS |  |  | N/A |  |  | N/A |  |  |
| NocNote 2 | dBm/15 kHz | 1,2,3 | -98+TT |  |  | -98+TT |  |  | -98+TT |  |  |
| NocNote 2 | dBm/SCS | 1 | -98+TT |  |  | -98+TT |  |  | -98+TT |  |  |
|  |  | 2 | -98+TT |  |  | -98+TT |  |  | -98+TT |  |  |
|  |  | 3 | -95+TT |  |  | -95+TT |  |  | -95+TT |  |  |
| EsIot | dB | 1,2,3 | 4 +TT | 4 +TT | -1.46 +TT | -∞ | -∞ | -1.46 +TT | 4 +TT | -∞ | -∞ |
| EsNoc | dB | 1,2,3 | 4 +TT | 4 +TT | 4 +TT | -∞ | -∞ | 4 +TT | 4 +TT | -∞ | -∞ |
| SS-RSRP Note 3 | dBm/SCS | 1,2 | -94 +TT | -94 +TT | -94 +TT | -∞ | -∞ | -94 +TT | -94 +TT | -∞ | -∞ |
|  |  | 3 | -91 +TT | -91 +TT | -91 +TT | -∞ | -∞ | -91 +TT | -91 +TT | -∞ | -∞ |
| Io | dBm/9.36 MHz | 1,2 | -64.60 +TT | -64.60 +TT | -62.25 +TT | -∞ | -∞ | -62.25 +TT | -64.60 +TT | -∞ | -∞ |
|  | dBm/38.16 MHz | 3 | -58.50 +TT | -58.50 +TT | -56.16 +TT | -∞ | -∞ | -56.16 +TT | -58.50 +TT | -∞ | -∞ |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |  |  |  |  |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T1 and T3. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. NOTE 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |  |

During T1, UE shall report corresponding HARQ-ACK/NACK for those PDSCHs scheduled in the slots that are not overlapped with the pre-MG occasions of MeasGapId #2; and

The UE shall send one Event A3 triggered measurement report of NR Cell 3 on RF channel 2, with a measurement reporting delay less than 1282 ms from the beginning of time period T1.

During T2 and starting from the first complete Pre-MG occasion after the beginning of PCell’s DL slot i+TBWPswitchDelay+7NR slot length as defined in clause 8.19.5.2, the UE shall report corresponding HARQ-ACK/NACK for those PDSCHs scheduled in the slots that are not overlapped with the pre-MG occasions of MeasGapId #1.

The UE shall send one Event A3 triggered measurement report of NR Cell 2 on RF channel 1, with a measurement reporting delay less than 802 ms from the beginning of time period T3.

During T3, the UE shall report corresponding HARQ-ACK/NACK for those PDSCHs scheduled in the slots that are not overlapped with the pre-MG occasions of MeasGapId #1.

The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

<End of Change 1>

<Start of Change 2>

6.6.23.1	NR SA FR1-FR1 event-triggered reporting with concurrent Type-2 measurement gap and NCSG

Editor’s Notes: This test case is incomplete in following aspects:

-	TT analysis is still missing;

6.6.23.1.1	Test purpose

The purpose of this test is to verify that the concurrent gaps with NCSG capable UE makes correct reporting of events. This test will partly verify the SA inter-frequency NR cell search requirements in TS 38.133 [6] clause 9.3.10.

6.6.23.1.2	Test applicability

This test applies to all types of NR UE release 18 and forward supporting concurrentMeasGapsNCSG-r18.

6.6.23.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.23.0.

The normative reference for this requirement is TS 38.133 [6] clauses A.6.6.23.1.

6.6.23.1.4	Test description

6.6.23.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.23.1.4.1-1.

Table 6.6.23.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 6.6.23.1-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.23.1-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.23.1-3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations NOTE 2: Target NR cells have the same SCS, BW and duplex mode as NR serving cells |  |

Configure the test requirement and the DUT according to the parameters in Table 6.6.23.1.4.1-2.

Table 6.6.23.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.23.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. |  |  |

1. The general test parameter settings are set up according to Table 6.6.23.1.4.1-3.

2. Message contents are defined in clause 6.6.23.1.4.3.

3. The test scenario comprises of 2 NR carriers and 3 cells. NR Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 6.6.23.1.4.1-3: General test parameters

| Parameter | Unit | Test config. | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | 1,2,3 | 1,2 | Two FR1 NR carrier frequencies are used. |
| Active cell |  | 1,2,3 | NR Cell 1 (PCell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | 1,2,3 | NR Cell 2 and 3 | NR Cell 2 is on NR RF channel number 2. NR Cell 3 is on NR RF channel number 3. |
| Measurement Gap Pattern Id |  | 1,2,3 | 0 | As specified in TS 38.133 [6] Table 9.1.2-1. |
| Measurement Gap offset |  | 1,2,3 | 39 |  |
| NCSG Pattern Id |  | 1,2,3 | 1 | As specified in TS 38.133 [6] Table 9.1.9.3-1. |
| NCSG offset |  | 1,2,3 | 4 |  |
| A3-Offset | dB | 1,2,3 | -6 |  |
| Hysteresis | dB | 1,2,3 | 0 |  |
| CP length |  | 1,2,3 | Normal |  |
| TimeToTrigger | s | 1,2,3 | 0 |  |
| Filter coefficient |  | 1,2,3 | 0 | L3 filtering is not used |
| DRX |  | 1,2,3 | OFF | DRX is not used |
| Time offset between Cell 1 and Cell 2 |  | 1,2,3 | 3s | Synchronous. The timing of Cell 2 is 3 s later than the timing of Cell 1. |
| Time offset between Cell 1 and Cell 3 |  | 1,2,3 | 5 ms | Asynchronous. The timing of Cell 3 is 5 ms later than the timing of Cell 1. |
| T1 | s | 1,2,3 | 5 |  |
| T2 | s | 1,2,3 | 1.5 |  |

6.6.23.1.4.2	Test procedure

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. The SS sets the parameters according to T1 in Table 6.6.23.1.5-1. T1 starts.

3. The SS starts to continuously schedule DL reception on Cell 1.

4. SS shall transmit an RRCReconfiguration message to provide measurement configuration.

5. The UE shall transmit an RRCReconfigurationComplete message.

6. The SS shall set the parameters according to T2 in Table 6.6.23.1.5-1 when T1 expires. T2 starts.

7. The UE shall,

-	transmit a MeasurementReport message triggered by event A3 for Cell 2 and the overall reporting delay measured from the beginning of time period T2 is less than 1282 ms, and

-	transmit a MeasurementReport message triggered by event A3 for Cell 3 and the overall reporting delay measured from the beginning of time period T2 is less than 1282 ms, and

-	The UE shall report ACK/NACK for every PDSCHs scheduled in the slots that are not overlapped with the VIL of NCSG occasions associated to MeasGapId #2 or non-dropped gap occasions associated to MeasGapId #1.

Then the number of successful tests is increased by one. If the UE fails to report the event within the overall delay requirement or fails to send valid ACK/NACK in the required slots, then the number of failure tests is increased by one.

8. After the SS receive the MeasurementReport messages in step 7) or when T2 expires, the SS shall

-	sends an RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources, or

-	switch the UE off.

9. Set NR Cell 2 physical cell identity = ((current NR Cell 2 physical cell identity + 3) mod 1008) and NR Cell 3 physical cell identity = ((current NR Cell 3 physical cell identity + 3) mod 1008) for next iteration of the test procedure loop.

10. Depending on the choice in Step 8, the SS:

-	if the RRCRelease message has been sent, transmits in NR Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5), or

-	if the UE has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

11. Repeat step 2-10 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.23.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.6.22.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-4 with A3-offset = -6dB |

Table 6.6.23.1.4.3-2: MeasConfig (Step 4, test procedure)

| Derivation path: Table H.3.1-2 with condition INTER-FREQ and GAP NEEDED |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { | 3 entries |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-DEFAULT specified in Table H.3.1-3 with condition INTRA-FREQ MO | MO for Cell 1 |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod[2] SEQUENCE { |  | entry 2 |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-DEFAULT specified in Table H.3.1-3 with condition INTER-FREQ MO except that - ssbFrequency is set as the ARFCN-ValueNR of the SSB associated to Cell 2 - associatedMeasGapSSB-r17 = 1 | MO for Cell 2 is associated to MeasGapId #1 |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod[3] SEQUENCE { |  | entry 3 |  |
| measObjectId | 3 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-DEFAULT specified in Table H.3.1-3 with condition INTER-FREQ MO except that - ssbFrequency is set as the ARFCN-ValueNR of the SSB associated to Cell 3 - associatedMeasGapSSB-r17 = 2 | MO for Cell 3 is associated to MeasGapId #2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 2 entries |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 2 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| MeasIdToAddMod[2] SEQUENCE { |  | entry 2 |  |
| measId | 2 |  |  |
| measObjectId | 3 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| measGapConfig | MeasGapConfig-ConNCSG | Table 6.6.23.1.4.3-3 |  |
| } |  |  |  |

Table 6.6.23.1.4.3-3: MeasGapConfig-ConNCSG (Table 6.6.23.1.4.3-2)

| Derivation Path: Table H.3.1-6 with condition MGe |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasGapConfig ::= SEQUENCE { |  |  |  |
| gapToAddModList-r17 SEQUENCE (SIZE (1..maxNrofGapId-r17)) OF GapConfig-r17 SEQUENCE{ | 2 entries |  |  |
| GapConfig-r17[1] SEQUENCE{ |  | entry 1 Type-2 MG |  |
| measGapId-r17 | 1 |  |  |
| gapType-r17 | perUE |  |  |
| gapOffset-r17 | 39 |  |  |
| mgl-r17 | ms6 |  |  |
| mgrp-r17 | ms40 | gap pattern 0 |  |
| mgta-r17 | ms0 |  |  |
| refServCellIndicator-r17 | Not present |  |  |
| refFR2-ServCellAsyncCA-r17 | Not present |  |  |
| preConfigInd-r17 | Not present |  |  |
| ncsgInd-r17 | Not present |  |  |
| gapAssociationPRS-r17 | Not present |  |  |
| gapSharing-r17 | Not present |  |  |
| gapPriority-r17 | 2 | Low priority |  |
| } |  |  |  |
| GapConfig-r17[2] SEQUENCE{ |  | entry 2 NCSG |  |
| measGapId-r17 | 2 |  |  |
| gapType-r17 | perUE |  |  |
| gapOffset-r17 | 4 |  |  |
| mgl-r17 | ms5 |  |  |
| mgrp-r17 | ms80 | NCSG pattern 1 |  |
| mgta-r17 | ms0 |  |  |
| refServCellIndicator-r17 | Not present |  |  |
| refFR2-ServCellAsyncCA-r17 | Not present |  |  |
| preConfigInd-r17 | Not present |  |  |
| ncsgInd-r17 | true |  |  |
| gapAssociationPRS-r17 | Not present |  |  |
| gapSharing-r17 | Not present |  |  |
| gapPriority-r17 | 1 | High priority |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.6.23.1.4.3-4: MeasResults (Step 7, test procedure, Cell 2 and Cell 3)

| Derivation Path: Table H.3.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  | Cell 2 |
|  | 2 |  | Cell 3 |
| measResultServingMOList | Not Checked |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListNR SEQUENCE (SIZE(1..maxCellReport)) OF MeasResultNR { | 1 entry |  |  |
| MeasResultNR[1] { |  | entry 1 |  |
| physCellId | PhysCellId of NR neighbour Cell 2 |  | Cell 2 |
|  | PhysCellId of NR neighbour Cell 3 |  | Cell 3 |
| measResult SEQUENCE { |  |  |  |
| cellResults | Checked |  |  |
| rsIndexResults | Not Checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.6.23.1.5	Test requirements

Table 6.6.23.1.4.1-3 and Table 6.6.23.1.5-1 define the primary level settings including test tolerances for the test.

Table 6.6.23.1.5-1: NR Cell specific test parameters

| Parameter | Unit | Test | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | config | T1 | T2 | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  | 1,2,3 | 1 |  | 2 |  | 1 |  |
| Duplex mode |  | 1 | FDD |  |  |  |  |  |
|  |  | 2,3 | TDD |  |  |  |  |  |
| TDD configuration |  | 1 | N/A |  |  |  |  |  |
|  |  | 2 | TDDConf.1.1 |  |  |  |  |  |
|  |  | 3 | TDDConf.2.1 |  |  |  |  |  |
| BWchannel | MHz | 1,2 | 10: NPRB,c = 52 |  |  |  |  |  |
|  |  | 3 | 40: NPRB,c = 106 |  |  |  |  |  |
| BWP BW | MHz | 1,2 | 10: NPRB,c = 52 |  |  |  |  |  |
|  |  | 3 | 40: NPRB,c = 106 |  |  |  |  |  |
| Initial BWP configuration |  | 1,2,3 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  | N/A |  |
| Dedicated BWP configuration |  |  | DLBWP.1.1 ULBWP.1.1 |  |  |  |  |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  |  |  |  |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |  |  |
| OCNG Patterns |  | 1,2,3 | OP.1 |  | OP.1 |  | OP.1 |  |
| PDSCH RMC |  | 1 | SR.1.1 FDD |  | N/A |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |  |  |
| RMSI CORESET RMC |  | 1 | CR.1.1 FDD |  |  |  |  |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |  |  |
| Dedicated CORESET Reference Channel |  | 1 | CCR.1.1 FDD |  |  |  |  |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |  |  |
| SSB parameters |  | 1,2,3 | SSB.1 FR1 |  | SSB.1 FR1 |  | SSB.1 FR1 |  |
| SMTC configuration |  | 1,2,3 | SMTC.2 |  | SMTC.2 |  | SMTC.2 |  |
| PDSCH/PDCCH subcarrier | kHz | 1,2 | 15 |  |  |  |  |  |
| spacing |  | 3 | 30 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | 1,2,3 | 0 |  | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |  |
| NocNote2 | dBm/ 15 kHz | 1,2,3 | -98+TT |  | -98+TT |  | -98+TT |  |
| NocNote2 | dBm/SCS | 1,2 | -98+TT |  | -98+TT |  | -98+TT |  |
|  |  | 3 | -95+TT |  | -95+TT |  | -95+TT |  |
| SS-RSRP Note 3 | dBm/SCS | 1,2 | -94+TT | -94+TT | -Infinity | -91+TT | -Infinity | -91+TT |
|  |  | 3 | -91+TT | -91+TT | -Infinity | -88+TT | -Infinity | -88+TT |
| EsIot | dB | 1,2,3 | 4 +TT | 4+TT | -Infinity | 7+TT | -Infinity | 7+TT |
| EsNoc | dB | 1,2,3 | 4+TT | 4+TT | -Infinity | 7+TT | -Infinity | 7+TT |
| IoNote3 | dBm/ 9.36 MHz | 1,2 | -64.59 +TT | -64.59 +TT | -70.05 +TT | -62.26 +TT | -70.05 +TT | -62.26 +TT |
|  | dBm/ 38.16 MHz | 3 | -58.49 +TT | -58.49 +TT | -63.94 +TT | -56.15 +TT | -63.94 +TT | -56.15 +TT |
| Propagation Condition |  | 1,2,3 | AWGN |  | AWGN |  | AWGN |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |

The UE shall send one Event A3 triggered measurement report for each neighboring cell, with a measurement reporting delay less than 1282 ms for Cell 2 and 1282 ms for Cell 3 from the beginning of time period T2.

During T2, UE shall send HARQ-ACK/NACK for the corresponding PDSCH scheduled in PCell in all the slots except for the case where PDSCH or PUCCH is overlapped with the VIL of NCSG pattern and the non-dropped measurement gap occasions after considering the collision between NCSG and measurement gap.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

6.6.23.2	NR SA FR1-FR1 event-triggered reporting with two concurrent NCSGs

Editor’s Notes: This test case is incomplete in following aspects:

-	TT analysis is still missing;

6.6.23.2.1	Test purpose

The purpose of this test is to verify that the concurrent gaps with NCSG capable UE makes correct reporting of events. This test will partly verify the SA inter-frequency NR cell search requirements in TS 38.133 [6] clause 9.3.10.

6.6.23.2.2	Test applicability

This test applies to all types of NR UE release 18 and forward supporting concurrentMeasGapsNCSG-r18.

6.6.23.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.23.0.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.23.2.

6.6.23.2.4	Test description

6.6.23.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.23.2.4.1-1.

Table 6.6.23.2.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 6.6.23.2-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.23.2-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.23.2-3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test requirement and the DUT according to the parameters in Table 6.6.23.2.4.1-2.

Table 6.6.23.2.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.23.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. |  |  |

1. The test parameters for PCell and neighbour cell are given in Table 6.6.23.2.4.1-3.

2. Message contents are defined in clause 6.6.23.2.4.3.

3. The test scenario comprises of 3 NR carriers and 3 cells. NR Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 6.6.23.2.4.1-3: General test parameters

| Parameter | Unit | Test config | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | 1,2,3 | 1,2,3 | Three FR1 NR carrier frequencies are used. |
| Active cell |  | 1,2,3 | NR Cell 1 (PCell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | 1,2,3 | NR Cell 2 and 3 | NR Cell 2 is on NR RF channel number 2. NR Cell 3 is on NR RF channel number 3. |
| NCSG Pattern Id |  | 1,2,3 | 0 for NCSGId #0 1 for NCSGId #1 | As specified in TS 38.133 [6] Table 9.1.9.3-1. |
| NCSG offset |  | 1,2,3 | 39 for NCSGId #0 4 for NCSGId #1 |  |
| A3-Offset | dB | 1,2,3 | -6 |  |
| Hysteresis | dB | 1,2,3 | 0 |  |
| CP length |  | 1,2,3 | Normal |  |
| TimeToTrigger | s | 1,2,3 | 0 |  |
| Filter coefficient |  | 1,2,3 | 0 | L3 filtering is not used |
| DRX |  | 1,2,3 | OFF | DRX is not used |
| Time offset between Cell 1 and Cell 2 |  | 1,2,3 | 3 s | Synchronous. The timing of Cell 2 is 3 s later than the timing of Cell 1. |
| Time offset between Cell 1 and Cell 3 |  | 1,2,3 | 5 ms | Asynchronous. The timing of Cell 3 is 5 ms later than the timing of Cell 1. |
| T1 | s | 1,2,3 | 5 |  |
| T2 | s | 1,2,3 | 1.5 |  |

6.6.23.2.4.2	Test procedure

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. The SS sets the parameters according to T1 in Table 6.6.23.2.5-1. T1 starts.

3. The SS starts to continuously schedule DL reception on Cell 1.

4. SS shall transmit an RRCReconfiguration message to provide measurement configuration.

5. The UE shall transmit an RRCReconfigurationComplete message.

6. The SS shall set the parameters according to T2 in Table 6.6.23.2.5-1 when T1 expires. T2 starts.

7. The UE shall,

-	transmit a MeasurementReport message triggered by event A3 for Cell 2 and the overall reporting delay measured from the beginning of time period T2 is less than 1282 ms, and

-	transmit a MeasurementReport message triggered by event A3 for Cell 3 and the overall reporting delay measured from the beginning of time period T2 is less than 1282 ms, and

-	The UE shall report ACK/NACK for every PDSCHs scheduled in the slots that are not overlapped with the VIL of NCSG occasions associated to MeasGapId #2 or VIL of the non-dropped NCSG occasions associated to MeasGapId #1.

Then the number of successful tests is increased by one. If the UE fails to report the event within the overall delay requirement or fails to send valid ACK/NACK in the required slots, then the number of failure tests is increased by one.

8. After the SS receive the MeasurementReport messages in step 7) or when T2 expires, the SS shall

-	sends an RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources, or

-	switch the UE off.

9. Set NR Cell 2 physical cell identity = ((current NR Cell 2 physical cell identity + 3) mod 1008) and NR Cell 3 physical cell identity = ((current NR Cell 3 physical cell identity + 3) mod 1008) for next iteration of the test procedure loop.

10. Depending on the choice in Step 8, the SS:

-	if the RRCRelease message has been sent, transmits in NR Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5), or

-	if the UE has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

11. Repeat step 2-10 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.23.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.6.23.2.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-4 with A3-offset = -6dB |

Table 6.6.23.2.4.3-2: MeasConfig (Step 4, test procedure)

| Derivation path: Table H.3.1-2 with condition INTER-FREQ and GAP NEEDED |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { | 3 entries |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-DEFAULT specified in Table H.3.1-3 with condition INTRA-FREQ MO | MO for Cell 1 |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod[2] SEQUENCE { |  | entry 2 |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-DEFAULT specified in Table H.3.1-3 with condition INTER-FREQ MO except that - ssbFrequency is set as the ARFCN-ValueNR of the SSB associated to Cell 2 - associatedMeasGapSSB-r17 = 1 | MO for Cell 2 is associated to MeasGapId #1 |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod[3] SEQUENCE { |  | entry 3 |  |
| measObjectId | 3 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-DEFAULT specified in Table H.3.1-3 with condition INTER-FREQ MO except that - ssbFrequency is set as the ARFCN-ValueNR of the SSB associated to Cell 3 - associatedMeasGapSSB-r17 = 2 | MO for Cell 3 is associated to MeasGapId #2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 2 entries |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 2 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| MeasIdToAddMod[2] SEQUENCE { |  | entry 2 |  |
| measId | 2 |  |  |
| measObjectId | 3 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| measGapConfig | MeasGapConfig-ConNCSG | Table 6.6.23.2.4.3-3 |  |
| } |  |  |  |

Table 6.6.23.2.4.3-3: MeasGapConfig-ConNCSG (Table 6.6.23.2.4.3-2)

| Derivation Path: Table H.3.1-6 with condition MGe |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasGapConfig ::= SEQUENCE { |  |  |  |
| gapToAddModList-r17 SEQUENCE (SIZE (1..maxNrofGapId-r17)) OF GapConfig-r17 SEQUENCE{ | 2 entries |  |  |
| GapConfig-r17[1] SEQUENCE{ |  | entry 1 NCSG |  |
| measGapId-r17 | 1 |  |  |
| gapType-r17 | perUE |  |  |
| gapOffset-r17 | 39 |  |  |
| mgl-r17 | ms5 |  |  |
| mgrp-r17 | ms40 | NCSG pattern 0 |  |
| mgta-r17 | ms0 |  |  |
| refServCellIndicator-r17 | Not present |  |  |
| refFR2-ServCellAsyncCA-r17 | Not present |  |  |
| preConfigInd-r17 | Not present |  |  |
| ncsgInd-r17 | true |  |  |
| gapAssociationPRS-r17 | Not present |  |  |
| gapSharing-r17 | Not present |  |  |
| gapPriority-r17 | 2 | Low priority |  |
| } |  |  |  |
| GapConfig-r17[2] SEQUENCE{ |  | entry 2 NCSG |  |
| measGapId-r17 | 2 |  |  |
| gapType-r17 | perUE |  |  |
| gapOffset-r17 | 4 |  |  |
| mgl-r17 | ms5 |  |  |
| mgrp-r17 | ms80 | gap pattern 1 |  |
| mgta-r17 | ms0 |  |  |
| refServCellIndicator-r17 | Not present |  |  |
| refFR2-ServCellAsyncCA-r17 | Not present |  |  |
| preConfigInd-r17 | Not present |  |  |
| ncsgInd-r17 | true |  |  |
| gapAssociationPRS-r17 | Not present |  |  |
| gapSharing-r17 | Not present |  |  |
| gapPriority-r17 | 1 | High priority |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.6.23.2.4.3-4: MeasResults (Step 7, test procedure, Cell 2 and Cell 3)

| Derivation Path: Table H.3.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  | Cell 2 |
|  | 2 |  | Cell 3 |
| measResultServingMOList | Not Checked |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListNR SEQUENCE (SIZE(1..maxCellReport)) OF MeasResultNR { | 1 entry |  |  |
| MeasResultNR[1] { |  | entry 1 |  |
| physCellId | PhysCellId of NR neighbour Cell 2 |  | Cell 2 |
|  | PhysCellId of NR neighbour Cell 3 |  | Cell 3 |
| measResult SEQUENCE { |  |  |  |
| cellResults | Checked |  |  |
| rsIndexResults | Not Checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.6.23.2.5	Test requirement

Table 6.6.23.2.4.1-3 and Table 6.6.23.2.5-1 define the primary level settings including test tolerances for the test.

Table 6.6.23.2.5-1: NR Cell specific test parameters

| Parameter | Unit | Test | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | config | T1 | T2 | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  | 1,2,3 | 1 |  | 2 |  | 3 |  |
| Duplex mode |  | 1 | FDD |  |  |  |  |  |
|  |  | 2,3 | TDD |  |  |  |  |  |
| TDD configuration |  | 1 | N/A |  |  |  |  |  |
|  |  | 2 | TDDConf.1.1 |  |  |  |  |  |
|  |  | 3 | TDDConf.2.1 |  |  |  |  |  |
| BWchannel | MHz | 1,2 | 10: NPRB,c = 52 |  |  |  |  |  |
|  |  | 3 | 40: NPRB,c = 106 |  |  |  |  |  |
| BWP BW | MHz | 1,2 | 10: NPRB,c = 52 |  |  |  |  |  |
|  |  | 3 | 40: NPRB,c = 106 |  |  |  |  |  |
| Initial BWP Configuration |  | 1,2,3 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  | N/A |  |
| Dedicated BWP Configuration |  | 1,2,3 | DLBWP.1.1 ULBWP.1.1 |  |  |  |  |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  |  |  |  |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |  |  |
| OCNG Patterns |  | 1,2,3 | OP.1 |  |  |  |  |  |
| PDSCH RMC |  | 1 | SR.1.1 FDD |  | N/A |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |  |  |
| RMSI CORESET RMC |  | 1 | CR.1.1 FDD |  |  |  |  |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |  |  |
| Dedicated CORESET RMC |  | 1 | CCR.1.1 FDD |  |  |  |  |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |  |  |
| SSB parameters |  | 1,2,3 | SSB.1 FR1 |  | SSB.1 FR1 |  | SSB.1 FR1 |  |
| SMTC configuration |  | 1,2,3 | SMTC.2 |  | SMTC.2 |  | SMTC.2 |  |
| PDSCH/PDCCH subcarrier | kHz | 1,2 | 15 |  |  |  |  |  |
| spacing |  | 3 | 30 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | 1,2,3 | 0 |  | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |  |
| NocNote2 | dBm/ 15 kHz | 1,2,3 | -98 +TT |  | -98 +TT |  | -98 +TT |  |
| NocNote2 | dBm/ SCS | 1,2 | -98 +TT |  | -98 +TT |  | -98 +TT |  |
|  |  | 3 | -95 +TT |  | -95 +TT |  | -95 +TT |  |
| SS-RSRP Note 3 | dBm/ SCS | 1,2 | -94 +TT | -94 +TT | -Infinity | -91 +TT | -Infinity | -91 +TT |
|  |  | 3 | -91 +TT | -91 +TT | -Infinity | -88 +TT | -Infinity | -88 +TT |
| EsIot | dB | 1,2,3 | 4 +TT | 4 +TT | -Infinity | 7 +TT | -Infinity | 7 +TT |
| EsNoc | dB | 1,2,3 | 4 +TT | 4 +TT | -Infinity | 7 +TT | -Infinity | 7 +TT |
| IoNote3 | dBm/9.36 MHz | 1,2 | -64.59 +TT | -64.59 +TT | -70.05 +TT | -62.26 +TT | -70.05 +TT | -62.26 +TT |
|  | dBm/38.16 MHz | 3 | -58.49 +TT | -58.49 +TT | -63.94 +TT | -56.15 +TT | -63.94 +TT | -56.15 +TT |
| Propagation Condition |  | 1,2,3 | AWGN |  | AWGN |  | AWGN |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |

The UE shall send one Event A3 triggered measurement report for each neighboring cell, with a measurement reporting delay less than 1282 ms for Cell 2 and 1282 ms for cell 3 from the beginning of time period T2.

During T2, UE shall send HARQ-ACK/NACK for the corresponding PDSCH scheduled in PCell in all the slots except for the case where PDSCH or PUCCH is overlapped with the VIL of non-dropped NCSG pattern after considering the collision between NCSGs.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

6.6.23.3	NR SA FR1 and FR1-FR1 event triggered reporting with concurrent Type-2 measurement gap and NCSG

Editor’s Notes: This test case is incomplete in following aspects:

-	TT analysis is still missing;

6.6.23.3.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the cell search requirements on SCC with deactivated SCell in TS 38.133 [6] clauses 9.2.7.1 and 9.2.7.2.

6.6.23.3.2	Test applicability

This test applies to all types of NR UE release 18 and forward supporting 2CC DL CA and concurrentMeasGapsNCSG-r18.

6.6.23.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.23.0.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.23.3.

6.6.23.3.4	Test description

6.6.23.3.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.23.3.4.1-1.

Table 6.6.23.3.4.1-1: Supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.6.23.3-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.23.3-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.23.3-3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test requirement and the DUT according to the parameters in Table 6.6.23.3.4.1-2.

Table 6.6.23.3.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E (Note 1), Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.23.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. |  |  |
| Note 1: For UE supporting intra-band contiguous CA, the test frequencies are selected according to E.1.4.2. For UE supporting intra-band non-contiguous CA, the test frequencies are selected according to E.1.4.3. For UE supporting inter-band CA, the the test frequencies are selected according to E.1.4.1 |  |  |  |

1. The test parameters for PCell and neighbour cell are given in Table 6.6.23.3.4.1-3 below.

2. Message contents are defined in clause 6.6.23.3.4.3.

3. The test scenario comprises of 3 NR carriers and 3 cells. NR Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 6.6.23.3.4.1-3: General test parameters

| Parameter | Unit | Test config | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1,2,3 | Cell 1 |  |
| Configured deactivated SCell |  | 1,2,3 | Cell 2 |  |
| Neighbour cell |  | 1,2,3 | Cell 3 | Cell to be identified. |
| RF Channel Number |  | 1,2,3 | 1: Cell 1 |  |
| RF Channel Number |  | 1,2,3 | 2: Cell 2 and Cell 3 |  |
| NCSG type |  | 1,2,3 | Per-UE NCSG |  |
| NCSG pattern Id |  | 1,2,3 | ID # 1 | Defined in TS 38.133 [6] Table 9.1.9.3-1 |
| NCSG offset |  | 1,2,3 | 39 |  |
| Measurement gap type |  | 1,2,3 | Per-UE MG |  |
| MG pattern Id |  | 1,2,3 | ID #1 | Defined in TS 38.133 [6] Table 9.1.2-1 |
| Measurement gap offset |  | 1,2,3 | 19 |  |
| MGTA | ms | 1,2,3 | 0 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| A6-Offset | dB | 1,2,3 | -4.5 |  |
| CP length |  | 1,2,3 | Normal |  |
| Hysteresis | dB | 1,2,3 | 0 |  |
| Time To Trigger | s | 1,2,3 | 0 |  |
| Filter coefficient |  | 1,2,3 | 0 | L3 filtering is not used |
| DRX |  | 1,2,3 |  | OFF |
| SCell measurement cycle (measCycleSCell) | ms | 1,2,3 | 640 |  |
| Cell 2 timing offset to Cell 1 | s | 1,2,3 | 0 |  |
| Time alignment error between Cell 2 and Cell 1 | s | 1,2,3 | ≤ Time alignment error as specified in TS 38.104 [13] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| Cell 3 timing offset to Cell 1 | s | 1,2,3 | 3 | Synchronous cells |
| T1 | s | 1,2,3 | 5 |  |
| T2 | s | 1,2,3 | 5 |  |

6.6.23.3.4.2	Test procedure

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. The SS sets the parameters according to T1 in Table 6.6.23.3.5-1. T1 starts.

3. The SS starts to continuously schedule DL reception on Cell 1.

4. SS shall transmit an RRCReconfiguration message to provide measurement configuration and add Cell 2 as a deactivated SCell.

5. The UE shall transmit an RRCReconfigurationComplete message.

6. The SS shall set the parameters according to T2 in Table 6.6.23.3.5-1 when T1 expires. T2 starts.

7. The UE shall,

-	transmit a MeasurementReport message triggered by event A6 and the overall reporting delay measured from the beginning of time period T2 is less than [6402] ms, and

-	The UE shall report ACK/NACK for every PDSCHs scheduled in the slots that are not overlapped with the VIL of NCSG occasions associated to MeasGapId #1 or gap occasions associated to MeasGapId #2.

Then the number of successful tests is increased by one. If the UE fails to report the event within the overall delay requirement or fails to send valid ACK/NACK in the required slots, then the number of failure tests is increased by one.

8. After the SS receive the MeasurementReport messages in step 7) or when T2 expires, the SS shall

-	sends an RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources, or

-	switch the UE off.

9. Set NR Cell 2 physical cell identity = ((current NR Cell 2 physical cell identity + 3) mod 1008) and NR Cell 3 physical cell identity = ((current NR Cell 3 physical cell identity + 3) mod 1008) for next iteration of the test procedure loop.

10. Depending on the choice in Step 8, the SS:

-	if the RRCRelease message has been sent, transmits in NR Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5), or

-	if the UE has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

11. Repeat step 2-10 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.23.3.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.6.23.3.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-7 with condition Deactivated SCell and INTRA-FREQ |

Table 6.6.23.3.4.3-2: MeasConfig (Step 4, test procedure)

| Derivation path: Table H.3.1-2 with condition Deactivated SCell and GAP NEEDED |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { | 2 entries |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-DEFAULT specified in Table H.3.1-3 with condition INTRA-FREQ MO | MO for Cell 1 |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod[2] SEQUENCE { |  | entry 2 |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-DEFAULT specified in Table H.3.1-3 with condition Deactivated SCell except that - associatedMeasGapSSB-r17 = 1 | MO for Cell 2 is associated to MeasGapId #1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR-A6 | Table 6.6.23.3.4.3-3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measGapConfig | MeasGapConfig-ConNCSG | Table 6.6.23.3.4.3-4 |  |
| } |  |  |  |

Table 6.6.23.3.4.3-3: ReportConfigNR (Table 6.6.23.3.4.3-2)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-142 with condition EVENT_A6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA6 SEQUENCE { |  |  |  |
| a6-Offset CHOICE { |  |  |  |
| rsrp | -9 | Acutal a6-offset = -4.5dB |  |
| } |  |  |  |
| hysteresis | 0 |  |  |
| timeToTrigger | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| reportAmount | r2 |  |  |
| maxReportCells | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.6.23.3.4.3-4: MeasGapConfig-ConNCSG (Table 6.6.23.3.4.3-2)

| Derivation Path: Table H.3.1-6 with condition MGe |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasGapConfig ::= SEQUENCE { |  |  |  |
| gapToAddModList-r17 SEQUENCE (SIZE (1..maxNrofGapId-r17)) OF GapConfig-r17 SEQUENCE{ | 2 entries |  |  |
| GapConfig-r17[1] SEQUENCE{ |  | entry 1 NCSG |  |
| measGapId-r17 | 1 |  |  |
| gapType-r17 | perUE |  |  |
| gapOffset-r17 | 39 |  |  |
| mgl-r17 | ms5 |  |  |
| mgrp-r17 | ms80 | NCSG pattern 1 |  |
| mgta-r17 | ms0 |  |  |
| refServCellIndicator-r17 | Not present |  |  |
| refFR2-ServCellAsyncCA-r17 | Not present |  |  |
| preConfigInd-r17 | Not present |  |  |
| ncsgInd-r17 | true |  |  |
| gapAssociationPRS-r17 | Not present |  |  |
| gapSharing-r17 | Not present |  |  |
| gapPriority-r17 | 1 | High priority |  |
| } |  |  |  |
| GapConfig-r17[2] SEQUENCE{ |  | entry 2 Type-2 MG |  |
| measGapId-r17 | 2 |  |  |
| gapType-r17 | perUE |  |  |
| gapOffset-r17 | 19 |  |  |
| mgl-r17 | ms6 |  |  |
| mgrp-r17 | ms80 | gap pattern 1 |  |
| mgta-r17 | ms0 |  |  |
| refServCellIndicator-r17 | Not present |  |  |
| refFR2-ServCellAsyncCA-r17 | Not present |  |  |
| preConfigInd-r17 | Not present |  |  |
| ncsgInd-r17 | Not present |  |  |
| gapAssociationPRS-r17 | Not present |  |  |
| gapSharing-r17 | Not present |  |  |
| gapPriority-r17 | 2 | Low priority |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.6.23.3.4.3-5: CellGroupConfig (Step 4, test procedure)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-19 with condition SCell_Add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| sCellToAddModList SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig { | 1 entry |  |  |
| SCellConfig[1] SEQUENCE { |  | entry 1 |  |
| sCellConfigDedicated SEQUENCE { |  |  |  |
| servingCellMO | 2 | Cell 2 is associated to MO #2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.6.23.3.5	Test requirement

Table 6.6.23.3.4.1-3 and Table 6.6.23.3.5-1 define the primary level settings including test tolerances for the test.

Table 6.6.23.3.5-1: NR Cell specific test parameters

| Parameter | Unit | Test | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | config | T1 | T2 | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  |  |  |  |  |
|  |  | 2 | TDDConf.1.1 |  |  |  |  |  |
|  |  | 3 | TDDConf.2.1 |  |  |  |  |  |
| PDSCH RMC |  | 1 | SR.1.1 FDD |  | N/A |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |  |  |
| RMSI CORESET RMC |  | 1 | CR.1.1 FDD |  |  |  |  |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |  |  |
| Dedicated CORESET RMC |  | 1 | CCR.1.1 FDD |  |  |  |  |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |  |  |
| OCNG Patterns |  | 1,2,3 | OP.1 |  | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  | 1,2,3 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |  |  |
| Active BWP configuration |  | 1,2,3 | DLBWP.1.1 ULBWP.1.1 |  | DLBWP.1.1 ULBWP.1.1 |  |  |  |
| NocNote2 | dBm/ SCS | 1,2 | -98 |  |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |  |
| NocNote2 | dBm/ 15 kHz | 1,2,3 | -98 |  |  |  |  |  |
| EsIot | dB | 1,2,3 | 4 +TT | 4 +TT | 4 +TT | -1.46 +TT | -Infinity | -1.46 +TT |
| EsNoc | dB | 1,2,3 | 4 +TT | 4 +TT | 4 +TT | 4 +TT | -Infinity | 4 +TT |
| SS-RSRP Note 3 | dBm/ SCS | 1,2 | -94 +TT | -94 +TT | -94 +TT | -94 +TT | -Infinity | -94 +TT |
|  |  | 3 | -91 +TT | -91 +TT | -91 +TT | -91 +TT | -Infinity | -91 +TT |
| Io | dBm/9.36 MHz | 1,2 | -64.60 +TT | -64.60 +TT | -64.60 +TT | -62.25 +TT | -64.60 +TT | -62.25 +TT |
|  | dBm/38.16 MHz | 3 | -58.50 +TT | -58.50 +TT | -58.50 +TT | -56.16 +TT | -58.50 +TT | -56.16 +TT |
| Propagation Condition |  | 1,2,3 | AWGN |  |  |  |  |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. NOTE 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

The UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than [6402] ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test.

The UE shall be scheduled on PCell continuously throughout the test. From the start of T1 until the measurement report is received during T2, UE shall send HARQ ACK/NACK for the corresponding PDSCH scheduled in PCell in all the slots except for the case where PDSCH or PUCCH is overlapped with the VIL of NCSG pattern and MGL of MG pattern.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

For a test to be considered successful requirements on both Event A6 detection and percentage of transmitted ACK/NACKs have to be fulfilled simultaneously.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

<End of Change 2>