3GPP TSG-RAN4 Meeting #116	R4-2510124

# Bengaluru, India, 25th – 29th, August, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 2949 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Big CR on TS 38.101-1 to introduce R19 ATG UE RF requirements (NR_ATG_enh-Core) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC, ZTE Corporation, Sanechips, Apple, Qualcomm Incorporated, Huawei, HiSilicon, LG Electronics France, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG_enh-Core |  |  |  |  |  | Date: |  |  | 2025-09-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduce R19 ATG enhancement UE RF requirements. And the following draft have been endorsed R4-2507992, R4-2507967, R4-2507965, R4-2507964, R4-2511785, R4-2510122, R4-2511784, R4-2511786 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce R19 ATG enhancement UE RF requirements for ATG UE with CA and UL MIMO. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | R19 ATG enhancement UE RF requirements would be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2J, 5.5J, 6.2J, 6.3J, 6.4J, 6.5J, 7.3J, 7.4J, 7.5J, 7.6J, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Start of Change#1>>

5.2J	Operating band for ATG

5.2J.1	General

NR operating bands n1, n3, n34, n39, n41, n78, n79, which are defined in Table 5.2-1, can be applied for ATG operation.

5.2J.1A	Operating band for ATG CA

NR carrier aggregation operating bands defined in Table 5.2J.1A.1-1 and Table 5.2J.1A.2-1, can be applied for ATG CA operation.

5.2J.1A.1	Operating band for ATG intra-band CA

Table 5.2A.1-1: ATG intra-band contiguous CA operating bands

| NR ATG CA Band | NR ATG Band (Table 5.2-1) |
| --- | --- |
| CA_n79 | n79 |
| NOTE: The minimum requirements only apply for non simultaneous Tx/Rx between all carriers for TDD combinations. |  |

5.2J.1A.2	Operating band for ATG inter-band CA

Table 5.2J.1A.2-1: ATG inter-band CA operating bands

| NR CA Band | NR Band (Table 5.2-1) | DL interruption allowed (Note 1) |
| --- | --- | --- |
| CA_n3-n39 | n3, n39 |  |
| NOTE 1: Applicable when dynamic Tx switching is conducted. The DL interruption requirement is specified in clause 8.2.2.2.10 of 38.133 [13]. |  |  |

5.2J.1D	Operating band for ATG UL MIMO

NR operating bands in Table 5.2J.1D-1 to support UL MIMO, can be applied for ATG UL MIMO operation.

Table 5.2J.1D-1: NR operating bands for UL MIMO in FR1

NR operating band

n1

n3

n34

n38

n39

n41

n78

n79

<<End of Change#1>>

<< Start of Change#2>>

5.5J	Configurations for ATG

5.5J.1A	Configurations for ATG CA

The ATG intra-band contiguous CA configurations and channel bandwidths are specified in clause 5.5J.1A.1. The ATG inter-band CA configurations and channel bandwidths are specified in clause 5.5J.1A.2.

5.5J.1A.1	Configurations for ATG intra-band contiguous CA

Table 5.5J.1A.1-1 Configurations and bandwidth combination set for ATG intra-band contiguous CA in FR1

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations or single uplink carrier | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
| CA_n79C | n79 | 50 | 60, 80, 100 |  |  |  | 200 | 0 |
|  |  | 60 | 60, 80, 100 |  |  |  |  |  |
|  |  | 80 | 80, 100 |  |  |  |  |  |
|  |  | 100 | 100 |  |  |  |  |  |
|  |  | See n79 channel bandwidths in Table 5.3.5-1 for each carrier |  |  |  |  | 200 | 4 and 5 |

5.5J.1A.2	Configurations for ATG inter-band CA

Table 5.5J.1A.2-1 Configurations and bandwidth combination set for ATG inter-band CA in FR1

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n39A | n3 n39 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n39 | 5, 10, 15, 20, 25, 30, 35, 40 |  |
| NOTE 1: For each channel bandwidth of each component carrier, refer to Table 5.3.5-1 for the applicable SCSs. For a given band, not all UE channel bandwidths support the same SCSs. |  |  |  |  |

<<End of Change#2>>

<< Start of Change#3>>

6.2J	Transmitter power for ATG

6.2J.0	Reserved

6.2J.0D	General

UE can indicate rated output power for the single configured UL CC with DL CA as specified in 6.2J.1 and if UE supports UL MIMO in this carrier, UE can indicate rated output power for the CA configuration as specified in 6.2J.1D.

6.2J.1	UE maximum output power for ATG

For the ATG UE, the rated maximum output power is reported via UE capability maxOutputPowerATG-r18 at maximum modulation order reported by ATG UE and full PRB configurations within the channel bandwidth of NR carrier unless otherwise stated. The period of measurement shall be at least one sub frame (1ms). UE capability maxOutputPowerATG-r18 is an integer value in the range 23 to 40 dBm.

For ATG UE with multiple omni-directional antennas not indicating the capability antennaArrayType-r18, the measured maximum output power Pmax,c,AC shall remain within +2 dB and -2 dB of the rated maximum output power Prated,c,AC  reported by the ATG UE.

For ATG UE with antenna array indicating the capability antennaArrayType-r18, the measured maximum output power Pmax,c,TABC shall remain within +2 dB and -2 dB of the rated maximum output power Prated,c,TABC reported by the ATG UE.

6.2J.1A	UE maximum output power for ATG CA

#### 6.2J.1A.1	UE maximum output power for ATG intra-band contiguous CA

For downlink intra-band contiguous carrier aggregation with a single uplink component carrier configured in the NR band, the rated output power specified in 6.2J.1 apply.

#### 6.2J.1A.2	UE maximum output power for ATG inter-band CA

For inter-band downlink carrier aggregation with one uplink carrier assigned to one NR band, the transmitter power requirements in 6.2J.1 apply.

#### 6.2J.1A.3	ΔTIB,c for ATG CA

For the UE which supports inter-band NR CA configuration, ΔTIB,c in tables below applies. Unless otherwise stated, ΔTIB,c is set to zero.

##### 6.2J.1A.3.1	ΔTIB,c for Inter-band ATG CA (two bands)

Table 6.2J.1A.3.1-1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB) |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration |  |
| CA_n3-n39 | 0.5 | 0.5 |

6.2J.1D	UE maximum output power for ATG UL MIMO

For UE supporting UL MIMO, the rated maximum output power is defined as the sum of the rated maximum output power from all UE antenna connectors or all UE TAB connectors, which is reported via UE capability maxOutputPowerATG-r18 at maximum modulation order reported by ATG UE and full PRB configurations within the channel bandwidth of NR carrier unless otherwise stated. The period of measurement shall be at least one sub frame (1 ms). UE capability maxOutputPowerATG-r18 is an integer value in the range 23 to 40 dBm.

- For ATG UE with multiple omni-directional antennas not indicating the capability antennaArrayType-r18, the measured maximum output power over all UE antenna connectors Pmax,c,AC shall remain within +2 dB and -2 dB of the rated maximum output power Prated,c,AC reported.

- For ATG UE with antenna array indicating the capability antennaArrayType-r18, the measured maximum output power over all UE TAB connectors Pmax,c,TABC shall remain within +2 dB and -2 dB of the rated maximum output power Prated,c,TABC reported.

For ATG UE with two transmit antenna connectors or two groups of TAB connectors (each of which supporting one layer) in closed-loop spatial multiplexing scheme, the requirements shall be met with the UL MIMO configurations specified in Table 6.2J.1D-1. The requirements shall be met with the UL MIMO configurations of using 2-layer UL MIMO codebook-based transmission with precoding matrix of W=. DCI Format for UE configured in PUSCH transmission mode for uplink single-user MIMO shall be used.

Table 6.2J.1D-1: UL MIMO configuration in closed-loop spatial multiplexing scheme

| Transmission scheme | DCI format | Number of layers | TPMI index |
| --- | --- | --- | --- |
| Codebook based uplink | DCI format 0_1 | 2 | 0 |
| NOTE 1: The UE is configured with one SRS resource with the parameter nrofSRS-Ports set to 2. |  |  |  |

For UE supporting uplink full power transmission (ULFPTx) for UL MIMO, the rated output power requirements shall be met with the PUSCH configurations specified in Table 6.2J.1D-2, based upon UE’s support of uplink full power transmission mode.

Table 6.2J.1D-2: PUSCH Configuration for uplink full power transmission (ULFPTx)

| ULFPTx Mode | Transmission scheme | DCI format | Modulation | Number of layers | Number of Tx Port | TPMI index |
| --- | --- | --- | --- | --- | --- | --- |
| Mode-1 | Codebook based uplink | DCI format 0_1 | DFT-s-OFDM, CP-OFDM NOTE3 | 1 | 2 NOTE1 | 2 |
| Mode-2 | Codebook based uplink | DCI format 0_1 | DFT-s-OFDM, CP-OFDM | 1 | 2 NOTE1 | 0 or 1NOTE2 |
| Mode-full power | Codebook based uplink | DCI format 0_1 | DFT-s-OFDM, CP-OFDM | 1 | 2 NOTE1 | 0,1 |
| NOTE 1: The UE is configured with one SRS resource with the parameter nrofSRS-Ports set to 2. NOTE 2: TPMI index selected shall be based upon the full power TPMI reported by the UE [8, TS 38.213]. |  |  |  |  |  |  |

If the UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for codebook based transmission with precoding matrix W=1 [6.3.1.5 TS 38.211], the requirements in clause 6.2J.1 apply for at least one antenna connector or one group of TAB connector for rated output power as indicated by the ue-PowerClass field in capability signalling.

A UE with 2Tx indicating the feature ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16 for a band shall meet the requirement in clause 6.2J.1 for at least one antenna connector when scheduled for single antenna-port transmission by DCI format 0_0 or by DCI format 0_1 for codebook-based transmission with precoding matrix W=1 [6.3.1.5 TS 38.211].

6.2J.2	Configured transmitted power for ATG

The UE is allowed to set its configured maximum output power PCMAX,f,c for carrier f of serving cell c in each slot. The configured maximum output power PCMAX,f,c is set within the following bounds:

PCMAX_L,f,c ≤  PCMAX,f,c  ≤  PCMAX_H,f,c with

PCMAX_L,f,c = MIN {PEMAX,c, Prated,c,AC or Prated,c,TABC}

PCMAX_H,f,c = PEMAX,c

where

PEMAX,c is the value given by ATG specific the p-Max IE or the field additionalPmax of the NR-NS-PmaxList IE], whichever is applicable according to TS 38.331[7]; It’s noted that the actual PEMAX,c value is (9 + field value) in ATG cell, according to p-Max IE definition in TS 38.331 [7];

Prated,c,AC is the rated maximum output power at maximum modulation order and full PRB configurations which is indicated by ATG UE capability maxOutputPowerATG-r18 for ATG UE with multiple omni-directional antennas not indicating the capability antennaArrayType-r18;

Prated,c,TABC is the rated maximum output power at maximum modulation order and full PRB configurations which is indicated by ATG UE capability maxOutputPowerATG-r18 for ATG UE with antenna array indicating the capability antennaArrayType-r18.

TREF and Teval are specified in Table 6.2J.2-0. For each TREF, the PCMAX,L,c for serving cell c are evaluated per Teval and given by the minimum  value taken over the transmission(s) within the Teval; the minimum PCMAX_L,f,c over one or more Teval is then applied for the entire TREF

Table 6.2J.2-0: Evaluation and reference periods for PCMAX

| TREF | Teval | Teval with frequency hopping |
| --- | --- | --- |
| Physical channel length | Physical channel length | Min(Tno_hopping, Physical Channel Length) |

The measured configured maximum output power PUMAX,f,c shall be within the following bounds:

PCMAX_L,f,c  – T(PCMAX_L,f,c)  ≤  PUMAX,f,c  ≤  PCMAX_H,f,c  +  T(PCMAX_H,f,c).

where the tolerance T(PCMAX,f,c) for applicable values of PCMAX,f,c is specified in Table 6.2J.2-1.

Table 6.2J.2-1: ATG PCMAX tolerance

| PCMAX,f,c (dBm) | Tolerance T(PCMAX,f,c) (dB) |
| --- | --- |
| 23 < PCMAX,c ≤ 40 | 2.0 |
| 21 ≤ PCMAX,c ≤ 23 | 2.0 |
| 20 ≤ PCMAX,c < 21 | 2.5 |
| 19 ≤ PCMAX,c < 20 | 3.5 |
| 18 ≤ PCMAX,c < 19 | 4.0 |
| 13 ≤ PCMAX,c < 18 | 5.0 |
| 8 ≤ PCMAX,c < 13 | 6.0 |
| -19 ≤ PCMAX,c < 8 | 7.0 |

6.2J.2D	Configured transmitted power for UL MIMO

For ATG UE supporting UL MIMO, the transmitted power is configured per each UE.

The definitions of configured maximum output power PCMAX,c, the lower bound PCMAX_L,c, and the higher bound PCMAX_H,c specified in clause 6.2J.2 shall apply to UE supporting UL MIMO.

The measured configured maximum output power PUMAX,c for serving cell c shall be within the following bounds:

PCMAX_L,c  –  MAX{TL, T LOW(PCMAX_L,c)}  ≤  PUMAX,c  ≤  PCMAX_H,c  +  T HIGH(PCMAX_H,c)

where TLOW(PCMAX_L,c) and THIGH(PCMAX_H,c) are defined as the tolerance and applies to PCMAX_L,c and PCMAX_H,c separately, while TL is the absolute value of the lower tolerance in 6.2J.1.

For UE with two transmit antenna connectors or two groups of TAB connectors (each of which supporting one layer) in closed-loop spatial multiplexing scheme, the tolerance is specified in Table 6.2J.2D-1. The requirements shall be met with UL MIMO configurations specified in Table 6.2J.1D-1.

For UE support uplink full power transmission (ULFPTx) for UL MIMO, the tolerance is specified in Table 6.2J.2D-1. The requirements shall be met with the PUSCH configurations specified in Table 6.2J.1D-2, based upon UE’s support of uplink full power transmission mode.

Table 6.2J.2D-1: PCMAX,c tolerance in closed-loop spatial multiplexing scheme

| PCMAX,c (dBm) | Tolerance TLOW(PCMAX_L,c) (dB) | Tolerance THIGH(PCMAX_H,c) (dB) |
| --- | --- | --- |
| 23 ≤ PCMAX,c ≤ 40 | 3.0 | 2.0 |
| 21 ≤ PCMAX,c < 23 | 5.0 | 2.0 |
| 20 ≤ PCMAX,c < 21 | 5.0 | 4.0 |
| 19 ≤ PCMAX,c < 20 | 3.5 |  |
| 18 ≤ PCMAX,c < 19 | 4.0 |  |
| 13 ≤ PCMAX,c < 18 | 5.0 |  |
| 18 ≤ PCMAX,c < 13 | 6.0 |  |
| -19 ≤ PCMAX,c < 8 | 7.0 |  |

If the UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook-based transmission, the corresponding requirements in clause 6.2J.1D apply for the rated output power as indicated by the maxOutputPowerATG-r18 field in capability signaling.

<<End of Change#3>>

<< Start of Change#4>>

6.3J	Output power dynamics for ATG

6.3J.1	Minimum output power for ATG

The minimum controlled output power of the UE is defined as the power in the channel bandwidth for all transmit bandwidth configurations (resource blocks) when the power is set to a minimum value.

The minimum output power is defined as the sum of the mean power from all antenna connectors or all TAB connectors in at least one sub-frame (1 ms). The minimum output power shall not exceed the values specified in Table 6.3J.1-1for ATG UE with omni-directional antenna and in Table 6.3J.1-2 for ATG UE with antenna array.

Table 6.3J.1-1: Minimum output power for ATG UE with omni-directional antenna

| Channel bandwidth | (MHz) | 5,10,15,20 | 25,30,35,40,45,50 | 60,70,80,90,100 |
| --- | --- | --- | --- | --- |
| REF_SCS | (kHz) | 15 |  | 30 |
| Minimum output power | (dBm) | -15 | -15+10log10 (BWChannel /20) | -15+10log10 (BWChannel /20) |
| Measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |  |  |
| NOTE: The minimum output power value is rounded to the nearest number down to one decimal point. |  |  |  |  |

Table 6.3J.1-2: Minimum output power for ATG UE with antenna array

| Channel bandwidth | (MHz) | 5,10,15,20 | 25,30,35,40,45,50 | 60,70,80,90,100 |
| --- | --- | --- | --- | --- |
| REF_SCS | (kHz) | 15 |  | 30 |
| Minimum output power | (dBm) | -19 | -19+10log10 (BWChannel /20) | -19+10log10 (BWChannel /20) |
| Measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |  |  |
| NOTE: The minimum output power value is rounded to the nearest number down to one decimal point. |  |  |  |  |

6.3J.1D	Minimum output power for ATG UL MIMO

For ATG UE with two transmit antenna connectors or two groups of TAB connectors (each of which supporting one layer), the minimum output power is defined as the sum of the mean power from all antenna connectors or all TAB connectors in one sub-frame (1ms). The minimum output power shall not exceed the values specified in Table 6.3J.1-1.

If ATG UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission with precoding matrix W=1 [6.3.1.5 TS 38.211], the requirements in clause 6.3J.1 apply when TxD is not indicated.

6.3J.2	Transmit OFF power for ATG

The transmit OFF power is defined as the mean power at each transmit antenna connector or each TAB connector in a duration of at least one sub-frame (1 ms) excluding any transient periods.

The transmit OFF power requirements as specified in clause 6.3.2 are applicable for ATG UE.

6.3J.2D	Transmit OFF power for ATG UL MIMO

The transmit OFF power is defined as the mean power at each transmit antenna connector or each TAB connector in a duration of at least one sub-frame (1ms) excluding any transient periods.

The transmit OFF power requirements as specified in clause 6.3J.2 are applicable for ATG UE with UL MIMO.

6.3J.3	Transmit ON/OFF time mask for ATG

The transmit ON/OFF time mask requirements as specified in clause 6.3.3 are applicable for ATG UE at each transmit antenna connector or each TAB connector.

6.3J.3D	Transmit ON/OFF time mask for ATG UL MIMO

The transmit ON/OFF time mask requirements as specified in clause 6.3J.3 are applicable for ATG UE at each transmit antenna connector or each TAB connector.

For ATG UE with two transmit antenna connectors or two groups of TAB connectors (each of which supporting one layer) in closed-loop spatial multiplexing scheme, the general ON/OFF time mask requirements specified in clause 6.3J.3 apply to each transmit antenna connector or each TAB connector. The requirements shall be met with the UL MIMO configurations described in clause 6.2J.1D.

If UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission with precoding matrix W=1 [6.3.1.5 TS 38.211], the requirements in clause 6.3J.3 apply when TxD is not indicated, and the requirements in clause 6.3G.3 apply when TxD is indicated.

6.3J.4	Power control for ATG

The power control requirements specified in clause 6.3.4 are applicable to the sum of output power at each transmit antenna connector for UE with omnidirectional antenna(s) or to the sum of output power at each transceiver array boundary (TAB) connectors for UE with antenna array for ATG UE.

6.3J.4D	Power control for ATG UL MIMO

For ATG UE supporting UL MIMO, the power control tolerance applies to the sum of output powers from all transmit antenna connectors of ATG UE with omni-directional antenna or the sum of output powers from all TAB connectors of ATG UE with antenna array.

The power control requirements specified in clause 6.3J.4 apply to ATG UE with all transmit antenna connectors or all TAB connectors in closed-loop spatial multiplexing scheme. The requirements shall be met with UL MIMO configurations described in clause 6.2J.1D.

If ATG UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission with precoding matrix W=1 [6.3.1.5 TS 38.211], the requirements in clause 6.3J.4 apply when TxD is not indicated.

<<End of Change#4>>

<< Start of Change#5>>

6.4J	Transmit signal quality for ATG

6.4J.0	Reserved

6.4J.0D	General

For a ATG UE supporting UL MIMO, the requirements in this section are defined per layer or as the sum of emissions from all UE antennas to account for the UL MIMO scheme.

Alternatively, when applicable, requirements may be verified per layer using 2-layer UL MIMO transmission with codebook of, and a configuration defined in Table 6.4J.0D-1.

Table 6.4J.0D-1: UL MIMO configuration for per connector measurements

| Transmission scheme | DCI format | Codebook Index |
| --- | --- | --- |
| Codebook based uplink | DCI format 0_1 | Codebook index 0 |

6.4J.1	Frequency error for ATG

The ATG UE basic measurement interval of modulated carrier frequency is 1 UL slot. The ATG UE pre-compensates the uplink modulated carrier frequency by the estimated Doppler shift. The mean value of basic measurements of ATG UE modulated carrier frequency per layer at each transmit antenna connector for UE with omni-directional antenna(s) or at each transceiver array boundary (TAB) connector for UE with antenna array shall be accurate to within ± 0.1 PPM observed over a period of 1 ms of cumulated measurement intervals compared to ideally pre-compensated reference uplink carrier frequency.

UE [shall] rely on the ATG BS location broadcasted by the SIB22 in TS 38.331 [7].

NOTE 1:	the ideally pre-compensated reference uplink carrier frequency consists of the UL carrier frequency signalled to the UE by ATG BS and UL precompensated doppler frequency shift.

6.4J.1D	Frequency error for ATG UL MIMO

For ATG UE supporting UL MIMO, the frequency error requirement as specified in clause 6.4J.1 apply per layer.

6.4J.2	Transmit modulation quality for ATG

The requirements for transmit modulation quality defined in clause 6.4.2 shall apply for ATG UE at each transmit antenna connector for UE with omni-directional antenna(s) or at each transceiver array boundary (TAB) connector for UE with antenna array, except for the phase continuity requirements for DMRS bundling in 6.4.2.5. The requirements for 256QAM modulation are only applicable to ATG UE indicating support of 256QAM.

6.4J.2D	Transmit modulation quality for ATG UL MIMO

6.4J.2D.0	General

For ATG UE supporting UL MIMO, general description for transmit modulation quality as specified in clause 6.4D.2.0 apply.

6.4J.2D.1	Error Vector Magnitude for ATG UL MIMO

For ATG UE with two transmit antenna connectors or two groups of TAB connectors (each of which supporting one layer) in closed-loop spatial multiplexing scheme, the Error Vector Magnitude requirements specified in clause 6.4.2.1 apply per layer. The requirements shall be met with the UL MIMO configurations specified in Table 6.2J.1D-1.

6.4J.2D.2	Carrier leakage for ATG UL MIMO

For ATG UE with two transmit antenna connectors or two groups of TAB connectors (each of which supporting one layer) in closed-loop spatial multiplexing scheme, the Relative Carrier Leakage Power requirements specified in Table 6.4.2.2-1 which is defined in clause 6.4.2.2 apply per layer. The requirements shall be met with the UL MIMO configurations specified in Table 6.2J.1D-1.

6.4J.2D.3	In-band emissions for ATG UL MIMO

For ATG UE with two transmit antenna connectors or two groups of TAB connectors (each of which supporting one layer) in closed-loop spatial multiplexing scheme, the In-band Emission requirements specified in Table 6.4.2.3-1 which is defined in clause 6.4.2.3 apply at each transmit antenna connector or each TAB connector. The requirements shall be met with the UL MIMO configurations specified in Table 6.2J.1D-1.

6.4J.2D.4	EVM equalizer spectrum flatness for ATG UL MIMO

For ATG UE with two transmit antenna connectors or two groups of TAB connectors (each of which supporting one layer) in closed-loop spatial multiplexing scheme, the EVM Equalizer Spectrum Flatness requirements specified in clause 6.4.2.4 apply per layer. The requirements shall be met with the UL MIMO configurations specified in Table 6.2J.1D-1.

6.4J.3D	Time alignment error for ATG

6.4J.3D.1	Time alignment error for ATG UL MIMO

For ATG UE with two transmit antenna connectors or two groups of TAB connectors (each of which supporting one layer) supporting UL MIMO, this requirement applies to frame timing differences between transmissions on two transmit antenna connectors or TAB connectors. The time alignment error (TAE) is defined as the average frame timing difference between any two transmissions on different transmit antenna connectors or TAB connectors belonging to different groups (each of which supporting one layer).

For ATG UE with two transmit antenna connectors or two groups of TAB connectors (each of which supporting one layer), the Time Alignment Error (TAE) shall not exceed 130 ns.

6.4J.3D	Requirement for ATG coherent UL MIMO

For ATG UE with two transmit antenna connectors or two groups of TAB connectors (each of which supporting one layer), the requirements for coherent UL MIMO as specified in clause 6.4D.4 apply.

<<End of Change#5>>

<< Start of Change#6>>

6.5J	Output RF spectrum emissions for ATG

6.5J.1	Occupied bandwidth for ATG

The requirements for occupied bandwidth in clause 6.5.1 apply. For ATG UE, the requirements for occupied bandwidth are defined at each transmit antenna connector or each TAB connector.

6.5J.1D	Occupied bandwidth for ATG UL MIMO

For ATG UE supporting UL MIMO, the requirements for occupied bandwidth apply to the sum of the powers from all UE transmit antenna connectors or all UE TAB connectors. The occupied bandwidth is defined as the bandwidth containing 99 % of the total integrated mean power of the transmitted spectrum on the assigned channel at each transmit antenna connector or each TAB connector.

For ATG UE with two transmit antenna connectors or two groups of TAB connectors (each of which supporting one layer) in closed-loop spatial multiplexing scheme, the occupied bandwidth shall be less than the channel bandwidth specified in Table 6.5.1-1. The requirements shall be met with UL MIMO configurations described in clause 6.2J.1D.

If ATG UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission with precoding matrix W=1 [6.3.1.5 TS 38.211], the requirements in clause 6.5J.1 apply when TxD is not indicated.

6.5J.2	Out of band emission for ATG

6.5J.2.1	General

This clause contains requirements for out of band emissions for ATG UE, the requirement defined in general part of clause 6.5.2.1 should apply.

6.5J.2.2	Spectrum emission mask

If the actual transmission power of ATG UE is less than or equal to 31dBm, the requirements for spectrum emission mask in clause 6.5.2.2 apply; if the actual transmission power of ATG UE is larger than 31dBm, the requirements of spectrum emission mask in clause 6.5.2.2 shall be relaxed with scaling factor equal to (the actual transmission power minus 31) dB. For ATG UE, the requirements for spectrum emission mask are defined as the sum of the emissions from all UE transmit antenna connectors or all TAB connectors.

NOTE:	This scaling factor is only applicable to ATG airborne UE.

6.5J.2.3	Adjacent channel leakage ratio

NR Adjacent Channel Leakage power Ratio (NRACLR) is the ratio of the filtered mean power centred on the assigned NR channel frequency to the filtered mean power centred on an adjacent NR channel frequency at nominal channel spacing. For ATG UE, the requirements for ACLR are defined as the sum of the emissions from all UE transmit antenna connectors or all TAB connectors.

The assigned NR channel power and adjacent NR channel power are measured with rectangular filters with measurement bandwidths specified in Table 6.5J.2.3-1.

If the measured adjacent channel power is greater than –50 dBm then the NRACLR shall be higher than the value 30dBc.

Table 6.5J.2.3-1: NR ACLR measurement bandwidth

| Channel bandwidth | (MHz) | 5,10,15,20,25,30,35,40,45,50 | 60,70,80,90,100 |
| --- | --- | --- | --- |
| REF_SCS | (kHz) | 15 | 30 |
| NR ACLR measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |  |
| NOTE: “NRB” in the formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. |  |  |  |

6.5J.2D	Out of band emission for ATG with UL MIMO

For ATG UE supporting UL MIMO or uplink full power transmission (ULFPTx) for UL MIMO, the requirements for Out of band emissions resulting from the modulation process and non-linearity in the transmitters is defined as the sum of the emissions from all transmit antenna connectors or all TAB connectors.

For ATG UEs with two transmit antenna connectors or two groups of TAB connectors (each of which supporting one layer) in closed-loop spatial multiplexing scheme, the requirements in subclause 6.5J.2 apply. The requirements shall be met with UL MIMO configurations described in clause 6.2J.1D.

For ATG UE support uplink full power transmission (ULFPTx) for UL MIMO, the requirements in clause 6.5J.2 shall apply. The requirements shall be met with the PUSCH configurations specified in Table 6.2J.1D-2, based upon UE’s support of uplink full power transmission mode.

If ATG UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission with precoding matrix W=1 [6.3.1.5 TS 38.211], the requirements in clause 6.5J.2 apply when TxD is not indicated.

6.5J.3	Spurious emissions for ATG

The requirements for spurious emission in general part of clause 6.5.3.0 and clause 6.5.3.1 apply. For ATG UE, the requirements for Spurious emissions are defined as the sum of the emissions from all UE transmit antenna connectors or all TAB connectors.

6.5J.3D	Spurious emissions for ATG with UL MIMO

For ATG UE supporting UL MIMO or uplink full power transmission (ULFPTx) for UL MIMO, the requirements for Spurious emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emissions, intermodulation products and frequency conversion products is defined as the sum of the emissions from all transmit antenna connectors or all TAB connectors.

For ATG UEs with two transmit antenna connectors or two groups of TAB connectors (each of which supporting one layer) in closed-loop spatial multiplexing scheme, the requirements specified in subclause 6.5J.3 apply. The requirements shall be met with the UL MIMO configurations described in clause 6.2J.1D.

For ATG UE support uplink full power transmission (ULFPTx) for UL MIMO, the requirements in clause 6.5J.3 shall apply. The requirements shall be met with the PUSCH configurations specified in Table 6.2J.1D-2, based upon UE’s support of uplink full power transmission mode.

If ATG UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission with precoding matrix W=1 [6.3.1.5 TS 38.211], the requirements in clause 6.5J.3 apply when TxD is not indicated.

<<End of Change#6>>

<< Start of Change#7>>

7.3J	Reference sensitivity for ATG

7.3J.1	General

For ATG UE with multiple omni-directional antennas not indicating the capability antennaArrayType-r18, the reference sensitivity power level REFSENS is the minimum mean power per polarization at antenna connector, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

For ATG UE with antenna array indicating the capability antennaArrayType-r18,the reference sensitivity power level REFSENS is the minimum mean power per polarization at TAB antenna connector, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

7.3J.2	Reference sensitivity power level

For a ATG UE(s) equipped with 2 Rx antenna ports, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in Annex A.5.1.1) with parameters specified in Table 7.3.2-1a and Table 7.3.2-1b for the applicable operating bands.

For ATG UE(s) equipped with 4 Rx antenna ports, reference sensitivity for 2Rx antenna ports shall be modified by the amount given in ΔRIB,4R in Table 7.3.2-2 for the applicable operating bands.

The reference sensitivity (REFSENS) requirement for a ATG UE shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3.2-3.

### 7.3J.2A	Reference sensitivity power level for ATG CA

#### 7.3J.2A.1	Reference sensitivity power level for ATG intra-band contiguous CA

For ATG UE intra-band contiguous carrier aggregation, the throughput of each component carrier shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3.2-1a, Table 7.3.2-1b, Table 7.3.2-2, Table 7.3.2-2a, and Table 7.3.2-3.

For UE(s) supporting one uplink carrier, the uplink configuration of the PCC shall be in accordance with Table 7.3.2-3 and the downlink PCC carrier center frequency shall be configured closer to uplink operating band than any of the downlink SCC center frequency.

#### 7.3J.2A.2	Reference sensitivity power level for ATG inter-band CA

For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one NR band the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2 and A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 with parameters specified in Table 7.3.2-1a, Table 7.3.2-1b, Table 7.3.2-2, Table 7.3.2-2a and Table 7.3.2-3. The reference sensitivity is defined to be met with all downlink component carriers active and one of the uplink carriers active. Exceptions to reference sensitivity are allowed in accordance with clause 7.3J.2A.3.

The reference sensitivity exceptions due to cross band isolation is applicable to the UL aggressor band configured with either one Tx antenna connector or two Tx antenna connectors with UL MIMO.

### 7.3J.3A	ΔRIB,c for ATG CA

For ATG UE supporting inter-band carrier aggregation, the minimum requirement for reference sensitivity in clause 7.3A.2 shall be increased by the amount given by ΔRIB,c defined in clause 7.3A.3.2 for the applicable operating bands. Unless otherwise stated, ΔRIB,c is set to zero.

### 7.3J.4A	Reference sensitivity exceptions due to cross band isolation for ATG CA

Sensitivity degradation is allowed for a band if it is impacted by UL of another band part which belongs to NR band of the same NR CA configuration due to cross band isolation issues. The reference sensitivity degradation for the victim band due to cross band isolation is specified only for the specific uplink and downlink test points specified in Table 7.3J.4A-1, and the reference sensitivity degradation values according to UL output power are specified in Table 7.3J.4A-1a.

Table 7.3J.4A-1: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from an ATG UE aggressor NR UL band for NR CA in FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n39 | n3 | 1900 | 30 | 15 | 160 (RBstart=0) | 1817.5 | 25 | A11 | >ACLR2 |
| n3 | n39 | 1722.5 | 25 | 15 | 50 (RBstart=83) | 1900 | 30 | A22 | >ACLR2 |
| NOTE1: MSD is A1 value corresponding to UL output power in Table 7.3J.2A.4-1a. NOTE2: MSD is A2 value corresponding to UL output power in Table 7.3J.2A.4-1a. NOTE3: MSD only applies for ATG UE targeted for CPE form factor. |  |  |  |  |  |  |  |  |  |

Table 7.3J.4A-1a: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from an ATG UE aggressor for different NR UL output powers

| UL Output power (dBm) | A1 (dB) | A2 (dB) |
| --- | --- | --- |
| 23 | 4.4 | 1.0 |
| 24 | 5.0 | 1.2 |
| 25 | 5.4 | 1.4 |
| 26 | 5.9 | 1.6 |
| 27 | 6.5 | 1.8 |
| 28 | 7.0 | 2.1 |
| 29 | 7.7 | 2.4 |
| 30 | 8.4 | 2.7 |
| 31 | 9.1 | 3.1 |
| 32 | 9.9 | 3.5 |
| 33 | 10.8 | 4.0 |
| 34 | 11.6 | 4.5 |
| 35 | 12.5 | 5.0 |
| 36 | 13.4 | 5.7 |
| 37 | 14.4 | 6.4 |
| 38 | 15.4 | 7.2 |
| 39 | 16.4 | 8.0 |
| 40 | 17.5 | 8.9 |

<<End of Change#7>>

<< Start of Change#8>>

7.4J	Maximum input level for ATG

7.4J.1	General

Maximum input level is defined as the maximum mean power received at the UE antenna port, at which the specified relative throughput shall meet or exceed the minimum requirements for the specified reference measurement channel. The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexs A.3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.4J-1.

Table 7.4J-1: Maximum input level for ATG

| Rx Parameter | Units | ATG UE Types |  | Reference measurement channel |
| --- | --- | --- | --- | --- |
|  |  | Omni-directional antenna: receiver characteristics specified at the antenna connector(s) | Antenna array: receiver characteristics specified at transceiver array boundary (TAB) connectors |  |
| Power in Transmission Bandwidth Configuration | dBm | -42 | -30 | A.3.2.3 or A.3.3.3 for 64 QAM |
|  |  | -44 | -32 | A.3.2.4 or A.3.3.4 for 256 QAM |
| The applicable channel bandwidths | MHz | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 with PCMAX_L,f,c as defined in clause 6.2J.2. |  |  |  |  |

7.4J.2A	Maximum input level for ATG CA

#### 7.4J.2A.1	Minimum requirement for ATG intra-band contiguous CA

For intra-band contiguous carrier aggregation maximum input level is defined as the maximum mean power received at the UE antenna port, over the Transmission bandwidth configuration of each CC.

The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexs A.3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.4J.2A-1 for each component carrier.

Table 7.4J.2A-1: Maximum input level for ATG intra-band contiguous CA

| Rx Parameter | Units | ATG UE Types |  |
| --- | --- | --- | --- |
|  |  | Omni-directional antenna | Antenna array |
| Power in largest transmission bandwidth configuration CC, Plargest BW | dBm | -422 | -302 |
|  |  | -443 | -323 |
| Power in each other CC | dBm | Plargest BW +10*log{(NRB,c*SCSc)/(NRB,largest BW*SCSlargest BW)} |  |
| The applicable NR CA Bandwidth Class | MHz | C |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 with PCMAX_L,f,c as defined in clause 6.2J.2. NOTE 2: Reference measurement channel is A.3.2.3 or A.3.3.3 for 64 QAM. NOTE 3: Reference measurement channel is A.3.2.4 or A.3.3.4 for 256 QAM. |  |  |  |

#### 7.4J.2A.1	Minimum requirement for ATG inter-band CA

For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one NR band, the maximum input level is defined with the uplink active on the band(s) other than the band whose downlink is being tested. For NR CA configurations including an operating band without uplink band or an operating band with an unpaired DL part (as noted in Table 5.2-1), the requirements for all downlinks shall be met with the single uplink carrier active in each band capable of UL operation. The UE shall meet the requirements specified in clause 7.4A.1 for each component carrier while all downlink carriers are active.

The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexs A.3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1) for each component carrier.

<<End of Change#8>>

<< Start of Change#9>>

7.5J	Adjacent channel selectivity for ATG

### 7.5J.1	General

Adjacent channel selectivity (ACS) is a measure of a receiver's ability to receive an NR signal at its assigned channel frequency in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel. ACS is the ratio of the receive filter attenuation on the assigned channel frequency to the receive filter attenuation on the adjacent channel(s).

The UE shall fulfil the minimum requirements specified in Table 7.5J-1 for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz and the minimum requirements specified in Table 7.5J-2 for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz. These requirements apply for all values of an adjacent channel interferer up to -42 dBm with omni-directional antenna and -30dBm with antenna array for any SCS specified for the channel bandwidth of the wanted signal. However, it is not possible to directly measure the ACS; instead the lower and upper range of test parameters are chosen as in Table 7.5J-3 and Table 7.5J-4 for verification of the requirements specified in Table 7.5J-1, and as in Table 7.5J-5 and Table 7.5J-6 for verification of the requirements specified in Table 7.5J-2. For these test parameters, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1).

Table 7.5J-1: ACS for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| RX parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 5, 10 | 15 | 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| ACS | dB | 33 | 30 | 27 – 10log10(BWChannel /20) |
| NOTE1: ACS value is rounded to the next higher 0.5dB value |  |  |  |  |

Table 7.5J-2: ACS for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| RX parameter | Units | Channel bandwidth (MHz) |
| --- | --- | --- |
|  |  | 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| ACS | dB | 33 |

Table 7.5J-3: Test parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz, case 1

| RX parameter | Units | Channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 5, 10 | 15 | 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| Power in transmission bandwidth configuration | dBm | REFSENS + 14 dB |  |  |  |
| Pinterferer4 | dBm |  | REFSENS + 45.5 dB | REFSENS + 42.5 dB | REFSENS + 39.5 – 10log10(BWChannel /20) |
| BWinterferer | MHz |  | 5 |  |  |
| Finterferer (offset) | MHz |  | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2J.2 NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the NR interferer RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. NOTE 4: 10log10(x) is rounded to the next higher 0.5dB value. |  |  |  |  |  |

Table 7.5J-4: Test parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz, case 2

| RX parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 5, 10 | 15 | 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| Power in transmission bandwidth configuration4 | dBm | -73.55 -61.56 | -70.55 -58.56 | -67.5 + 10log10(BWChannel /20)5 -55.5 + 10log10(BWChannel /20)6 |
| Pinterferer | dBm | -425 -306 |  |  |
| BWinterferer | MHz | 5 |  |  |
| Finterferer (offset) | MHz | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |  |  |
| NOTE 1: The transmitter shall be set to 24 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2J.2. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 NOTE 4: 10log10(x) is rounded to the next higher 0.5dB value. NOTE 5: Pinterferer shall be set to -42dBm for omni-directional antenna. NOTE 6: Pinterferer shall be set to -30dBm for antenna array. |  |  |  |  |

Table 7.5J-5: Test parameters for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, case 1

| RX parameter | Units | Channel bandwidth (MHz) |
| --- | --- | --- |
|  |  | 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| Power in transmission bandwidth configuration | dBm | REFSENS + 14 dB |
| Pinterferer | dBm | REFSENS + 45.5 dB |
| BWinterferer | MHz | BWChannel |
| Finterferer (offset) | MHz | BWChannel / -BWChannel |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2J.2 NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |

Table 7.5J-6: Test parameters for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, case 2

| RX parameter | Units | Channel bandwidth (MHz) |
| --- | --- | --- |
|  |  | 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| Power in transmission bandwidth configuration | dBm | -73.54 -61.55 |
| Pinterferer | dBm | -424 -305 |
| BWinterferer | MHz | BWChannel |
| Finterferer (offset) | MHz | BWChannel / -BWChannel |
| NOTE 1: The transmitter shall be set to 24 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2J.2 NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. NOTE 4: Pinterferer shall be set to -42dBm for omni-directional antenna. NOTE 5: Pinterferer shall be set to -30dBm for antenna array. |  |  |

### 7.5J.2A	Adjacent channel selectivity for ATG CA

#### 7.5J.2A.1	Minimum requirement for ATG intra-band contiguous CA

For intra-band contiguous carrier aggregation the downlink SCC(s) shall be configured at nominal channel spacing to the PCC. The UE shall fulfil the minimum requirement specified in Table 7.5J.2A.1-1 for an adjacent channel interferer on either side of the aggregated downlink signal at a specified frequency offset and for an interferer power up to -42 dBm for omni-directional antenna or -30dBm for antenna array.

The throughput of each carrier shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Tables 7.5J.2A.1-2 and 7.5J.2A.1-3.

Table 7.5J.2A.1-1: ACS for ATG intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

|  |  | NR CA bandwidth class |
| --- | --- | --- |
| Rx Parameter | Units | C |
| ACS | dB | 33.0 |

Table 7.5J.2A.1-2: Test parameters for ATG intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, case 1

| Rx Parameter | Units | NR CA bandwidth class |
| --- | --- | --- |
|  |  | C |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + 14 dB |
| PInterferer | dBm | Aggregated power + 31.5 dB |
| BWInterferer | MHz | BWchannel CA |
| FInterferer (offset) | MHz | BWchannel CA/2 + Foffset / -BWchannel CA/2 + Foffset |
| NOTE 1: The transmitter shall be set to 24 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2J.2 NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the carrier closest to the interferer in MHz. The interferer is an NR signal with an SCS equal to that of the closest carrier. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |

Table 7.5J.2A.1-3: Test parameters for ATG intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, case 2

| Rx Parameter | Units | NR CA bandwidth class |
| --- | --- | --- |
|  |  | C |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | -73.54 -61.55 |
| PInterferer | dBm | -424 -305 |
| BWInterferer | MHz | BWchannel CA |
| FInterferer (offset) | MHz | BWchannel CA / -BWchannel CA |
| NOTE 1: The transmitter shall be set to 24 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2J.2 NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the carrier closest to the interferer in MHz. The interferer is an NR signal with an SCS equal to that of the closest carrier. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. NOTE 4: Pinterferer shall be set to -42dBm for omni-directional antenna. NOTE 5: Pinterferer shall be set to -30dBm for antenna array. |  |  |

#### 7.5J.2A.2	Minimum requirement for ATG inter-band contiguous CA

For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one NR band, the adjacent channel requirements are defined with the uplink active on the band(s) other than the band whose downlink is being tested. For NR CA configurations including an operating band without uplink operation or an operating band with an unpaired DL part (as noted in Table 5.2-1), the requirements for all downlinks shall be met with the single uplink carrier active in each band capable of UL operation. The UE shall meet the requirements specified in clause 7.5J.1 for each component carrier while all downlink carriers are active.

The throughput of each carrier shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1).

<<End of Change#9>>

<< Start of Change#10>>

7.6J	Blocking characteristics for ATG

7.6J.1	General

The blocking characteristic is a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency in the presence of an unwanted interferer on frequencies other than those of the spurious response or the adjacent channels, without this unwanted input signal causing a degradation of the performance of the receiver beyond a specified limit. The blocking performance shall apply at all frequencies except those at which a spurious response occurs.

7.6J.2	In-band blocking for ATG

For ATG UE, the in-band blocking requirement defined in clause 7.6.2 applies.

7.6J.2A	 In-band blocking for ATG CA

#### 7.6J.2A.1	In-band blocking for ATG Intra-band contiguous CA

For ATG UE supporting intra-band contiguous CA operation, the in-band blocking requirement defined in clause 7.6A.2.1 applies.

#### 7.6J.2A.2	In-band blocking for ATG Inter-band CA

For ATG UE supporting inter-band CA operation, the in-band blocking requirement defined in clause 7.6A.2.3 applies.

7.6J.3	Out-of-band blocking for ATG

For ATG UE, the out-of-band blocking requirement defined in clause 7.6.3 applies.

NOTE:	In 3GPP, the ATG UE out-of-band blocking specification is defined to ensure the telecommunication link and there may be other sources of interference and regulatory issues that need to be considered when designing ATG UE, i.e. avionic equipment.

7.6J.3A	Out-of-band blocking for ATG CA

NOTE:	In 3GPP, the ATG UE out-of-band blocking specification is defined to ensure the telecommunication link and there may be other sources of interference and regulatory issues that need to be considered when designing ATG UE, i.e. avionic equipment.

#### 7.6J.3A.1	Out-of-band blocking for ATG Intra-band contiguous CA

For ATG UE supporting intra-band contiguous CA operation, the out-of-band blocking requirement defined in clause 7.6A.3.1 applies.

#### 7.6J.3A.2	Out-band blocking for ATG Inter-band CA

For ATG UE supporting inter-band CA operation, the out-of-band blocking requirement defined in clause 7.6A.3.3 applies.

<<End of Change#10>>

<< Start of Change#11>>

7.7J	Spurious response for ATG

7.7J.1	General

For ATG UE, the spurious response defined in clause 7.7 applies.

7.7J.1A	Spurious response for ATG CA

#### 7.7J.1A.1	Spurious response for ATG intra-band contiguous CA

For ATG UE supporting intra-band contiguous CA operation, the spurious response defined in clause 7.7A.1 applies.

#### 7.7J.1A.2	Spurious response for ATG inter-band CA

For ATG UE supporting inter-band CA operation, the spurious response defined in clause 7.7A.3 applies.

<<End of Change#11>>

<< Start of Change#12>>

7.8J	Intermodulation characteristics for ATG

7.8J.1	General

Intermodulation response rejection is a measure of the capability of the receiver to receive a wanted signal on its

assigned channel frequency in the presence of two or more interfering signals which have a specific frequency relationship to the wanted signal.

7.8J.2	Wide band intermodulation for ATG

For ATG UE, the wide band intermodulation requirement defined in clause 7.8.2 applies.

7.8J.2A	Wide band intermodulation for ATG CA

7.8J.2A.1	Wide band intermodulation for ATG intra-band contiguous CA

For ATG UE supporting intra-band contiguous carrier aggregation, the wide band intermodulation requirement defined in clause 7.8A.2.1 applies.

7.8J.2A.2	Wide band intermodulation for ATG inter-band CA

For ATG UE supporting inter-band carrier aggregation, the wide band intermodulation requirement defined in clause 7.8A.2.3 applies.

<<End of Change#12>>

<< Start of Change#13>>

7.9J	Spurious emissions for ATG

7.9J.1	General

For ATG UE, the spurious emissions as specified in clause 7.9 applies.

7.9J.1A	Spurious emissions for ATG CA

7.9J.1A.1	Spurious emissions for ATG inter-band CA

For ATG UE supporting inter-band carrier aggregation including an operating band without uplink band, the UE shall meet the Rx spurious emissions requirements specified in clause 7.9 for each component carrier while all downlink carriers are active.

<<End of Change#13>>