3GPP TSG RAN meeting #109 								RP-252444

Beijing, China, September 15-18,

## Status Report to TSG

Agenda item:			9.3.1.3

| WI / SI Name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_AIML_air_Ph2 |  |  |  |  |
| Unique ID | 1080085 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-251870 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: NA | Core part: March ‘27 | Performance part: Sept ‘27 | Testing part: NA |  |
| Overall Completion level | Study Item: NA | Core part: 4% | Performance Part: 0% | Testing part: NA |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | TSG RAN WG1 |
| --- | --- | --- |
| Rapporteur | Name | Juan Montojo (RAN1); Xiaofeng Liu (RAN4); Marco Belleschi (RAN2); Anna Pantelidou (RAN3). |
|  | Company | Qualcomm; CAICT; Ericsson; Nokia |
|  | Email | juanm@qti.qualcomm.com; liuxiaofeng1@caict.ac.cn; marco.belleschi@ericsson.com; anna.pantelidou@nokia.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | Yes |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

RAN3 TUs corrected per RAN3 Chair guidance: 0.5 TU/meeting -> 0.25 TU/meeting.

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

Agreement:

For CSI feedback via two-sided model, support at least precoding matrix as target CSI type.

- FFS whether consider raw channel matrix as further improvement, starting with evaluations, e.g. potential fusion with SRS measurement (SRS period/hopping, DL/UL reciprocity), frequency granularity of channel matrix, number of Rx antennas, etc.

Agreement

Specify the precoding matrix feedback via two-sided model as follows: For a certain layer l and rank v,

- A precoding matrix is mapped with a latent message , with l = 1, ..., v where l is the layer index.

  - The mapping is subject to [at least] a pairing ID configured by high-layer signalling.

  - FFS dzl,v is common or specific for different layers or ranks

  - Note: the terminology “latent message” is used for discussion purposes, the detailed name can be discussed further

- zl is quantized and mapped to bit sequence before reporting.

Agreement

For precoding matrix feedback, for a certain layer l of rank ??, the latent message zl contains dzl,?? real values

- If scalar quantization (SQ) supported, it refers to the quantization of each real value independently,

  - The payload size for layer l and rank ?? is determined by , where Ql,?? is the number of bits per scalar.

- If vector quantization (VQ) is supported, it refers to the quantization of a segment of L (L>1) consecutive real values jointly, and the segment(s) are separately quantized.

  - The payload size for layer l and rank ?? is determined by , where L is the length of the vector segment,  Ql,?? is the number of bits per segment

  - Note: Combining two real-values as a complex value and performing amplitude-phase quantization is precluded.

Agreement:

For precoding matrix feedback, consider the following aspects for payload determination

- Support only SQ, or only VQ or both

- dzl,?? and / or Ql,?? and / or L are fixed values or determined from multiple values based on NW configuration or UE reporting.

- dzl,??and / or Ql,?? is layer-common or layer-specific, rank-common or rank-specific.

- If configured, dzl,??and / or Ql,?? and / or L above are configured separately from pairing ID, or configured by Pairing ID.

  - The configuration may be via other parameters which are used to derive dzl,??, L and Ql,??.

Note: Consider limiting the total number of possible payload sizes considering the inter-vendor training challenges.

Note: dzl,??, Ql,?? and L are independent on subband and port configurations,

- FFS each combination of dzl,??, Ql,?? and L is applicable to a certain set of subband and port configurations

Agreement:

For inference report configuration, support separate configuration of subband, Tx port and payload size from pairing ID.

Agreement:

For quantization codebook used for quantizing the feedback,

- For inter-vendor collaboration Direction A, select one of the following:

  - Alt1: Exchange quantization codebook from NW-side to UE-side along with each exchanged dataset or model parameters

  - Alt2: Use a standardized quantization codebook decided by RAN1.

Agreement:

For model pairing procedure for inference, consider the applicability report procedure of Rel-19 AI/ML beam management for UE side model as a starting point.

- Pairing ID(s) (i.e., ID for pairing related discussion) is indicated (e.g., in CSI-reportConfig) in Step 3.

  - FFS whether/how to link the Pairing ID with the inter-vendor collaboration

  - FFS whether/how Paring ID(s) are reported in Step 4

- FFS whether to support both Option A (CSI-ReportConfig for inference configuration) and Option B (sets of inference related parameters for applicability report only).

  - FFS CSI compression specific inference configuration or inference related parameters.

Agreement:

For NW side data collection of CSI compression, discuss the content and format of the collected data by assuming higher layer based signalling.

- Details on signaling is up to RAN2.

Agreement:

To enable UE side data collection, CSI-ReportConfig can be used for configuring the resources for data collection purpose without CSI report

- P-CSI-RS and /SP CSI-RS are supported

Agreement:

For NW side data collection, study the solution for quantizing Target CSI, from the aspects of overhead, quantization loss/performance, and complexity. E.g.,:

- Option 0: Reusing legacy e-Type II codebook

- Option 1: Scalar quantization to the Target CSI. FFS number of bits for real/imaginary, whether coefficients for quantization are obtained from transformation of Target CSI.

- Option 2: Enhanced methods to reduce the SVD/EVD decomposition complexity, reduce frequency/spatial/beam domain basis searching, with potentially relaxed overhead, e.g., Type II like codebook with scalar quantized W2 similar to Option 1 scalar quantization.

- Option 3: Enhanced method for improved overhead saving, e.g., reporting the dominant eigenvalues/eigenvectors of per layer precoding matrix.

- Option 4: eType II like codebook with new parameter values (e.g. larger L, pv, beta, amplitude, phase)

- Note: the quantizaton format should consider the maximum payload size per reporting (e.g., 9KBytes) of the current higher layer signaling.

- Note: At least take precoding matrix as Target CSI type. FFS channel matrix.

- FFS: EVM, metric, benchmark (e.g., FP32, eT2 with PC6/8), by reusing R18 study EVM as baseline.

Agreement:

For Option 4-1 under Direction A in AI/ML based CSI compression, support at least target CSI and CSI feedback in the exchanged dataset(s).

- FFS: Target CSI type and format

- FFS: CSI feedback type and format

- FFS: Association between Target CSI and CSI feedback, including mapping for different number of Tx port, number of sub bands, and CSI payload size.

Agreement:

For Option 4-1 under Direction A in AI/ML based CSI compression, consider  the following methods for performance target in the exchanged dataset,

- SGCS

  - FFS: Average SGCS

  - FFS: SGCS values at X-percentiles

- NMSE:

  - FFS: When the exchanged CSI feedback is the floating-point values at the input of quantization

  - FFS: When the exchanged CSI feedback is the binary bit sequence at the output of quantization, the binary sequence will be mapped back to the floating-point values via quantization codebook

- FFS: Multiple performance targets for different layer when the target CSI type is precoding matrix, different configurations such as antenna ports, subband configuration and payload size configuration

Agreement:

For Option 4-1 under Direction A in AI/ML based CSI compression, for CSI feedback type and format, down select one of the following options:

- Option 1: The exchanged CSI feedback is the latent message before quantization.

- Option 2: The exchanged CSI feedback is the binary sequence at the output of quantization.

- Note: Quantization codebook is exchanged in addition if not specified in the spec.

- Note: Whether or not using the Quantization codebook for model training, is up to UE implementation.

Agreement:

For Option 4-1 under Direction A in AI/ML based CSI compression, support exchange of single pairing ID along with dataset exchange.

- One pairing ID is assigned to one dataset.

  - The dataset can have different number of Tx port, number of subbands, and CSI payload size configurations (including different quantization codebooks if needed).

- From RAN1 perspective, the uniqueness of the pairing ID needs further studied.

- FFS: the impact on pairing ID(s) when additional data samples are added to an exchanged dataset, if supported.

Agreement:

For Option 4-1 under Direction A in AI/ML based CSI compression,  if the quantization codebook for CSI feedback is not fixed by the specification, support exchange of quantization codebook along with dataset exchange,

- FFS: Quantization codebook exchange method (e.g, look up table, , or how to exchange quantization codebook related information, or other methods)

- FFS: Common or different quantization codebook for different CSI payload size

- Note: leveraging the discussion in agenda item 10.1.1.1 when applied

#### 2.1.2	Remaining Open issues

Specification support for CSI feedback enhancement (two-sided model):

Specification support for inter-vendor training collaboration for two-sided AI/ML models focusing on standardized dataset format/content plus dataset exchange (“Direction A, sub-option 4-1”) and, subject to RAN4 progress, on specification of standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing) leveraging defined/reference model of “Direction C” and taking RAN1 scalability study outcome into account.

## 2.2	RAN2

#### 2.2.1	Agreements

Not started.

#### 2.2.2	Remaining Open issues

## 2.3	RAN3

#### 2.3.1	Agreements

Not started.

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

Not started.

#### 2.4.2	Remaining Open issues

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

## 4.1	RAN1

R1-2506206	Work plan for NR_AIML_air_Ph2	Qualcomm Incorporated

R1-2505148	Discussion on inference related aspects for CSI spatial/frequency compression without temporal aspects (“Case 0”)	FUTUREWEI

R1-2505239	AI/ML CSI Spatial/Frequency Compression: Inference Aspects	InterDigital, Inc.

R1-2505133	Inference related aspects of AI/ML for CSI compression	Ericsson

R1-2505161	Discussion on AIML for CSI compression inference related aspects	Spreadtrum, UNISOC

R1-2505199	Inference related aspects for CSI compression	Huawei, HiSilicon

R1-2505260	Inference for AI/ML based CSI Compression	Google

R1-2505299	Specification support on inference related aspects of AI/ML-based CSI compression	CATT

R1-2505405	Discussion on inference related aspects for CSI compression	vivo

R1-2505452	Discussion on inference related aspects of two-sided AI/ML model based CSI feedback	Xiaomi

R1-2505477	Discussion on interference related aspects for CSI compression	TCL

R1-2505491	Discussion on inference related aspects of CSI compression	ZTE Corporation, Sanechips

R1-2505573	Views on inference related aspects of CSI compression	Samsung

R1-2505617	Discussion on inference related aspects for CSI compression	KT Corp.

R1-2505670	Discussion on Inference related aspects for AI CSI compression	Ofinno

R1-2505687	Inference related aspects for CSI compression	Lenovo

R1-2505699	Discussion on inference aspects for AI/ML-based CSI compression	Panasonic

R1-2505746	Inference related aspects for AI/ML CSI compression	OPPO

R1-2505802	CSI Compression: Inference Related Aspects	Nokia

R1-2505819	Discussion on inference related aspects for CSI compression	LG Electronics

R1-2505902	On inference related aspects for AI based CSI spatial/frequency domain compression	Apple

R1-2505935	Discussion on inference aspects of CSI compression	NEC

R1-2505944	Discussions on Inference Related Aspects for CSI Compression	Sharp

R1-2505945	Discussion on inference of AI/ML CSI compression	Transsion Holdings

R1-2505962	Discussion on inference related aspects in CSI compression with AI/ML	Fujitsu

R1-2506029	CSI spatial/frequency compression without temporal aspects (“Case 0”)- Inference related aspects			MediaTek Inc.

R1-2506057	Discussion on inference related aspects of CSI compression	ETRI

R1-2506088	Discussion on inference related aspects of CSI compression	CMCC

R1-2506122	Discussion on inference aspects of CSI compression	KAIST

R1-2506149	Discussion on Inference related aspects for AI/ML based CSI compression	ITL

R1-2506207	Specification of inference aspects of AIML CSI compression	Qualcomm Incorporated

R1-2506294	Discussion on the inference-related aspects of AI/ML CSI compression	NTT DOCOMO, INC.

R1-2506353	Discussion on AI/ML CSI Compression inference aspects	CEWiT

R1-2506377	Discussion on Inference-Related Aspects of Case-0 CSI Compression	Rakuten Mobile, Inc

R1-2506495	FL summary #1 on AIML inference related aspects	Moderator (Qualcomm)

R1-2506496	FL summary #2 on AIML inference related aspects	Moderator (Qualcomm)

R1-2506497	FL summary #3 on AIML inference related aspects	Moderator (Qualcomm)

R1-2506498	FL summary #3 on AIML inference related aspects	Moderator (Qualcomm)

R1-2505700	Discussion on other aspects for AI/ML-based CSI compression	Panasonic

R1-2505574	Views on other aspects of CSI compression	Samsung

R1-2505134	Other aspects of AI/ML for CSI compression	Ericsson

R1-2505149	Discussion on other aspects for CSI spatial/frequency compression without temporal aspects (“Case 0”)	FUTUREWEI

R1-2505162	Discussion on AIML for CSI compression other aspects	Spreadtrum, UNISOC

R1-2505200	Other aspects for CSI compression	Huawei, HiSilicon

R1-2505240	AI/ML CSI Spatial/Frequency Compression: Other Aspects	InterDigital, Inc.

R1-2505261	Other Aspects for AI/ML based CSI Compression	Google

R1-2505300	Specification support on other aspects of AI/ML-based CSI compression	CATT

R1-2505406	Discussion on other aspects for CSI compression	vivo

R1-2505453	Discussion on other aspects of CSI spatial/frequency compression	Xiaomi

R1-2505478	Discussion on other aspects for CSI compression	TCL

R1-2505492	Discussion on other aspects of CSI compression	ZTE Corporation, Sanechips

R1-2505685	Other aspects	Tejas Network Limited

R1-2505688	Other aspects for CSI compression	Lenovo

R1-2505747	Other aspects for AI/ML CSI compression	OPPO

R1-2505803	CSI Compression: Other Aspects	Nokia

R1-2505820	Discussion on other aspects for CSI compression	LG Electronics

R1-2505903	On other aspects for AI based CSI spatial/frequency domain compression	Apple

R1-2505930	Discussion on other aspects of CSI compression	NEC

R1-2505963	Discussion on other aspects of CSI compression	Fujitsu

R1-2506030	CSI spatial/frequency compression without temporal aspects (“Case 0”)- Other aspects	MediaTek Inc.

R1-2506058	Discussion on other aspects of CSI compression	ETRI

R1-2506089	Discussion on other aspects of CSI compression	CMCC

R1-2506108	Discussions on Performance Monitoring for AI/ML CSI Compression	Sony

Withdrawn

R1-2506208	Specification of other aspects of AIML CSI compression	Qualcomm Incorporated

R1-2506249	Discussions on other aspects for NR air interface enhancements	Sharp

R1-2506295	Discussion on other aspects of AI/ML CSI compression	NTT DOCOMO, INC.

R1-2506315	Other Aspects of CSI spatial/frequency compression	Indian Institute of Tech (M)

R1-2506339	Discussion on AIML based CSI compression	ASUSTeK

R1-2506354	Discussion on AI/ML CSI Compression  other aspects	CEWiT

R1-2506378	Discussion on Data-collection / Monitoring / Model-pairing	Rakuten Mobile, Inc

R1-2506386	Other aspects for CSI compression	IIT Kanpur

R1-2506482	FL summary#2 for other aspects of CSI compression	Moderator (Huawei)

R1-2506483	FL summary#3 for other aspects of CSI compression		Moderator (Huawei)

R1-2506484	FL summary#4 for other aspects of CSI compression		Moderator (Huawei)

R1-2506485	FL summary#5 for other aspects of CSI compression		Moderator (Huawei)

R1-2505964	Discussion on inter-vendor training collaboration for two-sided AI/ML models	Fujitsu

R1-2506090	Discussion on inter-vendor training collaboration for CSI compression	CMCC

R1-2505135	Inter-vendor training collaboration for two-sided AI/ML models	Ericsson

R1-2505150	Discussion on inter-vendor training collaboration for two-sided AI/ML models	FUTUREWEI

R1-2505163	Discussion on Inter-vendor training collaboration for two-sided AI/ML models	Spreadtrum, UNISOC

R1-2505201	Inter-vendor training collaboration for two-sided AI/ML models	Huawei, HiSilicon

R1-2505241	AI/ML CSI Spatial/Frequency Compression: Inter-vendor Collaboration	InterDigital, Inc.

R1-2505262	Inter-Vendor Collaboration for AI/ML based CSI Compression	Google

R1-2505301	Discussion on Inter-vendor training collaboration for CSI compression	CATT

R1-2505407	Discussion on inter-vendor training collaboration for two-sided AI/ML models	vivo

R1-2505454	Discussion on inter-vendor training collaboration for two-sided AI/ML models	Xiaomi

R1-2505479	Discussion on inter-vendor collaboration for CSI compression	TCL

R1-2505493	Discussion on inter-vendor training collaboration for two-sided AI/ML models	ZTE Corporation, Sanechips

R1-2505575	Views on inter-vendor training collaboration for two-sided AI/ML models	Samsung

R1-2505642	Discussion on Inter-vendor Training Collaboration for AI/ML models	NEC

R1-2505689	Inter-vendor training collaboration for two-sided AI/ML models	Lenovo

R1-2505701	Discussion on inter-vendor training collaboration for two-sided AI/ML models	Panasonic

R1-2505748	Inter-vendor training collaboration for AI/ML CSI compression	OPPO

R1-2505804	Inter-vendor training collaboration for two-sided AI/ML models	Nokia

R1-2505806	Reference Model for Data Generation	NTU

R1-2505821	Discussion on Inter-vendor training collaboration for two-sided models	LG Electronics

R1-2505904	Inter-vendor training collaboration for two sided AI/ML models	Apple

R1-2506012	Discussions on Inter-vendor training collaboration	Sharp

R1-2506031	Inter-vendor training collaboration for two-sided AI/ML models	MediaTek Inc.

R1-2506059	Discussion on inter vendor training collaboration for two-sided AI/ML models	ETRI

R1-2506109	Discussion on Inter-Vendor Training collaboration for Two-Side AI/ML Models	Sony

R1-2506209	Inter-vendor training collaboration for two-sided CSI compression use case	Qualcomm Incorporated

R1-2506296	Discussion on the inter-vendor training collaborations for two-sided AI/ML models	NTT DOCOMO, INC.

R1-2506467  Summary #1 on other aspects of AI/ML for CSI enhancement	Moderator (Apple)

R1-2506468  Summary #2 on other aspects of AI/ML for CSI enhancement	Moderator (Apple)

R1-2506469  Summary #3 on other aspects of AI/ML for CSI enhancement	Moderator (Apple)

R1-2506470	Summary #4 on other aspects of AI/ML for CSI enhancement	Moderator (Apple)

## 4.2	RAN2

## 4.3	RAN3

## 4.4	RAN4

16.02.2024		minor adaptations for RAN #103

10.11.2023		minor adaptations for RAN #102

02.08.2023		minor adaptations for RAN #101

26.04.2023		minor adaptations for RAN #100

01.02.2023		minor adaptations for RAN #99

27.10.2022		minor adaptations for RAN #98e

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template