3GPP TSG RAN #109			RP-252471

Beijing, China, Sept. 15-18, 2025

Agenda Item:	8.2.2

Source: 	LG Electronics

Title: 	Discussion on study on 6G Radio

Document for:	Discussion and decision

Introduction

At the RAN1#122 meeting on Rel-20 Study of 6GR [1], the following conclusion was made on the frame structure agenda.

Conclusion - Numerologies for sensing is up to sensing agenda discussion.

Similar discussions were made on the waveform agenda, but no conclusion was made.

Discussion

- 2.1	Waveform and frame structure on ISAC

In RAN1#122 meeting, there were discussions on whether the waveform and the frame structure for ISAC would be discussed together with those for communication. Some companies prefer to discuss ISAC-related issues separately on ISAC agenda while other companies showed concerns on totally independent discussion on ISAC in a later stage.

The main concern comes from the possibility that both waveforms and associated frame structures may have impact to each other. For example, if the communication frame structure is designed only with some SCSs and a different SCS is required for sensing later, there could be an issue on whether or not the basic communication frame structure can afford the different SCS for sensing.

To resolve this issue, we believe that the waveform and frame structure for communication needs to be designed considering the coexistence with those for sensing from the beginning of 6GR framework discussion. It will save the effort and time for 6GR design by avoiding unexpected consideration later for adopting the sensing feature. In this sense, it would be better to study the impact of the coexistence between communication and sensing from the initial stage of 6GR design.

For this purpose, we suggest to start ISAC related discussion earlier than the suggested schedule, i.e., April 2026. This does not violate the plenary guidance to avoid duplicated discussions on the same agenda because no new waveform or frame structure will be discussed in 5G-A ISAC SI.

Proposal 1: For 6GR, start discussion on ISAC-related waveform and frame structure earlier than April 2026. November 2025 can be considered as a candidate for discussion.

Summary

In this contribution, we discussed the direction on waveform and frame structure discussion on ISAC in 6GR, and the following proposal was made:

Proposal 1: For 6GR, start discussion on ISAC-related waveform and frame structure earlier than April 2026. November 2025 can be considered as a candidate for discussion.

Reference

[1]	RP-251881, “New SID: Study on 6G Radio,” NTT DOCOMO (Moderator), 3GPP TSG RAN Meeting #108, Prague, Czech Republic, June 9-13, 2025.