3GPP TSG RAN Meeting #109	RP-252621

Beijing, China, 15th to 18th September 2025

Source:	Ericsson

Title:	RAN4 BS RF Rel-20 work packages

Agenda item:	9.1.4.2

Document for:	Approval

# Introduction

At the last RAN plenary meeting (RAN#108 in Prague) the discussion about RAN4 Rel-20 5G-Advanced work packages started. All the input proposals were collected and categorized into different groups with the aim to further describe justification and objectives at coming meetings, with the intention to define relevant Rel-20 RAN4 work items at the September plenary meeting (RAN#109 in China).

The following work package categorizes have been identified by RAN:

- UE RF centric evolution

- BS RF centric evolution

- UE OTA enhancement

- RRM enhancement

- Demodulation performance evolution

- NTN enhancements

- Cross RF and baseband topics

In this contribution we provide proposals for work packages to be included in Rel-20 5G-Advanced for BS RF centric evolution. In addition to proposals, also additional information is provided to further stimulate the discussion to establish clear objectives for each work package within Rel-20 timeframe. It is essential to find relevant scope of each work package to be able to fit it within RAN4 Rel-20 Time Unit (TU) budget.

# Discussion

From the moderator summary [1], the work packages related to BS RF evolution was collected in a list without priority of high-level candidate objectives for BS RF centric evolution and/or enhancement in Rel-20. The list identified in moderator summary is cleaned up with the intention to be clearer to progress the work to establish justification and complete description of objective. The BS RF centric list can be summarized as:

- Evolve and improve co-location requirement concept, requirement levels and test methodologies to resolve identified technical issues above 3 GHz. Work is ongoing in Rel-19, the outcome could be used as foundation for normative work in Rel-20.

Further discuss to include work related to re-evaluation of transmitter spurious emission levels for bands in the upper region of FR1. Depending on the outcome from ongoing Rel-19 work.

- Down select from the following four options:

  - Introduce new additional BS type (BS type 1-H enhanced) enabling technologies required for large BS antenna array with mix of analog and digital beamforming.

  - Define new requirement for SBFD-to-SBFD adjacent channel co-existence considering realistic deployment scenarios.

  - Neither (a) nor (b).

  - Both (a) and (b).

The intention is to use the list of candidate objectives as the starting point for 5G-Advanced BS RF centric evolution and/or enhancement in Rel-20, considering the TU splitting between 5G and 6G in RAN4.

It should be noted that all relevant improvements of 5G-Advanced need to be prioritized within Rel-20 to ensure that the transition to 6G will benefit from these improvements in day one.

In following sections specific details related to work packages we see as essential for Rel-20 BS RF in 5G-Advanced is presented.

  - Evolution of co-location requirement concept

In Rel-15 BS RF core and conformance OTA requirements was defined for AAS BS. These requirements were also adopted for NR BS type 1-O. The OTA requirements are derived on a coupling loss assumption of 30 dB and often referred to as “co-location requirements”. For the OTA co-location requirements, a new requirement concept was introduced. The concept is based on a co-location situation where two base stations are located side-by-side with a separation of 10 cm. This scenario was defined to mimic the worst-case situation for co-location of two BS part of two different networks. It was assumed that a commercial of the shelf single column BS antenna could be used as reference for such co-location situation. It was decided that for this situation the coupling loss between the passive antenna (referred to as the co-location reference antenna) and the AAS BS is 30 dB.

Now in Rel-19 the situation can be summarized as:

- The availability of co-location reference antennas for bands over 3.5 GHz following description in TS 38.104 is limited and for bands in upper FR1 not available. This will have direct impact on selection of Co-Location Test Antenna (CLTA) for conformance testing. From this we conclude:

  - The BS type 1-O with OTA requirements cannot be used for bands above 3.5 GHz.

  - Alternatively, if a customized CLTA is designed to mimic a co-location reference antenna. The current specification is not complete and will not provide sufficient information. This would jeopardize the intention to guarantee tower/site sharing. To guarantee robust co-location requirements it is essential that the CLTA to be used by BS manufactures is having the same radiation characteristics in the near-field region.

- For bands above 3.5 GHz it can be seen from Rel-19 results that coupling loss varies and is typically larger than 30 dB. This would impact requirements derivation and BS RF core specification with the consequence that the requirement is too strict which will drive cost and complexity.

Based on above information it’s clear that the current approach used to define and test co-location requirements don’t work for a BS type 1-O BS supporting bands above 3.5 GHz. For bands below 3.5 GHz down to 1 GHz the CLTA concept may work as intended, however in this frequency region most manufacturers are currently following BS type 1-H requirements. Above 3.5 GHz adopting BS type 1-O requirement is more suitable due to the number of antenna elements in the array increases and corresponding number of TAB RF ports increases.

The Rel-15 CLTA concept was derived based on the assumption of 30 dB coupling loss. The corresponding requirement levels for emissions and interfering signal will put extreme challenges on the test equipment, which resulted in the CLTA concept. However, if the coupling loss increases as expected for higher frequencies it can be concluded that OTA testing using traditional parameters (such as TRP, EIRP and EIS) can be used instead.

It can also be noticed that when BS with large frequency offset is considered, like for the case where a 900 MHz passive antenna is co-located with a 3.5 GHz AAS BS as showed in Figure 2.1-1, there will be a significant different is antenna aperture length.

# Figure 2.1-1: Example situations, co-location between low-band (non-AAS) and mid-band (AAS)

Observation 2.1-1: The height difference between to co-located base stations operating at different bands (900 MHz and 3.5 GHz) is far larger than what is captured in the CTLA description in current specification.

Observation 2.1-2: The current concept used for BS type 1-O co-location requirements captured in TS 38.104, Clause 4.9 is not practical for conformance testing of product operating above 3.5 GHz since the current description of the co-location test antenna and corresponding CLTA don’t match real deployment situations. Hence, BS type 1-O is not a feasible option for AAS BS implementations above 3.5 GHz.

Today the guidance for selection of CLTA is described in the conformance test specification as listed in Table 2.1-1. If now that information is used to design a CLTA some additional technical issues will emerge.

Table 2.1-1: CLTA characteristics (TS 38.141-2)

| Parameter | In-band CLTA | Out-of-band CLTA |
| --- | --- | --- |
| Vertical radiating dimension (h) | Test object vertical radiating length ±30% | Test object vertical radiating length ±30% (Note 2) |
| Horizontal beam width | 65° ± 10° | 65° ± 10° |
| Vertical beam width | N/A | The half-power vertical beam width of the CLTA equals the narrowest declared (D.3) vertical beamwidth ±3° (Note 2) |
| Polarization (Note 3) | Match (Note 4) | Match to in-band (Note 4) |
| Conducted interface return loss | > 10 dB | > 10 dB |
| Note 1: If a multi-column or multi-band antenna is used the column closest to the NR BS shall be selected while other columns are terminated during testing. Note 2: The vertical radiating dimension definition shall be used instead of the vertical beam width definition when the test chamber dimensions limit the use of vertical beam width definition. Otherwise, the vertical beam width definition shall be used. Note 3: For BS type 1-O with dual polarization the CLTA has two conducted interfaces each representing one polarization. Note 4: Matched to the polarization of EUT antenna. |  |  |

Observation 2.1-3: It can be noticed that information provided to describe the CLTA in the specification is overlapping and contradicting. The description provides information on both vertical radiating dimension and vertical beamwidth for the out-of-band case. Both conditions cannot be met at the same time.

Observation 2.1-4: The CLTA antenna gain is indirectly specified via the horizontal beam width and the vertical radiating dimension, while the antenna efficiency is not specified. This means that large isolation between the test object and CLTA can be created by having a CLTA with poor antenna efficiency.

In the case where the CLTA following the description in TS 38.141-2, Clause 4.12 is not commercially available the remaining way-forward is to let the BS manufacturer to build a CLTA antenna according to the technical specification. For this situation where the CLTA needs to be designed we have observed following issues:

- The technical description of the CLTA provides information on how the beamforming characteristics (3-sector coverage in azimuth and height similar as test object gives beamwidth along vertical axis). However, there is no guidance on the antenna efficiency. Hence the isolation created between the test antenna and CLTA can be created by poor antenna efficiency.

- If the BS manufacturer is responsible to design the CLTA, special means to increase the isolation can be used. There will differ among different BS manufacturers. This means that there is no guarantee that different BS manufacturer will use the same CLTA for conformance testing. As a result, the co-location requirement will not guarantee co-location of multiple BS from different vendors at the same site as the co-location performance will be arbitrary and not repeatable, which was the original intention with the co-location requirements.

Observation 2.1-4: The CLTA antenna efficiency is not captured by the description in current specification.

Observation 2.1-5: Different types of CLTA from different vendors with very different characteristics would risk making the co-location requirements use-less. To guarantee co-location, the CLTA used should be of similar type and have the same characteristics.

In the Rel-19 BS RF evolution WI (NR_BS_RF_req_evo-Core) the study outcome indicates that the coupling loss to be considered for requirements derivation is significantly different to 30 dB considering coupling within and to bands in the upper region of FR1 including following NR bands n46, n47, n48, n77, n78, n79, n96, n102 and n104.

The BS-to-BS port coupling loss for two different co-location scenarios have been evaluated through measurements and simulations.

- Vertical separation (Often used for tower sharing situations)

- Horizontal separation (side-by-side separated 0.1 m, according to TS 38.104, Clause 4.9)

At previous RAN4 meeting the outcome from the coupling loss investigation was summarized for different situations in a way-forward [5]. In this contribution the information has been restructured to clearer map to different situations, as listed in Table 2.1-2 intra band situations and Table 2.1-3 for inter band situations.

Table 2.1-2: Intra band coupling loss summary

| Co-location scenario | Coupling loss type | Coupling loss (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 2.6 GHz | 3.5 GHz | 4.9 GHz | 7 GHz |
| Vertical separation | SA-to-SA | M: 54 (R4-2504358) | M: 53 (R4-2504358) | M: 56 (R4-2504358) |  |
|  | A-to-SA |  |  |  |  |
|  | A-to-A |  |  |  |  |
| Horizontal separation | SA-to-SA | M: 31 (R4-2504358) | M: 34 (R4-2504358) | M: 46 (R4-2504358) |  |
|  | A-to-SA |  | M: 35 (R4-2507633) S: 55±15 (R4-2506307) | M: 51 (R4-2507445) M: 69 (R4-2507511) | M: 55 (R4-2507633) S: 70±15 (R4-2506307) |
|  | A-to-A |  |  |  | S: 77±15 (R4-2506307) |

Table 2.1-3: Inter band coupling loss summary

| Co-location scenario | Coupling loss type | Coupling loss (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 2.6 GHz to 3.5 GHz | 3.5 GHz to 4.9 GHz | 4.9 GHz to 7 GHz | 3.5 GHz to 7 GHz |
| Vertical separation | SA-to-SA |  |  |  |  |
|  | A-to-SA |  |  |  |  |
|  | A-to-A |  |  |  |  |
| Horizontal separation | SA-to-SA |  |  |  |  |
|  | A-to-SA |  |  |  | S: 57±10 (R4-2506307) |
|  | A-to-A |  |  |  | S: 54±10 (R4-2506307) |

The result consists of measurement results (M) and simulation results (S) for different coupling types:

- Sub-Array-to-Sub-Array (SA-to-SA) coupling loss

- Array-to-Sub-Array (A-to-SA) coupling loss

- Array-to-Array (A-to-A) coupling loss

For the table entries left blank no information have been provided to RAN4, which would indicate that the work is not complete.

Even though the coupling loss evaluation is not complete for all relevant cases, it’s clear that the outcome indicates that the assumption of a fixed BS-to-BS port coupling loss assumed to 30 dB for FR1 is not relevant anymore, especially not for AAS BS operating in the upper region of FR1. From the combined result from measurements and simulations it can be concluded that the coupling loss varies considering all scenarios, bands and coupling types. The general trend indicates that coupling loss increases for bands in the upper region of FR1.

Observation 2.1-6: It can be noticed that the coupling loss tables created in Rel-19 are not complete. RAN4 need to further evaluate BS-to-BS coupling loss for different coupling situations relevant for considered requirements.

For MSR AAS BS and NR, co-location requirements are requirements which are based on a given BS-to-BS co-location scenario with a given assumption of port-to-port coupling loss of 30 dB. The intention with these requirements is to ensure that BSs serving different bands can be co-sited at the same site and/or tower. A set of co-location requirements have been defined for AAS BS in TS 37.105 and for NR BS type 1-C, BS type 1-H and BS type 1-O in TS 38.104. In Table 2.1-4, all the BS co-location requirements are listed together with references to individual requirements in TS 38.104.

Table 2.1-4: Overview of NR co-location requirements

| Requirement | Reference to clauses in TS 38.104 | Description |
| --- | --- | --- |
| Transmitter spurious emission | BS type 1-C and BS type 1-H in Clause 6.6.5.2.4. BS type 1-O in Clause 9.7.5.2.5. | This requirement is defined to protect BS operating in other 3GPP band(s) co-located at the same site. |
| Receiver out-of-band blocking | BS type 1-C and BS type 1-H in Clause 7.5.3. BS type 1-O in Clause 10.6.2.2. | This requirement is defined to guarantee UL operation when a BS is co-located to another BS operating at another 3GPP band(s). |
| Transmitter OFF power | BS type 1-C in Clause 6.4.1.2. BS type 1-H in Clause 6.4.1.3. BS type 1-O is Clause 9.5.2.2. | This requirement is defined to protect other BS(s) in the same network in neighbouring cells. |
| Transmitter intermodulation | BS type 1-C in Clause 6.7.2. BS type 1-H in Clause 6.7.3. BS type 1-O in Clause 9.8.2. | This requirement guarantees transmitter robustness with respect to an external interferer. |

Originally all requirements were derived for non-AAS BS (BS type 1-C). In Rel-13, hybrid AAS BS was introduced (In NR specifications referred to as BS type 1-H) and in Rel-15 all OTA AAS BS was introduce (In NR specifications referred to as BS type 1-O). For BS type 1-C and BS type 1-H, all requirements are defined at the transceiver ports (also known as the Transceiver Array Boundary (TAB) connector(s)). For BS type 1-O all co-location requirements are defined based on a concept of a co-location reference antenna. In conformance testing this antenna is referred to Co-Location Reference Antenna (CLTA).

Looking at the details related to the technical background for each induvial requirement it can be noticed that the BS-to-BS coupling loss assumed is of different type depending on requirement. In Tabel 2.1-5 some further details for each requirement in presented.

Table 2.1-5: Requirement derivation assumptions

| Requirement | Coupling type | Frequency aspect | Description |
| --- | --- | --- | --- |
| Transmitter spurious emission | A-to-A | Inter band | The requirement is defined to protect another band, hence coupling between bands are relevant. The approach used to derive the requirement level is defined to consider A-to-A coupling. |
| Receiver out-of-band blocking | A-to-SA | Inter band | The requirement is defined to protect another band, hence coupling between bands are relevant. Since the receiver blocking is related to induvial receiver branches, A-to-SA is considered when the requirement level is derived. |
| Transmitter OFF power | A-to-A | Intra band | The requirement is defined for TDD operation to protect the neighbouring BS in the same network. The approach used to derive the requirement level is defined to consider A-to-A coupling within the same band. The A-to-A coupling for this requirement should be based on a neighbouring BS in the same network and not side-by-side at the same site. |
| Transmitter intermodulation | A-to-SA | Intra band | This requirement is defined to guarantee than unwanted emission requirements is met in the case of an interferer signal. The interferer signal is injected within the band. Since the requirement is related to induvial transmitter branches, A-to-SA is considered when the requirement level is derived. |

It is worth to notice that the background for BS-to-BS coupling is different for all requirements listed in Table 2.1-5. A complete technical background for all requirements would require a coupling evaluation including following cases:

- Intra band A-to-A for relevant band combinations

- Inter band A-to-SA for relevant band combinations

- Intra band A-to-A for relevant bands considering coupling to neighbouring BS

- Intra band A-to-SA for relevant bands

Analysing the results presented in Table 2.1-2 and Table 2.1-3 would indicate that assuming BS-to-BS port coupling loss equal to a value larger than 30 dB would be reasonable for bands in the upper region of FR1. To be able to decide on a relevant coupling value for all cases, RAN4 need to adopt a common simulation methodology.

To guarantee co-location of two operator network sharing site or tower, the essential requirements are Transmitter spurious emission and Receiver out-of-band blocking.

Observation 2.1-7: To progress the evolution of co-location requirements the focus should be on transmitter spurious emission and receiver out-of-band blocking.

Observation 2.1-8: To establish relevant coupling assumptions for all requirements RAN4 need to establish a common simulation methodology and conduct a simulation campaign. It’s not possible to establish coupling values based on measurements results only, due to the large scope and different combination of bands and antenna configuration.

Observation 2.1-9: Improving co-location requirements will impact both BS RF core specification and Conformance test specification, since the coupling loss assumption needs to be re-evaluated and test issues related to current CLTA needs to be resolved.

Observation 2.1-10: Improving co-location requirement in the upper region of FR1 includes re-evaluation requirement levels for following bands: n46, n47, n48, n77, n78, n79, n96, n102 and n104.

Proposal 2.1-1: For BS RF in Rel-20 define a work package with clear goal to resolve open issues identified for BS type 1-O co-location requirements. Use outcome from Rel-19 coupling loss investigation as starting point and develop new co-location requirements based on regular OTA parameters for following bands: n46, n47, n48, n77, n78, n79, n96, n102 and n104.

As a starting point for WID preparation drafts for justification and objective are provided below:

Justification: To make BS type 1-O relevant for AAS BS operating in the upper region of FR1, RAN4 need to evolve and improve the requirement concept for which the co-location requirements are based. The requirements should be defined in such a way that multi-vendor BS co-location and tower sharing at a common site can be guaranteed, and conformance tests can be reproduced.

Objective: Evolve the co-location requirement for BS type 1-O for bands in the upper region of FR1 to be based on regular OTA parameters (such as TRP, EIRP, EIS, field strength or power density depending on co-location requirement in mind) instead of specified CLTA.

  - Derivation of SBFD-to-SBFD co-existence requirements

The major technical challenges for Sub-Band Full Duplex (SBFD) include handling self-interference, inter-sector interference, inter-site interference and inter-operator interference.

The scope of Rel-19 SBFD WI considers the co-existence of SBFD-capable base stations with legacy TDD base stations. For inter-operator interference, legacy TDD transmitters are not required to provide protection for SBFD receivers. For FR1 wide-area and medium-range high-power base stations, avoiding blockage of SBFD receivers necessitates a minimum separation of 288 meters between SBFD-capable base stations and regular TDD base stations, which makes same-band coexistence of SBFD-to-TDD challenging and is more practical when a single operator owns the entire band.

While inter-operator interference might be reduced in a certain scenario that all operators in the shared band deploy SBFD, i.e., co-existence of SBFD-to-SBFD, both transmitter and receiver meet specially designed requirements on in band emissions and blocking that consider the scenario of adjacent operator SBFD operation, where operators could be protected from transmitter emissions leakage and receiver blocking requirements to/from other operators.

The reason for deploying SBFD is that the existence of sub-bands allows for a significant frequency range within which a filter can roll-off. Examples for two operators and three operators are provided in Figures 2.2-1 and Figure 2.2-2.

Figure 2.2-1: SBFD-to-SBFD co-existence arrangement (three operators)

Figure 2.2-2: SBFD-to-SBFD co-existence arrangement (two operators)

An operator should provide suppression of unwanted emissions towards one another operator. For example, operator 1 must suppress emissions from their downlink sub-band towards operator 2. The filter roll-off can be located within the DL sub-band of operator 2. Similar ranges exist for suppression of DL sub-band emissions between operators.

Operator 2 must protect their receiver from the DL sub-band of operator 1. The attenuation can similarly be provided within the frequency range of the operator 2 DL sub-band. Similar ranges exist for supressing UL blocking towards the receiver for other operators.

By this means, SBFD operators can provide filtering to avoid interference between one another.

There exist some general constraints to enable this kind of co-existence between operators. Firstly, each operator must be protected from unwanted emissions from any other operator within the band. This means that every operator must suppress emissions in all of the band. This will not be guaranteed if there is any operator that has a deployment that is TDD and does not meet the additional requirements. Thus, all operators in the band need to deploy SBFD filtering.

Observation 2.2-1: All operators in the band must deploy SBFD filtering.

Observation 2.2-2: The proposed scenario will only suppress the inter-operator interference, meanwhile, the challenges on other types of interference still exist.

From the receiver point of view, an SBFD operator can protect their receiver as long as the blocker is supressed by a filter within their DL sub-bands. However, the more operators there are in the band, the greater will need to be the suppression in some cases; for example, if there are multiple operators that have BSs close to one another or on the same tower then each operator will experience the combined power from the other operators.

Although not directly part of operator-operator co-existence, it is important to note that SBFD can only function if an operator deploys SBFD at all sites on the same carrier. Consider, for example that Free-Space Path Loss (FSPL) (similar to the pathloss for a rooftop deployment) for 500 m is around 100 dB. If an operator would have SBFD deployed at one site, but not at the next then the SBFD uplink sub-band could experience interference of around -70 dBm/MHz (plus/minus antenna gains), which is 40 dB above the noise floor. Between sectors of the same site, the interference towards the UL sub-band if another sector would not deploy SBFD would be even larger, although it may be expected that SBFD would be deployed for all sectors of a site. Inter-site CLI would not be useful here if the other site would not be SBFD capable.

Taking these issues into account, in order for the paradigm under which the additional new emissions and blocking requirements for SBFD-to-SBFD would be designed to work, all operators would need to deploy SBFD BSs at every site.

Observation 2.2-3: Operators must deploy SBFD at every site to avoid sites at which SBFD does not work due to interference.

In our understanding, for operators to mutually invest in deploying SBFD at every site within a band, a strong commercial agreement would be required between the operators. Simply being interested would not be sufficient; it would not be justifiable for one operator to deploy SBFD BSs across their network without certainty that all of their neighbours would do, since to do so would risk an investment in a network that would fail due to interference, as long as any operator in the same band decides to deploy TDD without investing in analogue filters to suppress emissions to protect other operators.

As is discussed further below, the BS design to meet SBFD-to-SBFD co-existence requirements would need to incorporate analogue filters based on the sub-band frequencies of the own and other operators. Thus, the SBFD BSs deployed by the operators would need to assume a carrier and sub-band allocation between operators that would not always be adjustable without hardware changes by all involved operators.

Observation 2.2-4: The flexibility to configure carrier and sub-band allocations within a band without hardware changes would be restricted due to filtering and may need coordination between operators.

    - Operator deployment scenarios

The purpose of operators collaborating in order to deploy SBFD would be badly undermined if there would be constraints on the operators also having to co-ordinate their BS sites in order to avoid being close to one another. In case there would be three operators within a band, co-ordination would become even more complex, since every operator would need to maintain at least a certain distance from every other operator.

In our view, if additional requirements for SBFD-to-SBFD deployments would be considered, then they should guarantee full flexibility for operator site deployment. This should, for example enable SBFD to function when operators are within 10 m of one another, and also when they are sharing the same tower.

We remind that there is a general goal to be able to use existing site grids for e.g. 4 GHz or upper 6 GHz. If the new SBFD-to-SBFD requirements would require a complex redesign of the network layout with operators needing to co-ordinate and negotiate on site positions, then the deployment scenario would not be useful.

Observation 2.2-5: If new types of emissions and blocking requirements requiring operator collaboration are created then they should ensure that all kinds of deployment scenarios can work, including re-use of existing sites, close and co-located BS.

    - Spectrum and sub-band considerations

An SBFD-to-SBFD co-existence requirement would require analogue filtering to supress emissions and protect the SBFD receiver. The analogue filter characteristics would be designed based on the bandwidth of the DL and UL sub-bands, and those of neighbour operator sub-bands. Since BS design that meets, the requirements would need to take the widths of the sub-bands into account, the specification would need to fix sub-band and carrier sizes.

The sub-band sizes (and associated filter feasibility) would depend on the specific band and sub-band sizes. It should be noted that once operators have deployed SBFD, as noted above, hardware upgrades might be needed by all operators to change carrier or sub-band allocations.

The bands for which SBFD-to-SBFD co-existence requirements have been speculated include the upper 6 GHz and a potential future band of around 200 MHz in the US. Without concrete details of the bands, the operator allocations and the operator sub-band sizes, specific requirements cannot be designed.

Observation 2.2-6: Further details on band allocations, operator spectrum allocations and plans for sub-band sizes are needed to determine requirements and filtering feasibility.

    - Filtering feasibility considerations

As discussed above, for an SBFD-to-SBFD co-existence scenario to be workable, it should not place strong constraints on operators to maintain mutual distance between one another (especially not in the case of three operators) and should not prevent re-use of existing site grids. To achieve this, inter-operator BS distances of down to 10 m, and also co-location should not be precluded.

Existing co-location requirements assume 30 dB isolation between operators. For higher frequencies, this may be somewhat pessimistic, and a discussion on co-location requirements is taking place in the context of the Rel-19 BS RF evolution WI.

Much more investigation is needed into the impact of SBFD interference between operators, considering beamforming and deployment scenarios. It should be noted that for isolation levels of 30 to 70 dB, the emissions would need to be supressed to somewhere between -45 to -85 dBm. Blocking levels of between -20 to +20 dBm would need to be dealt with at the receiver. This implies filtering suppression of up to 50 to 70 dB; the exact filtering requirement needs further analysis.

Observation 2.2-7: Further analysis is needed on the filtering suppression and requirement levels.

Existing band- or sub-band filters need to supress emissions down to spurious emissions levels and out of band blocking within DfOBUE and DfOOB. It is quite likely that SBFD-to-SBFD scenario to work properly a larger amount of filtering is required than from sub-band filters. If the filtering requirements are stricter, this implies much higher Q factor, a large volume for the filters and potentially higher insertion loss and associated coverage compromise.

Although without extensive investigation it is not possible to determine how much emissions and suppression levels are needed, clearly there should also be a careful consideration of the feasibility of the filtering solution, considering both emissions and blocking suppression.

Use the two-operator case (see Figure 2.2-2) as an example, to protect operator 1’s UL sub-band, suppression of inter-operator interference from operator 2 is needed. This necessitates both a transmitter analogue filter for the whole carrier on operator 2’s side and a receiver analogue filter for the UL sub-band on operator 1’s side. The two analogue filters (one for transmission and one for reception) have different frequency positions, different past bands and different roll-offs, and are required per polarization per subarray.

Observation 2.2-8: Both transmitter and receiver analogue filters are required per polarization per subarray to suppress inter-operator interference. The two types of analogue filters have different frequency positions, different pass bands and different roll-offs.

Observation 2.2-9: An operator’s transmitter analogue filter is customized based on other operators’ sub-band configurations.

The theoretical UL coverage gain is 4.7 dB for SBFD compared with regular TDD (DDDDU pattern). It is reduced by self-interference, inter-sector and inter-site interference, and the fact that UE cannot transmit continuously. Therefore, it is important to avoid further losses. Adding TX and RX analogue filters will bring insertion loss, which should be carefully considered to maintain the rest of the gain.

Observation 2.2-10: The insertion-loss of both the TX and RX analogue filters should be carefully considered, as it can further reduce the theoretical UL coverage gain.

Observation 2.2-11: Filter feasibility in terms of roll-off, size, shape needs to be studied.

With all of the above in mind, it is important that any requirement provides scope for operators to negotiate but does not force operators to meet much stricter requirements in cases where there is no sharing agreement or TDD. Thus, if the SBFD co-existence is to be included, in our view it should be captured that it is declared for SBFD and does not impact TDD.

Proposal 2.2-1: If SBFD-to-SBFD co-existence requirements are created, it should be clarified that meeting the requirement is declared by the BS vendor and that TDD operation is not impacted.

  - Introduction of new AAS NR BS type

This BS type is intended for AAS BS with a large array antenna required to provide coverage operating within bands in the upper region of FR1. The new BS type will include more OTA requirements than BS type 1-H, which would allow for a smooth transition to full OTA and BS type 1-O.

In Rel-15, the concept of NR BS types was established to allow the specification to support different types of base stations for FR1 (non-AAS and AAS) and FR2 (AAS). The requirement applicability for the different base station types is different since requirements are defined at different requirement interfaces referred to as TAB and RIB. The interfaces are visualised in Figure 2.3-1.

# Figure 2.3-1: AAS BS architecture

The currently supported BS types and a potential new additional BS type (here referred to BS type 1-H-enh) are listed in Table 2.3-1.

Table 2.3-1: BS types

| BS type | Requirement interfaces | Note |
| --- | --- | --- |
| 1-C | TAB | FR1 non-AAS BS with few RF ports and passive antenna |
| 1-H | TAB, RIB | FR1 AAS BS with access to RF ports at TAB |
| 1-H-enh | TAB2, RIB | New BS type for FR1 AAS BS with access to RF ports at TAB2 |
| 1-O | RIB | FR1 AAS BS without access to RF ports |
| 2-O | RIB | FR2 AAS BS without access to RF ports |

Considering the coverage capability for AAS BS to be the same for bands in the upper region of FR1 as for mid FR1 bands would lead to larger number of antenna elements needed for AAS BS products in the upper region of FR1 compared with mid FR1 bands. Since the number of antenna elements needed will be significantly larger than the supported number of layer 1 logical ports (also referred to as baseband ports or CSI-RS ports), different types of port expansion techniques will be required to map signals from logical ports to physical antenna elements. Traditionally the port expansion is done by combining elements as vertical sub-arrays. This combination is done using passive combination networks in the Radio Distribution Network (RDN) and often in conjunction with digital port expansion between baseband and radio. For large arrays, the vertical sub-array tends be very large which would restrict the vertical steering range. Therefore, hybrid beamforming schemes are often considered to add steering capabilities in the RDN by adding analogue RF components, such as phase shifters, switched delay lines, etc.

The current BS architecture described in TS 38.104, Clause 4.3 does not explicitly describe type of beamforming schemes supported. However, for electrically large arrays it is essential to define the requirement interface such that all emission generated by the electronics is captured. In the case of having active analogue components in the RDN, a requirement interface should be defined after the RDN. With this interface TAB2 all emission generated by the radio electronics will be captured at TAB2 see Figure 2.3-1.

Another aspect to consider is that with very many AAS BS transceiver branches it is essential to define a relevant requirement level per branch. For BS type 1-H, the legacy requirement for the receiver is re-used with the consequence that the receiver for a AAS BS with a large array antenna the system will be over specified, which would increase cost and power consumption without providing benefit for the operator. Therefore, it would be natural to move requirements to the OTA domain for in-band receiver requirements and in-band transmitter requirements. Similar to the transmitter, additional considerations might be needed to ensure that receiver requirements are properly defined, and relevant test framework is in place as we move towards large electrically antennas including the architecture which cover such receiver beam forming and corresponding requirement interface.

In addition, the OTA test lab capabilities in the industry are not mature for AAS BS type 1-O all OTA conformance testing. Therefore, we suggest introducing a new BS type with a requirement set consisting of in-band requirements to be specified as OTA requirement while out of band requirements are defined as conducted requirements.

The general principles for establishing a complete requirement set for the new BS type are listed below:

- All carrier centric transmitter requirements are defined OTA at RIB.

- All receiver requirements are defined OTA at RIB.

- All out-of-band transmitter unwanted emission requirements and receiver spurious emission requirements are defined conducted at TAB2.

- TDD ON/OFF power is defined conducted at TAB2.

Based on the general principles above, a complete requirement set for TS 38.104 have been created in Table 2.3-2.

Table 2.3-2: New BS type requirement applicability mapping

| Requirement | BS type |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1-C | 1-H | 1-H-enh | 1-O | 2-O |
| BS output power | 6.2 | 6.2 | NA |  |  |
| Output power dynamics | 6.3 | 6.3 | NA |  |  |
| Transmit ON/OFF power | 6.4 | 6.4 | 6.4 |  |  |
| Transmitted signal quality | 6.5 | 6.5 | NA |  |  |
| Occupied bandwidth | 6.6.2 | 6.6.2 | NA |  |  |
| ACLR | 6.6.3 | 6.6.3 | 6.6.3 |  |  |
| Operating band unwanted emissions | 6.6.4 | 6.6.4 | 6.6.4 |  |  |
| Transmitter spurious emissions | 6.6.5 | 6.6.5 | 6.6.5 |  |  |
| Transmitter intermodulation | 6.7 | 6.7 | 6.7 | NA | NA |
| Reference sensitivity level | 7.2 | 7.2 | NA |  |  |
| Dynamic range | 7.3 | 7.3 | NA |  |  |
| In-band selectivity and blocking | 7.4 | 7.4 | NA |  |  |
| Out-of-band blocking | 7.5 | 7.5 | NA |  |  |
| Receiver spurious emissions | 7.6 | 7.6 | 7.6 |  |  |
| Receiver intermodulation | 7.7 | 7.7 | NA |  |  |
| In-channel selectivity | 7.8 | 7.8 | NA |  |  |
| Performance requirements | 8 | 8 | NA |  |  |
| Radiated transmit power |  | 9.2 | 9.2 | 9.2 | 9.2 |
| OTA base station output power |  |  | 9.3 | 9.3 | 9.3 |
| OTA output power dynamics |  |  | 9.4 | 9.4 | 9.4 |
| OTA transmit ON/OFF power |  |  | NA | 9.5 | 9.5 |
| OTA transmitted signal quality |  |  | 9.6 | 9.6 | 9.6 |
| OTA occupied bandwidth |  |  | 9.7.2 | 9.7.2 | 9.7.2 |
| OTA ACLR |  | NA | NA | 9.7.3 | 9.7.3 |
| OTA Operating band unwanted emissions |  |  | NA | 9.7.4 | 9.7.4 |
| OTA transmitter spurious emission |  |  | NA | 9.7.5 | 9.7.5 |
| OTA transmitter intermodulation |  |  | NA | 9.8 | NA |
| OTA spatial emission |  | 9.9 | 9.9 | 9.9 | NA |
| OTA sensitivity |  | 10.2 | 10.2 | 10.2 | NA |
| OTA reference sensitivity level | NA |  | 10.3 | 10.3 | 10.3 |
| OTA dynamic range |  |  | 10.4 | 10.4 | NA |
| OTA in-band selectivity and blocking |  |  | 10.5 | 10.5 | 10.5 |
| OTA out-of-band blocking |  | NA | 10.6 | 10.6 | 10.6 |
| OTA receiver spurious emission |  |  | NA | 10.7 | 10.7 |
| OTA receiver intermodulation |  |  | 10.8 | 10.8 | 10.8 |
| OTA in-channel selectivity |  |  | 10.9 | 10.9 | 10.9 |
| Radiated performance requirements |  |  | 11 | 11 | 11 |

With the given general principles and example requirement set applicability, further details related to the exact mapping required for the applicability table in TS 38.104, Clause 4.6 can be worked out.

Observation 2.3-1: Based on high level principles required to define a new BS type for AAS BS there is a direct link to work package related to improvements of co-location requirements. For the new BS type to be practically relevant, work to establish relevant and practically feasible co-location requirements will be needed.

Observation 2.3-2: The new BS type (BS type 1-H enh) would require a new requirement interface (TAB2 after the RDN) to be defined.

Proposal 2.3-1: For BS RF in Rel-20, include WI to define new BS type with relevant requirement applicability and corresponding requirement interface relevant for AAS BS with large array antenna.

If a new BS type for FR1 AAS BS is defined in addition to BS type 1-H and BS type 1-O, RAN4 need to work out general principles on requirement applicability and for when different BS types are relevant, e.g. defining caps on BS types based on number of ports, number of transceivers and supported beamforming schemes for each AAS BS type, to avoid having a specification with multiple alternatives for a given design.

For the OTA receiver requirement for BS type 1-O, declarations related to the element/sub-array is needed to translate requirement levels from legacy conducted requirement levels to OTA requirements levels. With the introduction of more advanced RDN structures with fast analogue phase shifters combining several sub-arrays in more complex sub-array structures it’s not clear how the declaration defined in current specification should be done. More work is needed to clarify interfaces in relating to the RDN and the antenna array and clearly describe what type of functionality is included in the RDN.

Observation 2.3-3: OTA receiver requirements rely on declarations of the element/sub-array characteristics. The description of the RDN needs to be enhanced to support more complex beamforming schemes (e.g. hybrid beamforming) to provide guidance for required declarations.

As a starting point for WID preparation drafts for justification and objective are provided below:

Justification: To properly capture mixture of digital and analog beamforming schemes and stimulate the industry to take the step to OTA conformance testing for AAS BS operating in the upper region of FR1, RAN4 need to define a new BS type with more OTA requirements than BS type 1-H.

Objective: Define a new BS type with more OTA requirements than BS type 1-H. As a principle it would be reasonable to define all in-band and receiver requirements as OTA requirements. If needed, update declarations to support complex beamforming schemes (e.g. hybrid beamforming including analogue and digital beamforming).

# Conclusion

In this contribution we provide technical background and proposals for Rel-20 5G-Advanced BS RF work package.

The observations and proposals are captured per topic below:

Evolution of co-location requirement concept:

Observation 2.1-1: The height difference between to co-located base stations operating at different bands (900 MHz and 3.5 GHz) is far larger than what is captured in the CTLA description in current specification.

Observation 2.1-2: The current concept used for BS type 1-O co-location requirements captured in TS 38.104, Clause 4.9 is not practical for conformance testing of product operating above 3.5 GHz since the current description of the co-location test antenna and corresponding CLTA don’t match real deployment situations. Hence, BS type 1-O is not a feasible option for AAS BS implementations above 3.5 GHz.

Observation 2.1-3: It can be noticed that information provided to describe the CLTA in the specification is overlapping and contradicting. The description provides information on both vertical radiating dimension and vertical beamwidth for the out-of-band case. Both conditions cannot be met at the same time.

Observation 2.1-4: The CLTA antenna gain is indirectly specified via the horizontal beam width and the vertical radiating dimension, while the antenna efficiency is not specified. This means that large isolation between the test object and CLTA can be created by having a CLTA with poor antenna efficiency.

Observation 2.1-4: The CLTA antenna efficiency is not captured by the description in current specification.

Observation 2.1-5: Different types of CLTA from different vendors with very different characteristics would risk making the co-location requirements use-less. To guarantee co-location, the CLTA used should be of similar type and have the same characteristics.

Observation 2.1-6: It can be noticed that the coupling loss tables created in Rel-19 are not complete. RAN4 need to further evaluate BS-to-BS coupling loss for different coupling situations relevant for considered requirements.

Observation 2.1-7: To progress the evolution of co-location requirements the focus should be on transmitter spurious emission and receiver out-of-band blocking.

Observation 2.1-8: To establish relevant coupling assumptions for all requirements RAN4 need to establish a common simulation methodology and conduct a simulation campaign. It’s not possible to establish coupling values based on measurements results only, due to the large scope and different combination of bands and antenna configuration.

Observation 2.1-9: Improving co-location requirements will impact both BS RF core specification and Conformance test specification, since the coupling loss assumption needs to be re-evaluated and test issues related to current CLTA needs to be resolved.

Observation 2.1-10: Improving co-location requirement in the upper region of FR1 includes re-evaluation requirement levels for following bands: n46, n47, n48, n77, n78, n79, n96, n102 and n104.

Proposal 2.1-1: For BS RF in Rel-20 define a work package with clear goal to resolve open issues identified for BS type 1-O co-location requirements. Use outcome from Rel-19 coupling loss investigation as starting point and develop new co-location requirements based on regular OTA parameters for following bands: n46, n47, n48, n77, n78, n79, n96, n102 and n104.

Derivation of SBFD-to-SBFD co-existence requirements:

Observation 2.2-1: All operators in the band must deploy SBFD filtering.

Observation 2.2-2: The proposed scenario will only suppress the inter-operator interference, meanwhile, the challenges on other types of interference still exist.

Observation 2.2-3: Operators must deploy SBFD at every site to avoid sites at which SBFD does not work due to interference.

Observation 2.2-4: The flexibility to configure carrier and sub-band allocations within a band without hardware changes would be restricted due to filtering and may need coordination between operators.

Observation 2.2-5: If new types of emissions and blocking requirements requiring operator collaboration are created then they should ensure that all kinds of deployment scenarios can work, including re-use of existing sites, close and co-located BS.

Observation 2.2-6: Further details on band allocations, operator spectrum allocations and plans for sub-band sizes are needed to determine requirements and filtering feasibility.

Observation 2.2-7: Further analysis is needed on the filtering suppression and requirement levels.

Observation 2.2-8: Both transmitter and receiver analogue filters are required per polarization per subarray to suppress inter-operator interference. The two types of analogue filters have different frequency positions, different pass bands and different roll-offs.

Observation 2.2-9: An operator’s transmitter analogue filter is customized based on other operators’ sub-band configurations.

Observation 2.2-10: The insertion-loss of both the TX and RX analogue filters should be carefully considered, as it can further reduce the theoretical UL coverage gain.

Observation 2.2-11: Filter feasibility in terms of roll-off, size, shape needs to be studied.

Proposal 2.2-1: If SBFD-to-SBFD co-existence requirements are created, it should be clarified that meeting the requirement is declared by the BS vendor and that TDD operation is not impacted.

Introduction of new AAS NR BS type:

Observation 2.3-1: Based on high level principles required to define a new BS type for AAS BS there is a direct link to work package related to improvements of co-location requirements. For the new BS type to be practically relevant, work to establish relevant and practically feasible co-location requirements will be needed.

Observation 2.3-2: The new BS type (BS type 1-H enh) would require a new requirement interface (TAB2 after the RDN) to be defined.

Observation 2.3-3: OTA receiver requirements rely on declarations of the element/sub-array characteristics. The description of the RDN needs to be enhanced to support more complex beamforming schemes (e.g. hybrid beamforming) to provide guidance for required declarations.

Proposal 2.3-1: For BS RF in Rel-20, include WI to define new BS type with relevant requirement applicability and corresponding requirement interface relevant for AAS BS with large array antenna.

# References

[1]	RP-251879, “WF for RAN4 led REL-20 topics”, RAN4 chairman (Apple)

[2] 	RP-251285, “Discussion on UE RF and BS RF proposals for Rel-20 5G-Advanced”, Ericsson

[3]	R4-2503725, “Considerations on Rel-20 BS RF package objectives”, Ericsson

[4]	R4-2510227, “On improvements of co-location requirements”, Ericsson

[5]	R4-2508657, “Way Forward for [115][303] NR_BS_RF_Part2_CLTA_SE: CLTA”, Huawei

[6]	R4-2510233, “On alternatives to current CLTA concept”, Ericsson