3GPP TSG RAN Meeting #109					RP-252667

Beijing, China, September 15-18, 2025

Agenda Item:	8.2.1.4

Source:	Ericsson, AT&T, BDBOS, DIST, Erillisverkot, FirstNet, Motorola Solutions, NIST, Nkom, Softil, SyncTechno Inc, Telstra, The Netherlands Police, T-Mobile USA

Title:	Requirements for supporting critical communications services in 6G

Document for:	Approval

# 1	Introduction

In this contribution, we discuss some of the requirements for supporting critical communications. Other aspects which are also relevant for critical communications such as resilience, sensing, NTN, aerials, emergency communications, etc. are discussed in separate papers. The corresponding references can be found at the end of this paper.

# 2	Discussion

## 2.1	General requirements

The existing 3GPP standards (e.g., 5G) provide a good baseline for mission critical users. Building on these capabilities, an ecosystem of devices, networks, servers, and applications has been created. The introduction of 6G will bring new capabilities and enable new services for critical communications, with new requirements. For some existing critical communication services, the new capabilities will allow for increased requirements. In addition, for some established critical communication services (e.g., voice), the requirements in 6G will remain the same. Given this and that the renewal rate is lower for critical-communications equipment than for consumer equipment, we believe that the migration of mission critical users to 6G will be slower than for consumers. Therefore, we expect to find a mixture of 4G-, 5-, and 6G-capable devices deployed in this scenario. For these existing services, ensuring service delivery to all users, irrespective of the supported capabilities, and guaranteeing interoperability between them will be a key requirement.

The commercial success of 6G for critical communication users depends not only on supporting their requirements on paper, but also on doing so without creating a different architecture or separate architecture options that may lead to product and market fragmentation. Similarly, 3GPP should reutilize existing elements (e.g., 5G features) whenever possible and avoid introducing redundant features in 6G, which fragment the market and solution space, especially for use cases with limited adoption.

It is important that the 6G RAN and its architecture can support the QoS model defined by 3GPP SA groups for 6G. Critical communications services are sometimes offered in congested scenarios. Given the nature of the operations they support, it is crucial to provide means to prioritize critical communication services already from the access phase. Prioritization will come at the expense of other users who, in some cases, may be pre-emptied from the network altogether.

## 2.2	Radio robustness

Communication networks, UEs and end user applications may be targeted by intentional or unintentional attacks ranging from e.g. physical attacks, harmful electromagnetic interference, end user information and communications security and privacy compromising attacks. The 6G system should enhance the protection of the access stratum against such disturbances (e.g. by improving the privacy of PHY channels and signals; by improving the security of system information, by avoiding vulnerable designs; by enabling GNSS independent operations etc.).

The 6G requirements for the related topics of resilience, sensing, and NTN are discussed in [2], [3], and [4], respectively.

## 2.3	Network coverage extensions and ad-hoc deployments

Critical communication services are also required in places where there is no network infrastructure or where network capabilities (e.g., availability, capacity, coverage, etc.) are insufficient to meet the requirements of these services. It is therefore crucial that 6G supports deploying additional ad-hoc sites fast and efficiently. The solutions should allow for extending the network at its coverage edge (e.g., by creating new cells adjacent to or co-located with other existing cells) or creating new isolated coverage areas. Moreover, some of these additional infrastructure nodes may be placed on moving platforms. 6G system and industry solutions must also support fast and flexible means to provision capacity whenever the critical communications need arises

To some extent, commercial availability of solutions for ad-hoc deployment is a matter of product development. However, it should be noted that standardization also plays an important role, and the standardized solutions have so far relied on network-based relaying via wireless backhaul. On the other hand, unnecessarily complex specifications or separate architecture tracks (e.g., new logical nodes, etc.) seldom find their way to the market. In addition, some of the scenarios impose limitations on the complexity, weight, energy consumption, etc. that the serving nodes can have (e.g., on drones or other light aircraft).

In our view, it is important that ad-hoc deployments can be used with limited backhaul means (e.g., with capacity limitations, with higher-latency links such as over NTN, etc.). This will allow for use in emergency support and disaster relief situations, etc., which are common scenarios for critical communication users.

## 2.4	Mission critical services

To provide critical communication services for mission critical users, 3GPP has developed and evolved a collection of mission critical services (e.g., MCPTT, MCVideo, MCData, etc.) and signalling/user data prioritization framework also for mission critical communications. Like in earlier generations, these applications and functions should be supported by 6G.

## 2.5	Public warning/emergency alert systems

Like for earlier generations, 6G needs to support public warning/emergency alert systems.

## 2.6	Broadcast and multicast communications

Support for broadcast and multicast communications has also been highlighted in the past by critical communication users as one relevant requirement. In the past, 3GPP has specified several BC/MC solutions (e.g., MBMS, SC-PTM, etc.), but there has been very limited product availability and market adoption.

Like for device-to-device (D2D) communication, we believe that support in 6G should be considered carefully, balancing the following requirements:

- Improve product availability, e.g., by avoiding further solution and product fragmentation.

- Ensure efficient delivery of communications to users supporting different 3GPP cellular generations. Spectral efficiency is particularly important given that many critical communication users have very limited spectrum allocations.

In our view, the critical communication requirements on BC/MC must be addressed by considering the availability of solutions in the different 3GPP cellular generations.

## 2.7	Device-to-device communication

Support for device-to-device (D2D) communication is one of the key requirements for critical communication services, both for communications without involving network infrastructure and for network coverage extension by means of UE-to-NW relaying.

D2D communication is subject to interoperability issues that need to be carefully considered before standardizing new solutions:

- At the time 6G is introduced for critical communication services, it is expected that D2D communications using 5G NR will be in use, likely sharing the spectrum with cellular communications. For many, if not all, cases, spectrum will be a very limited asset.

- The critical communication services use cases, including the ones using D2D, will involve multi-RAT (5G+6G-capable) devices as well as legacy (5G-only) devices.

- Sidelink relaying, will likely involve a combination of multi-RAT (5G+6G-capable) devices as well as legacy (5G-only) devices, as relays and as remote UEs. In addition, network coverage may be provided by 5G or 6G base stations.

In summary, 3GPP should ensure that 6G allows for device-to-device communication for critical services in a way that ensures interoperability with 5G D2D users without using the spectrum resources in an inefficient manner. Similarly, 3GPP should ensure that UE-to-NW relaying involving 5G and 6G UEs and base stations is possible.

## 2.8	Other requirements

Critical communications services rely on many other features and functionalities. These include:

- Resilience against failures, unintentional disturbances, and intentional attacks. It is crucial that the 6G architecture and procedures are robust in their design and provide functionality that facilitate the development of resilient products (e.g., capable of predicting issues and able to prevent them; capable of responding to failures; etc.). The 6G RAN requirements related to resilience are discussed in our contribution [2].

- Sensing capabilities will extend the applicability of networks beyond communication and connectivity. Critical communications services are typically operated in a broader context where situational awareness of the environment and surroundings are highly relevant. The 6G RAN requirements related to sensing are discussed in our contribution [3].

- Positioning requirements are currently being discussed at ITU. Our views are presented in [6]. Given that 3GPP will design 6G to meet the IMT-2030 requirements, the need for additional requirements is not clear at this point.

- Non-terrestrial networks can provide ubiquitous coverage. Although coverage is essential for any communication services, its relevance is even higher for mission critical communications. Non-terrestrial networks to provide coverage and as a tool for resilient connectivity are discussed in our contribution [4].

- Aerials are used for mission critical purposes (e.g., for surveillance, for site inspection, as a platform for deploying temporary nodes, etc.). Their requirements are discussed in our contribution [5].

- Emergency communications and support of emergency calls and their related requirements (e.g. E911, etc.) are discussed in [1].

- Private networks with their evolved requirements for 6G.

# 3	Text proposal

Based on the discussion above we propose the following text proposal.

## x.x Critical Communication services The 6G RAN architecture shall support critical communication services, avoiding a different architecture or multiple architectural options. The 6G Radio shall support the QoS framework in coordination with SA2, including e.g., prioritization, pre-emption, and access control mechanisms. ### x.x.1 Public safety communications The RAN design for the 6G Radio Access Technologies shall support Mission Critical Communications for diverse types of services (e.g. MCPTT, MCVideo, MCData). ### x.x.2 Radio robustness The 6G Radio shall provide mechanisms to enhance the privacy of radio transmissions of signals and channels. The 6G Radio shall provide mechanisms to enhance the security of radio transmissions of system information (e.g., encryption, integrity protection, etc.). ### x.x.3 Ad-hoc deployments The 6G Radio and the 6G RAN architecture shall enable simple implementations of network equipment that can be quickly and efficiently deployed, for the purpose of providing coverage, capacity, etc. The support shall include ad-hoc deployments co-located with other network deployments (e.g., extending the coverage at cell edge; subject to radio interface) and isolated ad-hoc deployments (e.g., creating a separate coverage area, possibly connected to the public CN; etc.). The support shall cover stationary ad-hoc deployments as well as ad-hoc deployments that change over time (e.g., due to moving nodes). The 6G RAN architecture shall make it possible to utilize backhaul solutions suitable for ad-hoc deployments (e.g., with capacity limitations, with longer latencies, etc.). ### x.x.4 Public warning/emergency alert systems The RAN design for the 6G Radio Access Technologies shall provide mechanisms to enable public warning services that provides warning/notifications to users meeting regional regulatory requirements. ### x.x.5 Broadcast and multicast communications The design of the 6G System shall allow for the 5G broadcast and multicast system to be reutilized, ensuring delivery to users of earlier generations and utilizing the spectrum resources in an efficient manner. ### x.x.6 Device-to-device communications The 6G system shall consider device-to-device communication for critical services (e.g., ProSe) in a way that ensures interoperability with 5G D2D users without using the spectrum resources in an inefficient manner (e.g., without duplicating traffic). UE-to-NW multi-hop relaying involving 6G and/or 5G UEs and networks shall be supported. UE-to-UE multi-hop relaying involving 6G and/or 5G UEs shall be supported.

# 4	Conclusion

In this contribution we propose to add the Text Proposal in section 3 to TR 38.914.

# References

- RP-252206, T-Mobile USA Inc. et al., “On support of Voice in 6G”, 3GPP TSG RAN Meeting #109, Beijing, China, September 15-18, 2025

- RP-251946, Ericsson et al. “Resiliency requirements in 6G”, 3GPP TSG RAN Meeting #109, Beijing, China, September 15-18, 2025

- RP-251989, T-Mobile USA Inc. et al., “6G Sensing”, 3GPP TSG RAN Meeting #109, Beijing, China, September 15-18, 2025

- RP-251949, Ericsson et al. “3GPP specific requirements on NTN”, 3GPP TSG RAN Meeting #109, Beijing, China, September 15-18, 2025

- RP-251948, Ericsson et al. “Requirements for supporting aerial UEs in 6G”, 3GPP TSG RAN Meeting #109, Beijing, China, September 15-18, 2025

- RP-251952, Ericsson, Technical Performance Requirements (TPR) for IMT-2030 related to WP5D, ”, 3GPP TSG RAN Meeting #109, Beijing, China, September 15-18, 2025