3GPP TSG RAN Meeting #109			                                     RP-252670

Beijing, China, September 15-18, 2025

Agenda Item:	9.1.4.6

Source:	CSCN

Title:	Views on NTN related RAN4 led R20 topics

Document for:	Discussion/Decision

Introduction

In RAN#108 meeting, some agreements about high level scope for WI on NTN enhancement in Rel-20 was summarized in RP-251879 [1] as following.

- NTN HPUE - Finish remaining PC1.5 (~29 dBm) work for hand-held devices for both NR NTN and IoT NTN (several studies and discussions already done in R19) - NR NTN with 2TX and/or FFS 1TX - NTN HD-FDD VSAT for Ka/Ku bands - RAN4 will start with HD-FDD mobile VSAT with small antennas (e.g 20x20cm). - Once the mobile VSAT with small antennas work is done, - NTN HD-FDD for fixed VSAT and the mobile VSAT with large/normal antenna will be discussed, including how the agreements for the mobile VSAT with small antenna can be leveraged as applicable - RAN1/2 impact is expected and will be further discussed in RAN#109 - Leverage RedCap over NTN HD-FDD solution to NTN as applicable. - NTN DL CA - Intra-SAN CA limited to DL intra-band contiguous and non-contiguous with 2CC - Example bands will be further decided within L/S-band and Ku/Ka-band as example bands - Depending on the available band combinations, interested companies can propose to extend this work to intra-band with more than 2CC in the check-point in June 2026 - RAN1/2 impact is expected and will be further discussed in RAN#109 - Spec clean-up/fixing: Missing Channel BW Common Core and Perf Requirements - 30MHz (DL/UL), 40MHz (priority DL), 25MHz with PC3 - Take one example band with minimum work with PC3 - The rest cases can be considered as part of spectrum work once the requirements on the example band is completed - Target 1TU in Q4’25 and 1.5 TU allocation including RF, RRM and demod from Q1’26

In this contribution, we provide our views about the scopes of candidate RAN4 led NTN topics in Rel-20 for further discussion and decision as per [2].

Discussion

  - NTN HPUE

Justification

For NTN operation, uplink coverage is one of the major issues, while HPUE is an effective solution which can significantly improve UL performance to meet deployment demands. Hence, HPUE for NTN is studied in Rel-19 regarding the coexistence analysis and HPUE for NR/IoT-NTN in certain bands for single UL carrier scenario, as well as the necessary signalling to support HPUE.

Device types supported for each power class depend on feasibility study and corresponding requirements, while PC1.5 hand-held UE for NR-NTN is precluded in Rel-19. In addition to non-handheld device with different power classes discussed in previous releases, PC1.5 handheld devices with great prospect for commercial applications should be studied further. Although there maybe potential implementation challenges on RF front-end design (e.g. Duplexer) for PC1.5 hand-held UE, PC1.5 has already been supported for handheld devices on existing TN TDD bands.

Similar to the existing studies on HPUE for NTN, even for hand-held UE in typical NTN use cases, the higher power requirements of PC1.5 is not a major issue. At the same time, the existing regulation and specifications, such as ECC/CEPT and FCC, allow significantly higher UE transmit powers for handheld and automotive UE. Therefore, regulation is not expected to be a barrier.

Both NR-NTN and IoT-NTN could be included since the techniques share the commonality for them.

Proposed Objectives

Objective of Core part WI

- Support PC1.5 handheld UE for FDD NR-NTN and IoT-NTN on the co-existence study.

- Specify 1TX and/or 2TX solution for PC1.5 NR-NTN handheld UE

- Specify NTN HPUE RF requirements for PC 1.5 NR-NTN and IoT-NTN Handheld UE.

- Power boosting scheme could be considered to achieve PC 1.5.

- Example bands include bands n255, n256, and n254.

Work Groups

RAN4

  - NTN HD-FDD mobile VSAT for Ka/Ku bands

Justification

Compared with full-duplex systems, half-duplex systems are fully capable of meeting the performance requirements in many realistic UE applications without high data rates, and achieving lower implementation complexity as the same time.

Considering the critical commercial interests of potential NTN enhancements in 5G-A Rel-20, half-duplex FDD mobile VSAT has a clear commercial demand which comes from the lower cost and smaller antenna. In this configuration, UE device can use one phased antenna array to fulfill the requirement of transmission and reception in half duplex mode. It can significantly reduce the size and weight of NTN device and simplify RF design with higher reliability, which is very necessary in the scenarios of vehicle mounted terminals, ship-borne terminals and portable terminals. Consequently, it is the key to making NTN VSAT devices more popular, and then achieves commercial success in the end.

In addition, compared with Ku band, Ka band can accommodate a wider channel bandwidth. Owe to the flat array antenna with even smaller size and lighter weight, mobile HD-FDD VSAT operating in Ka band may have better prospects and more potential applications.

In light of the similar RRM requirements for mobile VSAT, it is a feasible and more efficient approach to consider Ka band and Ku band within a unified framework. If the outcomes of the study on Ku band can be reused to Ka band as much as possible, or vice versa, the overall workload can be minimized to the greatest extent possible. At the same time, existing RedCap over NTN HD-FDD solution can be leveraged as much as possible to minimize the workload and exclude the impact on RAN1/2.

Proposed Objectives

Objective of Core part WI

- Consider HD-FDD mobile VAST with small phase array antenna antennas.

- Specify RF requirements for NTN HD-FDD VSAT, at least including

- Specify the RSD requirements on the example bands, if necessary

- Define Maximum output power limits and Minimum peak EIRP.

- Specify new VSAT type for NTN HD-FDD VSAT, at least including

- Mobile VSAT communicating with LEO with electronic steering antenna.

- Consider the conflict resolution strategy of NTN HD-FDD VSAT for Ka/Ku bands.

- It is recommended to reuse the HD-FDD conflict strategy for the Ka/Ku frequency bands in RedCap.

- Consider more collision conflict resolution strategy for the Ka/Ku bands if necessary.

- HD-FDD duplex mode should be defined and report to SAN for NTN.

- Example bands include bands n512, n511, n510.

- No RAN1/2 impact is expected.

Proposed Objectives

Objective of performance part WI

- Specify the necessary RRM performance requirements and test cases for NR-NTN in Ka/Ku bands

- Specify the necessary demodulation and CSI performance requirements for NR-NTN in Ka/Ku bands

Work Groups

RAN4

Conclusion

In this contribution, we propose the objectives related to candidate RAN4 led NTN topics NTN HPUE and NTN HD-FDD VSAT for Ka/Ku bands as above.

Reference

[1] RP-251879, WF for RAN4 led REL-20 topics

[2] RP-251892, Revised agenda for RAN #109