3GPP TSG RAN Meeting #109														RP-252698

Beijing, China, September 15-18, 2025

Agenda Item:			9.1.4.7

Source:					Ofinno

Title:							Low Band CA via Switching Enhancement

Document for:		Decision

# Introduction

The low band carrier aggregation (CA) via switching based on RRC based semi-static pattern has been recently completed and introduced in Rel-19 [1].

At RAN#108, a WF on RAN4 package on non-spectrum Rel-20 items, was endorsed [2]. According to the endorsed WF [2], the following high-level objectives related to the enhancement of low band CA via switching were endorsed:

11 Cross RF and baseband topics 11.1 Agreements: The following two WI are agreed to be introduced in R20. - Extension of LBCA (the exact scope and WID draft will be discussed and decided in RAN#109): - Further discussion on the possible scoping in RAN#109 based on the following two objectives - Study and if feasible, specify non-RRC based switching solution, e.g. dynamic switching, by taking the RAN1 impacts into consideration - Dual DL solution - RAN1/2 impact is expected and will be further discussed in RAN#109 - Target 0.5 TU (0.25 RF and 0.25 RRM) in Q4’25 and 0.75 TU allocation (0.25 RF and 0.5 RRM) from Q1’26

For further discussion on the scoping of the Rel-20 RAN4 work in RAN#109, we provide our views on the enhancement of low band CA via switching.

# Discussion

The study item in [3] conducted feasibility of various types of low band combinations (e.g., involving bands in 700/800/900MHz), which pose implementation challenges in the design of the UE RF architecture. Due to these implementation constraints, simultaneous CA operation on multiple bands is infeasible. Some of these challenging low band combinations as analyzed in [3-5], can be categorized as:

- SDL + FDD: large frequency separation between SDL and FDD (e.g., band n29 (SDL)+band n5, band n29+n14, n29+n71, etc.)

- SDL + FDD: SDL within the FDD duplex gap (e.g., n29+n12, n67 (SDL)+n28)

- FDD + FDD: UL of one FDD band overlapping with DL of the other FDD (e.g., band n5+band n8)

- FDD + FDD: UL of one FDD band close to DL of the other FDD (e.g., band n26+band n106)

The existing Rel-19 solution on the low band CA via switching based on RRC semi-static pattern [1] is suitable for the first type of low band combination stated above (e.g., n29+n5, etc.).

While Rel-19 semi-static pattern can also be used for the second type of low band combination stated above (e.g., n29+n12, etc.), but such pattern will not be optimal and will lead to user throughput loss. The “Dual DL” solution listed in the WF on the RAN4 package is aimed to address such SDL+FDD band combinations via new semi-static pattern. For example, the UE can receive/transmit (RX/TX) on FDD band (DL/UL) in one time period and receive (RX) on both SDL and DL of FDD in another time period of the semi-static pattern.

In our view, the CA operation in SDL+FDD combinations can also be performed using non-RRC based switching solution (i.e., based on dynamic switching). For example, the DCI (e.g., scheduling both PCell and SCell) via the PCell can indicate the UE to switch from the RX/TX mode (on FDD DL/UL) to RX mode (on FDD DL and SDL) or vice versa.

Furthermore, the dual DL solution will not address FDD+FDD combinations, which may typically require switching across different FDD bands (i.e., RX/TX on one FDD band at a time). Therefore, CA operation of low band FDD+FDD combinations can be more efficiently performed using non-RRC based switching solution.

Overall, we think that the non-RRC based switching solution (i.e., based on dynamic switching) is feasible for various types of low band CA combinations. Compared to the dual DL solution based on semi-static pattern, the non-RRC based switching solution will also provide more flexibility to the network in managing different types of physical layer procedures (e.g., scheduling, CSI reporting, RACH procedures, etc.). Furthermore, to limit RAN1 and RAN4 workload, it would be more suitable to limit the scope of the WI to a single solution based on the non-RRC based switching.

- Observation 1: Dual DL solution is suitable for SDL+FDD (with SDL within FDD duplex gap) but may not usable/optimal for other types of band combinations.

- Observation 2: The non-RRC based switching solution (i.e., based on dynamic switching) can be applied for various types of low band combinations (e.g., with appropriate DCI design).

- Observation 3: The non-RRC based switching solution (i.e., based on dynamic switching) is also less restricted, enabling the network to more efficiently handle different types of physical layer procedures (e.g., scheduling, RACH procedure, CSI reporting, etc.).

- Proposal 1: It is proposed to limit the scope of Rel-20 WI on enhancement of low band CA via switching to non-RRC based switching solution (i.e., based on dynamic switching).

# Summary and conclusion

Based on the analysis provided above, our observations and proposal related to the Rel-20 WI on enhancement of low band CA via switching are as follows:

- Observation 1: Dual DL solution is suitable for SDL+FDD (with SDL within FDD duplex gap) but may not usable/optimal for other types of band combinations.

- Observation 2: The non-RRC based switching solution (i.e., based on dynamic switching) can be applied for various types of low band combinations (e.g., with appropriate DCI design).

- Observation 3: The non-RRC based switching solution (i.e., based on dynamic switching) is also less restricted, enabling the network to more efficiently handle different types of physical layer procedures (e.g., scheduling, RACH procedure, CSI reporting, etc.).

- Proposal 1: It is proposed to limit the scope of Rel-20 WI on enhancement of low band CA via switching to non-RRC based switching solution (i.e., based on dynamic switching).

# References

- RP-251315, “Revised WID on Low NR band carrier aggregation via switching”, TELUS (Rapporteur).

- RP-251879, Way forward on RAN4 R20 package, RAN4 Chair (Apple).

- TR 38.872, “Study on enhancement for 700/800/900MHz band combinations”, Rel-18.

- RP-251668, Motivation for low-band carrier aggregation via switching enhancement in Rel-20, TELUS, Bell Mobility, Apple, BT plc, Skyworks Solutions, Inc, Murata, AT&T.

- RP-251337, Enhancement of the low NR band aggregation via switching framework, Apple.