3GPP TSG RAN Meeting #109		RP-252712

Beijing, China, Sep 15th – 18th, 2025

Title: 	Views on UE OTA works for Rel-20 5G-A

Agenda Item:	9.1.4.3

Source: 	CATT

Document for:	Discussion and Decision

# Introduction

UE OTA test methods and requirements continue to draw attention from operators worldwide. A well-defined scope for Rel-20 is essential to keep this work advancing. The following high-level scope for WI on UE OTA enhancement are extracted from guidance in [1] item 7.1:

- Agreements: The high-level scope for WI on UE OTA enhancement (the exact scope and WID draft will be discussed and decided in RAN#109): - The performance requirements of UE OTA, including TRP/TRS and MIMO OTA, for new bands - Handheld UE is the first priority, other UE types are 2nd priority if sufficient number devices is available - initial bands n7,n20, n79, needs operators input for other bands - the total number of band should be limited to [5] - prioritized UE type should be specified in the WID - Specify test method to support NTN VSAT - Define the measurement setup, test procedure, and preliminary MU - No RAN1/2 impact is expected - Target 0.5 TU allocation

In this contribution, we share our views on the two technical topics captured in the agreements: UE OTA performance requirements and NTN VSAT test methodology.

# Proposals on Rel-20 OTA WI

## 2.1	UE OTA performance requirements for new bands

Through standardization efforts from Rel-17 to Rel-19, UE OTA requirements—including TRP, TRS, and MIMO OTA—have been defined for only a limited number of NR bands, as summarized in Table - 1 (noting that the performance part in Release 19 is still under development). There is a growing demand to extend these performance requirements (TRP/TRS and MIMO OTA) to additional new frequency bands.

Table - 1 NR bands with defined TRP TRS and MIMO OTA requirements in early releases

| Requirements for NR band | Rel-17 | Rel-18 | Rel-192 |
| --- | --- | --- | --- |
| TRS and TRS | n41, n78 | n1, n28, n41, n78 | n3, n5, n7, n8, n20 |
| MIMO OTA | n41, n78 | n1, n5, n28 | n3, n7, n77 and n8 |
| Note 1: The PC2 with 1Tx requirement for n41 and n78 for browsing mode is defined in Rel-17 and for talk mode is defined in Rel-18. Note 2: Rel-19 Performance part is unfinished. Final Requirements and recommended TT for 1Tx TRP TRS and static MIMO OTA is incomplete. |  |  |  |

In Rel-20 5G-A works, defining performance requirements (TRP/TRS and MIMO OTA) for handheld UEs operating in the new bands n7, n20, and n79 should be treated as a top priority.

Moreover, it was already confirmed during Rel-19 discussions that XR OTA work is a key requirement from global operators. RAN4 agreements in Rel-19 on the test method and performance metric for XR devices is specified in approved WF [2] and extracted as below:

- Sub-topic 1-2 Test method and Performance metric for XR devices Issue 1-2-2: If TRP/TRS is kept as baseline, whether a new additional XR performance metric can be considered (as optional)? Agreements: - For XR, keep current TRP/TRS metric as baseline and add NHPRP/NHPRS into TR. RAN4 is not intended to preclude NHPRP/NHPRS as long as it is technically justified. - FFS future requirements, single metric will be used for future XR OTA requirements.

It is also agreed to keep TRP/TRS metric as baseline for XR devices. The specific OTA performance requirements remain undefined. Moreover, a proper head phantom for XR devices is still absent, as stated in clause 4.2.1 of [3].

### 4.2.1 UE use scenarios for TRP TRS test The following use scenarios are considered for TRP TRS test: - Talk mode using head & hand phantom for narrow phones between 56 mm and 72 mm and for wide phones with a width >72 mm and <92 mm. - Browsing mode using hand phantom for narrow and wide phones - Using forearm phantom for wrist-worn devices - Using XR Head phantom for headworn XR devices. FFS details - Free Space is used for devices not used in above-mentioned scenarios, other phantoms are not precluded for wearable devices

In Rel-20, RAN4 should take head-worn XR devices as secondary priority and specify OTA requirements with proper head phantom. In this way, the corresponding scope for UE OTA performance requirements should capture below contents:

Proposal 1:	Objectives for this performance part work item are as follows:

- Specify FR1 TRP and TRS requirements for NR bands for UE:

  - Handheld UEs with 1Tx at new bands n7, n20, n79 are first priority.

  - Handheld UEs with 2Tx (non-coherent UL MIMO) at n79 depending on measurement data availability are first priority

  - non-RedCap XR devices are secondary priority

    - Note: Suitable head phantom is required for headworn XR devices, consider the coordination with CTIA on this aspect

- Specify MIMO OTA performance requirements for NR FR1 bands:

  - Handheld UE (2 layers or 4 layers)

    - Bands n7, n79 are first priority

  - The requirements are specified based on Rel-17 static MPAC test method

## 2.2:	NTN VSAT requirements and test methodology

In Rel-19, the OTA test method has been defined for handheld NTN device. Based on the interest from industry, it’s time to develop OTA test methodology and performance metrics for NTN VSAT devices.

For NTN VSAT device OTA testing, the conflict between large-aperture far-field requirements and chamber size could be a challenge. Besides, according to the definition of NTN VSAT types in TS 38.101-5, NTN VSAT devices are generally divided into two main classes, with five distinct types in total.

TS 38.101-5 Table 9.2.1.0-1: The definitions of NTN VSAT Types NTN VSAT class; NTN VSAT type; Type description. Fixed VSAT; 1; Fixed VSAT communicating with GSO and LEO with mechanical steering antenna.. 22; Fixed VSAT communicating with GSO and LEO with electronic steering antenna.. 3; Fixed VSAT communicating with LEO only with electronic steering antenna.. Mobile VSAT; 4; Mobile VSAT communicating with GSO with mechanical steering antenna.. 52; Mobile VSAT communicating with GSO with electronic steering antenna.. NOTE 1: The NTN VSAT types are assuming NTN VSAT has only one antenna beam towards one satellite at a given time in this release. NOTE 2: NTN VSAT may need power reduction to comply with OFF-axis EIRP requirement defined in clause 9.2.2. There is no requirement for the potential power reduction..

The test methods and performance metrics for Fixed VSAT and Mobile VSAT may be different, so two separate test methodologies are likely to be required. Compared with Fixed VSAT, Mobile VSAT OTA testing should additionally consider mobility-specific factors such as time-variant steering antenna and angular rates, etc. Considering the limited TU allocation, we suggest to take Fixed VSAT as first priority in Rel-20. The corresponding scope for NTN VSAT OTA testing should capture below contents:

Proposal 2:	Objectives for this core part work item are as follows:

- Study and define test methodology and radiated performance metric for NTN VSAT devices:

  - Fixed VSAT devices are first priority.

  - Take band n512 as example band

  - Study the usage scenarios and develop test methodology for TRP/TRS requirements

  - Develop preliminary Measurement Uncertainty (MU)  (RAN5)

- Note: No RAN1/2 impact is expected

# 3 Conclusion

In this contribution, we propose the following exact scope for Rel-20 OTA WI:

Proposal 1:	Objectives for this performance part work item are as follows:

- Specify FR1 TRP and TRS requirements for NR bands for UE:

  - Handheld UEs with 1Tx at new bands n7, n20, n79 are first priority.

  - Handheld UEs with 2Tx (non-coherent UL MIMO) at n79 depending on measurement data availability are first priority

  - non-RedCap XR devices are secondary priority

    - Note: Suitable head phantom is required for headworn XR devices, consider the coordination with CTIA on this aspect

- Specify MIMO OTA performance requirements for NR FR1 bands:

  - Handheld UE (2 layers or 4 layers)

    - Bands n7, n79 are first priority

  - The requirements are specified based on Rel-17 static MPAC test method

Proposal 2:	Objectives for this core part work item are as follows:

- Study and define test methodology and radiated performance metric for NTN VSAT devices:

  - Fixed VSAT devices are first priority.

  - Take band n512 as example band

  - Study the usage scenarios and develop test methodology for TRP/TRS requirements

  - Develop preliminary Measurement Uncertainty (MU)  (RAN5)

- Note: No RAN1/2 impact is expected

# Reference

- RP-251879 WF for RAN4 led REL-20 topics, RAN4 Chair (Apple)

- R4-2512693 Way Forward for [116][337] TRP_TRS_MIMO_OTA_Ph3, VIVO

- TR 38.870 Enhanced Over-the-Air (OTA) test methods for NR FR1 Total Radiated Power (TRP) and Total Radiated Sensitivity (TRS), V19.1.0