3GPP TSG-RAN Meeting #109                                                 RP-252738

Beijing, China, 15th – 18th September, 2025

Source: 	Huawei, HiSilicon

Title: 	Proposals for RAN4-led Rel-20 UE RF topics

Agenda Item:	9.1.4.1

Document for:	Discussion and Decision

Introduction

In last RAN plenary meeting, Rel-20 RAN4 topics have been discussed and a WF can be found at [1]. In this contribution, we provide our views for topic of UE RF enhancement in Rel-20.

Discussion

Agreements in RAN#108:

The high-level scope of the WI on UE RF centric evolution (the exact scope and WID draft will be discussed and decided in RAN#109): - FR1 HPUE enhancement - Further consider/discuss until RAN#109 on the detailed scope - This work is to focus on the TN application - 6MHz channel bandwidth - It can be prioritized to FDD band <1GHz - Study and, if feasible, reduce UE MPR values with the relaxed Tx EVM requirement for 5G NR higher modulation orders, i.e., 64QAM, 256QAM - BS demod scope is included. - The study is performed based on non-AI-based approach at BS receiver as baseline and without RAN1 impact - The details for study is to be discussed until RAN#109 - This work will be started in Q1’26. If the similar scope (The examples include the enhancements enabled by BS advanced receiver and UE Tx signal modifications. It is noted that referring to these examples in this WF should not be deemed as taking these examples as the baseline or starting point in 6G study) is agreed by RAN4 to be studied in 6G, it will be removed from 5G-A package. In this case, it is possible that 6G related conclusion can be applied to 5G. - No RAN1/2 scope is expected - Target 0.5 RF TU in Q4’25 and 1.25 TU (0.75 RF and [0.5] Demod for EVM relaxation and 6MHz) from Q1’26.

HPUE

The main topics for UE RF centric evolution are clear, however, the scope for FR1 HPUE enhancement is not determined yet. The following HPUE enhancements have been proposed in last RAN meeting.

| HPUE SC | 4Tx PC1 FWA (4x26dBm) |
| --- | --- |
|  | 2Tx PC1 FWA (2x29dBm) |
|  | 2Tx PC1.5 FDD (2x26dBm) |
|  | 1Tx PC1.5 TDD (1x29dBm) |
|  | 3Tx PC1.5 (including FDD PC2) |
| HPUE CA/DC | PC2/PC1.5 EN-DC with 2Tx or 3Tx (PC2 in LTE band) |
|  | PC1.5 with 4Tx or 8Tx for UL intra-band and inter-band CA |
|  | 3UL CC in two bands for CA/EN-DC with PC2 / PC1.5 (2Tx) with 2CC in NR |
|  | PC1 with DL CA |
|  | PC1.5(2Tx) FR1 PDE for intra-band CA |

Many proposals for HPUE enhancements were discussed at the last RAN plenary meeting, covering both single carrier and CA/DC scenarios. However, due to limited available TU, it may be difficult to address all of these enhancements within Release 20. It should be noted that even for single carrier alone, completing the study of all proposed features is not feasible given the current TU constraints. Therefore, we believe it would be more practical to focus on single-carrier enhancements with a well-defined and reasonable scope.

Observation 1: Limited TU cannot support so many HPUE enhancement proposals in Rel-20.

The topic of 4Tx was initially discussed in Rel-18. At that time, only the requirements for Power Class 1.5 were finalized; however, higher power classes were not excluded from consideration from the very beginning. The fact that the extension of PC1.5 to other power classes was not addressed in Rel-18 was primarily due to time constraints.

Since then, there has been continued interest from operators in further expanding the power class for 4Tx FWA UE. Similar enhancements were discussed for NTN HPUE in Rel-19, where 4Tx requirements were specified for PC1. Although coexistence studies indicated that a smaller ACLR could ensure system performance while maintaining interference at an acceptable level, the final specified requirements were based on conservative assumptions, which do not fully exploit the uplink capability.

Given the shared interest among stakeholders, we propose that 4Tx PC1 FWA be considered as a key focus for HPUE enhancements in Rel-20.

Observation 2: Further enhancement of 4Tx PC1 FWA UE is well recognized by vendors and operators.

Observation 3: 4Tx PC1 for NTN HPUE was supported in Rel-19, but with stringent ACLR not aligned with the co-existence study result.

Proposal 1: 4Tx PC1 FWA RF as one major HPUE enhancement should be included in the UE RF WI for Rel-20.

Besides 4Tx PC1, there is also relatively strong interest in enhancements for 2Tx PC1.5 FDD and 1Tx PC1.5 TDD. While it is well understood that higher power can improve coverage and uplink performance, the associated implementation challenges—particularly for handheld UEs—must also be taken into account. Achieving a higher power class not only requires larger PA output power, but also poses significant challenges in thermal management for UE design and power handling capability for filter design. At present, we feel that the industry is not yet fully prepared to address these challenges from an RF component perspective.

Regarding 2Tx PC1, as previously mentioned, it depends on the use of a 1Tx 29dBm PA, a solution for which component-level implementation may not yet be mature. We therefore recommend focusing on PC1 FWA UE designs that utilize widely available PA technology, specifically those based on a 26dBm implementation.

Observation 4: Supporting 2Tx PC1.5 (2×26 dBm) for FDD and 1Tx PC1.5 (1×29 dBm) remains challenging due to thermal dissipation constraints and limitations in filter power capacity design.

Proposal 2: Given current implementation difficulties and the immaturity of key RF components, such as PAs and filters, it is recommended to prioritize and conduct a down-selection among the proposed enhancements for 2Tx PC1.5 FDD and 1Tx PC1.5 TDD HPUE.

Proposal 3: Study phase would be needed for HPUE enhancement for the challenged cases when handheld UE is taken into account.

With above observations and proposals, the proposed objectives for HPUE in Rel-20 WI are listed below:

High power UE (HPUE) for single carrier in terrestrial network (TN) - Specify the UE RF requirements to support PC1 4Tx for CPE/FWA UE with 4x26dBm PA configurations - Co-existence study for the proper ACLR requirement - Supporting 4x4 UL MIMO and TxD - The power fallback agreement for Release 18 4Tx configurations remains valid - MPR and A-MPR requirements if applicable for the example bands - Example bands: TDD n41, n77/n78; FDD n1 - Supporting release independent from Rel-18 - [Study and, if feasible, specify UE RF requirements to support PC1.5 FDD with 2Tx] - Feasibility study first and , if feasible, to continue specifying the following requirements - Supporting 2x2 UL MIMO and TxD - MPR and A-MPR requirements if applicable for the example bands - Example bands: FDD n1, n3 - Specify release independence requirements if needed - [Study and, if feasible, specify UE RF requirements to support PC1.5 TDD with 1Tx for CPE/FWA UE] - Feasibility study first and, if feasible, to continue specifying the following requirements - MPR and A-MPR requirements if applicable for the example bands - Example bands: TDD n77/n78, n41 - Specify release independence requirements if needed

6MHz CBW

Regarding 6MHz CBW, originally it is identified as a potential work for Rel-20 in RAN#106 [2] when 3GPP approved a similar WI for 7MHz CBW [3]. Though RRM and Demod parts are considered for 7MHz CBW WI in Rel-19, it is established as a spectrum related WI. Companies have the feeling that the work for 6MHz would be quite similar to that of 7MHz, hence not too much controversial study expected. We don’t have strong view 6MHz should be considered as a non-spectrum or spectrum WI, but it is not appropriate to be considered as part of the UE RF enhancement WI.

Observation 5: Study of 6MHz CBW would be similar to that of 7MHz, and most of the study outcome for 7MHz could be reused for 6MHz.

Observation 6: 7MHz CBW WI was established as a spectrum WI in Rel-19.

Proposal 4: 6MHz CBW is considered as a separate WI, rather than part of UE RF enhancement WI for Rel-20.

Tx EVM relaxation

The topic of Tx EVM relaxation was extensively discussed during the last RAN plenary meeting. As per the WF agreed in the meeting, setting a study phase on this topic has been considered. However, certain aspects remain uncertain, particularly the following clause:

“This work will be started in Q1’26. If the similar scope (The examples include the enhancements enabled by BS advanced receiver and UE Tx signal modifications. It is noted that referring to these examples in this WF should not be deemed as taking these examples as the baseline or starting point in 6G study) is agreed by RAN4 to be studied in 6G, it will be removed from 5G-A package.”

To our understanding, AI-based receivers are expected to be a major topic in 6G, making it likely that Tx EVM relaxation will be discussed as well in the 6G context. This overlap introduces ambiguity regarding how the corresponding study should be conducted.

The aim of Tx EVM relaxation in 5G-Advanced is to conduct research and develop specifications (if feasible) within the confines of existing requirements and assumptions, such as MPR and EVM requirements for higher modulation orders, while preserving the current DMRS design and avoiding impacts on RAN1 spec. In contrast, 6G-related studies are not expected to be carried out under these limitations. Moreover, even if a similar topic is initiated under 6G from the outset, it remains unclear how such a study would proceed without sufficient RAN1 inputs on fundamental aspects like waveform and reference signal design.

Therefore, as stipulated in the approved WF, even if Tx EVM relaxation is incorporated into 6G studies due to overlapping proposals, the 5G-A study should be conducted concurrently with 6G efforts, leveraging its clearly defined requirements and constraints.

Regarding the approach to Tx EVM relaxation within the 5G-A context, two potential ways could be considered:

- Proceed according to the current WF by incorporating this topic under the Rel-20 UE RF WI. Once the 5G-A study is completed, subsequent normative work would be considered within the WI.

- Alternatively, should the 5G-A study be conducted in parallel with 6G efforts on Tx EVM relaxation, a checkpoint is considered upon its completion time to determine whether a dedicated WI for 5G-A Tx EVM relaxation is necessary.

Either way is fine with us.

Observation 1: Regarding the Tx EVM relaxation topic, the relevant requirements and assumptions are well-defined for the 5G-Advanced study. In contrast, the corresponding 6G study may require additional inputs from RAN1, such as those related to waveform and reference signal design, before it can proceed effectively.

Observation 2: The normative work for the 5G-Advanced Tx EVM relaxation study could be handled through different approaches: either within the existing UE RF Work Item, or through a dedicated checkpoint to determine the necessity of a separate WI, particularly if the topic is also being studied within the 6G Study Item.

Proposal 1: The Tx EVM relaxation study for 5G-A should proceed in parallel with 6G research from Q1 2026, regardless whether under the Rel-20 UE RF WI or within a 6G SI.

Proposal 2: If the Tx EVM relaxation study for 5G-A is conducted within 6G SI, a checkpoint should be established upon completion to determine whether a dedicated WI for 5G-A is necessary.

With above observations and proposals, the proposed objectives for Tx EVM relaxation in Rel-20 WI are listed below:

Power enhancement with relaxed UE Tx EVM and advanced BS receiver - Study and, if feasible, reduce UE MPR values with the relaxed Tx EVM requirement for 5G NR higher modulation orders, i.e., 64QAM , 256QAM - BS demod scope is included. - The study is performed based on non-AI-based approach at BS receiver as baseline and without RAN1 impact - Note 1: This work will be started in Q1’26. If the similar scope is agreed by RAN4 to be studied in 6G, it will be removed from 5G-A package. In this case, the study for 5G-A and 6G will be carried out in parallel. A check point is established upon completion to determine whether a dedicated WI for 5G-A is necessary. - Note 2: No RAN1/2 scope is expected - Note 3: the proper TU can follow the agreement in RP-251879

Conclusion

This contribution provides our considerations for RAN4 led Rel-20 UE RF topics.

HPUE

Observation 1: Limited TU cannot support so many HPUE enhancement proposals in Rel-20.

Observation 2: Further enhancement of 4Tx PC1 FWA UE is well recognized by vendors and operators.

Observation 3: 4Tx PC1 for NTN HPUE was supported in Rel-19, but with stringent ACLR not aligned with the co-existence study result.

Proposal 1: 4Tx PC1 FWA RF as one major HPUE enhancement should be included in the UE RF WI for Rel-20.

Observation 4: Supporting 2Tx PC1.5 (2×26 dBm) for FDD and 1Tx PC1.5 (1×29 dBm) remains challenging due to thermal dissipation constraints and limitations in filter power capacity design.

Proposal 2: Given current implementation difficulties and the immaturity of key RF components, such as PAs and filters, it is recommended to prioritize and conduct a down-selection among the proposed enhancements for 2Tx PC1.5 FDD and 1Tx PC1.5 TDD HPUE.

Proposal 3: Study phase would be needed for HPUE enhancement for the challenged cases when handheld UE is taken into account.

The proposed objectives for HPUE in Rel-20 WI are listed below. The objectives in [ ] are to be further prioritized and down selected.

High power UE (HPUE) for single carrier in terrestrial network (TN) - Specify the UE RF requirements to support PC1 4Tx for CPE/FWA UE with 4x26dBm PA configurations - Co-existence study for the proper ACLR requirement - Supporting 4x4 UL MIMO and TxD - The power fallback agreement for Release 18 4Tx configurations remains valid - MPR and A-MPR requirements if applicable for the example bands - Example bands: TDD n41, n77/n78; FDD n1 - Supporting release independent from Rel-18 - [Study and, if feasible, specify UE RF requirements to support PC1.5 FDD with 2Tx] - Feasibility study first and , if feasible, to continue specifying the following requirements - Supporting 2x2 UL MIMO and TxD - MPR and A-MPR requirements if applicable for the example bands - Example bands: FDD n1, n3 - Specify release independence requirements if needed - [Study and, if feasible, specify UE RF requirements to support PC1.5 TDD with 1Tx for CPE/FWA UE] - Feasibility study first and, if feasible, to continue specifying the following requirements - MPR and A-MPR requirements if applicable for the example bands - Example bands: TDD n77/n78, n41 - Specify release independence requirements if needed

6MHz CBW

Observation 5: Study of 6MHz CBW would be similar to that of 7MHz, and most of the study outcome for 7MHz could be reused for 6MHz.

Observation 6: 7MHz CBW WI was established as a spectrum WI in Rel-19.

Proposal 4: 6MHz CBW is considered as a separate WI, rather than part of UE RF enhancement WI for Rel-20.

Tx EVM relaxation

Observation 7: Regarding the Tx EVM relaxation topic, the relevant requirements and assumptions are well-defined for the 5G-Advanced study. In contrast, the corresponding 6G study may require additional inputs from RAN1, such as those related to waveform and reference signal design, before it can proceed effectively.

Observation 8: The normative work for the 5G-Advanced Tx EVM relaxation study could be handled through different approaches: either within the existing UE RF Work Item, or through a dedicated checkpoint to determine the necessity of a separate WI, particularly if the topic is also being studied within the 6G Study Item.

Proposal 5: The Tx EVM relaxation study for 5G-A should proceed in parallel with 6G research from Q1 2026, regardless whether under the Rel-20 UE RF WI or within a 6G SI.

Proposal 6: If the Tx EVM relaxation study for 5G-A is conducted within 6G SI, a checkpoint should be established upon completion to determine whether a dedicated WI for 5G-A is necessary.

The proposed objectives for Tx EVM relaxation in Rel-20 WI are listed below:

Power enhancement with relaxed UE Tx EVM and advanced BS receiver Core part: - Study and, if feasible, reduce UE MPR values with the relaxed Tx EVM requirement for 5G NR higher modulation orders, i.e., 64QAM , 256QAM - The study is performed based on non-AI-based approach at BS receiver as baseline and without RAN1 impact - Specify reduced MPR or appliable MPR requirements upon relaxed Tx EVM requirement - Introduce the network signaling and UE capability to support the above objectives, if needed - Note 1: This work will be started in Q1’26. If the similar scope is agreed by RAN4 to be studied in 6G, it will be removed from 5G-A package. In this case, the study for 5G-A and 6G will be carried out in parallel. A check point is established upon completion to determine whether a dedicated WI for 5G-A is necessary. Perf part: - Specify BS demodulation requirements and test cases when necessary

- References

[1] R4-2310394, LS on support of UE configured with two serving cells, each with SUL, CMCC

[2] RP-243322 Irregular channel bandwidths Way Forward

[3] RP-243323 New WID on 7 MHz Channel Bandwidth for n26 and n5