3GPP TSG-RAN WG2 Meeting #131	R2-2505600

# Bangalore, India, 25th – 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 2107 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Miscellaneous corrections on Aggregated SP Positioning SRS Activation/Deactivation MAC CE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2025-08-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - There are incorrect references to IE names, i.e. SRS-PosRRC-AggBW-InactiveConfigList and PosResourceSetLinkedForAggBW-List do not exist in ASN.1. They need to be replaced by SRS-PosRRC-InactiveAggBW-ConfigList and SRS-PosResourceSetLinkedForAggBW-List. - Editorial issues can be fixed, e.g. removing “the field” for the referenced IE names. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The incorrect references to SRS-PosRRC-AggBW-InactiveConfigList and PosResourceSetLinkedForAggBW-List have been replaced by SRS-PosRRC-InactiveAggBW-ConfigList and SRS-PosResourceSetLinkedForAggBW-List. - Editorial issues have been fixed. Impact analysis Impacted 5G architecture options: NR SA, (NG)EN-DC, NE-DC, NR-DC Impacted functionality: Bandwidth aggregation for SRS for positioning Inter-operability: - There are no interoperability issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistencies in the description of Aggregated SP Positioning SRS Activation/Deactivation MAC CE remain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.83 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

#### 6.1.3.83	Aggregated SP Positioning SRS Activation/Deactivation MAC CE

The Aggregated SP Positioning SRS Activation/Deactivation MAC CE is identified by a MAC subheader with eLCID as specified in Table 6.2.1-1b. It has a variable size with following fields:

-	Positioning SRS Aggregation ID: This field indicates one of the combinations of linked Positioning SRS Resource Sets corresponding to SRS-PosResourceSetAggBW-CombinationList or SRS-PosRRC-InactiveAggBW-ConfigList specified in TS 38.331 [5]. Value 0 corresponds to the first entry within the list SRS-PosResourceSetAggBW-CombinationList or SRS-PosRRC-InactiveAggBW-ConfigList; value 1 corresponds to the second entry within the list SRS-PosResourceSetAggBW-CombinationList or SRS-PosRRC-InactiveAggBW-ConfigList and so on;

-	C1, C2, C3: These fields indicate the activation/deactivation status of each Positioning SRS Resource Set that is linked for SRS for positioning bandwidth aggregation configured in SRS-PosResourceSetLinkedForAggBW-List or SRS-InactivePosResourceSetLinkedForAggBW-List specified in TS 38.331 [5]. C1 corresponds to the first entry in SRS-PosResourceSetLinkedForAggBW-List or SRS-InactivePosResourceSetLinkedForAggBW-List, C2 corresponds to the second entry in SRS-PosResourceSetLinkedForAggBW-List or SRS-InactivePosResourceSetLinkedForAggBW-List and so on. The Ci field is set to 1 to indicate that the Positioning SRS Resource Set corresponding to Ci shall be activated. The Ci field is set to 0 to indicate that the Positioning SRS Resource Set corresponding to Ci shall be de-activated;

-	S: This field indicates whether the fields Spatial Relation for Resource IDi are present. If the field is set to 1, the fields Spatial Relation for Resource IDi are present; otherwise, they are absent;

-	Spatial Relation for Resource IDi: The field Spatial Relation for Resource IDi is only present if MAC CE is used for activation, i.e. at least one of the C1, C2, and C3 is set to 1. There can be as many as 16 entries of Spatial Relation for Resource IDi depending on the RRC configuration. There are 4 types of Spatial Relation for Resource IDi, which is indicated by the F (F0 and F1) field within, defined as in Figure 6.1.3.36-2 to 6.1.3.36-5 in clause 6.1.3.36. Spatial Relation for Resource IDi corresponds to the spatial relation of the ith SRS resource of the SRS resources within the first activated carrier indicated by the fields C1, C2 and C3, that is aggregated with SRS resources from the other aggregated carriers according to TS 38.214 [7];

-	C: This field indicates whether the octets containing Resource Serving Cell ID field(s) and Resource BWP ID field(s) within the field Spatial Relation for Resource ID i are present, except for Spatial Relation Resource IDi with DL-PRS or SSB. If the combination of the linked SRS resource sets is configured by SRS-PosResourceSetAggBW-CombinationList and this field is set to 1, the octets containing Resource Serving Cell ID field(s) and Resource BWP ID field(s) in the field Spatial Relation for Resource IDi are present, otherwise if this field is set to 0, they are not present; if the combination of the linked SRS resource sets is configured by SRS-PosRRC-InactiveAggBW-ConfigList, this field shall be set to 0 and the octets containing Resource Serving Cell ID field(s) and Resource BWP ID field(s) in the field Spatial Relation for Resource IDi are not present;

-	R: Reserved bit, set to 0.

Figure 6.1.3.83-1: Aggregated SP Positioning SRS Activation/Deactivation MAC CE

End of changes