3GPP TSG RAN meeting #109 									    RP-252916

Beijing, China, September 15-18, 2025							  (revision of RP-252860)

## Status Report to TSG

Agenda item:			9.6.1.3

| WI / SI Name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_AIML_air |  |  |  |  |
| Unique ID | 1020093 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-251186 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: NA | Core part: Sept Dec ‘25 | Performance part: March ‘26 | Testing part: NA |  |
| Overall Completion level | Study Item: NA | Core part: 90% | Performance Part: 25% | Testing part: NA |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | TSG RAN WG1 |
| --- | --- | --- |
| Rapporteur | Name | Juan Montojo (RAN1); Xiaofeng Liu (RAN4); Marco Belleschi (RAN2/RAN3) |
|  | Company | Qualcomm; CAICT; Ericsson |
|  | Email | juanm@qti.qualcomm.com; liuxiaofeng1@caict.ac.cn; marco.belleschi@ericsson.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | Yes |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

1 TU/meeting to address RAN4 Core moved from RAN4 Perf in Q4-25.

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

// Specification support for beam management

Agreement

Adopt the following TP for the clarification of ranking information.

Reason for change: The description of ranking information for P-CRIs or P-SSBRIs is unclear.

Summary of change: Clarify that the ranking information is for P-CRIs or P-SSBRIs.

Consequences if not approved: UE cannot generate the ranking information.

TP for TS 38.214 Clause 5.2.1.4.3a P-CRI, P-SSBRI, and P-L1-RSRP reporting <omitted texts> For a CSI-ReportConfig with reportQuantity-r19 set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19' or 'p-ssb-index-RSRP-r19', if nroftimeinstance-r19 is configured, the UE is configured with TimeGap-r19 indicating a time gap between a reference time and the earliest time instance of nroftimeinstance-r19 time instance(s) (defined in slot(s)). If nroftimeinstance-r19 is greater than 1, the same time gap is considered between two consecutive time instances. The UE considers the reference time to be the slot of the latest one of each CSI-RS/SSB resource, for channel measurement, respective latest CSI-RS/SSB transmission occasion no later than the corresponding CSI reference resource of the CSI report. For P-CRI or P-SSBRI reporting without P-L1-RSRP, the ranking information of the nrofreportedpredictedrs-r19 P-CRIs or P-SSBRIs (per time instance, if nroftimeinstance-r19 is configured) is conveyed by the order of the P-CRIs or P-SSBRIs reported in the CSI report, where the first reported P-CRI or P-SSBRI ranks first. <omitted texts>

Agreement

Adopt the following TP for CSI reporting for monitoring.

Reason for change: The condition associated with CSI reporting for RS-PAI is unclear.

Summary of change: Clarify that the timeinstanceformonitoring-r19-th time instance is used in case of BM-Case2 for condition check. Adding the description on resource mapping methods depending on the size of the set for monitoring is smaller than or the same as the size of Set A.

Consequences if not approved: gNB and UE may have different understanding on the condition associated with CSI reporting for RS-PAI.

TP for TS 38.214 Clause 5.2.1.4.3b RS-PAI Reporting ##### 5.2.1.4.3b RS-PAI Reporting <omitted texts> - at least one of the nrofBestBeamforMonitoring-r19 identified CSI-RS resources, or SS/PBCH Block resources is mapped one of the nrofreportedpredictedrs-r19 reported P-CRI(s) or P-SSBRI(s), if nroftimeinstance-r19 is not configured, or for the timeinstanceformonitoring-r19-th time instance if nroftimeinstance-r19 is configured, of the linked report of the first CSI Reporting Setting, wherein the mapping between CSI-RS resources, or SS/PBCH Block resources of Resource Set for channel measurement of the second CSI Reporting Setting and CSI-RS resources, or SS/PBCH Block resources of Resource Set given by resourcesForSetA-r19 of the first CSI Reporting Setting is provided by the higher layer parameter RSMappingtoSetA in the second CSI Reporting Setting; - if this condition is met, the transmission occasion is counted as an accurate reference signal prediction instance; otherwise, it is not counted as an accurate reference signal prediction instance; - determine RS-PAI as the total count of accurate reference signals prediction instance(s), and the UE shall report RS-PAI for the second Reporting Setting, with a -bit field (M is given by nroftransmissionOccasion-r19). <omitted texts>

Agreement

Adopt the following TP in section 5.2.1.6, TS 38.214.

Reason for change: The CPU occupation time of the monitoring report is unclear.

Summary of change: For UE-sided model, regarding a CSI report with CSI-ReportConfig for monitoring, Rel-15 CPU occupation time is reused for CPU occupation time of the CSI report for all types of CSI reports (i.e., AP/SP/P CSI report).

Consequence if not approved: The CPU occupation time of the monitoring report is not aligned between the gNB and UE, leading to inefficient CSI report scheduling.

#### 5.2.1.6 CSI processing criteria #### <Unchanged part is omitted> For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity not set to 'none', or a CSI report with LTM-CSI-ReportConfig, or reportQuantity not set to 'none-bm-r19' or 'none-csi-r19', the CPU(s) (including and/or , for CSI reports with reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19' or 'rs-pai-r19', and CSI reports configured with the higher layer parameter [RRC_name-r19] are occupied for a number of OFDM symbols as follows: - A periodic or semi-persistent CSI report (excluding an initial semi-persistent CSI report on PUSCH after the PDCCH triggering the report and a semi-persistent CSI report on PUSCH configured with the higher layer parameter codebookType set to 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18') occupies CPU(s) from the first symbol of the earliest one of each CSI-RS/CSI-IM/SSB resource, or each CSI-RS/CSI-IM resource associated with all configured sub-configurations for periodic CSI report corresponding to a CSI-ReportConfig that contains a list of sub-configurations provided by csi-ReportSubConfigToAddModList, or each CSI-RS/CSI-IM resource associated with all activated/triggered sub-configurations for semi-persistent CSI report corresponding to a CSI-ReportConfig that contains a list of sub-configurations provided by csi-ReportSubConfigToAddModList, for channel or interference measurement, respective latest CSI-RS/CSI-IM/SSB occasion no later than the corresponding CSI reference resource, until the last symbol of the configured PUSCH/PUCCH carrying the report. #### <Unchanged part is omitted>

Agreement

Adopt the following TP for Clause 6.3.1.1.2 in TS 38.212.

Reason for change: For NW-sided model, for a CSI-ReportConfig with the number of reported RS is the same as the size of the corresponding resource set for channel measurement, the mapping between the reported RSRPs and the resources within the resource set is unclear.

Summary of change: Clarify the mapping between the reported RSRPs and the resources within the resource set when the number of reported RS of a CSI-ReportConfig is the same as the size of the corresponding resource set for channel measurement.

Consequence if not approved: The understanding of resources corresponding to reported RSRPs is not aligned between the gNB and UE.

Table 6.3.1.1.2-8G: Mapping order of CSI fields of one report for CRI/RSRP or SSBRI/RSRP reporting, if nrofReportedRS is configured CSI report number; CSI fields. CSI report #n; CRI or SSBRI #1 as in Table 6.3.1.1.2-6. CRI or SSBRI #2 as in Table 6.3.1.1.2-6, if reported. …. CRI or SSBRI # M as in Table 6.3.1.1.2-6, if reported. RSRP #1 as in Table 6.3.1.1.2-6. Differential RSRP #2 as in Table 6.3.1.1.2-6, if reported. …. Differential RSRP # M as in Table 6.3.1.1.2-6, if reported. NOTE: The value of M is configured by the higher layer parameter nrofReportedRS. If M is equal to the number of SSB/CSI-RS resources in the corresponding resource set for channel measurement, Differential RSRP #m, m = 2, …, M, corresponds to the (m-1)-th SSB/CSI-RS resources among the remaining M-1 SSB/CSI-RS resources other than the resource corresponding to CRI/SSBRI#1 in the resource set..

Agreement

For one CSI resource setting for Set A for CSI report for inference for UE-side model

- Only a single resource set can be configured for the CSI resource setting.

Agreement

Support the following on the reporting condition of CSI reporting for inference

- For BM-Case2, after the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, UE transmit a CSI report for inference only if receiving at least K latest consecutive transmission occasion for each of the RS resources in Set B no later than the corresponding CSI reference resource, where K is indicated by a new UE capability.

Agreement

Support the following on the reporting condition of CSI reporting for monitoring

- After the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, UE transmit a CSI report for monitoring only if receiving at least nroftransmissionOccasion-r19 latest transmission occasion for each of the RS resources in the resource set for monitoring no later than the corresponding CSI reference resource.

Agreement

For AP CSI report for inference, regarding the associated ID for aperiodic CSI resource setting with multiple resource sets for Set B

- Associated ID in CSI report configuration is for the resource set(s) for Set B

  - Note: This does not have impact on RAN1 specifications

Agreement

Adopt the following TP for TS 38.214 Clause 5.2.2.5.

Reason for change: The condition of reporting CSI report for inference for BM-Case2 is unclear.

Summary of change: Adding the condition of reporting CSI report for inference for BM-Case2.

Consequences if not approved: gNB and UE may have different understanding on the condition of reporting CSI report for inference for BM-Case2.

5.2.2.5 CSI reference resource definition <omitted texts> For a CSI-ReportConfig configured with the higher layer parameter reportQuantity set to 'etypeII-Doppler-r19', after the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, the UE reports a CSI report only after receiving at least one aperiodic or 𝐾𝑝 consecutive periodic/semi-persistent CSI-RS transmission occasion(s) for each of the CSI-RS resources in each CSI-RS resource group in the corresponding CSI-RS resource set for channel measurement and at least one CSI-RS and/or CSI-IM resource transmission occasion for each of the CSI-RS and/or CSI-IM resources in the corresponding resource set for interference measurement no later than the CSI reference resource and within the same DRX active time, when DRX is configured, and drops the report otherwise. The value of 𝐾𝑝∈{1,2,4} is indicated by UE capability, as defined in clause 5.2.1.6. For a CSI-ReportConfig configured with the higher layer parameter reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19' or 'p-ssb-index-RSRP-r19' and with nroftimeinstance-r19 is configured, after the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, the UE reports a CSI report only if receiving at least 𝐾BM latest consecutive transmission occasion(s) for each of the CSI-RS resources or SS/PBCH Block resources in the corresponding resource set for channel measurement no later than the CSI reference resource, and drops the report otherwise. The value of 𝐾BM is indicated by UE capability. <omitted texts>

Agreement

Adopt the following TP for TS 38.214 Clause 5.2.2.5.

Reason for change: The condition of reporting RS-PAI is unclear.

Summary of change: Adding the condition of reporting RS-PAI.

Consequences if not approved: gNB and UE may have different understanding on the condition of reporting RS-PAI.

5.2.2.5 CSI reference resource definition <omitted texts> For a CSI-ReportConfig configured with the higher layer parameter reportQuantity set to 'etypeII-Doppler-r19', after the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, the UE reports a CSI report only after receiving at least one aperiodic or 𝐾𝑝 consecutive periodic/semi-persistent CSI-RS transmission occasion(s) for each of the CSI-RS resources in each CSI-RS resource group in the corresponding CSI-RS resource set for channel measurement and at least one CSI-RS and/or CSI-IM resource transmission occasion for each of the CSI-RS and/or CSI-IM resources in the corresponding resource set for interference measurement no later than the CSI reference resource and within the same DRX active time, when DRX is configured, and drops the report otherwise. The value of 𝐾𝑝∈{1,2,4} is indicated by UE capability, as defined in clause 5.2.1.6. For a CSI-ReportConfig configured with the higher layer parameter reportQuantity set to 'rs-pai-r19', after the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, the UE reports a CSI report only if receiving at least nroftransmissionOccasion-r19 latest transmission occasion(s) for each of the CSI-RS resources or SS/PBCH Block resources in the corresponding resource set for channel measurement no later than the CSI reference resource, and drops the report otherwise. <omitted texts>

Agreement

Adopt the following TP in section 5.2.1.6, TS 38.214.

Reason for change: To capture the agreement to support two AI/ML PU pools.

Summary of change: Adding the description to support two AI/ML PU pools.

Consequences if not approved: two AI/ML PU pools are not supported.

#### 5.2.1.6 CSI processing criteria <Unchanged part is omitted> For CSI reports with reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', and CSI reports configured with the higher layer parameter [RRC_name-r19], the UE may indicate a second value for the number of supported simultaneous CSI calculations NCPU,2 with parameter SecondValuesSimultaneousCSI-ReportsPerCC in a component carrier, and SecondValuesSimultaneousCSI-ReportsAllCC across all component carriers, and if applicable, a third value for the number of supported simultaneous CSI calculations NCPU,3 with parameter ThirdValuesSimultaneousCSI-ReportsPerCC in a component carrier, and ThirdValuesSimultaneousCSI-ReportsAllCC across all component carriers, in addition to NCPU. If a UE supports NCPU,2x simultaneous CSI calculations it is said to have NCPU,2x CSI processing units for processing CSI reports. If L Lx CPUs are occupied for calculation of CSI reports in a given OFDM symbol, the UE has NCPU,2x-LLx unoccupied CPUs. If N CSI reports start occupying their respective CPUs on the same OFDM symbol on which NCPU,2x-L Lx CPUs are unoccupied, where each CSI report corresponds to OCPU,2x(n), the UE is not required to update the N-M2x requested CSI reports with lowest priority (according to Clause 5.2.5), where is the largest value such that Lx holds. If only the second value is indicated, x = 2. If both of the second value and the third value are indicated, x = 2,3 where the CSI reports with reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19' corresponds to either x = 2 or x = 3 subject to UE capability and the CSI reports configured with the higher layer parameter [RRC_name-r19] corresponds to either x = 2 or x = 3 subject to UE capability. For CSI reports with reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', and CSI reports configured with the higher layer parameter [RRC_name-r19] if a CSI report is not considered within any of M and M2x, the values for OCPU,1 and OCPU,2x are considered to be 0, for the procedure previously described in this clause and the UE is not required to update the CSI report. <Unchanged part is omitted> - for a CSI report with CSI-ReportConfig with reportQuantity-r19 set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', - if nroftimeinstance-r19 is not configured, OCPU,1=X1 and OCPU,2x=X2x, where the value of X1and X2x are reported by UE capability. - if nroftimeinstance-r19 is configured, OCPU,1=Y1 and OCPU,2x=Y2x, where the value of Y1and Y2x are reported by UE capability. - OCPU,1=X1 and OCPU,2x=X2x, for a CSI report configured with the higher layer parameter [RRC_name-r19], where the values of X1 and X2x are reported by UE capability. For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity not set to 'none', or a CSI report with LTM-CSI-ReportConfig, or reportQuantity not set to 'none-bm-r19' or 'none-csi-r19', the CPU(s) (including OCPU,1and/or OCPU,2x, for CSI reports with reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', and CSI reports configured with the higher layer parameter [RRC_name-r19]) are occupied for a number of OFDM symbols as follows: <Unchanged part is omitted>

Agreement

Support the configuration of following parameters in a CSI-ReportConfig with the higher layer parameter reportQuantity set to'none-bm-r19':

- TimeGap-r19, i.e., the time gap between two consecutive predicted time instances and between the reference time and the earliest predicted time instance.

- nroftimeinstance-r19, i.e., number of predicted time instances for BM-Case 2.

// Specification support for positioning accuracy enhancement

Agreement:

Adopt the text proposal to TS 38.215 v19.0.0 in Section 3 of R1-2505531.

Agreement

Endorse the following text proposal to TS 38.214 v19.0.0.

| Reason for change | Currently PRU is captured within the subsection of CPP, and it is limited to only when the UE is provided with phase related measurement together with other measurement(s). However, AI/ML positioning does not require the activation of CPP, although CPP is not prevented from being activated simultaneously. |
| --- | --- |
| Summary of change | Addition of PRU data in the general section of 5.1.6.5. |
| Consequences if not approved | The data related to PRU will not be supported for AI/ML positioning if CPP method is not activated simultaneously. |
| Suggested text proposal | TS 38.214 (V19.0.0) 5.1.6.5 PRS reception procedure ================= text omitted=============================== Within each window indicated by NR-DL-PRS-MeasurementTimeWindowsConfig, the UE expects that the indicated DL PRS resource sets across all dl-PRS-IDs are from one DL PRS positioning frequency layer, and that the number of indicated DL PRS resource sets associated with each dl-PRS-ID are the same. The UE may be provided with NR-PRU-DL-Info which contains measurement(s) performed by a positioning reference unit (PRU) [20, TS 38.305], the timestamps associated with the measurement(s), and the location information of the PRU. ================= text omitted=============================== |

Agreement

Update the following LPP parameters in higher layer parameter list to RAN2.

|  | Sub-feature group | RAN2 Parent IE | Parameter name in the spec | New or existing? | Description | Value range | Per (UE, cell, TRP, …) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Associated ID for Info #7 | UE-based positioning Case 1 | FFS for RAN2 | AssociatedID-TRP-LocationInfo | New | The associated ID (optional) provides implicit indication of Info #7. For given TRP(s), the same associated ID implies that geographical coordinates of the TRP(s) can be understood as consistent by the UE. The associated ID is not expected to provide the real value of Info #7 (i.e., geographical coordinates of the TRP(s) are not disclosed). An associated ID is configured per-cell (e.g., NCGI-r15). Note: Info #7 can be provided explicitly (as in legacy UE-based DL-TDOA) or implicitly by Associated ID. | FFS for RAN2 (e.g., 0..255) | Per cell |

|  | Sub-feature group | RAN2 Parent IE | Parameter name in the spec | New or existing? | Description | Value range | Per (UE, cell, TRP, …) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Assistance data Info #7 | UE-based positioning Case 1 | FFS for RAN2 | (It is up to RAN2 to decide which existing IEs to include for providing Info #7) | Existing | LMF can provide to UE the assistance information (optional) from legacy UE-based DL-TDOA: Info #7 in Table 8.12.2.1.0-1 in 38.305 v18.3.0. Note: Info #7 can be provided explicitly (as in legacy UE-based DL-TDOA) or implicitly by Associated ID. | Reuse existing | Per UE |

|  | Sub-feature group | RAN2 Parent IE | Parameter name in the spec | New or existing? | Description | Value range | Per (UE, cell, TRP, …) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Assistance data Info #1 ~ Info #6, Info #8 ~ Info #15 | UE-based positioning Case 1 | FFS for RAN2 | (It is up to RAN2 to decide which existing IEs to include for providing Info #1 ~ Info #6, Info #8 ~ Info #15) | Existing | LMF can provide to UE the assistance information (optional) from legacy UE-based DL-TDOA: Info #1 ~ Info #6, Info #8 ~ Info #15 in Table 8.12.2.1.0-1 in 38.305 v18.3.0. | Reuse existing | Per UE |

Conclusion

RAN1 has no consensus whether the IE for assistance data Info#7 and AssociatedID-TRP-LocationInfo  can be simultaneously enabled.

It is upto RAN2 to decide whether the IE for assistance data Info#7 and AssociatedID-TRP-LocationInfo  can be simultaneously enabled.

Agreement

Include the IE for Rel-19 enhanced measurement (i.e., UL SRS-TDCT, UL SRS-TDCP in 38.215) in higher layer parameter list to RAN3.

|  | Sub-feature group | RAN3 Parent IE | Parameter name in the spec | New or existing? | Description | Value range | Per (UE, cell, TRP, …) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rel-19 enhanced measurement | NG-RAN node assisted positioning Case 3b | FFS for RAN3 | FFS for RAN3 | New | Rel-19 enhanced channel measurement, which can be either (a) or (b): (a) UL SRS-TDCT; (b) paired UL SRS-TDCT and UL SRS-TDCP. |  | Per UE |

// Specification support for CSI prediction

Agreement

Adopt following TPs for TS38.214 Section 5.2.1.4.6.

| Reason for change | In current specification, there is no SGCS quantization mapping table. |
| --- | --- |
| Summary of change | SGCS quantization mapping table is added. |
| Consequences if not approved | Incomplete capture of the agreement. |
| TS38.214 ##### 5.2.1.4.6 CSI-PAI reporting << Unchanged parts are omitted >> - each calculated SGCS value is quantized to a 4-bit field, where the first codepoint ‘0000’ is used to indicate (0 0.3] value range, and the value range [0.3, 1] is quantized in a linear scale with 0.05 step size. Detailed mapping is specified in [xx, TS 38.abc]Table 5.2.1.4.6-1. Table 5.2.1.4.6-1: SGCS Quantization Mapping Code Point; Range. 0; [0.0000, 0.3000]. 1; (0.3000, 0.3500]. 2; (0.3500, 0.4000]. 3; (0.4000, 0.4500]. 4; (0.4500, 0.5000]. 5; (0.5000, 0.5500]. 6; (0.5500, 0.6000]. 7; (0.6000, 0.6500]. 8; (0.6500, 0.7000]. 9; (0.7000, 0.7500]. 10; (0.7500, 0.8000]. 11; (0.8000, 0.8500]. 12; (0.8500, 0.9000]. 13; (0.9000, 0.9500]. 14; (0.9500, 1.0000]. 15; Reserved. << Unchanged parts are omitted >> |

Agreement

Adopt following TPs for TS38.214 Section 5.2.1.4.6.

| Reason for change | Based on the agreement, the reported wideband SGCS is the average of SGCS values across all subbands, not directly derived from the average precoder. |
| --- | --- |
| Summary of change | The definition of SGCS needs to be changed to the average of SGCS values across all subbands. |
| Consequences if not approved | Incorrect capture of the agreement. |
| TS38.214 5.2.1.4.6 CSI-PAI reporting << Unchanged parts are omitted >> to report CSI-PAI for the second Reporting Setting, the UE shall: - determine predicted PMI (see Clause 5.2.2.2.10 or Clause 5.2.2.2.11) for each of the subband(s) configured by csi-ReportingBand and numberOfPMI-SubbandsPerCQI-Subband for the S-th time instance configured by timeinstanceforCSImonitoring-r19 in the second Reporting Setting ( , with N4 provided in the first Reporting Setting), based on the CSI prediction performed by the UE using the channel measurement corresponding to the first Reporting Setting, - determine non-predicted PMI for each of the subband(s) configured by csi-ReportingBand and numberOfPMI-SubbandsPerCQI-Subband, which is corresponding to the S-th time instance of the report of the first Reporting Setting, based on the CSI-RS resource(s) in the resource set for channel measurement for the report corresponding to the second Reporting Setting. - determine non-predicted PMI for each of the subband(s) configured by csi-ReportingBand and numberOfPMI-SubbandsPerCQI-Subband, based on the latest CSI-RS resource transmission occasion, no later than the CSI reference resource of the CSI report, corresponding to the first Reporting Setting - for , v is the RI reported for the CSI report corresponding to the first Reporting Setting, calculate SGCS value(s) as, , , , , where is the predicted precoder represented by predicted PMI for k-th layer, averaged over all the subbands configured by csi-ReportingBand, n-th subband, and for S-th time instance corresponding to the report of the first Reporting Setting, wkn is the precoder represented by non-predicted PMI for k-th layer, averaged over all the subbands configured by csi-ReportingBand, n-th subband, based on the channel measurement corresponding to the second Reporting Setting, and wkn is the precoder represented by PMI for k-th layer and averaged over all the subbands configured by csi-ReportingBand n-th subband, corresponds to the subbands configured by csi-ReportingBand and numberOfPMI-SubbandsPerCQI-Subband, based on the latest CSI-RS resource transmission occasion, no later than CSI reference resource, corresponding to the report of the first Reporting Setting. << Unchanged parts are omitted >> |  |

Agreement

For CSI-PAI report, UE is expected to be configured with the same subband configuration with that of inference report.

Agreement

For CSI prediction, for monitoring report, SP CSI report is only supported on PUSCH.

Agreement

The following TP2.6-1 and TP2.6-2 are endorsed as editorial corrections for TS 38.214 for Clauses 5.2.1.4.6 and 5.2.1.4.2, respectively.

TP 2.6-1 < Unchanged parts are omitted > 5.2.1.4.6 CSI-PAI reporting When the UE is configured with a first CSI Reporting Setting for reporting CSI prediction, based on a CSI-ReportConfig configured with the higher layer parameter [RRC_name-r19] and a second CSI Reporting Setting for reporting CSI-PAI, based on a CSI-ReportConfig with reportQuantity-r19 set to 'csi-pai-r19', and the second Reporting Setting is linked to the first Reporting Setting by inferenceReportConfigId-r19, the reporting of CSI-PAI corresponding to the second CSI Reporting Setting shall consider the following: - when aperiodic Reporting Setting is configured for both first and second Reporting Settings: - if aperiodic CSI-RS resources are configured for channel measurement for the first Reporting Setting, the UE is expected to receive the PDCCH triggering a CSI report of the second Reporting Setting, if any, no later than the first symbol of the earliest occasion of, Ka??{4,8,12} , aperiodic CSI-RS resources in the resource set for channel measurement for the first Reporting Setting. - if a single periodic or semi-persistent CSI-RS resource is configured for channel measurement for the first Reporting Setting, the UE is expected to receive the PDCCH triggering a CSI report of second Reporting Setting, if any, no later than the first symbol of the earliest transmission occasion of the most recent, Kpa??{1,2,4}, periodic or semi-persistent CSI-RS resource, no later than the CSI reference resource of the CSI report, corresponding to the first Reporting Setting. - to report CSI-PAI for the second Reporting Setting, the UE shall: - determine predicted PMI (see Clause 5.2.2.2.10 or Clause 5.2.2.2.11) for each of the subband(s) configured by csi-ReportingBand for the S-th time instance configured by timeinstanceforCSImonitoring-r19 in the second Reporting Setting ( , with N4>1 provided in the first Reporting Setting), based on the CSI prediction performed by the UE using the channel measurement corresponding to the first Reporting Setting, - determine non-predicted PMI for each of the subband(s) configured by csi-ReportingBand, which is corresponding to the S-th time instance of the report of the first Reporting Setting, based on the CSI-RS resource(s) in the resource set for channel measurement for the report corresponding to the second Reporting Setting. - determine non-predicted PMI for each of the subband(s) configured by csi-ReportingBand, based on the latest CSI-RS resource transmission occasion, no later than the CSI reference resource of the CSI report, corresponding to the first Reporting Setting < Unchanged parts are omitted > --------------------------------------- End of Text Proposal ------------------------------------

TP 2.6-2 < Unchanged parts are omitted > 5.2.1.4.2 Report quantity configurations If the UE is configured with a CSI-ReportConfig with reportQuantity-r19 set to 'csi-pai-r19', - the UE shall be configured with inferenceReportConfigId-r19 to link another CSI-ReportConfig configured with a higher layer parameter [RRC_name-r19], - when semi-persistent Reporting Setting is configured for the CSI-ReportConfig configured with the higher layer parameter [RRC_name-r19], the UE is not expected to be configured with a periodic Reporting Setting for the CSI-ReportConfig with reportQuantity-r19 set to 'csi-pai-r19'. - when aperiodic Reporting Setting is configured for the CSI-ReportConfig configured with the higher layer parameter [RRC_name-r19], the UE is not expected to be configured with a periodic or semi-persistent Reporting Setting for the CSI-ReportConfig with reportQuantity-r19 set to 'csi-pai-r19'. < Unchanged parts are omitted > --------------------------------------- End of Text Proposal ------------------------------------

Agreement

For CSI-PAI report, when the UE is configured with two linked CSI report configurations, adopt UCI padding to fixed size

- based on the maximum rank determined from configuration of RI restriction (for simplicity denoted by max_rank here), the size of the CSI-PAI report is 2×max_rank×4 irrespective of the reported rank ‘v’ in the inference report.

- When the linked inference report indicates rank ‘v’, all ones is padded to LSB of the UCI sequence for each of max_rank-v fields of SGCS#1 or SGCS#2

Agreement

For PMI assumption on SGCS calculation of wk and wk ,support Rel-16 eType II CSI.

Agreement

For CSI-PAI report, support

- OCPU =X, X is reported by UE capability

- For AP monitoring report, CPU occupancy starts from the first symbol after the PDCCH triggering the CSI report until the last symbol of the scheduled PUSCH carrying the CSI-PAI report

- For SP monitoring report, CPU occupancy starts from the first symbol of the latest periodic/semi-persistent CSI-RS occasion of the inference report not later than CSI reference resource of the inference report, until the last symbol of the configured PUSCH carrying the CSI-PAI report

Agreement

For CSI prediction using UE-side model, for UE assisted performance monitoring,

- If the inference report is semi-persistent and the monitoring report is aperiodic or semi-persistent, UE is expected to receive the DCI triggering the monitoring report if any, no later than the first symbol of the earliest occasion of the most recent periodic or semi-persistent inference measurement resource transmission occasions that are no later than the CSI reference resource of the associated inference report.

#### 2.1.2	Remaining Open issues

None

## 2.2	RAN2

#### 2.2.1	Agreements

##### 2.2.1.1	RAN2#131

RAN2 endorsed the running CRs reflecting the agreements up to RAN2#130, i.e. the CRs to TS 37.320 (R2-2506078), 37.355 (R2-2505704), 38.300 (R2-2505191), 38.305 (R2-2505212), 38.321 (R2-2505501), 38.331 (R2-2506401)

RAN2#131 discussed LCM for UE-sided model for Beam Management use case and focused on the remaining issues mainly from the RRC specification point of view. The following agreements were reached in RAN2.

- Discussion on UE data collection procedures:

  - Multiple preferred configurations within the list of candidate configurations provided by NW can be indicated by the UE via UAI.

  - No prohibit timer is needed for UE indicating its preferred data collection configuration

  - On stop/start indication

    - The UE can send start indication (without a preferred list) to indicate preference to start data collection

    - The UE can send preferred list implying that it would like to start data collection on those configuration

    - The UE can send stop indication for all or a given actual CSI report config ID.

    - Rapporteur will determine best way of signaling.  This doesn’t preclude merging 1 and 2

  - Adopt below text in the field description of dataCollectionCandidateConfig:

    - The UE is not expected to perform measurements solely based on the configurations provided by this IE

  - Adopt the following solution:  OtherConfig contains a list of candidate configurations as a list of a new IE, where each candidate configuration contains at least an identifier of the candidate configuration, CSI-ResourceConfigId for Set A, CSI-ResourceConfigId for Set B, and related associated IDs, as agreed in RAN2#130. Each candidate configuration is associated with a cell ID.   We will also include individual IEs for CSI prediction case

  - Ask RAN1 what IEs are needed for CSI prediction and inform them of our agreements on BM and confirm if anything else is needed.

  - The procedures for UE data collection request for the CSI prediction use case are the same as for the beam management use case.

  - An LS was sent to RAN1 to notify them about the above agreements in R2-2506470.

- Discussion on UE-side inference configuration, applicability reports and associated IDs:

  - Applicability reports

    - The applicability reporting procedures for CSI prediction are the same as for beam management. RAN2 confirms that option B is not supported for CSI prediction, given no parameters were provided by RAN1.

    - No enhancements are pursued for reporting applicability in RRCReestablishmentComplete

    - The UE can report applicability via RRCResumeComplete for SCG inference configurations received in RRCResume, without specification impact beyond already agreed applicability reporting procedure.

    - Applicability reporting is added in RRCResumeComplete for inference configurations that exist at the UE based on legacy procedures (restored or received in RRCResume).

    - RRCReconfigurationComplete shall include applicability/inapplicability status for:

      - All inference configurations included in the immediately preceding RRCReconfiguration message, and

      - Any previously configured inference configurations for which applicability/inapplicability has already been reported and whose applicability status has changed since the last report.

    - Do not introduce a link, explicit or implicit, between a full inference configuration and a set of inference-related parameters. If the applicability of a full inference configuration changes and there is a corresponding set of inference-related parameters whose applicability changes at the same time, the UE shall report the applicability of both

  - Inference configurations

    - RAN2 understand that when network releases inference configurations of poor-performed applicable functionalities, network may also provide either non-AI/ML configuration in CSI-ReportConfig or may provide full inference configuration of other applicable functionalities, if previously not configured to UE. There is no spec impact and feedback from the NW to the UE to adjust the applicability determination procedure is not supported in Rel-19.

    - RAN2 assumes for no NR DC enhancements are considered and will not ask RAN3 work in Rel-19.

    - RAN2 confirms that UE receives RRCReconfiguration message including one set or multiple sets of inference related parameters via OtherConfig for option B.

    - For Option B for BM Case 1/2, one set or multiple sets of inference related parameters can be configured in OtherConfig, where each set in OtherConfig contains the following parameters according to RAN1#121 agreement:

      - associatedIDforSetA-r19, resourcesForSetA-r19, resourcesForChannelMeasurement, associatedIDforSetB-r19, reportQuantity-r19, reportConfigType, nrofreportedpredictedrs-r19, TimeGap-r19, nroftimeinstance-r19, applicabilityConfigId-r19.

    - For Option B for BM Case 1/2, inference related parameter set is configured per serving cell.

  - Discussion on the associated IDs

    - Both single cell and multi-cell associated ID can be supported based on NW implementation (i.e., the network may allocate an Associated ID to a single cell and/or to multiple cells).

    - Associated IDs shall be unique within a PLMN in that they can only be associated with one same/similar beam deployment.   FFS is we should have signalling indicating multi-cell.

    - We will not define areas.  The Associated ID is 24 bits.

    - If the network does not provide the associated ID, it is up to UE implementation how to determine the applicability.

Further, RAN2 discussed how to reply to the RAN4 LS on AI/ML functionality activation (R2-2505045), and made the following agreements:

- On the time duration for an AI functionality to become available for inference, RAN2 conclude that it is up to UE implementation from RAN2 point of view and no further RAN2 work.

- RRCReconfigurationComplete containing applicability reports has a processing latency requirement of 16 ms with respect to the reception of RRCReconfiguration, from RAN2 point of view.

- RRC processing delay shouldn’t be impacted by the model loading delay

- If the UE is ready for inference by end of RRC processing delay, it reports model applicable.  If not, it reports model inapplicable and doesn’t set the release flag.   The network is not expected to release inference configuration (this will not be added to stage 3 specification).

- Once the model is applicable, UE reports applicability to network via UAI (applicable to all CSI reporting).

- LS to RAN1 - RAN2 has identified a problem.  From RAN2 point of view this can be solved by option 2, but needs to check with RAN1.   RAN2 also discussed option 1 and couldn’t conclude as it is outside scope of RAN2.  Would like too ask RAN1 which one is best.

- Current RAN2 specifications will not be updated to cover this problem for now.

- In RAN4 LS, RAN2 will not mention interpretation but just provide agreement 1 – 6.

RAN2#131 discussed NW-side data collection for the beam management use case, and made the following agreements:

- RAN2 confirms that the network data logging is captured in a new clause (e.g. 5.5x) in the RRC specification.

- A hysteresis should be configured and used (alongside threshold and timeToTrigger) for event-triggered logging for NW-side data collection.

- The resource configuration does not have separate resources for Set A and Set B.

- RAN2 to send an LS to RAN1 to inform about the RAN2 agreements on solution for network data logging, including L1 related content for NW-side data collection.

- RAN2 to send an LS to RAN3 to inform about the RAN2 agreements on solution for network data logging

- Keep event-triggered logging

- Logging configuration is introduced as a new list of configurations under CSI-MeasConfig, based on TP1 in R2-2505860,

- Event evaluation for the event-triggered logging will be capturing within the existing A1/A2 events (in sub-clauses 5.5.4.2 and 5.5.4.3)

- For L1-related content for NW-side data collection, it is sufficient to collect the L1-RSRP and/or beam ID as agreed by RAN2.

- Multiplexing of legacy SON/MDT report and AIML logged data is not supported in the same UE information response message.  Up to the network to ensure that data is not requested at the same time

- The logging periodicity of a NW-side data collection configuration is configurable.

- No further indication/condition is specified (beyond already agreed ones) for the UE to inform source gNB about data availability before HO in Rel-19.

- The UE stores logged data for BM in a variable specific to L1 CSI related measurements.

- Only periodic CSI resources are used for NW sided data collection.  No need for new dynamic MAC CE mechanisms.

- If LoggedDataCollectionAssistanceConfig is configured, then full buffer and low power indications are configured by default (i.e., no additional fields/bits required to configure them). Data threshold is (optionally) configured by including the threshold in the loggedDataCollectionAssistanceConfig.

- Both the data collection configuration and the UAI configuration related to data collection are released when the UE transitions to IDLE/INACTIVE or initiates re-establishment (including RLF).

- If the buffer is not full or the data threshold is configured and the amount of data is below the threshold, UE does not send data availability indication when it sends low power indication.

- The possible values for the buffer threshold are 16KB, 32KB and 48KB.  FFS during CR phase in any higher value is needed depending on UE capability discussion

- No additional handling of logged data to be specified (apart from the already agreed release during state transition and RLF, and release upon successful delivery).

- No further discussion is needed on RRC issue 34 in re-19 as we have only one use case.

- The UE will indicate the presence of a gap (i.e. there will be no indication on the length of gap or time instance, etc).  Rapporteur will suggest a way to implement that as part of the RRC review.

- To define field names and IE based on the content of the logged data rather than the specific use case

- Data forwarding to OAM or source gNB after HO is not in RAN2 scope and understands that other groups don’t have time to work on it.

- UE discards the logged data upon inter-RAT handover.

- RAN2 confirm that the solution agreed in RAN2#130 is applicable to regular HO and CHO (i.e. 1-bit indication corresponding to each candidate cell configuration in RRCReconfiguration is provided).

- Do not introduce an indication from the UE to NW about unsuitable data collection configurations in Rel-19.

RAN2 also discussed how to reply to the LS from SA5 on OAM-centric solution for NW-side data collection (R2-2505043), and made the following agreements to be included in the LS reply:

- For network-side data collection for beam prediction, measurement reports include the following:

  - Cell identity: CGI or PCI of the cell to which the measurement results are related.

  - Logged L1 radio measurement results including the beam identifiers associated to CSI-RS resources or SSBs (CSI-RS IDs or SSB IDs) and the corresponding measured L1-RSRPs.

  - The UE will indicate the presence of a gap (i.e. there will be no indication on the length of gap or time instance, etc).

- The required L1 measurements (i.e. L1-RSRP of CSI-RS(s) or SSBs) are defined in TS 38.215 which is referenced in the definition of M1 measurement in TS 37.320.

- RAN2 needs to ask SA5 how to transfer the measurement report with the above content and whether the M1 measurement defined in clause 5.4.1.1 of TS 37.320 can be used to transfer the measurement reports with the above content or not.

- The gNB will configure the UE to log the above content via the NW-side data collection configuration, which is different from legacy immediate MDT configuration.

Further, the topic of user consent for NW-side data collection was brought up. RAN2 agreed to send an LS to SA3 indicating our work on NW-sided gNB and OAM centric data collection and the content of collected data.  The LS will indicate that RAN2 discussed the need for user consent and that SA3 should take this into account and decide on the need for user consent for NW-sided data collection.

RAN2#131 discussed LCM for Positioning use case and made the following agreements:

- Do not introduce a request for additional PRUs (e.g., a number of PRUs) in the Request Assistance Data message

- "Batch reporting", i.e., reporting of up to 32 location results in a single report as supported for the current NR positioning methods, is also applicable to "NR AI/ML Positioning Case 1".

- Keep NR-DL-AIML-RequestLocationInformation, excluding UE-assisted measurement parameters, and retain only UE-based and common parameters (e.g., nr-AssistanceAvailability).

- For AI/ML positioning Case 1, the LocationInformationType field in CommonIEsRequestLocationInformation shall be set to locationEstimateRequired. Other values, including locationMeasurementsRequired, locationEstimatePreferred, locationMeasurementsPreferred, and locationEstimateAndMeasurementsRequired, are not applicable and shall not be used.   No specification impact.

- We do not introduce new error cause for the target device error causes.

- Case 3a and Case 3b can be supported without new impact to LPP

- Introduce list of global cell information (i.e., NCGIs, or PCIs with ARFCN) and TRP ID, as the request associated information to ensure consistency between training and inference.

- The UE asks specific TRPs for PRS transmission with on-demand PRS configuration, i.e., within NR-On-Demand-DL-PRS-Request

- Similar to BM, UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device.  If nw side additional conditions are not provided then we follow BM conclusion.   No stage 3 impacts.

- Similar to AI PHY, when applicability changes the UE should report this to the LMF and only what changed.   For now capture this at least in stage 2.  Check offline if and how this would be implemented in stage 3.

- Wait for RAN1 for LPP-21.  Take what RAN1 gives us and we implemented.   Can compile an LS for next meeting if we have questions.

RAN2#131 discussed Rel.19 UE capabilities and made the following agreements:

- If UE supports NW side data collection, it is mandatory for UE to support the minimum AS layer memory size of 64kB for UE supporting AI/ML based beam management, which is shared across all use cases with NW-sided model.   The assumption is that this will be shared across all use cases (i.e. Rel-20 as well)

- FFS on whether UE can support other memory sizes and indicate to network via optional capability signaling.

- Include RAN2 feature ‘UE can provide update of applicability reporting via UAI’ as part of RAN1 FGs (e.g., 58-0-1 and/or FG 58-1-2/3/4/5, the details of those feature group depend on RAN1 progress) once implemented.

- Introduce two conditional mandatory capabilities (with signaling) for AI/ML based BM Option A and Option B, if UE supports FG58-0-1 and/or FG58-1-2/3/4/5 (the details of those feature group depend on RAN1 progress).

- Include RAN2 feature ‘providing UE preferred configuration for UE-side data collection’ as part of RAN1 FG58-1-7/FG58-3-4 (once implemented).

- UAI is mandatory for both Option A and B

- Introduce an optional per UE capability ‘loggedDataCollection-r19’ to indicate supporting logged measurements of data collection for NW-side model, which includes the following components:

  - the minimum 64kB AS layer memory size

  - periodic logging

  - Provide full buffer indication, low power indication

- Event-based logging is an optional per UE capability separate from ‘loggedDataCollection-r19’. UE supporting this feature shall also indicate the support of ‘loggedDataCollection-r19’.  If UE supports event-based logging it shall support data threshold-based data availability indication.

- RAN2 will not introduce separate CSI resource capability for logged NW-side data collection. Legacy capability will be used for logged NW-side data collection. Check with RAN1 on whether this assumption is ok.

- Data threshold-based data availability indication is an optional per UE capability with signaling. A UE supporting this feature shall also indicate support of the basic logged NW-side data collection.

RAN2 finally agreed that the AI/ML for PHY WI is considered complete from RAN2 point of view.

The following post-meeting email discussions were agreed to finalize the CRs for the impacted RAN2 specifications, and to finalize the content of the LSs to be sent:

[POST131][003][AI PHY] Functionality activation (Apple)

Intended outcome: LS to RAN4  and LS to RAN1

Deadline:  Sept. 5th, 10:00 UTC

[POST131][023][AI PHY] 37.320 (Huawei)

Intended outcome: Agree to final CR

Deadline: Sept. 5th, 10:00 UTC

[POST131][024][AI PHY] 37.355 (Qualcomm)

Intended outcome: Agree to final CR and then trigger LS to RAN1 on questions related to LPP21

Deadline:  Sept. 5th, 10:00 UTC for CR and 2 weeks after for LS

[POST131][025][AI PHY] 38.300 (Vivo)

Intended outcome: Agree to final CR

Deadline:  Sept. 5th, 10:00 UTC

[POST131][026][AI PHY] 38.305 (CATT)

Intended outcome: Agree to final CR

Deadline: Sept. 5th, 10:00 UTC

[POST131][027][AI PHY] 38.331 (Ericsson)

Intended outcome: Agree to final CR

Deadline: Sept. 5th, 10:00 UTC

[POST131][028][AI PHY] 38.321 (Apple)

Intended outcome: Agree to final CR

Deadline: Sept. 5th, 10:00 UTC

[POST131][036][AI PHY] Reply to SA5/RAN3 (Huawei)

Intended outcome: agree to LS

Deadline: Sept. 5th, 10:00 UTC

[POST131][037][AI PHY] User Consent LS to SA3 (NTT Docomo)

Intended outcome: agree to LS and cc RAN3 ccSA5

Deadline: Sept. 5th, 10:00 UTC

[POST131][038][AI PHY] LS to RAN1/RAN3 on nw-sided data collection (ZTE)

Intended outcome: agree to RAN1 and RAN3 LS capturing relevant RAN2 agreements

Deadline: Sept. 5th, 10:00 UTC

[POST131][043][AI PHY] UE capabilities (Xiaomi)

Intended outcome: align on remaining open issues and agreable proposals

Deadline:  Sept. 4th

#### 2.2.2	Remaining Open issues

RAN2 agreed that the WI is completed. 
The remaining FFSs and the corrections to the endorsed running CRs will be addressed during the next RAN2 meetings.

## 2.3	RAN3

#### 2.3.1	Agreements

##### 2.3.1.2	RAN3#129

During RAN3#129 meeting, RAN3 discussed the topic of AI/ML for Air Interface supporting Positioning Accuracy Enhancements and treated the received LSes R3-255009 (RAN1), R3-255016 (RAN2), R3-255023 (SA2) and R3-255029 (SA5).

A work plan was presented in R3-255569.

The following BL CRs were endorsed:

- R3-255095 (BL CR to 38.305) Introduction of AI/ML air (CATT, Ericsson, Nokia, Huawei, Xiaomi, ZTE, CMCC, Samsung, CEWiT, Jio Platforms)

- R3-255096 (BL CR to 38.401) Support of AI/ML assisted positioning (ZTE Corporation, Xiaomi, CATT, Ericsson, Huawei, NEC, Nokia, Jio Platforms)

- R3-255097 (BL CR to 38.413) Support of Case 3a (Huawei, ZTE, Lenovo, Nokia, Ericsson, Jio Platforms)

- R3-255098 (BL CR to 38.455) Introduction of AI/ML air (Ericsson, Nokia)

- R3-255099 (BL CR to 38.473) Support of Sample-based measurement (case 3b) (Xiaomi, Ericsson, ZTE, CATT, Samsung, Nokia, Jio Platforms)

- R3-255100 (BL CR to 38.300) Stage 2 for case 3a (Nokia, CATT, Xiaomi, Ericsson, CMCC, Huawei, ZTE, Lenovo, Jio Platforms)

Based on the discussion, RAN3 made the following agreements:

- Case 3a NW-side model:

- The "LoS/NLoS indicator" can also be a gNB inferred measurement. - In a POSITIONING DATA COLLECTION REPORT message, a LMF can provide the LoS/NLoS label information independently of any timing information label. The current semantics description in IE Positioning Data Information is removed. - It should be possible for an LMF to specifically request for AI/ML inferred measurements from TRPs. - Add the following error causes to the Positioning Data Unavailable IE: ‘Not Supported’, ‘Not Available’

The following TPs capturing the offline discussion were agreed:

- R3-255823 (TP to NRPPa BL CR): addition of UL SRS time domain channel measurement (Ericsson, et al.)

- R3-255813 (TP to BL CR TS 38.473) addition of UL SRS time domain channel measurement (Xiaomi, et al.)

- R3-255934 (TP to BL CR for TS38.305) Support of AI Positioning (CATT, et al.)

- R3-255873 (TP to NRPPa BL CR): Support of Case 3a remaining issue (Huawei et al.)

- R3-255456 (TP to TS 38.300) AI/ML Training in OAM for case 3a (Nokia, et al.)

- R3-255503 (TP to BL CR to 38.401) AI/ML assisted Positioning (ZTE Corporation, et al.)

The following LS were approved:

- R3-255935  Reply LS on AI/ML Positioning Case 3a

- R3-255824  Reply LS on Logged Data Handling During Handover

#### 2.3.2	Remaining Open issues

The WI is considered complete from RAN3’s perspective.

## 2.4	RAN4

#### 2.4.1	Agreements

##### 2.4.1.1	RAN4#114bis

CSI reporting requirement framework for CSI prediction

Issue 1-1: Performance monitoring

Agreement:

RAN4 will introduce requirements for Type 3 performance monitoring for CSI prediction

FSS which requirements are to be introduced.

Issue 1-2: Requirement baseline for monitoring

Agreement:

Introduce the following requirements for CSI performance monitoring:

- Reporting delay

  - FFS how the delay is defined

- Reporting accuracy

  - FFS on whether accuracy requirement can be defined/checked

- Mapping table – To be checked whether RAN1 captures or RAN4

- If it is not feasible to define reporting accuracy requirement or test it, RAN4 will send an LS to RAN1 to inform RAN1 about this

Issue 1-4: Reporting delay requirement

Agreement:

wait for RAN1 decision

Issue 1-5: Scheduling delay

Agreement:

- The delay from when the UE prediction is sent until the time the TE applies is n+4 (slots) for FDD

Testability and interoperability issues for beam management

Issue 2-1: Measurement period for inference

Agreement:

observation period for prediction: (observation period is the amount of time during which UE samples the reference signals to make 1 prediction report)

- For case 1 reuse M,N,P from legacy measurement requirements

- For case 2, use T*M, N P  (M,N,P same as legacy measurement requirement)

  - FFS on T value, T can also be 1. T can also be based on capability

- This observation period is to be used for the prediction delay requirement

Issue 2-2: Prediction report delay

Agreement:

- For aperiodic report, the overall prediction reporting delay at least includes measurement delay +reporting delay

  - The reporting delay includes the inference delay

- For semi-persistent and periodic report, the overall prediction reporting delay at least includes measurement delay +inference delay+time for the first available reporting occasion

Issue 2-3: TCI State Handling

Agreement:

- On Detectability and SNR conditions:

  - The UE has sent at least 1 L1-RSRP inference report for the target TCI state before the TCI state switch command

  - The TCI state remains detectable during the TCI state switching period

  - The SSB associated with the TCI state remain detectable during the TCI switching period

    - SNR of the TCI state ≥ -3 dB

- Time conditions:

  - TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam predict reporting or measurement

- Agreement on QCL relationship

  - If the predicted Tx beam in Set A is QCL Type-D to a known measured Tx beam, where TX beam can be both inside or outside set B, the corresponding Rx beam is known.

  - If the predicted Tx beam in Set A is not QCL Type-D to a known Tx beam, known TCI state conditions shall be based on UE capability.

Issue 2-4: Activation delay

Agreement:

- UE has to be ready to start measurements for inference after sending RRC reconfiguration complete

- Inference delay (might include also model loading depending on UE implementation) will be included in the inference reporting delay

- This applies to all reporting schemes

Issue 2-6: Test system setup

Agreement:

Consider multi AoA based RRM testing setup as baseline(enhanced IFF system with 4 probes used for FR2 RRM conformance testing)

- Single AoA tests can also be performed in the multi AoA based RRM test setup

Further analyze what spatial channel models can be emulated with this test setup

- Companies are invited to bring analysis on what simplified spatial channel (multiple clusters coming from multiple directions) model can be emulated in this test setup

  - Single AoA tests are not precluded

Issue 2-7: Test system channel model

Agreement:

Companies to bring analysis into what simplifications to CDL channels are needed to be able to emulate the channels in the enhanced IFF chamber.

- Take CDL-C Umi as one example

- Other channel models can be further discussed

Issue 2-9: Metrics/KPIs for beam ID prediction

Agreement:

- If RSRP prediction is reported, absolute/relative RSRP requirements applies to

  - reported beams where SNR side condition is met

  - The exact RSRP reporting including both absolute and relative will be defined in RAN1

  - The related test can only apply to the top-K, where the value of K is FFS, beams.

    - The legacy beam management test and parameters can be taken as the reference to decide the exact value of K

    - The feasibility of the test will be further discussed and decided.

Requirements for positioning

Issue 3-1: Reporting Delay Requirements for case 1

Agreement:

Take the framework of the existing reporting delay requirements

- Reporting delay includes measurement delay, inference delay and the time needed until the UE send the report

- Check until next meeting on the number of samples needed for the measurements

Issue 3-3: Report mapping for UL SRS-TDCP measurement

Agreement:

Adopt the mapping for UL SRS-TDCP as proposed in R4-2510759

Issue 3-4: Report mapping for UL SRS-TDCT and UL SRS-TDCP measurements

Agreement:

Technically endorse CR in R4-2510760, to be later merge with the rest of positioning requirements

#### 2.4.2	Remaining Open issues

- CSI reporting requirement and testing framework for CSI prediction

  - Requirements baseline

  - PMI report and scheduled PDSCH separation

  - SNR

  - Simulation results and next steps

  - Generalization

  - LCM

- RRM core requirement and testing framework for beam management Beam prediction KPIs

  - Beam ID prediction KPIs

  - Measurement period for inference

  - Rx beam knowledge and TCI state handling

  - Requirements based on relative difference

  - Absolute RSRP accuracy applicability

  - Simulation results

  - Measurement error modelling

  - LCM

  - Test system channel model

  - Test system setup

- RRM core requirement and testing framework for Positioning accuracy enhancement

  - Requirements for case 1

  - Report mapping for sample based timing measurement reporting

  - Case 3b requirements

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

## 4.1	RAN1

## 4.2	RAN2

### 4.2.1	RAN2#131

100 contributions (for details see agenda items 8.1.1, 8.1.2, 8.1.3 in Tdoc list)

## 4.3	RAN3

### 4.3.1	RAN3#129

56 contributions have been submitted (for details see agenda items 20 in Tdoc list)

## 4.4	RAN4

#### 4.4.1	RAN4#116

// General aspects

R4-2509244	On AI/ML Model LCM and Performance Monitoring Requirement

Type: discussion		For: Approval
					Source: NTU

Decision:		Noted.

R4-2509423	On general issues on AI/ML

Type: discussion		For: Discussion
					Source: Apple

Decision:		Noted.

R4-2509760	Discussion on general aspect for AI

Type: discussion		For: Discussion
					Source: Xiaomi

Decision:		Noted.

R4-2509920	On General Aspects of AI/ML for NR Air Interface Phase 1

Type: discussion		For: Discussion
					Source: Nokia

Decision:		Noted.

R4-2510104	Discussion on General aspects

Type: discussion		For: Discussion
					Source: MediaTek Inc.

Decision:		Noted.

R4-2510163	Discussion on general aspects for AI/ML for NR air interface

Type: discussion		For: Discussion
					Source: CMCC

Decision:		Noted.

R4-2510335	Discussion on general aspects for AI/ML for NR air interface

Type: discussion		For: Discussion
					Source: vivo

Decision:		Noted.

R4-2510804	Discussion on general aspects on AI/ML for NR air interface

Type: discussion		For: Approval
					Source: OPPO

Decision:		Noted.

R4-2510871	Discussion on general aspects on AIML for NR air interface

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision:		Noted.

R4-2511150	On general issues for AIML

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

On general issues for AIML

Decision:		Noted.

R4-2511569	Generalization aspects of AI-ML

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision:		Noted.

// CSI reporting requirement framework for CSI prediction

R4-2509133	Feasibility of reduction in PMI report and scheduled PDSCH separation for AI/ML CSI prediction

Type: discussion		For: Discussion
					Source: Anritsu Corporation

Decision:		Noted.

R4-2509411	View on CSI reporting requirement and testing framework for CSI prediction

Type: discussion		For: Discussion
					Source: Samsung

Decision:		Noted.

R4-2509424	View on CSI reporting requirements framework for CSI prediction

Type: discussion		For: Discussion
					Source: Apple

Decision:		Noted.

R4-2510161	Discussion on CSI reporting requirement and testing framework for CSI prediction

Type: discussion		For: Discussion
					Source: CMCC

Decision:		Noted.

R4-2510336	Discussion on CSI reporting requirement and testing framework for CSI prediction

Type: discussion		For: Discussion
					Source: vivo

Decision:		Noted.

R4-2510671	Discussion on PMI reporting delay

Type: other		For: Approval
					Source: ROHDE & SCHWARZ

Decision:		Noted.

R4-2510805	CSI reporting requirement and testing framework for CSI prediction

Type: discussion		For: Approval
					Source: OPPO

Decision:		Noted.

R4-2510835	Discussion on CSI prediction

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discusses the AI/ML-based CSI prediction.

Decision:		Noted.

R4-2510872	Discussion on CSI reporting requirement and testing framework for CSI prediction

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision:		Noted.

R4-2511221	CSI reporting requirement and testing framework for CSI prediction

Type: discussion		For: Discussion
					Source: Nokia

Decision:		Noted.

R4-2511568	CSI Prediction: Testing framework

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision:		Noted.

// RRM core requirement for beam management

R4-2509298	Discussion on RRM core requirements and testing framework for BM

Type: discussion		For: Discussion
					Source: CATT

Decision:		Noted.

R4-2509425	RRM core requirements and testing framework Based Beam Management

Type: discussion		For: Discussion
					Source: Apple

Decision:		Noted.

R4-2509761	Discussion on core part requirement and testing framework for AI beam management

Type: discussion		For: Discussion
					Source: Xiaomi

Decision:		Noted.

R4-2509921	RRM core requirements for AI/ML Based Beam Management

Type: discussion		For: Discussion
					Source: Nokia

Decision:		Noted.

R4-2510105	Discussion on RRM core requirement and testing framework for beam management

Type: discussion		For: Discussion
					Source: MediaTek Inc.

Decision:		Noted.

R4-2510159	Discussion on RRM core requirement and testing framework for beam management

Type: discussion		For: Discussion
					Source: CMCC

Decision:		Noted.

R4-2510234	Discussion on AI/ML beam management testability

Type: other		For: Approval
					Source: Qualcomm Incorporated

Decision:		Noted.

R4-2510334	Draft CR on RRM core requirements for support of AIML for NR Air Interface for beam management

Type: draftCR		For: Endorsement
					38.133 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: vivo

Decision:		Noted.

R4-2510337	Discussion on RRM core requirement and testing framework for beam management

Type: discussion		For: Discussion
					Source: vivo

Decision:		Noted.

R4-2510374	Discussion on RRM core requirements and test framework for beam management

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Discussion on RRM core requirements and test framework for beam management

Decision:		Noted.

R4-2510806	RRM core requirement and testing framework for beam management

Type: discussion		For: Approval
					Source: OPPO

Decision:		Noted.

R4-2510873	Discussion on RRM core requirement for AIML Beam management

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision:		Noted.

R4-2511018	Discussion on the RRM requirements of AI/ML Beam management

Type: other		For: Approval
					Source: ZTECorporation,Sanechips

Decision:		Noted.

R4-2511249	Discussion on RRM core requirement and testing framework for beam management

Type: other		For: Approval
					Source: Samsung

Decision:		Noted.

R4-2511570	RRM Core Requirements for AI-ML based Beam Management

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision:		Noted.

R4-2511597	Discussion on test setup for AI/ML based beam management

Type: discussion		For: Discussion
					Source: Rohde & Schwarz

Decision:		Noted.

// RRM core requirement for Positioning accuracy enhancement

R4-2509299	Discussion on RRM core requirements and testing framework for positioning

Type: discussion		For: Discussion
					Source: CATT

Decision:		Noted.

R4-2509426	RRM core requirement and testing framework for Positioning accuracy enhancement

Type: discussion		For: Discussion
					Source: Apple

Decision:		Noted.

R4-2510164	Discussion on RRM requirements for positioning accuracy enhancement

Type: discussion		For: Discussion
					Source: CMCC

Decision:		Noted.

R4-2510338	Discussion on RRM core requirement and testing framework for Positioning accuracy enhancement

Type: discussion		For: Discussion
					Source: vivo

Decision:		Noted.

R4-2510758	On core requirements for AI/ML based positioning

Type: other		For: Approval
					Source: Ericsson

Abstract:

This paper discusses issues related to the core requirements for AI/ML based positioning use case #1.

Decision:		Noted.

R4-2510807	RRM core requirement and testing framework for Positioning accuracy enhancement

Type: discussion		For: Approval
					Source: OPPO

Decision:		Noted.

R4-2510874	Discussion on RRM core requirement for AIML Positioning

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision:		Noted.

R4-2511019	Discussion on the RRM requirements aspects of AI/ML positioning

Type: other		For: Approval
					Source: ZTECorporation,Sanechips

Decision:		Noted.

R4-2511222	RRM core requirements and testing framework for AI/ML positioning accuracy enhancement

Type: discussion		For: Discussion
					Source: Nokia

Decision:		Noted.

// RRM performance requirements for beam management and positioning accuracy

R4-2509134	FR2 test setup for AI/ML beam management evaluation

Type: other		For: Approval
					Source: Anritsu Corporation

Decision:		Noted.

R4-2509242	Beam Prediction Performance Metrics for Monitoring and Accuracy Requirement

Type: discussion		For: Approval
					Source: NTU

Decision:		Noted.

R4-2509300	Discussion on RRM performance requirements for BM and positioning

Type: discussion		For: Discussion
					Source: CATT

Decision:		Noted.

R4-2509427	RRM performance requirements for beam management and positioning accuracy

Type: discussion		For: Discussion
					Source: Apple

Decision:		Noted.

R4-2509762	Discussion on performance requirement for AI beam management

Type: discussion		For: Discussion
					Source: Xiaomi

Decision:		Noted.

R4-2509932	RRM performance requirements for beam management and positioning accuracy

Type: discussion		For: Discussion
					Source: Nokia

Decision:		Noted.

R4-2510160	Discussion on RRM performance requirements for beam management

Type: discussion		For: Discussion
					Source: CMCC

Decision:		Noted.

R4-2510339	Discussion on RRM performance requirements for beam management and positioning accuracy

Type: discussion		For: Discussion
					Source: vivo

Decision:		Noted.

R4-2510759	On performance requirements for AI/ML based positioning

Type: other		For: Approval
					Source: Ericsson

Abstract:

This paper discusses issues related to performance requirements for AI/ML based positioning use cases.

Decision:		Noted.

R4-2510760	draftCR to 38.133 on report mapping for UL SRS-TDCT and UL SRS-TDCP measurements (Rel. 19)

Type: draftCR		For: Endorsement
					38.133 v19.1.0	  CR-  rev  Cat: B (Rel-19)
					Source: Ericsson

Abstract:

This draftCR introduces report mapping for new measurements introduced by RAN1 for AI/ML based positioning.

Decision:		Endorsed.

R4-2510808	RRM performance requirements for beam management

Type: discussion		For: Approval
					Source: OPPO

Decision:		Noted.

R4-2510875	Discussion on RRM performance requirements for beam management and positioning accuracy

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision:		Noted.

R4-2511574	AI-ML BM: Simulation Results

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision:		Revised to R4-2511794 (from R4-2511574).

R4-2511794	AI-ML BM: Simulation Results

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision:		Noted.

// Demodulation and/or CSI reporting requirements for CSI prediction

R4-2509412	Discussion and simulation results for AI based CSI prediction

Type: discussion		For: Discussion
					Source: Samsung

Decision:		Noted.

R4-2509428	Discussion on demodulation for CSI prediction

Type: discussion		For: Discussion
					Source: Apple

Decision:		Noted.

R4-2509655	Discussion on CSI reporting requirements for CSI prediction

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision:		Noted.

R4-2510162	Discussion on demodulation for CSI prediction

Type: discussion		For: Discussion
					Source: CMCC

Decision:		Noted.

R4-2510340	Discussion on Demodulation and CSI reporting requirements for CSI prediction

Type: discussion		For: Discussion
					Source: vivo

Decision:		Noted.

R4-2510809	Demodulation and CSI reporting requirements for CSI prediction

Type: discussion		For: Approval
					Source: OPPO

Decision:		Noted.

R4-2510836	PMI reporting requirements with CSI prediction

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discusses the AI/ML-based CSI prediction.

Decision:		Noted.

R4-2510876	Discussion on demodulation andor CSI reporting requirements for CSI prediction

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision:		Noted.

R4-2510994	On Demodulation and CSI Reporting Requirements for CSI Prediction

Type: discussion		For: Discussion
					Source: Nokia

Decision:		Noted.

R4-2511573	CSI Prediction: Simulation results

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision:		Noted.

// Moderator summary and conclusions

R4-2509098	Topic summary for [116][131] NR_AIML_air_part1

Type: other		For: Information
					Source: Moderator (Qualcomm)

Abstract:

Topic summary Main session

Decision:		Noted.

Minutes and agreements in the online session and ad hoc

Please refer to the hyperlink below for the detailed minutes of online session:

R4-2509098

R4-2511795	Summary of CSI prediction simulation results

Type: other		For: information
					Source: OPPO

Decision:		Revised to R4-2511889 (from R4-2511795).

R4-2511889	Summary of CSI prediction simulation results

Type: other		For: information
					Source: OPPO

Decision:		Noted.

R4-2511865	Summary of beam management simulation results

Type: other		For: information
					Source: vivo

Decision:		Noted.

R4-2511796	WF on updated simulation assumption for AI based CSI prediction

Type: other		For: Approval
					Source: OPPO, CAICT, vivo, Nokia, Ericsson, Qualcomm, CMCC, CTC, Huawei, Hisilicon, Mediatek, CATT, Samsung, ZTE Corporation, Apple

Decision:		Approved.

R4-2511797	WF on updated simulation assumption for AI based beam management

Type: other		For: Approval
					Source: vivo, NTU, Nokia, Ericsson, Qualcomm, Xiaomi, Huawei, Hisilicon, Mediatek, OPPO, APPLE, Rohde & Schwarz, CATT, Samsung, Intel, ZTE Corporation, Sanechips, CAICT

Decision:		Approved.

R4-2511890	WF on AI/ML for air interface [116][131]

Type: other		For: Approval
					Source: Qualcomm

Decision:		Approved.

R4-2511891	AH minutes for AI/ML for air interface [116][131]

Type: other		For: Information
					Source: Moderator (Qualcomm)

Abstract:

This contribution provides the summary of topics and recommended summary.

Decision:		Noted.

R4-2511892	AH minutes for BM simulation assumption in AI/ML for air interface [116][131]

Type: other		For: Information
					Source: vivo, Qualcomm

Abstract:

This contribution provides the summary of topics and recommended summary.

Decision:		Noted.

16.02.2024		minor adaptations for RAN #103

10.11.2023		minor adaptations for RAN #102

02.08.2023		minor adaptations for RAN #101

26.04.2023		minor adaptations for RAN #100

01.02.2023		minor adaptations for RAN #99

27.10.2022		minor adaptations for RAN #98e

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template