3GPP TSG RAN Meeting #110         	                                         RP-253213

Baltimore, US, Dec 8th – 11th, 2025

Agenda Item:	8.2.1.4

Source:	Xiaomi

Title:	Considerations on 6G NTN’s functionalities and services

Document for:	Discussion and Decision

Introduction

Until RAN#109 meeting, following services have been agreed and captured in the TR 38.914 [1].

- Mobile broadband

- Immersive Communication

- Massive Communication (IoT)

- Sensing

- AI

- Voice

- Regulatory services

For 6G NTN, services to be supported need to be discussed in RANP so that working groups can have clearer guidance for further study.

In this contribution, we discuss and provide our view on 6G NTN’s functionalities and services.

Discussion

GNSS

In 5G, UE’s GNSS capabilities are required for NTN operation. That is, UL time and frequency synchronization relies on UE’s GNSS location information, i.e. GNSS-based time and frequency pre-compensation. In the last RAN2 meeting, there was discussion on whether 6G should study NTN with or without GNSS and RAN2 think some RAN plenary guidance would be needed on this aspect. From UE vendor’s perspective, relying on GNSS functionalities on one hand increase the device’s cost, especially for low-end IoT devices. On the other hand, it would sometimes cause NTN communication malfunctioning, e.g., when GNSS coverage becomes unavailable. In 6GR NTN, we should remove this kind of GNSS requirement and taking GNSS free as baseline for common TN and NTN design. Of course, we can still have optimization for UEs that have GNSS capabilities.

Proposal 1: In 6G NTN, GNSS free operation is taken as baseline for common TN and NTN design.

Positioning, navigation and timing

In 5G, positioning for NTN is only discussed in Rel-18 to support network verified UE location and the solution is to use the multi-RTT positioning method. In the last RAN plenary meeting, there were discussions on positioning for TN, e.g. some proposals to support ECID based positioning for regulatory requirements, but there is no conclusion yet. Note that ECID-based and multi-RTT-based positionings are quite different as the latter requires PRS provisioning from LMF and transmission in RAN. In 6G, if NTN positioning is to be supported, we think that it should be considered jointly with TN to have common design. Therefore, we can wait for TN’s conclusion on positioning first.

Proposal 2: On the support of positioning, navigation and timer for 6G NTN, wait for TN’s conclusion to have common TN and NTN design.

Broadcast and multicast services

For broadcast and multicast services, although we understand that NTN’s large cell coverage is naturally suitable to provide MBS, we think that in 6G there should be no major difference from TN’s design in terms of the MBS services provisioning. Similar as positioning, we prefer to have common design of MBS, if supported, between TN and NTN, and we can wait for TN’s conclusion first.

Proposal 3: On the support of broadcast and multicast services for 6G NTN, wait for TN’s conclusion to have common TN and NTN design.

ISAC

According to RAN plenary agreements and the latest TR [1], ISAC has been supported in 6G and would be a 6G day-1 feature. We understand that the existing 6G study on ISAC is mainly focused on TN scenarios. For 6G NTN, ISAC is not that essential for day-1. After finishing TN study and knowing how ISAC works in TN scenarios, we can further study how ISAC can be supported or adapted for NTN.

Proposal 4: ISAC for NTN can be considered later (in 6G day-2) after RAN finishing ISAC study for TN.

Store and forward

In the last RAN2 meeting, there were few contributions proposing to support store and forward operation in 6G NTN. Note that this feature is being specified in IoT NTN Rel-19, and it enables NTN operation with no simultaneously available feeder link and service link due to sparse gateway deployment in the ground. Due to satellite’s storing before delivering data to the other end point, store and forward operation would increase the traffic latency and thus only suitable to those delay tolerant services for NB-IoT and eMTC. Note that for 6G IoT, RAN plenary has clearly clarified the difference from NB-IoT in the TR.

### 5.4.3 Massive Communication (IoT) The 6GR and 6G RAN architecture shall support the following minimum requirements for Massive Communication (IoT): - 6G Massive Communication (IoT) shall be supported for FR1. - 6GR should have a common/scalable design that supports the above usage scenario in addition to eMBB - Prioritize 6GR design for eMBB - The above usage scenario should not overlap with Ambient IoT and NB-IoT - [PHY or MAC] [minimum] peak data rate is [TBD] Mbps in DL and [TBD] Mbps in UL for lowest-tier device.

Although the minimum bandwidth requirement has not been agreed yet for 6G IoT devices, we assume that the lowest-tier device would be like RedCap in NR. Therefore, we don’t need think store and forward should be supported for 6G NTN.

Proposal 5: Store and forward operation is not supported for 6G NTN.

Conclusion

Based on the discussion, we have following proposals:

Proposal 1: In 6G NTN, GNSS free operation is taken as baseline for common TN and NTN design.

Proposal 2: On the support of positioning, navigation and timer for 6G NTN, wait for TN’s conclusion to have common TN and NTN design.

Proposal 3: On the support of broadcast and multicast services for 6G NTN, wait for TN’s conclusion to have common TN and NTN design.

Proposal 4: ISAC for NTN can be considered later (in 6G day-2) after RAN finishing ISAC study for TN.

Proposal 5: Store and forward operation is not supported for 6G NTN.

Reference

[1] RP‑252882, 3GPP TR 38.914 V0.2.0 (2025-09)

[2] RP-252943	Moderator's summary for RAN led 6G SI: Requirements of new and existing services, CMCC