3GPP TSG RAN Meeting #110									RP-253247

Baltimore, USA, December 8-11, 2025

Agenda Item:	8.2.1.0

Source:	Xiaomi

Title:	Deployment scenarios for ISAC

Document for:	Discussion and Decision

- Introduction

After the RAN #109 meeting, the TR 38.914 (Study on 6G Scenarios and Requirements) has been updated on the deployment scenarios [1]. There is a general sentence on the sensing aspects in the beginning of section 4 in TR 38.914.

Editor note: Sensing related aspects will be included in section 4

In this contribution, we provide our views on 6G deployment scenarios for the Integrated Sensing And Communication (ISAC) study and we suggest how ISAC can be captured in TR 38.914.

- Discussions

ISAC is a key usage scenario for 6G. It extends the communication oriented 3GPP system with new sensing capabilities for broader business opportunities. Various use cases are identified in SA1 TR 22.837 and TR 22.870. Considering the commercial request for the time being and for near future, as well as existing work on ISAC in RAN, it was agreed in RAN#109 to focus on object detection/tracking use cases and to study all 6 sensing modes, i.e., including bistatic and monostatic sensing. In our view, also cooperative sensing and/or multi-static sensing can be regarded as being inside this scope.

Even when focusing on object detection and tracking, the list of use cases is still very large. In TR 22.837, for example, seventeen different applications are listed, which are too many for evaluation. Multiple applications may share same/similar performance requirements. Certain representative deployment scenarios can be defined by having the most urgent commercial needs in mind, and by considering what RAN already has developed during Rel-19, i.e., the ISAC channel model for object detection and tracking.

In the current TR 38.914, twelve deployment scenarios are listed. Considering all of them for all sensing modes and target types, would give too many combinations for evaluation. Therefore, the following representative deployment scenarios are proposed.

- UAV detection and tracking should be evaluated in urban macro (UMa-AV in NR). This is also the scenario for which the ISAC Rel-19 channel model has been calibrated and it is also the baseline scenario for performance evaluation for ISAC in Rel-20 5GA.

- Human intruder detection or health care are also important for sensing. Indoor hotspot is preferred for their evaluation since there are more applications for indoor users than for outdoor.

- For automotive use cases, we prefer urban grid over highway, since more advanced automotive scenarios can be modelled, e.g., the crossroad or the blocking/shadowing of building, etc.

- A fourth target type is AGV, which motivates to evaluate the Indoor Factory deployment scenario.

Proposal 1: The following representative deployment scenarios can be considered for the evaluation of object detection/tracking:

- Urban macro (UMa-AV in NR) for UAV detection and tracking

- Indoor hotspot for human intruder detection or health care,

- Urban grid for automotive use cases.

- Indoor Factory for AGV use cases.

In the following we discuss the parameters that can be used for the evaluation of sensing in the above proposed representative deployment scenarios, except for AGV use cases, which we think can be discussed later after the communication related parameters for Indoor Factory have been agreed.

Motivated by the joint design of sensing and communication, it is favourable to capture the evaluation parameters for sensing and communication in one common table. But it is possible that not all assumptions for communication also are needed for sensing, also there are some parameters, e.g., on target types, that only are needed for sensing. Therefore, in deployment scenarios that can be used for both communication and ISAC, it is preferred to indicate which parameters can be used for both, only for communication and only for ISAC.

Proposal 2: For deployment scenarios that are used for evaluation of both sensing and communication use cases, the assumptions should be captured in one common table.

- Not all assumptions may be applicable for both sensing and communication. These should be highlighted in the corresponding tables.

When defining the evaluation assumptions for communication and sensing in a deployment scenario, there are some differences that should be taken into account.

- Carrier frequencies: Compared with communication, 700MHz and 2GHz are too low frequencies which may not need to be studied for ISAC since they may not provide good sensing performance. Also, sensing evaluation for UAV detection and tracking in purely urban micro should have lower priority in our view. On the other hand, macro + micro layer could be considered, for example for cooperative sensing.

- Minimum UE-UE distance: This parameter is not necessary for communication evaluation, but is required for UE-UE bistatic sensing.

- Target related parameters: Should be provided for sensing, not needed for communication.

On the other hand, with the guideline on possible attributes for RAN WG studies in the RAN TR 38.914, it is still allowed that RAN WG may further elaborate certain parameters, including simulation of a subset of proposed attribute values. The selection of certain combinations of attributes can be up to RAN WG discussions.

  - Urban macro for communication and sensing

Urban macro is a scenario that can be used for evaluation of communication and sensing.

For sensing, TRP monostatic sensing mode for the UAV use case is studied in 5GA Rel-20, other sensing modes or cooperative sensing of all sensing modes should be further considered in 6G.

To reflect that Urban macro can be used for joint evaluation of sensing and communication, the existing section [4.4 Urban macro] can be updated as follows.

-------------------- TP #1 on Urban macro for ISAC ----------------------

4.4	Urban macro

The urban macro deployment scenario focuses on large cells and continuous coverage. The key characteristics of this scenario are continuous and ubiquitous coverage in urban areas. This scenario will be interference-limited, using macro TRxPs (i.e. radio access points above rooftop level). Joint operation of sensing and communication can be supported in Urban macro deployment scenario. It focuses on UAV detection/tracking with a large 3D coverage. The sensing Tx/Rx can be BS and/or UE.

Some of the attributes are listed in Table 4.4.

Table 4.4: Attributes for urban macro

| Attributes |  | Values or assumptions |
| --- | --- | --- |
| Carrier Frequency |  | Macro layer: Around 700 MHz Around 2 GHz Around 4 GHz Around 7 GHz Around 15 GHz Around 2 GHz + Around 4 GHz Around 2 GHz + Around 7 GHz Around 4 GHz + Around 7 GHz Around 2 GHz + Around 4 GHz + Around 7 GHz Around 7GHz +Around 4GHz + Around 2GHz+Around 700MHz Micro: Around 7 GHz Around 15 GHz Around 30 GHz Macro + Micro: Around 2 GHz + Around 30GHz Around 4 GHz + Around 30 GHz Around 7 GHz + Around 30 GHz |
| Aggregated system bandwidth |  | Around 700 MHz: Up to 60 MHz (DL+UL) Around 2 GHz: Up to 200 MHz (DL+UL) Around 4 GHz: Up to 300 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) Around 15 GHz: Up to 400 MHz (DL+UL) Around 30 GHz: Up to 1 GHz (DL+UL) |
| Layout |  | Single layer: - Hex. Grid Two layers: - Macro layer: Hex. Grid - Micro layer: Random drop |
| ISD |  | Macro: 500m [TBD on micro layout] |
| BS antenna elements |  | Around 700 MHz: Up to 64 Tx and Rx antenna elements Around 2 GHz: Up to 288 Tx and Rx antenna elements Around 4 GHz: Up to 576 Tx and Rx antenna elements Around 7 GHz: Up to 2304 Tx and Rx antenna elements Around 15 GHz: Up to 2304 Tx and Rx antenna elements Around 30 GHz: Up to 4096 Tx and Rx antenna elements |
| UE antenna elements |  | TBD |
| User distribution and UE speed |  | TBD [10] users per TRxP |
| Service profile |  | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |
| Minimum UE-UE distance (ISAC) |  | TBD |
| Sensing target (ISAC) | Type | UAV |
|  | LOS/NLOS | LOS and NLOS |
|  | Outdoor/indoor | Outdoor |
|  | Mobility | Horizontal: {0 km/h, Uniform distribution between 0 and 180km/h} Vertical: {0, 20, 40} km/h Trajectory: straight-line |
|  | Distribution | N targets per sector within the center cell, N = 5 Horizontal: uniformly distributed within a sector Vertical: Uniform between 1.5m and 300m. |
|  | Orientation | Random in horizontal domain |
|  | Physical characteristics | Size: UAV with {small size, large size} defined in Rel-19 |

-------------------- End of TP #1 ---------------------------------------------

Proposal 3: Adopt TP #1 for deployment scenario Urban macro in TR 38.914 for the evaluation of communication and UAV sensing scenarios.

  - Indoor hotspot for communication and ISAC

For human intruder detection or health care, indoor hotspot can be a typical scenario. The sensing Tx/Rx can be BS or UE. The attributes of Indoor hotspot for ISAC can reuse those from indoor hotspot. The sensing target related parameters need to be added.  The existing section [4.1 Indoor hotspot] can be updated as follows.

-------------------- TP #2 on Indoor hotspot for ISAC ----------------------

4.1	Indoor hotspot

The indoor hotspot deployment scenario focuses on small coverage per site/TRxP (transmission and reception point) and high user throughput or user density in buildings. The key characteristics of this deployment scenario are high capacity, high user density and consistent user experience indoor. Joint operation of sensing and communication can be supported in Indoor hotspot deployment scenario. The focus is on human as sensing target with large LOS probability in small coverage. The sensing Tx/Rx can be BS and/or UE.

Some of its attributes are listed in Table 4.1A.

Table 4.1: Attributes for indoor hotspot

| Attributes |  | Values or assumptions |
| --- | --- | --- |
| Carrier Frequency NOTE1 |  | Around 2 GHz Around 4 GHz Around 7 GHz Around 15 GHz Around 30 GHz |
| Aggregated system bandwidth NOTE2 |  | Around 2 GHz: Up to 200 MHz (DL+UL) NOTE3 . Around 4 GHz: Up to 300 MHz (DL+UL) NOTE3 Around 7 GHz: Up to 400 MHz (DL+UL) NOTE3 Around 15 GHz Up to 400 MHz (DL+UL) Around 30 GHz: Up to 1GHz (DL+UL) NOTE3 |
| Layout |  | Single layer: - Indoor floor (Open office) , 120m x 50m |
| ISD |  | 20m for around 30GHz TBD on other carrier frequencies (Equivalent to [12]TRxPs per 120m x 50m) |
| BS antenna elements |  | Around 700 MHz: Up to 64 Tx and Rx antenna elements Around 2 GHz: Up to 288 Tx and Rx antenna elements Around 4 GHz: Up to 576 Tx and Rx antenna elements Around 7 GHz: Up to 2304 Tx and Rx antenna elements Around 15 GHz: Up to 2304 Tx and Rx antenna elements Around 30 GHz: Up to 4096 Tx and Rx antenna elements |
| UE antenna elements |  | TBD |
| User distribution and UE speed |  | 100% Indoor, 3km/h, [10] users per TRxP NOTE4 |
| Service profile |  | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |
| Sensing target (ISAC) | Type | Human |
|  | LOS/NLOS | LOS and NLOS |
|  | Outdoor/indoor | Indoor |
|  | Mobility | Uniform distribution between 0km/h and 3km/h Trajectory: straight-line |
|  | Distribution | N targets uniformly distributed over the horizontal area of the convex hull of the TRP deployment, N = 5 |
|  | Orientation | Random over the horizontal area |
|  | Physical characteristics | Adult: 0.5m x 0.5m x 1.75m |
| NOTE 1: TBD NOTE 2: TBD NOTE 3: TBD NOTE 4: TBD |  |  |

-------------------- End of TP #2 ----------------------

Proposal 4: Adopt TP #2 for deployment scenario Indoor hotspot in TR 38.914 for the evaluation of communication and human sensing scenarios.

  - Urban grid for ISAC

Urban grid deployment scenario was introduced for V2X use cases in 5G. It is expected that Urban grid deployment scenario in 6G is used mainly for sensing. All sensing modes including cooperative sensing can be considered. The sensing transmitter/receiver can be TRP, vehicle UE, pedestrian UE and/or RSU type UE. On the other hand, to verify the proper design of the ISAC operation, it is also important to check the communication performance in Urban grid deployment scenario. The attributes for Urban grid for communication and ISAC are provided below. The attributes except for those for sensing target are obtained by referring to the attributes of Urban macro. The existing section [4.11 Urban grid] can be updated as follows.

-------------------- TP #3 on Urban grid for ISAC ----------------------

4.11	[Urban grid]

The Urban grid deployment scenario focuses on vehicle/human detection/tracking in the dense urban area. The key characteristics of this deployment scenario is the rich presence of scatterers in the environment. Certain environment objects may be deterministically modeled. The sensing Tx/Rx can be BS, vehicle UE, pedestrian UE and/or RSU type UE. Joint operation of sensing and communication can be supported in Urban grid deployment scenario.

Some of its attributes are listed in Table 4.11.

Table 4.11: Attributes for Urban grid for ISAC

| Attributes |  | Values or assumptions |  |
| --- | --- | --- | --- |
| Carrier Frequency |  | Around 4 GHz Around 7 GHz Around 30GHz Around 4 GHz + Around 7GHz Around 4 GHz + Around 30GHz |  |
| System bandwidth |  | Around 4 GHz: Up to 300 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) Around 30 GHz: 1GHz (DL+UL) |  |
| Layout |  | Single layer: - Hex. Grid Two layers for Around 4 GHz + Around 30 GHz: - Macro layer: Hex. Grid, 4GHz - Micro layer: Random drop, 30GHz |  |
| ISD |  | Macro: 500m [TBD on micro layout] |  |
| BS antenna elements |  | TBD |  |
| UE antenna elements |  | TBD |  |
| User distribution and UE speed |  | Based on TR37.885 |  |
| Minimum UE-UE distance |  | TBD |  |
| Service profile |  | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |  |
| Sensing target, i.e., UAV | Type | Vehicle | Human |
|  | LOS/NLOS | LOS and NLOS (including NLOSv) | LOS and NLOS |
|  | Outdoor/indoor | Outdoor | Outdoor |
|  | Mobility | Based on TR37.885 | Uniform distribution between 0km/h and 10km/h Trajectory: straight-line |
|  | Distribution | Based on dropping in TR37.885 | Based on dropping in TR37.885 |
|  | Orientation | Lane direction in horizontal plane | Random over the horizontal area |
|  | Physical characteristics | Type 1/2 (passenger vehicle) Vehicle type distribution per TR 37.885 | Adult Pedestrian: 0.5m x 0.5m x 1.75m |
| Environment Objects |  | Optional: Type-2 EO - 4 walls modelled as Type-2 EO, per building of size 413m x 230m x 20m. |  |

-------------------- End of TP #3 ----------------------

Proposal 5: Adopt TP #3 for deployment scenario Urban grid in TR 38.914 for evaluation of automotive sensing scenarios.

- Conclusion

In this contribution, we discuss the deployment scenarios for ISAC and make the following proposals.

Proposal 1: The following representative deployment scenarios can be considered for the evaluation of object detection/tracking:

- Urban macro (UMa-AV in NR) for UAV detection and tracking

- Indoor hotspot for human intruder detection or health care,

- Urban grid for automotive use cases.

- Indoor Factory for AGV use cases.

Proposal 2: For deployment scenarios that are used for evaluation of both sensing and communication use cases, the assumptions should be captured in one common table.

- Not all assumptions may be applicable for both sensing and communication. These should be highlighted in the corresponding tables.

Proposal 3: Adopt TP #1 for deployment scenario Urban macro in TR 38.914 for the evaluation of communication and UAV sensing scenarios.

Proposal 4: Adopt TP #2 for deployment scenario Indoor hotspot in TR 38.914 for the evaluation of communication and human sensing scenarios.

Proposal 5: Adopt TP #3 for deployment scenario Urban grid in TR 38.914 for evaluation of automotive sensing scenarios.

- References

- 3GPP TR 38.914, Study on 6G Scenarios and Requirements, V0.2.0, (2025-09)

- RP-253249, Requirements of Integrated Sensing And Communication (ISAC), Xiaomi