3GPP TSG RAN Meeting #110	RP-253262

Baltimore, USA, December 8-11, 2025

# Source:	vivo (moderator)

Title:			   Revised WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 4

# Document for:	Approval

Agenda Item:	10.3.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: New WI: Non-Terrestrial Networks (NTN) for Internet of Things(IoT) Phase 4

Acronym: IoT_NTN_Ph4

Unique identifier: 1080090

NOTE:	For new WIs/SIs leave the Unique identifier empty and make a proposal for an Acronym.

For a revised WI/SI: Take Unique identifier and acronym as shown in 3GPP workplan.

If this is a RAN WID including Core and Perf. part, then Title, Acronym and Unique identifier refer to the feature WI.

Please tick (X) the applicable box(es) in the table below:

Either:

| This WID includes a Core part | X |
| --- | --- |
| This WID includes a Performance part |  |

or:

| This WID includes a Testing part |  |  |
| --- | --- | --- |
| and it addresses the following 3GPP work area: | Radio Access |  |
|  | Core Network |  |
|  | Services |  |

Potential target Release:	Rel-20

NOTE: In case of contradiction with the target dates of clause 5, clause 5 determines the target release.

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X |  |  |
| No | X |  |  | X |  |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This description is either a …

|  | Study Item |
| --- | --- |

or a

| Normative Work Item: tick applicable boxes below |  |
| --- | --- |
|  | Stage 1 |
| X | Stage 2 |
| X | Stage 3 |
|  | Other (e.g. testing) |

2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

NOTE:	RAN agreed some time ago, that it describes the feature WI + Core/Perf. part WI or Testing part WI in one 	WID. Therefore the table above should include the feature WI data (In case the feature covers Core and Perf. 	part, please list under Working Group the leading WG of the Core part).

2.3	Other related Work Items and dependencies

| Other related Work/Study Items (if any) |  |  |  |
| --- | --- | --- | --- |
| Acronym | Unique ID | Title | Nature of relationship |
| LTE_NBIOT_eMTC_NTN | 920169 | NB-IoT/eMTC support for Non-Terrestrial Networks | This WI enhanced the IoT usage via NTN in Rel-17 LTE |
| IoT_NTN_enh | 941004 | IoT NTN enhancements | This WI enhanced the IoT usage via NTN in Rel-18 LTE |
| IoT_NTN_Ph3 | 1020096 | Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3 | This WI enhanced the IoT usage via NTN in Rel-19 LTE |

NOTE:	Also related or dependent WIs/SIs in other TSGs shall be indicated here.

Dependency on non-3GPP (draft) specification:

3	Justification

Non-Terrestrial Networks (NTN) for Internet of Things (IoT) has been specified in 3GPP RAN Rel-17, and since then been optimised in Rel-18 and Rel-19. However, the current specification does not support IMS voice call over NB-IoT-NTN.

SA1 has studied the use case of IMS voice call using GSO satellite access, and the results are documented in TR 22.887. Normative service requirements and KPIs on IMS voice call using GSO satellite access is captured in TS 22.261, as shown in Table 1 below.

Table 1: Performance requirements for satellite access

| Scenario | Experienced data rate (DL) | Experienced data rate (UL) | Area traffic capacity (DL) (note 1) | Area traffic capacity (UL) (note 1) | Overall user density | Activity factor | UE speed | UE type | Others |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IMS voice call using GEO (note 7) | [1-3] kbit/s | [1-3] kbit/s | N/A | N/A | N/A | N/A | TBD | Handheld | Call set up time (note 8) ≤30 s (note 9) |
| Note 7: All the values defined for IMS voice call using GEO satellite access are generic for both regular and emergency calls. Note 8: call set up time refers to [56]; Note 9: 30s is the upper bound that is derived based on the user’s patience suggestions (30s) in [57]; |  |  |  |  |  |  |  |  |  |  |

SA has approved a new SA4-led SID on Ultra Low Bitrate Speech Codec in SP-250378. According to the agreed contribution in SA4 in S4-251123, the PHY bite rate is set to 1~5kbps and codec bite rate is set to 0.8~4.8kbps.

SA also approved a new SA2-led SID on IMS voice call over GSO via NB-IoT-NTN in SP-250400. As WT#1 of the SID, both CP and UP based solutions are being studied and the SI is be concluded by TSG#110 (Dec. 2025), including the down-selection between CP and UP solutions.

To enable end-to-end support of IMS voice call over NB-IoT-NTN via GSO, RAN specification work is required.

This work item aims to specify the RAN related enhancements to support IMS voice call over NB-IoT-NTN via GSO.

4	Objective

4.1	Objective of SI or Core part WI or Testing part WI

The aim of this work item is to specify the enhancement of NB-IoT-NTN to support IMS voice call over GSO with the following objectives:

- Specify UP solution for voice support over NB-IoT NTN [RAN2]

  - Specify necessary solution to address the scheduling for variable sized voice packets [RAN2]

  - Specify RoHC profile configuration for voice packet [RAN2]

  - Other necessary changes if needed

- Support of semi-persistent scheduling for NB-IoT-NTN for DL and UL data transmission for voice packets [RAN2, RAN1]

- Support of necessary modifications to RRC connection setup procedure for NB-IoT-NTN [RAN2]

- Support of necessary modifications for emergency call for voice over NB-IoT-NTN [RAN2]

- Study and if feasible, specify UE transmit power higher than PC1 (e.g. up to 37dBm) for NB-IoT-NTN [RAN4]

- Note: The enhancements to support voice over NB-IoT-NTN via GSO in this work item may be also applicable to NGSO cases without additional specification enhancements

- Note: Coordination with SA4/2 is expected

4.2	Objective of Performance part WI

NOTE:	Leave empty if the WI proposal does not contain a RAN performance part.

4.3	RAN time budget request (not applicable to RAN5 WIs/SIs)

NOTE:	For all new RAN related WIs/SIs which are not led by RAN WG5 the WI/SI rapporteur has to fill out the attached Excel table to request time budgets for corresponding RAN WG meetings.
The Excel table has to be filled out for all affected RAN WGs and up to the target date of the WI/SI.
One time unit (TU) corresponds to ~ 2 hours in the meeting.
If no TU is needed, then leave the field empty otherwise enter a number >0 in the field.

For revisions of already approved WI/SI descriptions: Please remove the Excel table from the WID/SID's zip file. The time budgets are already recorded. If you want to modify them, then this has to be done via the status report and not via a revised WID/SID.

If this WID is covering Core and Performance part, then please fill out one line for each part in the attached Excel table.

additional comments to the time budget request in the attached Excel table:

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
By default a new specs can only be new for one of both parts.

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 36.213 | To support IMS voice call over GSO via NB-IoT-NTN | RAN#114 | Core part |
| 36.300 | To support IMS voice call over GSO via NB-IoT-NTN | RAN#115 | Core part |
| 36.304 | To support IMS voice call over GSO via NB-IoT-NTN | RAN#115 | Core part |
| 36.306 | To support IMS voice call over GSO via NB-IoT-NTN | RAN#115 | Core part |
| 36.321 | To support IMS voice call over GSO via NB-IoT-NTN | RAN#115 | Core part |
| 36.331 | To support IMS voice call over GSO via NB-IoT-NTN | RAN#115 | Core part |
| 36.102 | To support UE transmit power higher than PC1 for NB-IoT-NTN | RAN#115 | Core part |

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
If an existing spec is affected by both (Core part and Perf. part), then it has to be listed twice with appropriate approval dates.

6	Work item Rapporteur(s)

Xiaodong Yang, vivo, yangxiaodong5g@vivo.com

7	Work item leadership

Leading working group: RAN2

Secondary working groups: RAN1, RAN4

8	Aspects that involve other WGs

NOTE:	For RAN WIs: Section 8 applies only toWGs outside of TSG RAN because all RAN WG aspects have to be covered in section 4.

9	Supporting Individual Members

Supporting IM name

vivo

CMCC

KT Corp.

Verizon

Spreadtrum

UNISOC

ZTE Corporation

Sanechips

CATT

Samsung

Interdigital

NTT DOCOMO

Skylo

Transsion Holdings

III

China Telecom

MediaTek Inc.

Intel Corporation

China Unicom

CAICT

Ericsson

SK Telecom

CSCN

Huawei

HiSilicon

Xiaomi

LG Electronics

Qualcomm

Nokia

China Broadnet

NEC

Viasat

FirstNet

Semtech

Lenovo

Motorola Mobility

OPPO

Sharp

ETRI

EchoStar

Cybercore

Gatehouse Satcom

Aalyria

Toyota ITC

Sony

Novamint

Sateliot

ESA

CBN