3GPP TSG RAN meeting #110										RP-253456

Baltimore, USA, December 8-11, 2025

Status Report to TSG

Agenda item:			9.3.1.2

| WI / SI Name | NR coverage enhancements Phase 3 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_cov_Ph3 |  |  |  |  |
| Unique ID | 1080084 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-252824 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: NA | Core part: 12/2026 | Performance part: 03/2027 | Testing part: NA |  |
| Overall Completion level | Study Item: NA | Core part: 28% | Performance Part: 0% | Testing part: NA |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG1 |
| --- | --- | --- |
| Rapporteur | Name | Hang Yin |
|  | Company | China Telecom |
|  | Email | yinh6@chinatelecom.cn |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

2.1	RAN1

2.1.1	Agreements

RAN1#122bis

PRACH coverage enhancement

Agreement

Support to use separate ROs or separate preambles on shared RO to differentiate multiple PRACH transmissions with same Tx beam and different Tx beams.

  - FFS: details

Working assumption

The multiple PRACH transmission with different Tx beams is supported for CBRA, ReconfigurationWithSync case in CFRA and SI request.

- RAN1 assumes that all the cases are supported with a common solution

Agreement

Reuse at least the following definitions and mechanisms in multiple PRACH transmissions with same Tx beam for multiple PRACH transmissions with different Tx beams.

- Definition and determination of RO group

- definition of time period

- SSB-to-RO mapping rule

- Note: the terminology RO group stands for the ROs for multiple PRACH transmissions as specified in TS 38.213, i.e., “set consists of Npreamblerep valid PRACH occasions…”.

Agreement

Reuse the definition of time offset between RO groups in multiple PRACH transmissions with same Tx beam for multiple PRACH transmissions with different Tx beams.

Agreement

Reuse the PRACH power control (power ramping between different RACH attempts is FFS) rule of multiple PRACH transmissions with same Tx beam for multiple PRACH transmissions with different Tx beams.

- FFS: whether/how power ramping should be supported for multiple PRACH transmissions with different Tx beams.

Agreement

Support indicating the UL beam information via one or a combination of multiple options from following options, for down-selection:

- Option 1: implicitly indicated by RA-RNTI

- Option 2: explicitly indicated by DCI format 1_0 with CRC scrambled by RA-RNTI (e.g., reserved bits)

- Option 3: explicitly indicated by repurposing field(s) in UL Grant in RAR

- Option 4: explicitly indicated by introducing new field in MAC RAR

- Option 5: explicitly indicated by repurposing bit(s) in MAC RAR

FFS: the exact content of UL beam information

Agreement

For the determination of Tx beam utilization for multiple PRACH transmissions with different Tx beams,

- It is up to UE implementation to determine which Tx beams to use, subject to any necessary restriction

  - FFS: any necessary restriction

- FFS: whether/how gNB can configure UE to use same Tx beams in part of the PRACH transmissions

Agreement

For multiple PRACH transmissions with different Tx beams, the maximum candidate value of the total number of PRACH transmissions is 8 per RACH attempt.

- FFS: the exact candidate values and other details

PUSCH repetition scheduled by DCI 0_0 with C-RNTI

Agreement

Reuse the following rules of Msg3 repetition for PUSCH repetition scheduled by DCI 0_0 with C-RNTI.

- RV determination rule, including first RV id determination, RV sequence selection, RV cycling, etc.

- Available slot determination rule

- Only applicable to PUSCH Repetition Type A

Agreement

Down-select one of the following options to indicate the number of repetitions of PUSCH scheduled by DCI 0_0 with C-RNTI

- Option 1: Using at most 2 MSB of MCS field in DCI format 0_0 with CRC scrambled by C-RNTI

- Option 2: Based on TDRA

- Option 3: Using the padding bits in DCI format 0_0 with CRC scrambled by C-RNTI

- Option 4: Using the 2 MSB of HPN field in DCI format 0_0 with CRC scrambled by C-RNTI

- Option 5: Using the DAI field in the DCI 1_0 with CRC scrambled by TC-RNTI

- Other options are not precluded

Agreement

Strive for a single mechanism for PUSCH repetition scheduled by DCI 0_0 with C-RNTI.

Agreement

Support [request or capability report] of PUSCH repetition scheduled by DCI 0_0 with C-RNTI at least before receiving RRCReconfiguration via one of the following signalling.

- Option 1: Msg3

- Option 2: HARQ-ACK of Msg4

- FFS: whether to configure a RSRP of DL reference signal for UE to trigger the [request or capability report].

FFS: other options and details.

Extension of pi/2 BPSK to more MCS entries

Agreement

Introduce RRC signaling to enable the extension of pi/2-BPSK to more MCS entries.

Agreement

LLS should be conducted to decide which MCS entries pi/2-BPSK can be extended to.

- Both LLS performance gain and power boosting gain due to lower PAPR should be taken into consideration.

- Note: LLS performance gain refers to the performance gain/loss when using pi/2-BPSK compared with QPSK with same SE according to LLS; power boosting gain refers to the performance gain acquired due to the low PAPR of pi/2-BPSK compared with QPSK.

Agreement

Pi/2-BPSK is extended to more MCS entries in MCS tables with spectrum efficiency no larger than N (N <= 0.8770).

- FFS: value of N

Agreement

When extending pi/2-BPSK to more MCS entries in MCS tables, the code rate should be doubled to keep spectral efficiency the same.

RAN1#123

PRACH coverage enhancement

Agreement

Confirm the following working assumption in RAN1 #122bis,

Working assumption The multiple PRACH transmission with different Tx beams is supported for CBRA, ReconfigurationWithSync case in CFRA and SI request. - RAN1 assumes that all the cases are supported with a common solution

Agreement

At least for initial Msg3 transmission, support indicating the UL beam information via one of multiple options from following options, for down-selection:

- Option 1: based on implicit indication by RA-RNTI

- Option 3: based on explicit indication by repurposing field(s) in UL Grant in RAR

- Option 4: based on explicit indication by introducing new field in MAC RAR

Agreement

For multiple PRACH transmissions with different Tx beams, candidate values of the total number of PRACH transmissions are {2,4,8} in one RACH attempt.

Agreement

For the power ramping between different RACH attempt for multiple PRACH transmission with different Tx beams, down-select from the following options:

- Option 1: when any of the Tx beams used in the next attempt is changed, Layer 1 (may) notifies higher layer to suspend the power ramping counter.

- Option 2: when all the Tx beams used in the next attempt are changed, Layer 1 notifies higher layer to suspend the power ramping counter

- Option 3: when M>1 UL Tx beams used in the next attempt are changed, Layer 1 (may) notifies higher layer to suspend the power ramping counter, FFS the value of M.

- Option 4: multiple power ramping counters are used for each PRACH transmission in the RO group.

Agreement

Support to configure separate ROs or separate preambles on shared RO to differentiate different number of multiple PRACH transmissions with different Tx beams via RRC parameters.

- FFS: Other details

Agreement

At least for initial RACH attempt, the determination of total number of PRACH transmissions per RACH attempt is down-selected based on the following options

- Option 1: A single RSRP threshold is introduced to determine whether multiple PRACH transmissions with different Tx beams can be applied

- Option 2: each of the values of multiple RPSP thresholds corresponding to each of the values of the total number of multiple PRACH transmissions

- Option 3: each of the values of multiple RSRP thresholds corresponding to each of the values of the number of repetitions of a RACH transmission per Tx beam.

- Option 4: UE determines total number of RACH transmission based on the number of beams it supports.

- Other options are not precluded.

- Combination of options not precluded.

PUSCH repetition scheduled by DCI 0_0 with C-RNTI

Agreement:

Support [request or capability report] of PUSCH repetition scheduled by DCI 0_0 with C-RNTI before receiving RRCReconfiguration in Msg3.

Agreement

Support PUSCH repetitions scheduled by DCI format 0_0 with C-RNTI both before and after receiving RRCReconfiguration.

Note: Strive for a single mechanism for PUSCH repetition scheduled by DCI 0_0 with C-RNTI

Agreement

The first RV id of PUSCH repetition scheduled by DCI format 0_0 with C-RNTI should follows the indication in DCI format 0_0.

Note: this agreement intends following update for the agreement made at RAN1#122bis

Reuse the following rules of Msg3 repetition for PUSCH repetition scheduled by DCI 0_0 with C-RNTI.

- RV determination rule, including first RV id determination, RV sequence selection, RV cycling, etc.

- Available slot determination rule

- Only applicable to PUSCH Repetition Type A

Agreement

For PUSCH repetition scheduled by DCI 0_0 with C-RNTI, inter-slot frequency hopping is supported

- Note: intra-slot frequency hopping is not supported unless the repetition time is 1

Agreement

Down-select one of the following options to indicate the number of repetitions of PUSCH scheduled by DCI 0_0 with C-RNTI both before and after receiving RRCReconfiguration

- Option 1: Using at most 2 MSB of MCS field in DCI format 0_0 with CRC scrambled by C-RNTI

  - FFS: details

- Option 2: Based on TDRA

  - FFS: details

Extension of pi/2 BPSK to more MCS entries

Agreement

For extension of pi/2-BPSK to more MCS entries, down-select one of the following Options:

- Option 1：Pi/2-BPSK is extended to all MCS entries in MCS table(s) with spectrum efficiency equals to or smaller than N = 0.8770.

  - gNB can configure the maximum MCS up to which pi/2 BPSK is applicable.

    - Details on configuration to be discussed further

  - FFS: UE signaling to the gNB for this configuration

    - FFS: whether/how to provide assistance information and what is the assistance information

- Option 2：Support extending Pi/2-BPSK to all MCS entries in MCS table(s) with spectrum efficiency equals to or smaller than X.

  - Where X the fixed value select from:

a)	X = 0.8770

b)	X = 0.6016

  - Extension is captured in the modified MCS table.

  - gNB enables/disables this feature via RRC signalling.

2.1.2	Remaining Open issues

PRACH coverage enhancements

- Details on supporting multiple PRACH transmissions with different Tx beams for 4-step RACH procedure.

PUSCH repetition scheduled by DCI 0_0 with C-RNTI

- Details on supporting PUSCH repetition scheduled by DCI 0_0 with C-RNTI.

Extension of pi/2-BPSK to more MCS entries

- Details on improving PUSCH coverage for higher uplink data rate by extending pi/2-BPSK to more MCS entries in MCS tables

2.2	RAN2

2.2.1	Agreements

2.2.2	Remaining Open issues

Specify following PRACH coverage enhancements

- Multiple PRACH transmissions with different Tx beams for 4-step RACH procedure.

  - UE may receive UL beam information after transmission of MSG1

    - UL beam information is to assist the UE decision on Msg3 beam selection.

- Note 1: “different Tx beams” is for the purpose of future RAN1 discussions

- Note 2: The enhancements of PRACH are targeting for FR2 and can also apply to FR1 when applicable.

- Note 3: The enhancements of PRACH are targeting short PRACH formats and can also apply to other formats when applicable.

- Note 4: The PRACH repetitions are transmitted over ROs associated with the same SSB.

- Note 5: The procedure for repetitions of a PRACH transmission is as in Rel-18

Specify enhancements to support PUSCH repetition scheduled by DCI 0_0 with C-RNTI

2.3	RAN3

2.3.1	Agreements

2.3.2	Remaining Open issues

2.4	RAN4

2.4.1	Agreements

2.4.2	Remaining Open issues

Specify the following performance requirements

- Base station demodulation performance requirements

- Base station conformance testing

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

4.	References

RAN 122bis

- R1-2506810, Discussion on NR Coverage enhancement Phase 3, Spreadtrum, UNISOC, RAN1 #122bis, Prague, Czech, Oct. 2025

- R1-2506894, Discussions on NR phase 3 coverage enhancements, vivo, RAN1 #122bis, Prague, Czech, Oct. 2025

- R1-2506916, Discussion on coverage enhancement, ZTE Corporation, Sanechips, RAN1 #122bis, Prague, Czech, Oct. 2025

- R1-2506947, Coverage enhancements for NR Phase 3	Huawei, HiSilicon, RAN1 #122bis, Prague, Czech, Oct. 2025

- R1-2506985, Discussion on coverage enhancement, Xiaomi, RAN1 #122bis, Prague, Czech, Oct. 2025

- R1-2507055, NR coverage enhancement Phase 3, Tejas Network Limited, RAN1 #122bis, Prague, Czech, Oct. 2025

- R1-2507056, Coverage enhancements for NR Phase 3, Nokia, RAN1 #122bis, Prague, Czech, Oct. 2025

- R1-2507115, Discussion on coverage enhancement, CATT, RAN1 #122bis, Prague, Czech, Oct. 2025

- R1-2507172, Discussion on further enhancement for coverage enhancement, OPPO, RAN1 #122bis, Prague, Czech, Oct. 2025

- R1-2507187, Discussion on coverage enhancement, LG Electronics, RAN1 #122bis, Prague, Czech, Oct. 2025

- R1-2507191, Discussion on coverage enhancement, Transsion Holdings, RAN1 #122bis, Prague, Czech, Oct. 2025

- R1-2507216, Coverage enhancement for NR Phase 3, InterDigital, Inc. , RAN1 #122bis, Prague, Czech, Oct. 2025

- R1-2507249, Discussion on Coverage Enhancement, Samsung, RAN1 #122bis, Prague, Czech, Oct. 2025

- R1-2507289, Discussions on the Rel. 20 coverage enhancement, KT Corp. , RAN1 #122bis, Prague, Czech, Oct. 2025

- R1-2507329, Discussion on NR coverage enhancements Phase 3	China Telecom, RAN1 #122bis, Prague, Czech, Oct. 2025

- R1-2507375, Discussion on 5G NR coverage enhancement phase 3, Panasonic, RAN1 #122bis, Prague, Czech, Oct. 2025

- R1-2507432, Discussion on coverage enhancement, TCL, RAN1 #122bis, Prague, Czech, Oct. 2025

- R1-2507464, Views on Coverage Enhancement Phase 3, Ofinno, RAN1 #122bis, Prague, Czech, Oct. 2025

- R1-2507502, Discussion on coverage enhancements, ETRI, RAN1 #122bis, Prague, Czech, Oct. 2025

- R1-2507530, Coverage enhancements, Lenovo, RAN1 #122bis, Prague, Czech, Oct. 2025

- R1-2507554, Discussion on coverage enhancement, DENSO CORPORATION, RAN1 #122bis, Prague, Czech, Oct. 2025

- R1-2507673, Discussion on Coverage enhancements for NR Phase 3, Apple, RAN1 #122bis, Prague, Czech, Oct. 2025

- R1-2507717, Coverage enhancement Phase 3, Qualcomm Incorporated, RAN1 #122bis, Prague, Czech, Oct. 2025

- R1-2507783, Discussion on Rel-20 Coverage Enhancement, Ericsson (China), RAN1 #122bis, Prague, Czech, Oct. 2025

- R1-2507809, Discussions on coverage enhancement, NTT DOCOMO, INC. , RAN1 #122bis, Prague, Czech, Oct. 2025

- R1-2507822, Discussions on further coverage enhancement for NR, Sharp, RAN1 #122bis, Prague, Czech, Oct. 2025

- R1-2507867, Discussion on coverage enhancement, ASUSTeK, RAN1 #122bis, Prague, Czech, Oct. 2025

- R1-2507900, Discussion on Coverage Enhancements Phase 3, CEWiT, RAN1 #122bis, Prague, Czech, Oct. 2025

- R1-2507330, FL's summary #1 on NR coverage enhancements Phase 3, China Telecom, RAN1 #122bis, Prague, Czech, Oct. 2025

- R1-2507331, FL's summary #2 on NR coverage enhancements Phase 3, China Telecom, RAN1 #122bis, Prague, Czech, Oct. 2025

- R1-2508116, FL's summary #3 on NR coverage enhancements Phase 3, China Telecom, RAN1 #122bis, Prague, Czech, Oct. 2025

- R1-2508158, Session Notes of AI 10.4, Ad-Hoc Chair (Huawei), RAN1 #122bis, Prague, Czech, Oct. 2025

RAN 123

- R1-2508383, Discussion on NR Coverage enhancement, Spreadtrum, UNISOC, RAN1 #123, Dallas, USA, Nov. 2025

- R1-2508393, Discussion on coverage enhancement, LG Electronics, RAN1 #123, Dallas, USA, Nov. 2025

- R1-2508427, Further discussions on NR phase 3 coverage enhancements, vivo, RAN1 #123, Dallas, USA, Nov. 2025

- R1-2508498, Coverage enhancements for NR Phase 3, Huawei, HiSilicon, RAN1 #123, Dallas, USA, Nov. 2025

- R1-2508592, Discussion on coverage enhancement, CATT, RAN1 #123, Dallas, USA, Nov. 2025

- R1-2508607, Discussion on NR coverage enhancements Phase 3, China Telecom, RAN1 #123, Dallas, USA, Nov. 2025

- R1-2508679, Discussion on coverage enhancement, Xiaomi, RAN1 #123, Dallas, USA, Nov. 2025

- R1-2508722, Discussion on further enhancement for coverage enhancement, OPPO, RAN1 #123, Dallas, USA, Nov. 2025

- R1-2508755, NR coverage enhancement Phase 3, Tejas Network Limited, RAN1 #123, Dallas, USA, Nov. 2025

- R1-2508797, Discussion on Coverage Enhancement, Samsung, RAN1 #123, Dallas, USA, Nov. 2025

- R1-2508837, Discussion on coverage enhancement, Transsion Holdings, RAN1 #123, Dallas, USA, Nov. 2025

- R1-2508853, Discussion on coverage enhancement, ZTE Corporation, Sanechips, RAN1 #123, Dallas, USA, Nov. 2025

- R1-2508913, MCS entries for pi/2-BPSK, Wisig, IITH, RAN1 #123, Dallas, USA, Nov. 2025

- R1-2508968, Discussion on coverage enhancements, ETRI, RAN1 #123, Dallas, USA, Nov. 2025

- R1-2508998, Discussion on 5G NR coverage enhancement phase 3, Panasonic, RAN1 #123, Dallas, USA, Nov. 2025

- R1-2509005, Coverage enhancements for NR Phase 3, Nokia, RAN1 #123, Dallas, USA, Nov. 2025

- R1-2509015, Coverage enhancement for NR Phase 3, InterDigital, Inc. , RAN1 #123, Dallas, USA, Nov. 2025

- R1-2509038, Views on Coverage Enhancement Phase 3, Ofinno, RAN1 #123, Dallas, USA, Nov. 2025

- R1-2509105, Discussion on coverage enhancements for NR Phase 3, Apple, RAN1 #123, Dallas, USA, Nov. 2025

- R1-2509138, Discussions on NR coverage enhancements Phase 3, KT Corp. , RAN1 #123, Dallas, USA, Nov. 2025

- R1-2509179, Coverage enhancements, Lenovo, RAN1 #123, Dallas, USA, Nov. 2025

- R1-2509181, Discussions on further coverage enhancement for NR, Sharp, RAN1 #123, Dallas, USA, Nov. 2025

- R1-2509226, Coverage enhancement Phase 3, Qualcomm Incorporated, RAN1 #123, Dallas, USA, Nov. 2025

- R1-2509275, Discussions on coverage enhancement, NTT DOCOMO, INC. , RAN1 #123, Dallas, USA, Nov. 2025

- R1-2509308, Discussion on NR Coverage enhancement Phase 3, ITRI, Acer Incorporated, RAN1 #123, Dallas, USA, Nov. 2025

- R1-2509334, Discussion on Rel-20 Coverage Enhancement, Ericsson (China) , RAN1 #123, Dallas, USA, Nov. 2025

- R1-2509346, Discussion on Coverage Enhancements Phase 3, CEWiT, RAN1 #123, Dallas, USA, Nov. 2025

- R1-2509365, Discussion on coverage enhancement, ASUSTeK, RAN1 #123, Dallas, USA, Nov. 2025

- R1-2509379, Discussion on coverage enhancement, DENSO CORPORATION, RAN1 #123, Dallas, USA, Nov. 2025

- R1-2509420, Discussion on Rel-20 Coverage Enhancement, Ericsson, RAN1 #123, Dallas, USA, Nov. 2025

- R1-2508608, FL's summary #1 on NR coverage enhancements Phase 3, China Telecom, RAN1 #123, Dallas, USA, Nov. 2025

- R1-2508609, FL's summary #2 on NR coverage enhancements Phase 3, China Telecom, RAN1 #123, Dallas, USA, Nov. 2025

- R1-2508610, FL's summary #3 on NR coverage enhancements Phase 3, China Telecom, RAN1 #123, Dallas, USA, Nov. 2025

- R1-2508611, FL's summary #4 on NR coverage enhancements Phase 3, China Telecom, RAN1 #123, Dallas, USA, Nov. 2025

- R1-2508612, Final FL's summary on NR coverage enhancements Phase 3, China Telecom, RAN1 #123, Dallas, USA, Nov. 2025

- R1-2509451, Session Notes of AI 10.4, Ad-Hoc Chair (NTT DOCOMO, INC.) China Telecom, RAN1 #123, Dallas, USA, Nov. 2025