3GPP TSG-RAN5 Meeting #109	R5-256109

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5498 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of TC 20.7 IPsec tunnel establishment procedure/MPS Priority |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MPS_WLAN-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TC 20.7 “IPsec tunnel establishment procedure/MPS Priority” is needed according to the WP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New TC 20.7 was added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work plan of MPS when access to EPC/5GC is WLAN will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 20.7 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

20.7	IPsec tunnel establishment procedure/MPS Priority

20.7.1	Test Purpose (TP)

(1)

with { UE is configured for high priority access in the seleted PLMN and connects to an untrusted WLAN }

ensure that {

when { UE receives an IKE_SA_INIT response message }

then { UE transmits an IKE_AUTH request message including HPA_INFO Notify payload with the MPS_PRI bit set as 1 }

}

20.7.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.302, clause 7.2.2.1. Unless otherwise stated these are Rel-18 requirements.

[TS 24.302, clause 7.2.2.1]

Once the ePDG has been selected, the UE shall initiate the IPsec tunnel establishment procedure using the IKEv2 protocol as defined in IETF RFC 7296 [28] and 3GPP TS 33.402 [15].

The UE shall send an IKE_SA_INIT request message to the selected ePDG in order to setup an IKEv2 security association. Upon receipt of an IKE_SA_INIT response, the UE shall send an IKE_AUTH request message to the ePDG, including:

-	The type of IP address (IPv4 address or IPv6 prefix or both) that needs to be configured in an IKEv2 CFG_REQUEST Configuration Payload. If the UE requests for both IPv4 address and IPv6 prefix, the UE shall send two configuration attributes in the CFG_REQUEST Configuration Payload: one for the IPv4 address and the other for the IPv6 prefix;

-	The "IDr" payload, containing the APN in the Identification Data, for non-emergency session establishment. For emergency session establishment, the UE shall format the "IDr" payload according to clause 7.2.5. The UE shall set the ID Type field of the "IDr" payload to ID_FQDN as defined in IETF RFC 7296 [28]. The UE indicates a request for the default APN by omitting the "IDr" payload, which is in accordance with IKEv2 protocol as defined in IETF RFC 7296 [28];

-	The "IDi" payload containing the NAI; and

-	The "CERTREQ" payload containing "X.509 certificate – Signature" if the UE is provisioned with the ePDG root certificate as specified in 3GPP TS 33.402 [15].

The IKE_AUTH request message may also contain:

-	An indication in a notify payload that MOBIKE is supported by the UE;

-	The INTERNAL_IP6_DNS or the INTERNAL_IP4_DNS attribute in the CFG_REQUEST Configuration Payload. The UE can obtain zero or more DNS server addressed in the CFG_REPLY payload within the IKE_AUTH response message as specified in IETF RFC 7296 [28]; or

-	The P_CSCF_IP6_ADDRESS attribute, the P_CSCF_IP4_ADDRESS attribute or both in the CFG_REQUEST Configuration Payload. The UE can obtain zero or more P-CSCF server addresses in the CFG_REPLY Configuration Payload within the IKE_AUTH response message as specified in IETF RFC 7651 [64].

If the UE's USIM indicates that the UE is a UE configured for high priority access in the selected PLMN as specified in 3GPP TS 31.102 [45], the UE shall send the HPA_INFO Notify payload, as defined in clause 8.1.2.3, within the IKE_AUTH request message, with the MPS_PRI bit and AC_PRI bit set as defined in clause 8.2.9.22.

20.7.3	Test description

20.7.3.1	Pre-test conditions

System Simulator:

-	WLAN Cell 27 according to Table 4.4.8-1 in TS 36.508 [18].

UE:

-	UE is configured for high priority access in the selected PLMN in the USIM according to Table 4.9.3.6-1 in TS 36.508 [18].

Preamble:

-	The UE is in state Switched OFF (state 1) according to TS 36.508 [18].

20.7.3.2	Test procedure sequence

Table 20.7.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - |  | - | - |
| 2 | The UE associates with the WLAN AP and obtains the local IP address. | - |  | - | - |
| 3 | The UE performs a dynamic selection of ePDG using DNS query. | - |  | - | - |
| 4 | The UE transmits an IKE_SA_INIT request message. | --> | IKE_SA_INIT request | - | - |
| 5 | The SS transmits an IKE_SA_INIT response message. | <-- | IKE_SA_INIT response | - | - |
| 6 | Check: Does the UE transmit the HPA_INFO Notify payload within IKE_AUTH request message with the MPS_PRI bit and AC_PRI bit set to 1? | --> | IKE_AUTH request | 1 | P |
| 7 | The SS transmits an IKE_AUTH response message with EAP-Request/AKA-Challenge. | <-- | IKE_AUTH response | - | - |
| 8 | The UE transmits an IKE_AUTH request message with EAP-Response/AKA-Challenge. | --> | IKE_AUTH request | - | - |
| 9 | The SS transmits an IKE_AUTH response message with EAP-Success. | <-- | IKE_AUTH response | - | - |
| 10 | The UE transmits an IKE_AUTH request message with AUTH parameter. | --> | IKE_AUTH request | - | - |
| 11 | The SS transmits an IKE_AUTH response message with AUTH parameter, configuration payload, security associations and the rest of the IKEv2 parameters and the IKEv2 negotiation terminates. | <-- | IKE_AUTH response | - | - |
| 12-20 | The UE performs the IMS registration procedure according TS 34.229-1 [35] subclause C.2c (steps 2-9). | - |  | - | - |

20.7.3.3	Specific message contents

Table 20.7.3.3-1: IKE_AUTH Request (step 6, Table 20.7.3.2-1)

| Derivation path: TS 36.508 [18] , Table 4.7G-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| IKE Header |  |  |  |
| Next Payload | ‘00101001’B | N |  |
| Exchange Type | ‘00100011’B | IKE_AUTH |  |
| Notify Payload |  |  |  |
| Next Payload | ‘00000000’B | No Next Playload |  |
| Protocol ID | ‘00000001’B | For IKE-SA |  |
| SPI Size | ‘00000000’B | IKE SA |  |
| Notify Message Type | ‘1101101001100111’B | HPA_INFO |  |
| Length | Not checked |  |  |
| Data | ‘00000011’B | MPS_PRI bit: 1 AC_PRI bit: 1 |  |

==============End of change==============