3GPP TSG-RAN5 Meeting #109	R5-256662

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 5154 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of MPS when access to EPC/5GC is WLAN TC 9.2.1.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MPS_WLAN-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TC 9.2.1.3 IKE SA and signalling IPsec SA establishment procedure/MPS Priority is needed according to the WP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new TC 9.2.1.3 IKE SA and signalling IPsec SA establishment procedure/MPS Priority |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work Plan will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.1.3(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.523-2 CR 0632 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R5-255678r1 |  |  |  |  |  |  |  |  |

==============First change==============

9.2.1.3	IKE SA and signalling IPsec SA establishment procedure/MPS Priority

9.2.1.3.1	Test Purpose (TP)

(1)

with { UE has a valid Access Identity 1 }

ensure that {

when { UE  connects to untrusted non-3GPP access network and finishes IKE_SA_INIT exchange and initiates an IKE_AUTH exchange }

then { UE includes the HPA_INFO Notify payload in the initial IKE_AUTH request message }

}

9.2.1.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.502, clause 7.3.2.1. Unless otherwise stated these are Rel-18 requirements.

[TS 24.502, clause 7.3.2.1] The UE proceeds with the establishment of IKE SA and signalling IPsec SA with the selected N3IWF by initiating an IKE_SA_INIT exchange according to IETF RFC 7296 [6]. All the IKE messages following the IKE_SA_INIT exchange are encrypted and integrity protected using the cryptographic algorithms and keys negotiated in the IKE_SA_INIT exchange as specified in IETF RFC 7296 [6].

Upon completion of the IKE_SA_INIT exchange, the UE shall initiate an IKE_AUTH exchange as specified in IETF RFC 7296 [6] to establish an IKE SA and first child SA (signalling IPsec SA). In the initial IKE_AUTH request message, the UE shall:

-	indicate the intention to use EAP by not including the AUTH payload;

-	include the IDi payload with the ID type set to ID_KEY_ID and value set to any random number;

-	include CERTREQ payload to request N3IWF's certificate if the UE is provisioned with the N3IWF root certificate; and

-	include the HPA_INFO Notify payload, as defined in 3GPP TS 24.302 [7], within the IKE_AUTH request message if the UE has a valid Access Identity 1 as specified in clause 4.5.2 of 3GPP TS 24.501 [4];

as specified in IETF RFC 7296 [6].

Upon reception of the IKE_AUTH request message, the N3IWF shall respond with an IKE_AUTH response message including:

-	an EAP-Request/5G-Start packet to inform the UE an EAP-5G session that will be used to convey the initial NAS messages (see the EAP-5G procedure described in clause 7.3.3);

-	the IDr payload with the value set to N3IWF identifier; and

NOTE:	The N3IWF identifier is the IP address or the FQDN of the N3IWF.

-	the CERT payload containing the N3IWF's certificate if the CERTREQ payload is included in the IKE_AUTH request message.

Upon reception of the IKE_AUTH request message containing an HPA_INFO Notify Payload, as defined in 3GPP TS 24.302 [7], with the HPA_INFO field indicating a UE configured for MPS access, the N3IWF may, if allowed by operator policy, treat further messages for the UE with MPS priority. Unless doing so would cause network instability, the N3IWF should not reject requests from UEs which the N3IWF is treating with MPS priority access.

9.2.1.3.3	Test description

9.2.1.3.3.1	Pre-test conditions

System Simulator:

-	WLAN Cell 27

UE:

-	UE is configured for high priority access in the selected PLMN in the USIM according to Table 4.9.3.6-1 in TS 36.508 [18].

Preamble:

-	The UE is in state Switched OFF (state 0W-B) according to TS 38.508-1 [4].

9.2.1.3.3.2	Test procedure sequence

Table 9.2.1.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | THE UE associates with the WLAN AP and obtains the local IP address. | - | - | - | - |
| 3 | The UE performs a dynamic selection of N3IWF using DNS query. | - |  | - | - |
| 4 | The UE transmit an IKE_SA_INIT message to the N3IWF. | --> | IKE_SA_INIT Request | - | - |
| 5 | The SS transmits an IKE_SA_INIT message | <-- | IKE_SA_INIT Response | - | - |
| 6 | Check: Does the UE transmit an IKE_AUTH Request message including HPA_INFO Notify payload with MPS_PRI bit and AC_PRI set as 1 accordingly. | --> | IKE_AUTH Request | 1 | P |
| 7 | The SS transmits an IKE_AUTH Response message including an EAP-Request/5G-Start packet and the IDr payload with the value set to N3IWF identifier . | <-- | IKE_AUTH Response | - | - |
| 8-15 | The registration procedure is performed by Executing steps 3-10 of Table 4.5.2.2-3 in TS 38.508-1 [4] | - |  | - | - |

9.2.1.3.3.3	Specific message contents

FFS

==============End of change==============