3GPP TSG RAN meeting #58										                                     RP-12 1976

Barcelona, Spain 4th – 7th Dec, 2012

Title	[DRAFT] 	Letter to ITU-R WP5D [on Preliminary Draft New Report on THE USES OF IMT FOR BROADBAND PPDR APPLICATIONS

Response to:   	Invitation to external organizations to input material for development of Working document towards a Preliminary Draft New Report on THE USE OF IMT FOR BROADBAND PPDE APPLICATIONS [RP-121490]

Source:	Motorola Solutions,

To:	RAN

Cc:

Contact Person:

#### Name:	Edgar FERNANDES

Tel. Number:	+44 7785 245205

E-mail Address:	edgar.fernandes@motorolasolutions.com

For review:

In: RAN	>	feedback LS before:	 7 Dec 2012	to: SA: cc RAN4

In: SA 	>	feedback LS before:	 12 Dec 2012 	to: PCG cc: RAN / RAN4

This ITU input will become part of ITU deliverable(s):		Yes

Responsible 3GPP group for final output to ITU:			PCG

Deadline for the final output to ITU:								23 January 2013 (16:00 hours UTC)

way to make this document available for ITU (see Art.51 of 3GPP working procedures)

[  ]	a.	via OPs as a deliverable from their organizations (for PCG review only)

[x]	b.	via Individual Members (for PCG or TSG review) coordinated by ITU sector convener

[  ]	c.	via 3GPP LS coordinator (for TSG or WG review)

Attachment None

1. Overall Description:

3GPP TSG RAN and SA are kindly requested to review the proposed response and associated material to be attached, and to provide comment as appropriate to PCG

RAN should work with SA in responding to this liaison. RAN has a number of WI/SI related to PPDR which may need to be included in future revision of this response.  SA is also currently working on PPDR related projects including; Proximity Services (ProSe) and Group Communications system enablers for LTE (GCSE_LTE) in addition to others being considered including EPC-less E-UTRAN Operation for Public Safety (EEOp) and Public Safety and Critical Communications support (PubS).

PCG needs to approve the final contribution that will be submitted to ITU-R WP5D#15 via Individual members. Since there will be no face-to-face meeting of PCG before the deadline for the submission to ITU-R WP5D#15 it is suggested that the RAN Chairman coordinates the PCG approval by correspondence

----------------- [remove upper part before submission to ITU   -----------------

[ITU MEMBERS] *

Reply to ITU-R WP5D on its Liaison Statement on the Draft Outline of a Working Document towards a Preliminary  Draft New Report on The Use of International Mobile Telecommunications (IMT) for Broadband Public Protection and Disaster Relief (PPDR) Applications

1. Overall Description:

3GPP TSG RAN thanks ITU-R WP5D for the invitation to provide information on the use of IMT for Broadband Public Protection and Disaster Relief (PPDR) applications.

3GPP understands that WP5D has initiated the development of a PDNR (Preliminary Draft New Report) on the use of IMT for Broadband Public Protection and Disaster Relief (PPDR) applications and is seeking input from Standards Development Organization towards the further development of this document.

2.	Action:

3GPP proposes the revisions in the attached document to reflect the applicability of 3GPP technologies of IMT.

3GPP TSG RAN is looking forward to continuing fruitful cooperation with ITU-R WP5D.

3. Date of Next 3GPP TSG Meetings:

TSG-RAN4#66	     	28th January – 1st February 2013      				Malta, Republic of Malta

TSG-RAN#59 			26th February - 1st March 2013 		        			Vienna, Austria

TSG-RAN4#66BIS		15th April – 19th April 2013 		        				Chicago, USA

Attachment:

This contribution was developed in 3GPP TSG RAN. (Source: RP-xxxxx)

ATTACHMENT

Draft Outline of a Working Document toward a Preliminary Draft New Report on

The Use of International Mobile Telecommunications (IMT) for Broadband Public Protection and Disaster Relief (PPDR) Applications

(Question ITU-R 229-3/5)

# [Editor Note, 3GPP suggested text shown in light blue]

# 1	Scope

This Report offers a study of the applications of IMT technologies to PPDR using as a baseline the PPDR requirements as listed in Report ITU-R M.2033. The main focus is on the use of IMT technologies as a solution to meet the requirements of Public Safety agencies. [text to be further developed, as appropriate.]

# 2	Introduction

IMT specifications and related systems have on a global scale drastically changed the mobile communications landscape around the world, in particular for commercial markets, where the general consumer in many countries has gained broadband access to new advanced applications making use of information databases, media and general internet capabilities.

In the past public safety radio communications and other governmental mission critical requirements have been met by narrow band systems.  However, requirement for mobile radio communications of video and high bit rate data has increased considerably all over the world, with the results that the need for mobile broadband capabilities has become essential to public safety organizations to save lives and property. In addition governments responsibilities to ensure an effective protection of vital national infrastructure such as power plans (nuclear), railroads, the road network, gas, water and electricity supply, etcetera.

Therefore, it is useful to understand how IMT based specifications and standards can play a vital role by enabling professional broadband PPDR applications.

# 3	Background

# 4	IMT capabilities needed to meet broadband PPDR

# 5	Existing and planned capabilities of IMT that support broadband PPDR needs

5.1 Application of 3GPP IMT Technologies to PPDR

It is useful to correlate Report M.2033 to the 3GPP technology based system capabilities including those 3GPP technologies incorporated in the IMT-2000 and IMT-Advanced radio interfaces.

It should be noted that the 3GPP technologies are continually being enhanced.  New capabilities or enhancements to existing capabilities may be under consideration in 3GPP that could enhance the capability and / or performance of the 3GPP technology when utilized in a PPDR application

This response from 3GPP is based strictly on a review of Report ITU-R M.2033 and does not address any specific performance requirements or operational thresholds that might be announced by relevant PPDR entities

- The philosophies related to the general, technical, and operational capabilities indicated in Annex 1 of Report M.2033 are in general supported by the 3GPP technology based system capabilities.

- In relation to Section 1 of Annex 2 of Report M.2033 the same 3GPP technology based system capabilities are believed to satisfy the narrow band, wide band and broadband related communication requirements outline therein.

- In relation to Table 2 of Section 3 of Annex 2 of Report M.2033 3GPP provides information regarding the ability of the 3GPP technology to support the PPDR Applications and Examples contained therein. This information is provided as an extra column to Table 2 as presented below (extracted from M.2033). The term “supported” as utilized in the new column in the table (3GPP Technology Based System Capability) includes not only the inherent capabilities of the radio technology and related 3GPP system but also the ability to support a varied range of “applications” that could provide the indicated service/capability.  Some new capabilities may be under consideration for development in 3GPP and others may be the subject of work to improve or enhance the capability and/or performance.

- In relation to Table 3 of Section 3 of Annex 2 of Report M.2033 3GPP provides information regarding the ability of the 3GPP technology to support the User Requirements contained therein. This information is provided as an extra column to Table 3 as presented below (extracted from M.2033). Please note that the term “supported” as utilized in the new column in the table (3GPP Technology Based System Capability) includes not only the inherent capabilities of the radio technology and related 3GPP system but also the ability to support a varied range of “applications” that could provide the indicated service/capability.  Some new capabilities may be under consideration for development in 3GPP and others may be the subject of work to improve or enhance the capability and/or performance. “Supported” in this context does not imply any specific performance or conformance level.

Table 2 and Table 3 provides examples of LTE technology capabilities which can be used as a base to develop mission critical applications as currently supported by narrow band PPDR technologies and services.

Extract from Report ITU-R M.2033

### .......

### 3.1.3	Envisioned applications

Table 2 lists the envisioned applications with particular features and specific PPDR examples. The applications are grouped under the narrowband, wideband or broadband headings to indicate which technologies are most likely to be required to supply the particular application and their features. Furthermore, for each example, the importance (high, medium or low) of that particular application and feature to PPDR is indicated. This importance factor is listed for the three radio operating environments identified in Annex 2, § 2.1 “Day-to-day operations”, § 2.2 “Large emergency and/or public events”, and § 2.3 “Disasters”, represented by PP (1), PP (2) and DR, respectively.

TABLE 2

PPDR Applications and Examples

| Application | Feature | PPDR Example | Importance(1) |  |  | 3GPP Technology Based System Capability (See Note A, ,B) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | PP (1) | PP (2) | DR |  |
| 1. Narrowband |  |  |  |  |  |  |
| Voice | Person-to-person | Selective calling and addressing | H | H | H | Supported |
|  | One-to-many | Dispatch and group communication | H | H | H | Proposals being considered in 3GPP for this capability in LTE |
|  | Talk-around/direct mode | Groups of portable to portable (mobile-mobile) in close proximity without infrastructure | H | H | H | Proposals being considered in 3GPP for this capability in LTE |
|  | Push-to-talk | Push-to-talk | H | H | H | Proposals being considered in 3GPP for this capability in LTE |
|  | Instantaneous access to voice path | Push-to-talk and selective priority access | H | H | H |  |
|  | Security | Voice encryption/scrambling | H | H | M | Supported |
| Facsimile | Person-to-person | Status, short message | L | L | H | Supported |
|  | One-to-many (broadcasting) | Initial dispatch alert (e.g. address, incident status) | L | L | H | Supported |
| Messages | Person-to-person | Status, short message, short e-mail | H | H | H | Supported |
|  | One-to-many (broadcasting) | Initial dispatch alert (e.g. address, incident status) | H | H | H | Supported |
| Security | Priority/instantaneous access | Man down alarm button | H | H | H | Supported |
| Telemetry | Location status | GPS latitude and longitude information | H | M | H | Supported |
|  | Sensory data | Vehicle telemetry/status | H | H | M | Supported |
|  |  | EKG (electrocardiograph) in field | H | H | M | Supported |
| Database interaction (minimal record size) | Forms based records query | Accessing vehicle license records | H | H | M | Supported |
|  |  | Accessing criminal records/missing person | H | H | M | Supported |
|  | Forms based incident report | Filing field report | H | H | H | Supported |
| 2. Wideband |  |  |  |  |  |  |
| Messages | E-mail possibly with attachments | Routine e-mail message | M | M | L | Supported |
| Data Talk-around/direct mode operation | Direct unit to unit communication without additional infrastructure | Direct handset to handset, on-scene localized communications | H | H | H | Proposals being considered in 3GPP for this capability in LTE |
| Database interaction (medium record size) | Forms and records query | Accessing medical records | H | H | M | Supported |
|  |  | Lists of identified person/missing person | H | H | H | Supported |
|  |  | GIS (geographical information systems) | H | H | H | Supported |
| Text file transfer | Data transfer | Filing report from scene of incident | M | M | M | Supported |
|  |  | Records management system information on offenders | H | M | L | Supported |
|  |  | Downloading legislative information | M | M | L | Supported |
| Image transfer | Download/upload of compressed still images | Biometrics (finger prints) | H | H | M | Supported |
|  |  | ID picture | H | H | M | Supported |
|  |  | Building layout maps | H | H | H | Supported |
| Telemetry | Location status and sensory data | Vehicle status | H | H | H | Supported |
| Security | Priority access | Critical care | H | H | H | Supported |
| Video | Download/upload compressed video | Video clips | M | L | L | Supported |
|  |  | Patient monitoring (may require dedicated link) | M | M | M | Supported |
|  |  | Video feed of in-progress incident | H | H | M | Supported |
| Interactive | Location determination | 2-way system | H | H | M | Supported |
|  |  | Interactive location data | H | H | H | Supported |
| 3. Broadband |  |  |  |  |  |  |
| Database access | Intranet/Internet access | Accessing architectural plans of buildings, location of hazardous materials | H | H | H | Supported |
|  | Web browsing | Browsing directory of PPDR organization for phone number | M | M | L | Supported |
| Robotics control | Remote control of robotic devices | Bomb retrieval robots, imaging/video robots | H | H | M | Supported |
| Video | Video streaming, live video feed | Video communications from wireless clip-on cameras used by in building fire rescue | H | H | H | Supported |
|  |  | Image or video to assist remote medical support | H | H | H | Supported |
|  |  | Surveillance of incident scene by fixed or remote controlled robotic devices | H | H | M | Supported |
|  |  | Assessment of fire/flood scenes from airborne platforms | M | H | M | Supported |
|  |  | Assessment of fire/flood scenes from airborne platforms | M | H | M | Supported |
| Imagery | High resolution imagery | Downloading Earth exploration-satellite images | L | L | M | Supported |
|  |  | Real-time medical imaging | M | M | M | Supported |
| (1) The importance of that particular application and feature to PPDR is indicated as high (H), medium (M), or low (L). This importance factor is listed for the three radio operating environments: “Day-to-day operations”, “Large emergency and/or public events”, and “Disasters”, represented by PP (1), PP (2) and DR, respectively. |  |  |  |  |  |  |

Note A: The term “supported” as utilized in the new column in the table (3GPP Technology Based System Capability) includes not only the inherent capabilities of the radio technology and related 3GPP system but also the ability to support a varied range of “applications” that could provide the indicated service/capability.  Some new capabilities may be under consideration for development in 3GPP and others may be the subject of work to improve or enhance the capability and/or performance.

Note B: This response from 3GPP is based strictly on a review of Report ITU-R M.2033 and does not address any specific performance requirements or operational thresholds that might be announced by relevant PPDR entities.

TABLE 3

User requirements

| Requirement | Specifics | Importance(1) |  |  | 3GPP Technology Based System Capability (See Note A, ,B) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | PP (1) | PP (2) | DR |  |  |
| 1. System |  |  |  |  |  |  |
| Support of multiple applications |  | H | H | M | Supported |  |
| Simultaneous use of multiple applications | Integration of multiple applications (e.g. voice and low/medium speed data) | H | H | M | Supported |  |
|  | Integration of local voice, high speed data and video on high speed network to service localized areas with intensive on-scene activity | H | H | M | Supported |  |
| Priority access | Manage high priority and low priority traffic load shedding during high traffic | H | H | H | Supported |  |
|  | Accommodate increased traffic loading during major operations and emergencies | H | H | H |  |  |
|  | Exclusive use of frequencies or equivalent high priority access to other systems | H | H | H |  |  |
| Grade of service | Suitable grade of service | H | H | H | Supported |  |
|  | Quality of service | H | H | H | Supported |  |
|  | Reduced response times of accessing network and information directly at the scene of incidence, including fast subscriber/network authentication | H | H | H | Supported |  |
| Coverage | PPDR system should provide complete coverage within relevant jurisdiction and/or operation | H | H | M | Supported |  |
|  | Coverage of relevant jurisdiction and/or operation of PPDR organization whether at national, provincial/state or at local level | H | H | M | Supported |  |
|  | Systems designed for peak loads and wide fluctuations in use | H | H | M | Supported |  |
|  | Enhancing system capacity during PP emergency or DR by techniques such as reconfiguration of networks with intensive use of direct mode operation | H | H | H | Proposals being considered in 3GPP for this capability in LTE |  |
|  | Vehicular repeaters (NB, WB, BB) for coverage of localized areas | H | H | H | Supported |  |
|  | Reliable indoor/outdoor coverage | H | H | H | Supported |  |
|  | Coverage of remote areas, underground and inaccessible areas | H | H | H | Supported |  |
|  | Appropriate redundancy to continue operations, when equipment/infrastructure fails | H | H | H |  |  |
| Capabilities | Rapid dynamic reconfiguration of system | H | H | H |  |  |
|  | Control of communications including centralized dispatch, access control, dispatch (talk) group configuration, priority levels and pre-emption. | H | H | H |  |  |
|  | Robust OAM offering status and dynamic reconfiguration | H | H | H | Supported |  |
|  | Internet Protocol compatibility (complete system or interface with) | M | M | M | Supported |  |
|  | Robust equipment (hardware, software, operational and maintenance aspects) | H | H | H | Supported |  |
|  | Portable equipment (equipment that can transmit while in motion) | H | H | H | Supported |  |
|  | Equipment requiring special features such as high audio output, unique accessories (e.g. special microphones, operation while wearing gloves, operation in hostile environments and long battery life) | H | H | H | Supported |  |
|  | Fast call set-up and instant push-to-talk operation | H | H | H |  |  |
|  | Communications to aircraft and marine equipment, control of robotic devices | M | H | L | Supported |  |
|  | One touch broadcasting/group call | H | H | H | Proposals being considered in 3GPP for this capability in LTE |  |
|  | Terminal-to-terminal communications without infrastructure (e.g. direct mode operations/talk-around), vehicular repeaters | H | H | H | Proposals being considered in 3GPP for this capability in LTE |  |
|  | Appropriate levels of interconnection to public telecommunication network(s) | M | M | M | Supported |  |
| 2. Security | End-to-end encrypted communications for mobile-mobile, dispatch and/or group calls communications | H | H | L | Supported |  |
| 3. Cost related | Open standards | H | H | H | Supported |  |
|  | Cost effective solution and applications | H | H | H | Supported |  |
|  | Competitive marketplace | H | H | H | Supported |  |
|  | Reduction in deployment of permanent network infrastructure due to availability and commonality of equipment | H | H | L | Supported |  |
| 4. EMC | PPDR systems operation in accordance with national EMC regulations | H | H | H | Supported |  |
| 5. Operational |  |  |  |  |  |  |
| Scenario | Support operation of PPDR communications in any environment | H | H | H | Supported |  |
|  | Implementable by public and/or private operator for PPDR applications | H | H | M | Supported |  |
|  | Robust OAM offering status and dynamic reconfiguration | H | H | H | Supported |  |
|  | Rapid deployment of systems and equipment for large emergencies, public events and disasters (e.g. large fires, Olympics, peacekeeping) | H | H | H | Supported |  |
|  | Information to flow to/from units in the field to the operational control center and specialist knowledge centers | H | H | H | Supported |  |
|  | Greater safety of personnel through improved communications | H | H | H | Supported |  |
| Interoperability | Intra-system: Facilitate the use of common network channels and/or talkgroups | H | H | H | Proposals being considered in 3GPP for this capability in LTE |  |
|  | Inter-system: Promote and facilitate the options common between systems | H | H | H |  |  |
|  | Coordinate tactical communications between on-scene or incident commanders of the multiple PPDR agencies | H | H | H |  |  |
| 6. Spectrum usage and management | Share with other terrestrial mobile users | L | L | M |  |  |
|  | Suitable spectrum availability (NB, WB, BB channels) | H | H | H | Supported |  |
|  | Minimize interference to PPDR systems | H | H | H |  |  |
|  | Efficient use of spectrum | M | M | M | Supported |  |
|  | Appropriate channel spacing between mobile and base station frequencies | M | M | M | Supported |  |
| 7. Regulatory compliance | Comply with relevant national regulations | H | H | H | Supported |  |
|  | Coordination of frequencies in border areas | H | H | M | Supported |  |
|  | Provide capability of PPDR system to support extended coverage into neighbouring country (subject to agreements) | M | M | M | Supported |  |
|  | Ensure flexibility to use various types of systems in other Services (e.g. HF, satellites, amateur) at the scene of large emergency | M | H | H |  |  |
|  | Adherence to principles of the Tampere Convention | L | L | H | Supported |  |
| 8. Planning | Reduce reliance on dependencies (e.g. power supply, batteries, fuel, antennas, etc.) | H | H | H | Supported |  |
|  | As required, have readily available equipment (inventoried or through facilitation of greater quantities of equipment) | H | H | H | Supported |  |
|  | Provision to have national, state/provincial and local (e.g. municipal) systems | H | H | M | Supported |  |
|  | Pre-coordination and pre-planning activities (e.g. specific channels identified for use during disaster relief operation, not on a permanent, exclusive basis, but on a priority basis during periods of need) | H | H | H |  |  |
|  | Maintain accurate and detailed information so that PPDR users can access this information at the scene | M | M | M | Supported |  |
| (1) The importance of that particular application and feature to PPDR is indicated as high (H), medium (M), or low (L). This importance factor is listed for the three radio operating environments: “Day-to-day operations”, “Large emergency and/or public events”, and “Disasters”, represented by PP (1), PP (2) and DR, respectively. |  |  |  |  |  |

Note A: The term “supported” as utilized in the new column in the table (3GPP Technology Based System Capability) includes not only the inherent capabilities of the radio technology and related 3GPP system but also the ability to support a varied range of “applications” that could provide the indicated service/capability.  Some new capabilities may be under consideration for development in 3GPP and others may be the subject of work to improve or enhance the capability and/or performance.

Note B: This response from 3GPP is based strictly on a review of Report ITU-R M.2033 and does not address any specific performance requirements or operational thresholds that might be announced by relevant PPDR entities

# 5.2	Other capabilities

# 3GPP is also working on LTE technology support for Public Warning System (PWS).  This 3GPP capability has been developed to ensure that the public has the capability to receive timely and accurate alerts, warnings and critical information regarding disasters and other emergencies As has been learned from disasters such as earthquakes, tsunamis, hurricanes and wild fires; such a capability is essential to enable the public to take appropriate action to protect their families and themselves from serious injury, or loss of life or property.

3GPP is also in the process of incorporating a High Power UE (Band 14) capabilty in the E-UTRAN specifications.

3GPP E-UTRAN technology is specified to operate in a number of frequency bands, particularly in some of the bands identified by ITU–R for PPDR under resolution 646 (WRC-12).

# 6		Approaches for addressing broadband PPDR requirements using IMT

Many countries around the world have already decided for or are expected to announce the standardization of 3GPP LTE technology to build dedicated nationwide public safety mobile broadband networks. In particular announcements made in USA and Australia are supporting the use of 3GPP LTE as the technology of choice for dedicated broadband PPDR networks. LTE for PPDR is also under consultation in Canada. In Europe, the PPDR community through CEPT ECC FM49 has already decided as a working assumption that LTE is the broadband PPDR technology of choice.

The FCC in USA, which had been looking to establish a nationwide public safety mobile broadband network for many years, has now unanimously decided that Long Term Evolution (LTE) will be the one technology to apply in the realization of broadband public safety radio services in the 700 MHz band. 3GPP has now standardized this band as Band class 14.

These decisions to dedicate spectrum to deploy LTE for broad band Public Safety services in the US, Canada and Australia  in 700 and 800 MHz bands are now sparking the desire to open up activities in 3GPP to specify new advanced functionalities of LTE to support the special requirements public safety and disaster relief

# 7	Case studies of applications of IMT technologies to broadband PPDR

# 8	Summary and conclusions

# 9	Acronyms, abbreviations

[To be developed, as needed.]

# 10	References

[To be developed, as needed.]

3GPP document RP-121490 (TSG-RAN#58)