TSG-RAN Working Group 4 (Radio) meeting #67	RP-150150

Shanghai, China, 9 - 12 March, 2015

Source:	Ericsson

Title:	Motivation for WID on AWS-extension for LTE

Agenda item:	12.

Document for:	Discussion

# 1	Introduction

A SI on Advanced Wireless Services was specified in [1]. The output of this SI was expected to be a band plan considering technical impediments and interoperability. RAN4 concluded on the band plan at RAN4#73 and the SI was closed at RAN#66.

A WI is the next step in the 3GPP process.

# 2				Discussion

The spectrum 1755-1780 paired with 2155-2180MHz has currently been auctioned in USA. The figure below summarizes all the blocks within this spectrum. In addition, unpaired spectrum within 1695-1710MHz has been auctioned.

A SI [1] was approved to specify a band plan by taking these blocks, along with the 2180-2200 MHz block (AWS-4), into consideration. The outcome of the SI was as follows:

<From TR 36.849>

# 12	Conclusions

Various AWS extension candidate band plan options have been studied in the study item [2,3]:

- Option #1:  70+70 (1710 - 1780 MHz / 2110 - 2180 MHz, fixed duplex)

- Option #2:  70+90 (1710 - 1780 MHz / 2110 - 2200 MHz, variable duplex)

- Option #3:  85+90 (1695 - 1780 MHz / 2110 - 2200 MHz, variable duplex)

- Option #4:  70+90 (1710 - 1780 MHz / 2110 - 2200 MHz, fixed duplex with downlink intra-band CA across 90 MHz span)

It is concluded that Option 4 is selected.  If during the WI for option 4 it is concluded that interoperability across any part of the band cannot be achieved, then RAN4 agrees to address those interoperability issues.

Auctions were finalized on Jan. 29, 2015.   A WI can thus commence following the SI outcome. This is to specify a new operating band with UL 1710 - 1780 MHz and DL 2110 - 2200 MHz with fixed duplex spacing of 400MHz and support for 5, 10, 15 and 20MHz bandwidths. In addition, interoperability across the complete band (1710-1780/2110-2200MHz) by means of CA should be addressed.

The unpaired spectrum 1695-1710MHz remains outside this work item but we note that possible options for pairing will need to be considered in a separate SI/WI

# Conclusion

A WID offered in [2] proposes to define a new operating band 1710 - 1780 MHz / 2110 - 2200 MHz, with fixed duplex spacing. Interoperability across the band is expected to be ensured by means of CA as part of the WI phase.

# References

[1]	RP-141037, “Study on Advanced Wireless Services (AWS) - Extension band for LTE”, Qualcomm

[2]	RP-150148, “New WID on AWS extension”, Ericsson