3GPP TSG-RAN #67	RP-150399

Shanghai, China, March, 9-12, 2015

      - Agenda Item:	12.4.4

- Source:	CMCC

- Title:	TP for TR36.825 section 4.1

Document for:	Text Proposal

The proposed text proposal for TR36.825v0.0.1 section 4.1 (scenarios part) is as follows,

------------------------------Start Text Proposal---------------------------------

# 4	Deployment scenarios of possible additional TDD configuration(s)

## 4.1	Scenario(s)

<Editor: This section summarizes the intended deployment scenario(s) for the possible additional TDD configurations.>

The additional TDD configuration(s) should provide flexibilities for operators to deploy in different scenarios, the following scenarios are of interests from operators’ deployment perspective

- Operator deploys macro cell operating PCell on carrier frequency f0 and SCell on carrier frequency f1 (collocated). Both of the two carriers are aggregated in the UE. ([RP-151047, VF][RP-150185, CMCC][RP-150252, CATT])

- Operator deploys small cell (f1) overlaid with macro cell (f0) using a remote radio head (RRH), i.e., CA scenario #4 [TR 36.300v12.0.0]. Both of the two carriers are aggregated in the UE. ([RP-151047][RP-150208, DCM][RP-150185][RP-150140, NEC])

- Operator deploys small cell operating PCell on carrier frequency f0 and Scell on carrier frequency f1. Both of the two carriers are aggregated in the UE. ([RP-151047][RP-150185])

- Operator deploys macro cell operating PCell on carrier frequency f0 and SCell on carrier frequency f1 (non-collocated via RRH) . Both of the two carriers are aggregated in the UE. ([RP-151047, VF])

In all of the above cases, the Scell is operating the additional TDD configuration, and the Pcell is operating TDD or FDD.

Inter-band carrier aggregation is a pre-requisite for a UE to receive data from a non-standalone TDD carrier.

------------------------------End Text Proposal---------------------------------