3GPP TSG-RAN #67	RP-150403

Shanghai, China, March, 9-12, 2015

      - Agenda Item:	12.4.4

- Source:	CMCC

- Title:	TP for TR36.825 section 4.1

Document for:	Text Proposal

The proposed text proposal for TR36.825v0.0.1 section 4.1 (scenarios part) is as follows,

------------------------------Start Text Proposal---------------------------------

# 4	Deployment scenarios of possible additional TDD configuration(s)

## 4.1	Scenario(s)

<Editor: This section summarizes the intended deployment scenario(s) for the possible additional TDD configurations.>

The following scenarios are of interests for operators who want to deploy additional TDD configuration(s). In all of the following cases, the SCell is operating the additional TDD configuration(s), and the PCell is operating FDD or TDD using the existing TDD configuration(s).

- Scenario 1a: Operator deploys macro cell operating as PCell on carrier frequency f0 and macro cell operating as SCell on carrier frequency f1 (co-located), e.g. CA scenario 2 or 3 in TS36.300 v12.4.0. The macro cell on f1 uses the additional TDD configuration(s). Both of the two carriers can be aggregated by UE.

- Scenario 1b: Operator deploys macro cell operating as PCell on carrier frequency f0 and macro cell as SCell on carrier frequency f1 (non-collocated). Both of the two carriers can be aggregated by UE.

- Scenario 2: Operator deploys macro cell operating as PCell on carrier frequency f0 and small cell operating as SCell on carrier frequency f1 (non-co-located), e.g., CA scenario 4 in TS36.300 v12.4.0. Both of the two carriers can be aggregated by UE.

- Scenario 3: Operator deploys small cell operating as PCell on carrier frequency f0 and small cell as Scell on carrier frequency f1 (co-located). Both of the two carriers can be aggregated by UE.

Based on the current specifications,

inter-band carrier aggregation would be a pre-requisite for a UE to receive data on the SCell operating the additional TDD configuration(s) .

------------------------------End Text Proposal---------------------------------