3GPP TSG RAN Meeting #72								RP-161013

Busan, Korea, June 13 - 16, 2016

Agenda Item:		9.2.2

Source:			EBU, IRT, BBC, DISH, QUALCOMM

Title:				pCR to TR 38.913 on Multimedia Broadcast/Multicast Service

Document for:		Approval

### 1	Introduction

This pCR is based on TR 38.913 v0.3.0 as agreed by RAN #71 in RP-160689 and it proposing to add text to the so far empty section "9.1 Multimedia Broadcast/Multicast Service". Changes are shown with revision marks in section 2.

### 2	Text proposal to TR 38.913 v0.3.0

# 9	Supplementary-Service related requirements

## 9.1	Multimedia Broadcast/Multicast Service

The NR shall support deployment of an independent (standalone) multicast/broadcast radio network, such as TV (found in [12]) with no signaling or resource allocation limits.

The NR shall allow dedication of up to 100% of DL resournces for MBMS without control signaling restriction and also allow the user terminals (receivers) to comunicate via Non-MBMS carrier for reverse link if needed

The NR shall support concurrent delivery of both unicast service (e.g. eMBB) and broadcast/multicast to the users.

The NR shall make it possible to cover, to any required level of service, large geographical areas up to the size of an entire country in SFN mode, and will allow cell radii of up to 50km if required to facilitate that objective (see [12]).

The NR shall leverage its MIMO capabilities and apply them to the MBMS services to improve capacity and reliability.

-------------------------------------------------- END TEXT PROPOSAL -------------------------------------------------------