3GPP TSG RAN Meeting #72								RP-161242

Busan, Korea, June 13 - 16, 2016

Agenda Item:		9.2.2

Source:		EBU, IRT, BBC, Dish, Qualcomm, Fraunhofer IIS, Deutsche Telekom, Telstra,    	AT&T

Title:				pCR to TR 38.913 on Multimedia Broadcast/Multicast Service

Document for:		Approval

### 1	Introduction

This pCR is based on TR 38.913 v0.3.0 as agreed by RAN #71 in RP-160689 and it proposing to add text to the so far empty section "9.1 Multimedia Broadcast/Multicast Service". Changes are shown with revision marks in section 2.

### 2	Text proposal to TR 38.913 v0.3.0

# 9	Supplementary-Service related requirements

## 9.1	Multimedia Broadcast/Multicast Service

The new RAT shall support existing Multicast/Broadcast services (e.g. download, streaming, group communication, TV, etc.) and new services (e.g. V2X, etc).

The new RAT shall support dynamic adjustment of the Multicast/Broadcast area based on e.g. the user distribution or service requirements.

The new RAT shall support concurrent delivery of both unicast and Multicast/Broadcast services to the users.

The new RAT shall support efficient multiplexing with unicast transmissions in at least frequency domain and time domain.

The new RAT shall support static and dynamic resource allocation between Multicast/Broadcast and unicast; the new RAT shall in particular allow support of up to 100% of DL resources for Multicast/Broadcast.

The new RAT shall support operation of a dedicated carrier MBMS for downlink only broadcasting.

The new RAT shall make it possible to cover large geographical areas up to the size of an entire country in SFN mode with network synchronisation and shall allow cell radii of up to 100 km if required to facilitate that objective. It shall also support local, regional and national broadcast areas.

The new RAT shall support Multicast/Broadcast services for fixed, portable and mobile UEs. Mobility up to 250 km/h shall be supported.

The new RAT shall leverage usage of RAN equipement (hard- and software) including e.g. multi-antenna capabilities (e.g. MIMO) to improve Multicast/Broadcast capacity and reliability.

The new RAT shall support Multicast/Broadcast services for mMTC devices.

-------------------------------------------------- END TEXT PROPOSAL -------------------------------------------------------