3GPP TSG-RAN4 Meeting #80	R4-166812

# Gothenburg, Sweden, 22nd Aug 2016 - 26th Aug 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 3974 | rev | - | Current version: | 14.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on eD2D RRM tests |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eD2D_Prox-Perf |  |  |  |  |  | Date: |  |  | 2016-08-11 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Introduction of eD2D RRM tests |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change 1: Resource pool configurations for Discovery Change 2: (Test 1) Interruptions due to ProSe Direct Discovery with discovery period less than 320ms Change 3: (Test 2) Interruptions with discovery with PCell + SCell, with discovery on PCell Change 4: (Test 3) Cell reselection and timing accuracy for ProSe Direct Discovery transmission on non-serving frequency Change 5: (Test 4) Interruptions due to ProSe Direct Discovery reception on non-serving frequency Change 6: (Test 5) Interruptions due to ProSe Direct Discovery transmission on non-serving frequency Change 7: (Test 6) Interruptions due to ProSe Direct Communication on non-serving frequency Change 8: (Test 7) Selection / Reselection of ProSe relay UE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRM core requirements for eD2D will not be verified |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | Y |  | Test specifications |  |  |  | TS 36.521-3 |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

< START OF CHANGE #1 >

### A.3.12.4	Reference resource pool configurations for ProSe Direct Discovery

Table A.3.12.4-1: ProSe Direct Discovery configuration for E-UTRA FDD (Configuration #1)

| Information Element |  |  |  | Value |
| --- | --- | --- | --- | --- |
| discRxPool | cp-Len |  |  | Normal |
|  | discPeriod |  |  | rf32 |
|  | numRetx |  |  | 0 |
|  | numRepetition |  |  | 1 |
|  | tf-ResourceConfig | prb-Num |  | 12 |
|  |  | prb-Start |  | 0 |
|  |  | prb-End |  | 23 |
|  |  | offsetIndicator |  | 160 |
|  |  | subframeBitmap |  | 11000000 00000000 00000000 00000000 00000000 |
|  | txParameters |  |  | not present |
|  | rxParameters |  |  | not present |
| discTxPoolCommon | cp-Len |  |  | Normal |
|  | discPeriod |  |  | rf32 |
|  | numRetx |  |  | 0 |
|  | numRepetition |  |  | 1 |
|  | tf-ResourceConfig | prb-Num |  | 2 |
|  |  | prb-Start |  | 3 |
|  |  | prb-End |  | 5 |
|  |  | offsetIndicator |  | 160 |
|  |  | subframeBitmap |  | 10000000 00000000 00000000 00000000 00000000 |
|  | txParameters | txParametersGeneral | alpha | al0 |
|  |  |  | p0 | 31 |
|  |  | ue-SelectedResourceConfig | poolSelection | random |
|  |  |  | txProbability | p100 |
|  | rxParameters |  |  | not present |
| discTxPowerInfo | discMaxTxPower |  |  | 23 |
| SL-SyncConfig | syncCP-Len |  |  | Normal |
|  | syncOffsetIndicator |  |  | 155 |
|  | slssid |  |  | 30 |
|  | txParameters | txParametersGeneral | alpha | al0 |
|  |  |  | p0 | 31 |
|  |  | syncTxThreshIC |  | 0 (-infinity) |
|  | rxParamsNCell |  |  | not present |
| discInterFreqList |  |  |  | not present |

Table A.3.12.4-2: ProSe Direct Discovery configuration for E-UTRA FDD (Configuration #2)

| Information Element |  |  |  | Value |
| --- | --- | --- | --- | --- |
| discRxPool | cp-Len |  |  | Normal |
|  | discPeriod |  |  | rf32 |
|  | numRetx |  |  | 0 |
|  | numRepetition |  |  | 1 |
|  | tf-ResourceConfig | prb-Num |  | 12 |
|  |  | prb-Start |  | 0 |
|  |  | prb-End |  | 23 |
|  |  | offsetIndicator |  | 160 |
|  |  | subframeBitmap |  | 11000000 00000000 00000000 00000000 00000000 |
|  | txParameters |  |  | not present |
|  | rxParameters | tdd-Config |  | not present |
|  |  | syncConfigIndex |  | 0 |
| discTxPoolCommon |  |  |  | not present |
| discTxPowerInfo | discMaxTxPower |  |  | 23 |
| SL-SyncConfig | syncCP-Len |  |  | Normal |
|  | syncOffsetIndicator |  |  | 140 |
|  | slssid |  |  | 30 |
|  | txParameters |  |  | not present |
|  | rxParamsNCell | physCellId |  | 1 |
|  |  | discSyncWindow |  | w1 |
| discInterFreqLis |  |  |  | not present |

Table A.3.12.4-3: ProSe Direct Discovery configuration for E-UTRA TDD Config 0 (Configuration #3)

| Information Element |  |  |  | Value |
| --- | --- | --- | --- | --- |
| discRxPool | cp-Len |  |  | Normal |
|  | discPeriod |  |  | rf32 |
|  | numRetx |  |  | 0 |
|  | numRepetition |  |  | 1 |
|  | tf-ResourceConfig | prb-Num |  | 12 |
|  |  | prb-Start |  | 0 |
|  |  | prb-End |  | 23 |
|  |  | offsetIndicator |  | 163 |
|  |  | subframeBitmap |  | 11000000 00000000 00000000 00000000 00000000 00 |
|  | txParameters |  |  | not present |
|  | rxParameters |  |  | not present |
| discTxPoolCommon | cp-Len |  |  | Normal |
|  | discPeriod |  |  | rf32 |
|  | numRetx |  |  | 0 |
|  | numRepetition |  |  | 1 |
|  | tf-ResourceConfig | prb-Num |  | 2 |
|  |  | prb-Start |  | 3 |
|  |  | prb-End |  | 5 |
|  |  | offsetIndicator |  | 163 |
|  |  | subframeBitmap |  | 10000000 00000000 00000000 00000000 00000000 00 |
|  | txParameters | txParametersGeneral | alpha | al0 |
|  |  |  | p0 | 31 |
|  |  | ue-SelectedResourceConfig | poolSelection | random |
|  |  |  | txProbability | p100 |
|  | rxParameters |  |  | not present |
| discTxPowerInfo | discMaxTxPower |  |  | 23 |
| SL-SyncConfig | syncCP-Len |  |  | Normal |
|  | syncOffsetIndicator |  |  | 158 |
|  | slssid |  |  | 30 |
|  | txParameters | txParametersGeneral | alpha | al0 |
|  |  |  | p0 | 31 |
|  |  | syncTxThreshIC |  | 0 (-infinity) |
|  | rxParamsNCell |  |  | not present |
| discInterFreqList |  |  |  | not present |

Table A.3.12.4-4: ProSe Direct Discovery configuration for E-UTRA FDD for PS discovery (Configuration #4)

| Information Element |  |  |  | Value |
| --- | --- | --- | --- | --- |
| discConfig |  |  |  | not present |
| discInterFreqList |  |  |  | not present |
| discConfig-v1310 |  |  |  | not present |
| discConfigPS | discRxPoolPS | cp-Len |  | Normal |
|  |  | discPeriod-v1310 |  | rf4 |
|  |  | numRetx |  | 0 |
|  |  | numRepetition |  | 1 |
|  |  | tf-ResourceConfig | prb-Num | 12 |
|  |  |  | prb-Start | 0 |
|  |  |  | prb-End | 23 |
|  |  |  | offsetIndicator | 20 |
|  |  |  | subframeBitmap | 11000000 00000000 00000000 00000000 00000000 |
|  |  | txParameters |  | not present |
|  |  | rxParameters | tdd-Config | not present |
|  |  |  | syncConfigIndex | 0 |
|  | discTxPoolPS-Common/ discTxPoolPS-Dedicated |  |  | not present |
| discConfigRelay | relayUE-Config | threshHigh |  | not present |
|  |  | threshLow |  | not present |
|  |  | hystMax |  | not present |
|  |  | hystMin |  | not present |
|  | remoteUE-Config | threshHigh |  | 0 (-130dBm) |
|  |  | hystMax |  | dB0 |
|  |  | reselectionInfoIC | q-RxLevMin | (-94dBm) |
|  |  |  | filterCoefficient | fc0 |
|  |  |  | minHyst | dB3 |

Table A.3.12.4-5: ProSe Direct Discovery configuration for E-UTRA FDD for inter-frequency discovery (Configuration #5)

| Information Element |  |  | Value |
| --- | --- | --- | --- |
| discConfig |  |  | not present |
| discInterFreqList | carrierFreq |  | Note 1 |
|  | plmn-IdentityList |  | not present |
| discConfig-v1310 |  |  |  |
| discInterFreqList-v1310 |  |  |  |
| discResourcesNonPS | discRxResourcesInterFreq | cp-Len | Normal |
|  |  | discPeriod | rf32 |
|  |  | numRetx | 0 |
|  |  | numRepetition | 1 |
|  |  | tf-ResourceConfig |  |
|  |  | prb-Num | 12 |
|  |  | prb-Start | 0 |
|  |  | prb-End | 23 |
|  |  | offsetIndicator | 160 |
|  |  | subframeBitmap | 11000000 00000000 00000000 00000000 00000000 |
|  |  | txParameters | not present |
|  |  | rxParameters | not present |
|  |  | rxParamsAddNeighFreq |  |
|  |  | physCellId | Note 1 |
|  | discTxResourcesInterFreq | discTxPoolCommon |  |
|  |  | cp-Len | Normal |
|  |  | discPeriod | rf32 |
|  |  | numRetx | 0 |
|  |  | numRepetition | 1 |
|  |  | tf-ResourceConfig |  |
|  |  | prb-Num | 2 |
|  |  | prb-Start | 3 |
|  |  | prb-End | 5 |
|  |  | offsetIndicator | 160 |
|  |  | subframeBitmap | 10000000 00000000 00000000 00000000 00000000 |
|  |  | txParameters |  |
|  |  | txParametersGeneral |  |
|  |  | alpha | al0 |
|  |  | p0 | 31 |
|  |  | ue-SelectedResourceConfig |  |
|  |  | poolSelection |  |
|  |  | txProbability | p100 |
|  |  | rxParameters | not present |
|  |  | rxParamsAddNeighFreq | not present |
|  |  | txParamsAddNeighFreq |  |
|  |  | physCellId | Note 1 |
|  |  | p-Max | 23 |
|  |  | tdd-Config | not present |
|  |  | tdd-Config-v1130 | not present |
|  |  | freqInfo |  |
|  |  | ul-CarrierFreq | Note 1 |
|  |  | ul-Bandwidth | n25 |
|  |  | additionalSpectrumEmission | NS_01 |
|  |  | referenceSignalPower | Note 2 |
|  |  | syncConfigIndex | not present |
| discResourcesPS |  |  | not present |
| discConfigOther | txPowerInfo | discMaxTxPower | 23 |
|  | refCarrierCommon |  | not present |
|  | discSyncConfig |  | not present |
|  | discCellSelectionInfo | q-RxLevMin | -70 (-140dBm) |
|  |  | q-RxLevMinOffset | not present |
|  |  | q-Hyst | dB0 |
|  |  | q-RxLevMinReselection | -70 (-140dBm) |
|  |  | t-ReselectionEUTRA | 0 |
| gapRequestsAllowedCommon |  |  | True |
| discConfigRelay |  |  | not present |
| discConfigPS |  |  | not present |
| NOTE 1: As specified in the test NOTE 2: As configured by the test system |  |  |  |

< END OF CHANGE #1 >

< START OF CHANGE #2 >

### A.7.5.6	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery with discovery period less than 320ms

#### A.7.5.6.1	Test Purpose and Environment

The purpose of this test is to verify the requirements related to the allowed PCell interruptions due to ProSe Direct Discovery defined in clause 7.16.3.3 when the discovery period less than 320ms. In the test the UE under test is configured only with PCell with the ProSe operation on uplink carrier of the PCell.

This test is applicable to ProSe Direct Discovery capable UEs that support discovery periods of less than 320ms.

For this test, the UE is triggered by the test loop function or the upper layers to monitor ProSe Direct Discovery.

The test parameters are given in Table A.7.5.6.1-1, Table A.7.5.6.1-2, and Table A.7.5.6.1-3 below. There is one active cell (PCell) in this test and 24 active Sidelink transmissions in this test (with 12 active Sidelink UEs per configured discovery subframe).

The tests consist of three successive time periods, with time duration of T1, T2 and T3 respectively.

During T1, the UE is in RRC_IDLE and monitoring the ProSe Direct Discovery announcements from other active Sidelink UEs on the ProSe Direct Discovery resources.

During T2, the test system establishes a RRC connection with the UE. No PDSCH traffic is scheduled for UE during T2, and the UE is expected to transmit SidelinkUEInformation indicating discRxInterest during T2. On reception of SidelinkUEInformation, the test system shall RRC reconfiguration message to the UE and wait for the UE to respond with RRC reconfiguration complete message before transitioning to T3. If the UE does not transmit SidelinkUEInformation for up to [2] sec, the test system shall transition to T3.

During T3, the UE is scheduled with PDSCH traffic on PCell downlink. The test system will count the missed ACK/NACKs during T3 to verify the allowed interruptions during ProSe Direct Discovery.

Table A.7.5.6.1-1: Test parameters for interruption due to ProSe Direct Discovery test with discovery period less than 320ms for E-UTRAN FDD

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 |  |
| Channel Bandwidth (BWchannel) | MHz | 5 |  |
| Active cell |  | Cell 1 | E-UTRA FDD Cell1 on RF channel number 1 |
| CP length of Cell 1 |  | Normal |  |
| T1 | s | 0.64 |  |
| T2 | s | Up to receiving RRC reconfiguration setup complete from the UE, or up to [2] sec if UE does not transmit SidelinkUEInformation during this period. |  |
| T3 | s | 1.28 |  |

Table A.7.5.6.1-2: ProSe Direct Discovery configuration for interruption test due to ProSe Direct Discovery with discovery period less than 320ms for E-UTRAN FDD

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | UL carrier frequency |
| Channel Bandwidth (BWchannel) | MHz | 5 |  |
| ProSe Direct Discovery resource pool configuration |  | As specified in Table A.3.12.4-4 (Configuration #4) | IE values unless specified otherwise in this test. |
| Active Sidelink UEs Configuration |  | PDP.1.FDD As specified in Table A.3.12.8.2-1 | Transmitting ProSe Direct Discovery |

Table A.7.5.6.1-3: Cell specific test parameters for interruption due to ProSe Direct Discovery test with discovery period less than 320ms for E-UTRAN FDD

| Parameter |  | Unit | Cell 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  |  | 1 |  |  |
| BWchannel |  | MHz | 5 |  |  |
| UE RRC state |  |  | IDLE | CONNECTED |  |
| Paging configuration | defaultPagingCycle |  | rf256 | N/A |  |
|  | nB |  | T / 32 |  |  |
| DRX |  |  | N/A | OFF |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1Note1 |  |  | N/A | None | R.5 FDD |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1Note1 |  |  | R.11 FDD |  |  |
| OCNG Pattern |  |  | OP.16 FDD |  | OP.15 FDD |
| PBCH_RA |  | dB | 0 |  |  |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |  |
| Note2 |  | dBm/15 kHz | -98 |  |  |
|  |  | dB | 16 |  |  |
| RSRP Note3 |  | dBm/15 kHz | -82 |  |  |
| SCH_RP Note 3 |  | dBm/15 kHz | -82 |  |  |
| Propagation Condition |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

#### A.7.5.6.2	Test Requirements

The UE shall be scheduled on PCell continuously during T3.

The test system shall verify that at least 98.75% of all expected ACK/NACKs during T3 shall be transmitted by the ProSe UE. The missed ACK/NACKs can occur only on subframe ‘n’, if either n±1 subframe is a discovery subframe, or if n-3, or n-5 is a discovery subframe.

NOTE:	For the test configuration in Table A.7.5.6.1-1 and Table A.7.5.6.1-2, the specific subframes where missed ACK/NACKs are allowed are when (subframe mod 40) = 19, 23, 22, 26, corresponding to allowed interruptions on subframe (subframe mod 40) = 19 and 22.

< END OF CHANGE #2 >

< START OF CHANGE #3 >

### A.7.5.7	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery

#### A.7.5.7.1	Test Purpose and Environment

The purpose of this test is to verify the requirements related to the allowed PCell interruptions due to ProSe Direct Discovery defined in clause 7.16.3.1 and clause 7.16.3.3. In the test the UE under test is configured with PCell and one SCell, with ProSe operation configured on the PCell.

A UE that meets the requirements of this clause is not required to be tested for the requirements of clause 7.5.3.

For this test, the UE is triggered by the test loop function or the upper layers to monitor ProSe Direct Discovery.

The test parameters are given in Table A.7.5.7.1-1, Table A.7.5.7.1-2, and Table A.7.5.7.1-3 below. There are two configured component carriers: PCC and SCC, and two active cells: Cell 1 and Cell2. Cell1 is the PCell on PCC, and Cell 2 is the SCell on SCC.  Sidelink operation is configured on the PCC UL. There are 24 active Sidelink transmissions in this test (with 12 active Sidelink UEs per configured discovery subframe). Two tests (Test 1 and Test 2) are defined to verify interruptions due to synchronous (Test 1) and asynchronous (Test 2) ProSe Direct Discovery.

The tests consist of three successive time periods, with time duration of T1, T2 and T3 respectively.

During T1, the UE is in RRC_IDLE and monitoring the ProSe Direct Discovery announcements from other active Sidelink UEs on the ProSe Direct Discovery resources.

During T2, the test system establishes a RRC connection with the UE. The test system shall configure the UE with the SCC. No PDSCH traffic is scheduled for UE during T2, and the UE is expected to transmit SidelinkUEInformation indicating discRxInterest during T2. On reception of SidelinkUEInformation, the test system shall RRC reconfiguration message to the UE and wait for the UE to respond with RRC reconfiguration complete message before transitioning to T3. If the UE does not transmit SidelinkUEInformation for up to [2] sec, the test system shall transition to T3.

During T3, the UE is scheduled with PDSCH traffic on both PCell and SCell downlink. The test system will count the missed ACK/NACKs during T3 to verify the allowed interruptions during ProSe Direct Discovery.

Table A.7.5.7.1-1: Test parameters for interruption due to ProSe Direct Discovery tests

| Parameter | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |  |
| E-UTRA RF Channel Number |  | 1 |  |  |
| Channel Bandwidth (BWchannel) | MHz | 5 |  |  |
| Active cell |  | Cell 1, Cell 2 |  | E-UTRA FDD Cell1 and Cell 2 on RF channel number 1 and 2, respectively |
| CP length of Cell 1, Cell 2 |  | Normal |  |  |
| T1 | s | 5.12 |  |  |
| T2 | s | Up to receiving RRC reconfiguration setup complete from the UE, or up to [2] sec if UE does not transmit SidelinkUEInformation during this period. |  |  |
| T3 | s | 10.24 |  |  |

Table A.7.5.7.1-2: ProSe Direct Discovery configuration for interruption due to ProSe Direct Discovery tests

| Parameter | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |  |
| E-UTRA RF Channel Number |  | 1 |  | UL carrier frequency |
| Channel Bandwidth (BWchannel) | MHz | 5 |  |  |
| ProSe Direct Discovery resource pool configuration |  | As specified in Table A.3.12.4-1 (Configuration #1) | As specified in Table A.3.12.4-2 (Configuration #2) | IE values unless specified otherwise in this test. |
| Active Sidelink UEs Configuration |  | PDP.1.FDD As specified in Table A.3.12.8.2-1 | PDP.2.FDD As specified in Table A.3.12.8.2-1 | Transmitting ProSe Direct Discovery (Test 1 and 2) and SLSS (for Test 2) |

Table A.7.5.7.1-3: Cell specific test parameters for interruption due to ProSe Direct Discovery tests

| Parameter |  | Unit | Cell 1 |  |  |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |  | T1 | T2 |  | T3 |
| E-UTRA RF Channel Number |  |  | 1 |  |  |  | 2 |  |  |  |
| BWchannel |  | MHz | 5 |  |  |  | 5 |  |  |  |
| UE RRC state |  |  | IDLE | CONNECTED |  |  | N/A |  |  |  |
| Paging configuration | defaultPagingCycle |  | rf256 | N/A |  |  | N/A |  |  |  |
|  | nB |  | T / 32 |  |  |  |  |  |  |  |
| DRX |  |  | N/A | OFF |  |  | N/A | OFF |  |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1Note1 |  |  | N/A | None | R.5 FDD |  | N/A | None |  | R.5 FDD |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1Note1 |  |  | R.11 FDD |  |  |  | R.11 FDD |  |  |  |
| OCNG Pattern |  |  | OP.16 FDD |  |  | OP.15 FDD | OP.16 FDD |  | OP.15 FDD |  |
| PBCH_RA |  | dB | 0 |  |  |  | 0 |  |  |  |
| PBCH_RB |  |  |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15 kHz | -98 |  |  |  | -98 |  |  |  |
|  |  | dB | 16 |  |  |  | 16 |  |  |  |
| RSRP Note3 |  | dBm/15 kHz | -82 |  |  |  | -82 |  |  |  |
| SCH_RP Note 3 |  | dBm/15 kHz | -82 |  |  |  | -82 |  |  |  |
| Propagation Condition |  |  | AWGN |  |  |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |

#### A.7.5.7.2	Test Requirements

The UE shall be scheduled on PCell and SCell downlink continuously during T3.

In Test 1, at least 98.75% of all expected ACK/NACKs during T3 shall be transmitted by the ProSe UE. The missed ACK/NACKs can occur only on subframe ‘n’, if either n±1 subframe is a discovery subframe, or if n-3, or n-5 is a discovery subframe.

NOTE:	For the test configuration in Table A.7.5.12.1-1 and Table A.7.5.12.1-2, the specific subframes where missed ACK/NACKs are allowed are when (subframe mod 320) = 159, 163, 162, 166, corresponding to allowed interruptions on subframe 159 and 162.

In Test 2, at least 97.5% of all expected ACK/NACKs during T3 shall be transmitted by the ProSe UE. The missed ACK/NACK can occur only on subframe ‘n’, if either n±5 subframe is a discovery or SLSS subframe, or if n+1, or n-9 is a discovery or SLSS subframe.

NOTE:	For the test configuration in Table A.7.5.7.1-1 and Table A.7.5.7.1-2, the specific subframes where missed ACK/NACKs are allowed are when (subframe mod 320) = 135, 139, 145, 149, 155, 159, 166, 170, corresponding to allowed interruptions on subframes 135, 145, 155 and 166.

< END OF CHANGE #3 >

< START OF CHANGE #4 >

### A.7.5.8	E-UTRAN FDD-FDD - Cell reselection and timing accuracy for ProSe Direct Discovery transmission on non-serving frequency

#### A.7.5.8.1	Test Purpose and Environment

The purpose of this test is to verify the requirements related to cell reselection and timing accuracy for ProSe Direct Discovery transmissions on a non-serving frequency defined in clauses 7.16.4 and 7.16.2.1.2, respectively. In the test the UE under test is configured with PCell on a serving frequency, and the PCell provides the ProSe Direct Discovery resources for a non-serving frequency.

This test is applicable for ProSe Direct Discovery capable UEs that support concurrent inter-band E-UTRAN and E-UTRAN ProSe operation, and indicates the support of inter-frequency discovery transmission using discInterFreqTx.

For this test, the UE is triggered by the test loop function or the upper layers to announce ProSe Direct Discovery.

The test parameters are given in Table A.7.5.8.1-1, Table A.7.5.8.1-2, and Table A.7.5.8.1-3 below. The test consists of one active serving cell (cell 1) on the serving RF channel 1, and two active non-serving cells (cell 2 and cell3) on the non-serving discovery RF channel 2.

The tests consist of three successive time periods, with time duration of T1, T2 and T3 respectively.

The serving cell (cell 1 on RF channel 1) is active during the entire test duration (T1, T2, T3) without any changes to cell 1 RSRP. Prior to start of the test, the test system shall verify that the UE is transmitting ProSe Direct Discovery transmissions on the non-serving RF channel 2.

During T1, only one non-serving cell (cell 2 on RF channel 2) is active. The UE is expected to be following the timing of cell 2 for its discovery transmissions on RF channel 2. During T2, cell 3 on the non-serving RF channel 2 is also turned ON and is configured to be better ranked than cell 2. The UE is expected to reselect to cell 3 for discovery transmit timing. During T3, RSRP of cell 2 is increased so that it becomes better ranked than cell 3. The UE is supposed to reselect back to cell 2 and follow its timing for discovery transmissions.

Table A.7.5.8.1-1: Test parameters for cell reselection and timing accuracy for ProSe Direct Discovery transmission on non-serving frequency test for E-UTRAN FDD-FDD

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2 | RF channel 1 is serving RF channel 2 is non-serving |
| Channel Bandwidth (BWchannel) | MHz | 5 |  |
| Active cell |  | Cell 1 | Serving E-UTRA FDD cell on RF channel number 1 |
| Non-serving cell |  | Cell 2, Cell 3 | Non-serving E-UTRA FDD cells on RF channel number 2 |
| CP length of Cell 1, 2, 3 |  | Normal |  |
| T1 | s | 2 |  |
| T2 | s | 14 |  |
| T3 | s | 5 |  |

Table A.7.5.8.1-2: ProSe Direct Discovery configuration for cell reselection and timing accuracy for ProSe Direct Discovery transmission on non-serving frequency test for E-UTRAN FDD-FDD

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 2 | UL carrier frequency |
| Channel Bandwidth (BWchannel) | MHz | 5 |  |
| ProSe Direct Discovery resource pool configuration |  | As specified in Table A.3.12.4-5 (Configuration #5) | IE values unless specified otherwise in this test. Broadcasted in SIB19 by Cell 1 on RF channel 1 |
| discInterFreqList/ carrierFreq |  | RF channel 2 (UL frequency) |  |
| freqInfo/ul-CarrierFreq |  | RF channel 2 (UL frequency) |  |
| rxParamsAddNeighFreq/physCellId |  | {phyCellId of Cell 2, phyCellId of Cell 3} |  |
| txParamsAddNeighFreq/physCellId |  | {phyCellId of Cell 2, phyCellId of Cell 3} |  |

Table A.7.5.8.1-3: Cell specific test parameters for cell reselection and timing accuracy for ProSe Direct Discovery transmission on non-serving frequency test for E-UTRAN FDD-FDD

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |  | Cell 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |  | T1 |  | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  |  | 2 |  |  |  |
| Serving / Non-serving |  | Serving |  |  | Non-serving |  |  |  | Non-serving |  |  |  |
| BWchannel | MHz | 5 |  |  | 5 |  |  |  | 5 |  |  |  |
| drx-Configuration |  | DRX_P1 As specified in Table A.3.12.2-1 |  |  | N/A |  |  |  | N/A |  |  |  |
| OCNG Patterns defined in A.3.2.1.2 |  | OP.16 FDD |  |  | OP.19 FDD |  |  |  | OP.19 FDD |  |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |  | 0 |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |
| Timing offset | us | 0 |  |  | 0 |  |  |  | 2 |  |  |  |
| Note 2 | dBm/15 kHz | -98 |  |  |  |  |  |  |  |  |  |  |
| RSRP Note 3 | dBm/15 kHz | -82 |  |  | -82 | -85 |  | -82 | -infinity | -82 |  | -85 |
|  | dB | 16 |  |  | 16 | -3.11 |  | 2.79 | -infinity | 2.79 |  | -3.11 |
|  | dB | 16 |  |  | 16 | 13 |  | 16 | -infinity | 13 |  | 16 |
| SCH_RP Note 3 | dBm/15kHz | -82 |  |  | -82 | -85 |  | -82 | -infinity | -82 |  | -85 |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |  |  |

#### A.7.5.8.2	Test Requirements

During T1, the test system shall verify that the transmit timing offset of discovery transmission on RF channel 2 is within ± 12×TS with respect to the first detected path (in time) of the corresponding downlink frame of cell 2.

During T2, the UE is expected to reselect to cell 3 for discovery timing synchronization after cell reselection delay to a newly detectable cell from start of T2. After the period of cell reselection delay to a newly detectable cell from the start of T2, the test system shall verify that the transmit timing offset of discovery transmission on RF channel 2 are within ± 12×TS with respect to the first detected path (in time) of the corresponding downlink frame of cell 3.

The cell reselection delay to a newly detectable cell for discovery transmission on non-serving carrier shall be 10.56s.

NOTE: The cell reselection delay to a newly detectable cell for discovery transmission on non-serving carrier can be expressed as (Tdetect,EUTRAN_ProSe_Intra + 1 discovery period).

During T3, the UE is expected to reselect back to cell 2 for discovery timing synchronization after cell reselection delay to an already detected cell from start of T3. After the period of cell reselection delay to a newly detectable cell from the start of T3, the test system shall verify that the transmit timing offset of discovery transmission on RF channel 2 are within ± 12×TS with respect to the first detected path (in time) of the corresponding downlink frame of cell 2.

The cell reselection delay to an already detected cell for discovery transmission on non-serving carrier shall be 5.44s.

NOTE: The cell reselection delay to an already detected cell for discovery transmission on non-serving carrier can be expressed as (Tevaluate, E-UTRAN_ProSe_Intra + 1 discovery period).

The rate of correct events observed during repeated tests shall be at least 90%.

< END OF CHANGE #4 >

< START OF CHANGE #5 >

### A.7.5.9	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery reception on non-serving frequency

#### A.7.5.9.1	Test Purpose and Environment

The purpose of this test is to verify the requirements related to interruptions due to ProSe Direct Discovery reception on a non-serving frequency as defined in clause 7.16.3.3 and 7.16.3.4. In the test the UE under test is configured with PCell on a serving frequency, and the PCell provides the ProSe Direct Discovery resources for a non-serving frequency.

This test is applicable for ProSe Direct Discovery capable UEs that support concurrent inter-band E-UTRAN and E-UTRAN ProSe operation.

For this test, the UE is triggered by the test loop function or the upper layers to monitor ProSe Direct Discovery.

The test parameters are given in Table A.7.5.9.1-1, Table A.7.5.9.1-2, and Table A.7.5.9.1-3 below. The test consists of one active serving cell (cell 1) on the serving RF channel 1, and one active non-serving cells (cell 2) on the non-serving discovery RF channel 2. There are 96 active Sidelink transmissions in this test (with 12 active Sidelink UEs per configured discovery subframe) on RF channel 2.

After the test system establishes a RRC connection with the UE, the UE is expected to transmit SidelinkUEInformation indicating discRxInterest. On reception of SidelinkUEInformation, the test system shall RRC reconfiguration message to the UE and wait for the UE to respond with RRC reconfiguration complete message before start of the test. Further, depending on UE implementation, the UE may request for discovery reception gaps (using discRxGapReq) for the ProSe Direct Discovery reception operation on the non-serving frequency. If gaps are requested, the test system shall configure the gaps as requested and modify the PDSCH scheduling on cell 1 for this UE such that the UE is not scheduled on the DL on the subframes configured as reception gaps.

The test shall start after the completion of the RRC reconfiguration is complete following the SidelinkUEInformation message transmission from the UE. The test system shall then continuously schedule the UE on DL of cell 1 (apart from any subframes that are configured as discovery gaps).

Table A.7.5.9.1-1: Test parameters for interruptions due to ProSe Direct Discovery reception on non-serving frequency test for E-UTRAN FDD-FDD

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2 | RF channel 1 is serving RF channel 2 is non-serving |
| Channel Bandwidth (BWchannel) | MHz | 5 |  |
| Active cell |  | Cell 1 | Serving E-UTRA FDD cell on RF channel number 1 |
| Non-serving cell |  | Cell 2 | Non-serving E-UTRA FDD cell on RF channel number 2 |
| CP length of Cell 1, 2 |  | Normal |  |

Table A.7.5.9.1-2: ProSe Direct Discovery configuration for interruptions due to ProSe Direct Discovery reception on non-serving frequency test for E-UTRAN FDD-FDD

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 2 | UL carrier frequency |
| Channel Bandwidth (BWchannel) | MHz | 5 |  |
| ProSe Direct Discovery resource pool configuration |  | As specified in Table A.3.12.4-5 (Configuration #5) | IE values unless specified otherwise in this test. Broadcasted in SIB19 by Cell 1 on RF channel 1 |
| discInterFreqList/carrierFreq |  | RF channel 2 (UL frequency) |  |
| freqInfo/ul-CarrierFreq |  | RF channel 2 (UL frequency) |  |
| rxParamsAddNeighFreq/physCellId |  | {phyCellId of Cell 2} |  |
| txParamsAddNeighFreq/physCellId |  | {phyCellId of Cell 2} |  |
| discResourcesNonPS/ discRxResourcesInterFreq/ tf-ResourceConfig/ subframeBitmap |  | 11111111 00000000 00000000 00000000 00000000 |  |
| Active Sidelink UEs Configuration |  | PDP.1.FDD As specified in Table A.3.12.8.2-1 | Transmitting ProSe Direct Discovery on RF channel 2 |

Table A.7.5.9.1-3: Cell specific test parameters for interruptions due to ProSe Direct Discovery reception on non-serving frequency test for E-UTRAN FDD-FDD

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | 2 |
| Serving/Non-serving |  | Serving | Non-serving |
| BWchannel | MHz | 5 | 5 |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low | 1x2 Low |
| drx-Configuration |  | None | N/A |
| PDCCH/PCFICH/PHICH Reference measurement channelNote1 |  | R.11 FDD | R.11 FDD |
| PDSCH Reference measurement channel defined in A.3.1.1.1Note1 |  | R.7 FDD (NOTE 5 applies) | N/A |
| OCNG Pattern defined in A.3.2.1 |  | OP.20 FDD | OP.19 FDD |
| PCFICH_RB | dB | 0 | 0 |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PBCH_RA | dB |  |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote1 | dB |  |  |
| OCNG_RBNote1 | dB |  |  |
|  | dB | 16 | 16 |
|  | dBm/15 kHz | -98 | -98 |
| RSRP Note4 | dBm/15 kHz | -82 | -82 |
| SCH_RP Note 4 | dBm/15 kHz | -82 | -82 |
| Propagation condition |  | AWGN | AWGN |
| Note 1: For the reference measurement channels, see clause A.3.1. Note 2: For the OCNG pattern, see clause A.3.2. Note 3: OCNG shall be used such that cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: Scheduling pattern for PDSCH modified to omit the subframes configured as Discovery Gaps in the test. |  |  |  |

#### A.7.5.9.2	Test Requirements

The test system shall verity the allowed interruptions for ProSe Direct Discovery reception on non-serving frequency, and depends on the discovery gap configuration for the UE.

If no discovery gaps are configured, the test system shall verify that the total number of missed ACK/NACKs on the serving cell are less than 0.5%.

If discovery gaps are configured as requested by the UE, then test system shall verify that the missed ACK/NACKs, if any, correspond to locations as specified in subclause 7.16.3.4:

- Missed ACK/NACKs is allowed on a subframe n, if subframe n is configured as downlink reception gap (using discRxGapConfig) and either the subframe immediately preceding or immediately following that subframe is not configured as reception gap, and

- One additional missed ACK/NACK per discovery period is allowed on a subframe m, such that the subframe m is configured as reception gap.

< END OF CHANGE #5 >

< START OF CHANGE #6 >

### A.7.5.10	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery transmission on non-serving frequency

#### A.7.5.10.1	Test Purpose and Environment

The purpose of this test is to verify the requirements related to interruptions due to ProSe Direct Discovery reception on a non-serving frequency as defined in clause 7.16.3.3 and 7.16.3.4. In the test the UE under test is configured with PCell on a serving frequency, and the PCell provides the ProSe Direct Discovery resources for a non-serving frequency.

This test is applicable for ProSe Direct Discovery capable UEs that support concurrent inter-band E-UTRAN and E-UTRAN ProSe operation, and indicates the support of inter-frequency discovery transmission using discInterFreqTx.

For this test, the UE is triggered by the test loop function or the upper layers to announce ProSe Direct Discovery.

The test parameters are given in Table A.7.5.10.1-1, Table A.7.5.10.1-2, and Table A.7.5.10.1-3 below. The test consists of one active serving cell (cell 1) on the serving RF channel 1, and one active non-serving cells (cell 2) on the non-serving discovery RF channel 2.

After the test system establishes a RRC connection with the UE, the UE is expected to transmit SidelinkUEInformation. On reception of SidelinkUEInformation, the test system shall RRC reconfiguration message to the UE and wait for the UE to respond with RRC reconfiguration complete message before start of the test. Further, depending on UE implementation, the UE may request for discovery transmission and/or reception gaps (using discTxGapReq and/or discRxGapReq) for the ProSe Direct Discovery transmission operation on the non-serving frequency. If transmission gap is requested, the test system shall configure the transmission gap as requested. The test system shall not configure any reception gap for the UE for the purpose of this test.

The test shall start after the completion of the RRC reconfiguration is complete following the SidelinkUEInformation message transmission from the UE. The test system shall then continuously schedule the UE on DL of cell 1.

Table A.7.5.10.1-1: Test parameters for interruptions due to ProSe Direct Discovery transmission on non-serving frequency test for E-UTRAN FDD-FDD

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2 | RF channel 1 is serving RF channel 2 is non-serving |
| Channel Bandwidth (BWchannel) | MHz | 5 |  |
| Active cell |  | Cell 1 | Serving E-UTRA FDD cell on RF channel number 1 |
| Non-serving cell |  | Cell 2 | Non-serving E-UTRA FDD cell on RF channel number 2 |
| CP length of Cell 1, 2 |  | Normal |  |

Table A.7.5.10.1-2: ProSe Direct Discovery configuration for interruptions due to ProSe Direct Discovery transmission on non-serving frequency test for E-UTRAN FDD-FDD

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 2 | UL carrier frequency |
| Channel Bandwidth (BWchannel) | MHz | 5 |  |
| ProSe Direct Discovery resource pool configuration |  | As specified in Table A.3.12.4-5 (Configuration #5) | IE values unless specified otherwise in this test. Broadcasted in SIB19 by Cell 1 on RF channel 1 |
| discInterFreqList/carrierFreq |  | RF channel 2 (UL frequency) |  |
| freqInfo/ul-CarrierFreq |  | RF channel 2 (UL frequency) |  |
| txParamsAddNeighFreq/physCellId |  | {phyCellId of Cell 2} |  |
| discConfigOther/ refCarrierCommon |  | pCell | Configure UE to use pCell as reference for synchronization and measurements for discovery transmission on non-serving carrier |
| discResourcesNonPS/ discRxResourcesInterFreq |  | Not configured |  |
| discResourcesNonPS/ discTxResourcesInterFreq/ tf-ResourceConfig/ subframeBitmap |  | 11111111 00000000 00000000 00000000 00000000 |  |
| discRxGapConfig |  | Not configured | Within SL-DiscConfig in RRC reconfiguration message |
| discTxGapConfig |  | Configured as requested by UE in discTxGapReq | Within SL-DiscConfig in RRC connection reconfiguration message |

Table A.7.5.10.1-3: Cell specific test parameters for interruptions due to ProSe Direct Discovery transmission on non-serving frequency test for E-UTRAN FDD-FDD

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | 2 |
| Serving/Non-serving |  | Serving | Non-serving |
| BWchannel | MHz | 5 | 5 |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low | 1x2 Low |
| drx-Configuration |  | None | N/A |
| PDCCH/PCFICH/PHICH Reference measurement channelNote1 |  | R.11 FDD | R.11 FDD |
| PDSCH Reference measurement channel defined in A.3.1.1.1Note1 |  | R.7 FDD | N/A |
| OCNG Pattern defined in A.3.2.1 |  | OP.20 FDD | OP.19 FDD |
| PCFICH_RB | dB | 0 | 0 |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PBCH_RA | dB |  |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote1 | dB |  |  |
| OCNG_RBNote1 | dB |  |  |
|  | dB | 16 | 16 |
|  | dBm/15 kHz | -98 | -98 |
| RSRP Note4 | dBm/15 kHz | -82 | -82 |
| SCH_RP Note 4 | dBm/15 kHz | -82 | -82 |
| Propagation condition |  | AWGN | AWGN |
| Note 1: For the reference measurement channels, see clause A.3.1. Note 2: For the OCNG pattern, see clause A.3.2. Note 3: OCNG shall be used such that cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |

#### A.7.5.10.2	Test Requirements

The test system shall verity the allowed interruptions for ProSe Direct Discovery trannsmission on non-serving frequency.

If no discovery transmission gaps are configured, the test system shall verify that the total number of missed ACK/NACKs on the serving cell are less than 0.5%.

If discovery transmission gaps are configured as requested by the UE,  the test system shall verify that the number of missed ACK/NACKs are less than or equal to 5 missed ACK/NACKs during a discovery period (configured as 320ms in this test). Corresponding to discovery transmission on s subframe n (with respect to PCell) on the non-serving carrier, the missed ACK/NACKs are allowed only on subframes (n-1), n, (n+1), (n+3), (n+5).

< END OF CHANGE #6 >

< START OF CHANGE #7 >

### A.7.5.11	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Communication on non-serving frequency

#### A.7.5.11.1	Test Purpose and Environment

The purpose of this test is to verify the requirements related to interruptions due to ProSe Direct Communication on a non-serving frequency as defined in clause 7.16.3.5. In the test the UE under test is configured with PCell on a serving frequency, and is pre-configured with ProSe Direct Communication resources for a non-serving frequency.

This test is applicable for ProSe Direct Communication capable UEs that support concurrent inter-band E-UTRAN and E-UTRAN ProSe operation.

For this test, the UE is triggered by the test loop function or the upper layers to receive ProSe Direct Communication.

The test parameters are given in Table A.7.5.11.1-1, Table A.7.5.11.1-2, Table A.7.5.11.1-3 and Table A.7.5.11.1-4 below. The test consists of one active serving cell (cell 1) on the serving RF channel 1, and there are no active cells on RF channel 2. On RF channel 2, the test consists of one active SyncRef UE (SyncRef UE 1) transmitting synchronization signals and channels, and 12 (5MHz) or 16 (10 MHz) active Sidelink UEs in this test transmitting ProSe Direct Communication.

The serving cell (cell 1) on RF channel 1 is not broadcasting SIB18, and the UE is expected to use its preconfigured parameters for ProSe Direct Communication operation on RF channel 2.

The UE is continuously scheduled with PDSCH traffic on PCell downlink in RF channel 1.

Table A.7.5.11.1-1: Test parameters for interruptions due to ProSe Direct Communication on non-serving frequency test for E-UTRAN FDD-FDD

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2 | RF channel 1 is serving RF channel 2 is non-serving |
| Channel Bandwidth (BWchannel) | MHz | 5 or 10 | According to principle defined in clause A.3.12.3 |
| Active cell |  | Cell 1 | Serving E-UTRA FDD cell on RF channel number 1 (Not broadcasting SIB18) |
| Active SyncRef UEs |  | SyncRef UE 1 | Transmitting SLSS+MIB-SL on uplink of RF channel number 2 |
| CP length of Cell 1 |  | Normal |  |

Table A.7.5.11.1-2: ProSe Direct Communication configuration for interruptions due to ProSe Direct Communication on non-serving frequency test for E-UTRAN FDD-FDD

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 2 | UL carrier frequency |
| Channel Bandwidth (BWchannel) | MHz | 5 or 10 | According to principle defined in clause A.3.12.3 |
| ProSe Direct Communication resource pool configuration |  | As specified in Table A.3.12.5-2 (Configuration #2) | IE values unless specified otherwise in this test. (Preconfigured) |
| Active Sidelink UEs ConfigurationNote 1 |  | PCP.1.FDD As specified in Table A.3.12.8.1-1 | Transmitting ProSe Direct Communication (PSCCH + PSSCH) Using SyncRef UE1 as synchronization source |
| Note 1: This test is according to the principle defined in section A.3.12.3. |  |  |  |

Table A.7.5.11.1-3: SyncRef UE specific test parameters for interruptions due to ProSe Direct Communication on non-serving frequency test for E-UTRAN FDD-FDD

| Parameter | Unit | SyncRef UE 1 |
| --- | --- | --- |
| E-UTRA RF Channel Number |  | 2 |
| BWchannelNote 4 | MHz | 5 or 10 |
| ProSe Direct Communication resource pool configuration |  | As specified in Table A.3.12.5-1 (Configuration #1) Note resource pool is same as Configuration #2 used by ProSe UE. |
| syncOffsetIndicator |  | Set same as syncOffsetIndicator1 in ProSe Direct Communication preconfiguration |
| slssid |  | 30 |
| inCoverage |  | TRUE |
| networkControlledSyncTx |  | ON |
| Note1 | dBm/15 kHz | -98 |
|  | dB | 16 |
| S-RSRP Note2, Note3 | dBm/15 kHz | -82 |
| Propagation Condition |  | AWGN |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SSSS Es/Iot is set the same as PSSS/PSBCH Es/Iot. Note 4: This test is according to the principle defined in section A.3.12.3. |  |  |

Table A.7.5.11.1-4: Cell specific test parameters for interruptions due to ProSe Direct Communication on non-serving frequency test for E-UTRAN FDD-FDD

| Parameter | Unit | Cell 1 |
| --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 |
| Serving/Non-serving |  | Serving |
| BWchannel Note 5 | MHz | 5 or 10 |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |
| drx-Configuration |  | None |
| PDCCH/PCFICH/PHICH Reference measurement channelNote1, Note 5 |  | 5 MHz: R.11 FDD 10 MHz: R.6 FDD |
| PDSCH Reference measurement channel defined in A.3.1.1.1Note1, Note 5 |  | 5MHz: R.7 FDD 10MHz: R.3 FDD |
| OCNG Pattern defined in A.3.2.1 |  | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD |
| PCFICH_RB | dB | 0 |
| PDCCH_RA | dB |  |
| PDCCH_RB | dB |  |
| PBCH_RA | dB |  |
| PBCH_RB | dB |  |
| PSS_RA | dB |  |
| SSS_RA | dB |  |
| PHICH_RA | dB |  |
| PHICH_RB | dB |  |
| PDSCH_RA | dB |  |
| PDSCH_RB | dB |  |
| OCNG_RANote1 | dB |  |
| OCNG_RBNote1 | dB |  |
|  | dB | 16 |
|  | dBm/15 kHz | -98 |
| RSRP Note4 | dBm/15 kHz | -82 |
| SCH_RP Note 4 | dBm/15 kHz | -82 |
| Propagation condition |  | AWGN |
| Note 1: For the reference measurement channels, see clause A.3.1. Note 2: For the OCNG pattern, see clause A.3.2. Note 3: OCNG shall be used such that cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: This test is according to the principle defined in section A.3.12.3. |  |  |

#### A.7.5.11.2	Test Requirements

The test system shall verify that the total number of missed ACK/NACKs on the serving cell on RF channel 1 are less than 0.5%.

< END OF CHANGE #7 >

< START OF CHANGE #8 >

### A.7.5.12	E-UTRAN FDD - Selection / Reselection of ProSe relay UE

#### A.7.5.12.1	Test Purpose and Environment

The purpose of this test is to verify the requirements related to selection / reselection of ProSe relay UE defined in clauses 7.16.5. In the test the UE under test is configured with PCell and is configured with ProSe Direct Discovery and ProSe Direct Communication resources as required for remote UE operation.

This test is applicable to UEs capable of ProSe Direct Discovery and ProSe Direct Communication, and further support the optional feature of sidelink remote UE operation.

The test parameters are given in Table A.7.5.12.1-1, Table A.7.5.12.1-2, Table A.7.5.12.1-3, Table A.7.5.12.1-4, and Table A.7.5.12.1-5 below. The test consists of one active serving cell (cell 1), and two active Sidelink relay UEs (Sidelink Relay UE 1, Sidelink Relay UE 2). The Sidelink relay UEs are configured to be transmitting relay discovery messages every discovery period.

The test system shall ensure that the remote UE under test has transmitted SidelinkUEInformation message and has been configured with the ProSe Direct Discovery resources for relay operation prior to the start of the test.

The tests consist of five successive time periods, with time duration of T1, T2, T3, and T4 respectively.

During T1, RSRP of cell 1 is kept higher than threshHigh (within remoteUE-Config), and the remote UE is not required to perform relay UE selection.

During T2, RSRP of cell 1 is configured to be lower than threshHigh. The UE is expected to start looking for relay UE and request the serving cell for ProSe Direct Communication resources for communicating with a candidate Sidelink Relay UE. The test system shall ensure that the UE under test transmits the SidelinkUEInformation message (requesting the ProSe Direct Communication resources) and has been configured the resource pool prior to end of T2 duration. During T2, the SD-RSRP of Sidelink Relay UE 1 and Sidelink Relay UE 2 is configured to be lower than the detection threshold and no relay UE will be available for the remote UE under test.

During T3, the SD-RSRP of Sidelink Relay UE 1 is raised above the detection threshold and the UE is expected to perform relay selection to Sidelink Relay 1. The test system can determine that the remote UE has selected a relay by monitoring the configured ProSe Direct Communication resource for the direct communication setup message to the relay UE.

During T4, the UE is expected to complete the direct communication setup with the relay UE. Note that the RSRP of the serving cell (cell 1) and the SD-RSRP of sidelink relay UEs is kept unchanged during T3. The period T3 ends when Sidelink Relay UE1 sends the direct communication accept message back to the remote UE.

During T5, SD-RSRP of Sidelink Relay UEs are modified such that the remote UE is expected to reselect to Sidelink Relay UE2.

Table A.7.5.12.1-1: Test parameters for selection / reselection of ProSe relay UE test for E-UTRAN FDD

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 |  |
| Channel Bandwidth (BWchannel) | MHz | 5 |  |
| Active cell |  | Cell 1 | Serving E-UTRA FDD cell on RF channel number 1 (Broadcasting SIB18 and SIB19) |
| Active Relay UEs |  | Relay UE 1, Relay UE 2 | Transmitting relay discovery message |
| CP length of Cell 1 |  | Normal |  |
| T1 | ms | 100 |  |
| T2 | s | Up to receiving RRC reconfiguration setup complete from the UE, or up to [1] sec if UE does not transmit SidelinkUEInformation during this period. |  |
| T3 | s | 1 |  |
| T4 | s | [2] |  |
| T5 | s | 1 |  |

Table A.7.5.12.1-2: ProSe Direct Discovery configuration for selection / reselection of ProSe relay UE test for E-UTRAN FDD

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | UL carrier frequency |
| Channel Bandwidth (BWchannel) | MHz | 5 |  |
| ProSe Direct Discovery resource pool configuration |  | As specified in Table A.3.12.4-4 (Configuration #4) | IE values unless specified otherwise in this test. |
| discRxPoolPS/numRetx |  | 3 |  |
| discRxPoolPS/ tf-ResourceConfig/ offsetIndicator |  | 0 | To align with communications resource pool offset |
| discRxPoolPS/ tf-ResourceConfig/ subframeBitmap |  | 00111100 00000000 00000000 00000000 00000000 | To avoid collision with communication resource pool, and allow for 4 retransmissions |
| remoteUE-Config/threshHigh |  | -91dBm |  |
| Note 1: This test is according to the principle defined in section A.3.12.3. |  |  |  |

Table A.7.5.12.1-3: ProSe Direct Communication configuration for selection / reselection of ProSe relay UE test for E-UTRAN FDD

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | UL carrier frequency |
| Channel Bandwidth (BWchannel) | MHz | 5 |  |
| ProSe Direct Communication resource pool configuration |  | As specified in Table A.3.12.5-1 (Configuration #1) | IE values unless specified otherwise in this test. |
| commRxPool/ sc-TFResourceConfig/ subframeBitmap |  | 00000011 00000000 00000000 00000000 00000000 | To avoid collision with discovery resource pool |
| commTxPoolNormalCommon/ sc-TFResourceConfig/ subframeBitmap |  | 00000011 00000000 00000000 00000000 00000000 |  |
| commTxAllowRelayCommon |  | true | Configured in SIB18 |
| Note 1: This test is according to the principle defined in section A.3.12.3. |  |  |  |

Table A.7.5.12.1-4: Sidelink Relay UE specific test parameters for selection / reselection of ProSe relay UE test for E-UTRAN FDD

| Parameter | Unit | Relay UE 1 |  |  |  |  | Relay UE 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 | T1 | T2 | T3 | T4 | T5 |
| E-UTRA RF Channel Number |  | 1 (UL frequency) |  |  |  |  |  |  |  |  |  |
| BWchannel Note 4 | MHz | 5 |  |  |  |  |  |  |  |  |  |
| ProSe Direct Discovery resource pool configuration |  | As specified in Table A.7.5.12.1-2 |  |  |  |  |  |  |  |  |  |
| Transmission frequency |  | Every discovery period (40ms) with 3 retransmissions per period |  |  |  |  |  |  |  |  |  |
| Resource allocation |  | Non-overlapping PRBs |  |  |  |  |  |  |  |  |  |
| Note1 | dBm/15 kHz | -97 |  |  |  |  |  |  |  |  |  |
|  | dB | -inf | -inf | 10.5 | 10.5 | -1.5 | -inf | -inf | -inf | -inf | 10.5 |
| SD-RSRP Note2, Note 3 | dBm/15 kHz | -inf | -inf | -86.5 | -86.5 | -98.5 | -inf | -inf | -inf | -inf | -86.5 |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |  |  |  |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |  |

Table A.7.5.12.1-5: Cell specific test parameters for interruptions due to ProSe Direct Communication on non-serving frequency test for E-UTRAN FDD-FDD

| Parameter | Unit | Cell 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  |
| Serving/Non-serving |  | Serving |  |  |  |  |
| BWchannel Note 5 | MHz | 5 |  |  |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  |  |  |
| drx-Configuration |  | DRX_P1 As specified in Table A.3.12.2-1 |  |  |  |  |
| PDCCH/PCFICH/PHICH Reference measurement channelNote1, Note 5 |  | R.11 FDD |  |  |  |  |
| OCNG Pattern defined in A.3.2.1 |  | OP.16 FDD |  |  |  |  |
| PCFICH_RB | dB | 0 |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |
| PBCH_RA | dB |  |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |
| OCNG_RANote1 | dB |  |  |  |  |  |
| OCNG_RBNote1 | dB |  |  |  |  |  |
|  | dBm/15 kHz | -97 |  |  |  |  |
|  | dB | 10.5 | 1.5 | 1.5 | 1.5 | 1.5 |
| RSRP Note4 | dBm/15 kHz | -86.5 | -95.5 | -95.5 | -95.5 | -95.5 |
| Propagation condition |  | AWGN |  |  |  |  |
| Note 1: For the reference measurement channels, see clause A.3.1. Note 2: For the OCNG pattern, see clause A.3.2. Note 3: OCNG shall be used such that cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: This test is according to the principle defined in section A.3.12.3. |  |  |  |  |  |  |

#### A.7.5.12.2	Test Requirements

Sidelink relay UE selection delay is defined as the time from the beginning of time period T3 to the moment when the UE selects the Sidelink Relay UE1 and transmits the PC5-SP direct communication setup message using ProSe Direct Communications.

The test system shall verify that the sidelink relay UE selection delay is less than 680ms.

NOTE: The sidelink relay UE selection delay can be expressed as (Tevaluate, ProSe_Relay_intra + 1 sc-period).

Sidelink relay UE reselection time is defined as the time from the beginning of time period T5 to the moment when the UE reselects to Sidelink relay UE2 and transmits the direct communication setup message using ProSe Direct Communications.

The test system shall verify that the sidelink relay UE reselection delay is less than 800ms.

NOTE: The sidelink relay UE reselection delay can be expressed as (Tmeasure,ProSe_Relay_Intra + Tevaluate, ProSe_Relay_intra + 1 sc-period).

< END OF CHANGE #8 >