3GPP TSG RAN Meeting #75	RP-170388

# Dubrovnik, Croatia, Mar 6 - 9, 2017

# Source:	ZTE, ZTE Microelectronics.

# Title:	Considerations on LTE-NR uplink sharing

# Document for:	Discussion

Agenda Item:	9.1

## 1	Introduction

Recently LTE-NR uplink sharing has been discussed in the scope of LTE-NR coexistence, where an NR uplink shares the same frequency resources as an LTE uplink, provided that LTE and NR base stations are collocated [1] [2]. Furthermore some specific band combinations for LTE uplink and NR downlink were proposed, e.g., band 28 (700MHz) LTE uplink and C-band (3.5GHz) NR downlink [3].

In this document, we discuss some potential issues with LTE-NR uplink sharing, and propose that 3GPP may consider such deployment only after these issues and challenges are fully addressed and enough overall benefits are justified.

## 2	Discussion

### 2.1	Overview

One of fundamental principles in cellular networks is the balanced uplink and downlink coverage. This is required by the bi-directional nature of telecommunications:

- Most services provided by cellular networks are bi-directional, e.g., voice call, video conference etc.

- The data transmission service provided by cellular networks is guaranteed with certain quality measures. And this is ensured by transmission in one direction and acknowledgement in the other direction from technical prospective.

This requires that uplink and downlink coverage need to be carefully calculated and designed by considering practical situation and implementation impairment etc. In any wireless cellular communication system, in order to make sure that a UE can successfully receive the signal transmitted from the base station, while at the same time, the base station can successfully receive the signal transmitted from the UE. Differences between the transceivers at BS and UE side and frequencies for both downlink and uplink should be taken into account:

- BS side:

  - Higher transmit power

    - Shared by multiple UEs

    - Shared by necessary common downlink channels

  - Multiple antennas

    - Higher transmit antenna gain

    - Higher receive antenna gain

  - Higher minimum guaranteed downlink data rate

- UE side

  - Lower transmit power

  - Less antennas

  - Lower minimum guaranteed uplink data rate

- Downlink and uplink frequencies

  - In TDD, downlink and uplink have the same frequency

  - In FDD, the frequency of downlink is chosen in the same band as uplink, therefore the statistical properties of path-loss are similar for both downlink and uplink as shown in Table 4.1 in [8]

Table – 4.1 [8] Simulation frequencies for FDD mode E-UTRA frequency bands

| E-UTRA Band | UL frequencies (MHz) |  | DL frequencies (MHz) |  | Simulation frequency (MHz) | Path loss difference (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | lowest | highest | lowest | highest |  | lowest UL frequency | highest DL frequency |
| 1 | 1920 | 1980 | 2110 | 2170 | 2000 | 0.37 | 0.74 |
| 2 | 1850 | 1910 | 1930 | 1990 | 2000 | 0.71 | 0.05 |
| 3 | 1710 | 1785 | 1805 | 1880 | 2000 | 1.43 | 0.56 |
| 4 | 1710 | 1755 | 2110 | 2155 | 2000 | 1.43 | 0.68 |
| 5 | 824 | 849 | 869 | 894 | 900 | 0.80 | 0.06 |
| 6 | 830 | 840 | 875 | 885 | 900 | 0.74 | 0.15 |
| 7 | 2500 | 2570 | 2620 | 2690 | 2000 | 2.04 | 2.70 |
| 8 | 880 | 915 | 925 | 960 | 900 | 0.20 | 0.59 |
| 9 | 1749.9 | 1784.9 | 1844.9 | 1879.9 | 2000 | 1.22 | 0.56 |
| 10 | 1710 | 1770 | 2110 | 2170 | 2000 | 1.43 | 0.74 |
| 11 | 1427.9 | 1452.9 | 1475.9 | 1500.9 | 2000 | 3.07 | 2.62 |

Therefore we may ensure balanced downlink and uplink coverage by designing a balanced link budget for downlink and uplink because of their similar large scale statistical properties of radio propagation.

As one example, Figure 1 shows the downlink and uplink coverage in LTE for different scenarios [4], and similar observations for different systems can also be found in [5][6].

Figure 1. LTE coverage for different scenarios [4]

### 2.2	Unbalanced uplink and downlink coverage in LTE-NR uplink sharing

In LTE-NR uplink sharing, NR uplink is carried on an LTE uplink of much lower frequency than that of NR downlink. It means that for the same maximum allowed pathloss in downlink and uplink, the achieved downlink and uplink coverage could differ a lot due to the different attenuation as shown in Figure 2, which shows the coverage against maximum allowed pathloss according to the macro rural model specified in TR36.942 [8].

Figure 2. Coverage vs maximum allowed pathloss

Figure 3. Unbalanced uplink and downlink coverage in LTE-NR uplink sharing

Consequently this may lead to an inherent issue that the NR uplink coverage is not balanced with NR downlink coverage, as illustrated in Figure 3. In Figure 3, for UE #1 which is located within the LTE uplink coverage but not in NR downlink coverage, even it can transmit NR uplink signal via LTE uplink frequency and base station can successfully receive the signal, but the UE cannot receive NR downlink signal correctly, therefore LTE-NR uplink sharing is not feasible. Only UEs within both LTE uplink coverage and NR downlink coverage can operate LTE-NR uplink sharing, e.g., UE #2 in the figure. So the claim is actually not valid that LTE-NR uplink sharing can enhance NR uplink coverage.

Furthermore, if the band combination of LTE uplink and NR downlink is dynamically configurable and flexible, it becomes very complicated for the network planning and operation, and we can anticipate the huge potential challenges. The inherent issue and the challenges should be addressed.

### 2.3	Standardization consideration

Since NR is also OFDM based as that in LTE in spite that new numerologies aligned with LTE can be supported in NR, the co-existence between LTE and NR is actually not a co-existence issue between two different RATs, therefore operations supported by the current specifications may be applied and extended to NR by treating NR as an LTE carrier.

So for the scenario discussed above where only UEs located in both LTE uplink and NR downlink coverage can operate LTE-NR uplink sharing, there are also other options, which are illustrated in Figure 4. In LTE-NR uplink sharing shown in Figure 4(a), NR uplink shares the same LTE uplink band F1, and two downlinks running at LTE downlink band F3 and NR downlink band F2. Figure 4(b) shows another option of dual connectivity, in which two radio bearers are carried by NR F2 in TDD and LTE F1/F3 in FDD.

In the case of Option 1 (Fig. 4a), the multiplexing between NR and LTE in an LTE uplink subframe would be FDM, since most of LTE uplink physical channels span over an entire subframe and are confined in frequency (instead of the entire system bandwidth). Even with LTE SRS, its power spectral density is typically low when spanning over large bandwidth, and it occupies only the last SC-OFDM symbol. Hence, it can coexist with NR uplink in FDM manner, without significantly losing the performance. The choice is up to the scheduler and it can be transparent to UE. From standardization prospective further enhancement can be added only if the sufficient overall benefits are justified [7].

- Option 1: LTE-NR uplink sharing

- Option 2: Dual connectivity

Figure 4. Different options for the UE located both in LTE uplink and NR downlink coverage

## 3	Conclusion

This document provides our main technical concerns and views on LTE-NR uplink sharing. Technically the big differences between LTE uplink frequency and NR downlink frequency in LTE-NR uplink sharing may lead to an inherent issue of unbalanced uplink and downlink coverage, thus bring huge complexity and challenges on the network planning and operation. In addition, due to the same nature of OFDM based modulation in NR, the co-existence between LTE and NR can be treated as if they were the same RAT and thus be covered under the current framework from specification prospective. Further enhancement can be considered only after its overall benefits can be justified.

## Reference

- 3GPP, R1-1613665, Way forward on LTE-NR coexistence in a dual connectivity scenario, Source, RAN1#??

- 3GPP, R1-1701361, Way forward on LTE-NR coexistence scenarios with uplink sharing, Source, RAN1#??

- 3GPP, R4-1702503, Way forward on NR band structure, Source, RAN4#??

- Stefania Sesia, Issam Toufik, Matthew Baker, Long term evolution: from theory to practice 2nd edition, A John Wiley & Sons, Ltd, Publication, 2011

- H.Holma & A.Toskala, WCDMA for UMTS: HSPA Evolution and LTE, John Wiley & Sons, 2010

- H.Holma & A.Toskala, LTE for UMTS: OFDMA and SC-FDMA based radio access, John Wiley & Sons, 2009

- 3GPP, R1-1703817, LTE/NR UL co-existence, ,Ericsson, RAN1#88

- 3GPP TR 36.942 vd.0.0

form change history:

2013-12-06 v1.14.1 modified §11 to read: <FamilyName>, <GivenName>, (If the person is new to 3GPP work, give full contact coordinates, in particular, email address.)

2013-10-03 v1.14.0 removal of embedded help text

v1.13.2: adds tdoc header

v1.13.1: minor changes resulting from discussions at CT#41 & SA#41

v1.13.0: mods to enforce linkage amongst stages 1, 2, 3

draft mods Scarrone-Meredith 2008-07 ff

v1.12.1: removes revision marks following approval at SP-29
v1.12.0: includes provision for Study Items (SP-29)

v1.11.0: includes those changes from v1.8.0 agreed at SP-25.

v1.10.0: full circle

v1.9.0: a clean sheet

v1.8.0: includes comments from SA#24

v1.7.0: includes comments from RAN, CN and T #24; also includes “early implementation” data

v1.6.0: includes comments made during review period prior to TSGs#24

v1.5.0: includes comments made at TSGs#23 (Phoenix)

v1.4.0: offered to SA#23 for approval

v1.3.0: offered to CN#23, RAN#23 and T#23 for comments

DRAFT4 v1.3.0: 2004-03-09: Incorporation of comments from Leaders list

DRAFT3 v1.3.0: 2004-02-19: Incorporation of comments from MCC members

DRAFT2 v1.3.0: 2004-01-29: Complete redraft:

v1.2.0: 2002-07-04: "USIM" box changed to "UICC apps"

2003-05-28: spelling of “rapporteur” corrected

2002-07-04: "USIM" box changed to "UICC apps"