3GPP TSG RAN#75	RP-170780

March  6th – 9th, 2017 – Dubrovnik, Croatia

Agenda item:		Joint SA – RAN meeting (08.03.2017)

Source:	Deutsche Telekom, BT, Telecom Italia, MediaTek

Title:	Discussion on inter-RAT interworking between NR and other 3GPP RATs

Document for:		Discussion & Decision

# 1	Introduction

Currently 3GPP maintains 4 radio technologies (RATs), which have evolved over the last 25 years:

- GERAN

- UTRAN (UMTS/HSPA)

- E-UTRAN (LTE/LTE-A)

- New Radio

With each development of a new technology or cellular generation, at a certain time it is necessary to question the extent to which inter-working is needed between the different technologies.

The term “inter-working” needs to be differentiated in such discussion to (at least):

- Cell Selection

- Cell Reselection

- Cell Change Order

- Handover

Between different RATs different level of inter-working is supported, but generally cell selection (incl. RAT change) is always supported, cell reselection inside a RAT is always supported, network controlled cell change order is supported in GERAN and UTRAN networks, while network controlled Handover is always supported inside a particular RAT and to and from different RATs.

# 2	Discussion

2.1	Principle of simplicity for the 5G design

One of the fundamental design principles for 5G is to base it on a simplified architecture and where possible try to avoid inheriting legacy functionally if possible.

The requirements for NR were initially discussed in a RAN Workshop on 5G in Phoenix back in Sept. 2015, followed by a workshop initially defining requirements for NR in Barcelona back in Feb. 2016.

In the following we only discuss the mobility / inter-working principles with NR.

2.2		Status: Requirements in SA1 (TS 22.261 v2.0.0)

[1] clearly defines the exceptions of inter-working with legacy:

So it is clear from this that there is the requirement to exclude connected mode mobility (i.e. Handover) between NR and GERAN / UTRAN.

And further:

So it is clear from this that the only requirement is to support mobility between NGC and EPC, hence no mobility procedures between NCG and 3G or 2G core networks.

2.3		Status: Requirements in SA2 (TS 23.501 v0.3.1)

According to our understanding the work in SA2 on the 5G CN architecture only considers inter-RAT interworking with E-UTRAN (no mention of UTRAN or GERAN in the entire TS)

2.4		Status: Requirements in RAN (TR 38.913 v14.1.0)

The NR requirements in RAN [2] request tight interworking only between the new RAT (NR) and LTE:

Further in [2] only a KPI definition in chapter 7 refers further to inter-system interworking:

2.5		Status: Agreements in RAN2 (TR 38.804v1.0.0)

On the architecture the TR [3] only shows a potential interface between the EPC and the NGC which is marked as FFS

Interestingly the state diagram  shows cell reselection between GERAN and UTRAN to/from NR:

Observation: Obviously there is a kind of mismatch between the different groups involved.

2.6		Market Observations

It has become very clear that operators around the world are considering when 2G and 3G networks should be closed and at least one operator has already closed their 2G network [5], with timeline announcements from at least one other operator for closure of 3G [6] and 2G [7].  Other operators have not made any information public and may continue to operate 2G and 3G networks for many years but it is clear that support for modern smartphones will be far superior on LTE and 5G networks.

Observation: the value of a large volume of work on fallback to technologies that are ultimately going to be shut down is questionable.

# 3	Conclusion

As it seems to be a widely established view that there will only be tight interworking (i.e. handover) between NR and LTE (but not UTRAN or GERAN) and there is no requirement on idle mode inter-RAT-interworking the co-signing companies propose to document this in the appropriate TR/TS [2] and change [3].

If this proposal is not approved 3GPP consequently needs to add additional work on support of idle mode inter-RAT interworking from GERAN and UMTS (as system information for NR cell reselection needs to be defined), as there is currently no WI proposal, neither related to GERAN nor UTRAN, we even believe stronger that there is no need adding this complexity.

For the NR design it would also ease the work as a significant amount of work for the inter-RAT cell reselection from NR to UTRAN / GERAN can be avoided.

No need to mention the relief on terminal implementation, terminal testing.

Proposal: The co-signing companies kindly propose to (re-)confirm that neither connected mode nor idle mode interworking (besides cell selection) to / from GERAN and UTRAN to / from NR is standardised.

# References

- TS 22.261 v2.0.0

- TS 23.501 v0.3.1

- TR 38.913 v14.0.0

- TR 38.804 v1.0.0

- http://about.att.com/innovationblog/2g_sunset

- http://www.arnnet.com.au/article/610472/telstra-flags-3g-network-shutdown-early-2020/

- http://www.arnnet.com.au/article/607766/vodafone-bids-farewell-its-2g-network/

- R2-167493