3GPP TSG-RAN Meeting #77         													          RP-171768

Sapporo, Japan, September 11 – 14, 2017

Source: 	Huawei, HiSilicon, Ericsson, Telecom Italia

Title: 	Summary of ITU-R evaluation criteria and consideration for self evaluation

Agenda Item:	9.3.2

Document for:	Discussion/Decision

# Introduction

At RAN#75 meeting in March 2017, New Study Item on Self Evaluation towards IMT-2020 submission was approved [1] and further revised in [2] at RAN#76 meeting. The objective of this study item is to provide self evaluation results towards IMT-2020 submission against the technical performance requirements defined by Report ITU-R M.[IMT-2020. TECH PERF REQ], using the evaluation criteria defined in Report ITU-R M.[IMT-2020. EVAL], and complete the compliance template and description template defined in Report ITU-R M.[IMT-2020. SUBMISSION]. By these steps, self evaluation should verify that 3GPP proposed IMT-2020 radio interface technology (RIT) or Set of RIT (SRIT) passes the criteria for entry and criteria for acceptance as defined in Document IMT-2020/02 (Rev. 1).

To achieve the above objective, a common understanding on ITU-R evaluation criteria among the contributors to the study on self-evaluation would be helpful. Therefore, this contribution summarizes the ITU-R evaluation criteria which aims to build the common understanding as well as to figure out the issues that should be further considered for self evaluation.

# ITU-R evaluation criteria

In summary, the ITU-R evaluation criteria towards IMT-2020 submission are defined in the following ITU-R Document and Reports [3-6], as announced in [7] and [8],

–	Document IMT-2020/02 (Rev.1) – Submission, evaluation process and consensus building for IMT-2020, which defines the criteria for entry and criteria for acceptance of the candidate IMT-2020 radio interface technology (RIT) or Set of RIT (SRIT).

–	Report ITU-R M.[IMT-2020.Submission] – Requirements, evaluation criteria and submission templates for the development of IMT-2020, which defines the overall IMT-2020 requirements and submission templates for IMT-2020 submission.

–	Report ITU-R M.[IMT-2020.TECH PERF REQ] – Minimum requirements related to technical performance for IMT-2020 radio interface(s), which defines detailed targets for technical performance requirements.

–	Report ITU-R M.[IMT-2020.EVAL] – Guidelines for evaluation of radio interface technologies for IMT-2020, which defines evaluation critieria, including evaluation method, test environment and its related evaluation configurations that should be used to test the candidate IMT-2020 proposal.

The following sections provide detailed summary on the aspects defined in the above ITU-R Document and Reports.

## IMT-2020 submission and evaluation process

IMT-2020 submission and evaluation process is defined in Document IMT-2020/02 (Rev.1) [3]. There are eight steps for a candidate RIT (radio technology interface) or SRIT (set of RITs) to be identified as IMT-2020 terrestrial components radio interface Recommendation(s).

These steps are illustrated in Figure 1from the perspective of an IMT-2020 proponent. Self evaluation activities are highly related to Step 2 and 3. The proponent should provide complete information including self evaluation results and completed templates (including description and compliance templates, see below) to complete its submission to ITU-R as defined in Step 3. Before that, proponent can provide description template to share information to ITU-R on its development of IMT-2020.

In addition, the proponent should take the criteria for entry and criteria for acceptance into account for its development and self evaluation of its proposal. The criteria for entry the submission process for a candidate RIT/SRIT is defined in Step 2, and the criteria for acceptance of the candidate RIT/SRIT as (part of) IMT-2020 specification is defined in Step 7.

Figure 1 Illustration of IMT-2020 process and potential proponent activity

Criteria for entry (see Step 2 in [3]):

- An RIT needs to fulfil the minimum requirements for at least three test environments; two test environments under eMBB and one test environment under mMTC or URLLC.

- An SRIT consists of a number of component RITs complementing each other, with each component RIT fulfilling the minimum requirements of at least two test environments and together as an SRIT fulfilling the minimum requirements of at least four test environments comprising the three usage scenarios (i.e., eMBB, mMTC and URLLC).

Criteria for acceptance (see Step 7 in [3]):

- The RIT or SRIT meets the requirements for the five test environments comprising the three usage scenarios (i.e., eMBB, mMTC and URLLC).

Completeness of submission of candidate RIT/SRIT (Step 3):

Three major components should be provided to complete the submission:

- Completed templates, including description templates and compliance templates (see details in Section 2.3). Compliance templates will be based on self evaluation results.

- Self-evaluation results against the IMT-2020 requirements (including service, spectrum and technical performance; see details in Section 2.2).

- Proponents and IPR holders should indicate their compliance with the ITU policy on intellectual property rights.

The test environments for eMBB, mMTC and URLLC usage scenario are defined in Report ITU-R M.[IMT-2020. EVAL], and is summarized in Table 1 (see Section 8.2 in [6]).

Table 1 Mapping of test environments and usage scenarios

| Usage scenarios | eMBB |  |  | mMTC | URLLC |
| --- | --- | --- | --- | --- | --- |
| Test environments | Indoor Hotspot – eMBB | Dense Urban – eMBB | Rural – eMBB | Urban Macro – mMTC | Urban Macro – URLLC |

Observation:

3GPP should provide complete information to complete its submission to ITU-R in Step 3

- Self evaluation should provide two major components: completed templates and self evaluation results.

- Self evaluation should demonstrate the proposed RIT/SRIT can pass criteria for entry and acceptance against the IMT-2020 requirements.

## IMT-2020 requirements and their evaluation method

IMT-2020 requirements are defined in Report ITU-R M.[IMT-2020.SUBMISSION]. It includes three aspects [4]:

- Service requirements

- Spectrum requirements

- Technical performance requirements

The detailed targets for technical performance requirements are defined in Report ITU-R M.[IMT-2020. TECH PERF REQ] (i.e., [5]). And the mapping of these technical performance requirements to test environments is also defined.

The evaluation method for the IMT-2020 requirements (including service, spectrum, and technical performance) is defined in Report ITU-R M.[IMT-2020.EVAL].

A summary of evaluation method and their applicability to RIT/SRIT is provided in Table 2. The summary of technical performance requirements and their related evaluation method and test environments is provided in Table 3.

It is noted from Table 2 and Table 3 that five technical performance requirements need simulation efforts (highlighted in red fonts), and six technical performance requirements require analytical way (via calculation) for evaluation. For user experience data rate, if single layer (macro layer) configuration is employed, analytical way can be applied. Otherwise system level simulation is required.

Observation:

3GPP should evaluate the fulfillment of service requirement and spectrum requirement, and test the technical performance requirements in related test environment(s) using evaluation method as illustrated in Table 2 and Table 3.

- For different test environment, different need of technical performance evaluation is observed.

- Six technical performance requirements will need simulation efforts in related test environments.

- 3GPP should review the evaluation method defined by ITU-R, and might need to develop additional details on evaluation method and/or models as necessary to provide self evaluation results.

Table 2 Service and spectrum requirements and their evaluation method

| Requirement | Sub-items | Evaluation method | Remarks |
| --- | --- | --- | --- |
| Service requirement | Support for wide range of services | Inspection | See 5.2.4.1.1 in [4] for details |
| Spectrum requirement | Frequency bands identified for IMT | Inspection | This is related to the support of below 6GHz (see 5.2.4.2.1 in [4]). |
|  | Higher Frequency range/band(s) | Inspection | This is related to the support of above 24.25GHz (see 5.2.4.2.2 in [4]). According to the note in 5.2.4.2.2 in [4], - For an RIT, this requirement should be fulfilled. - For an SRIT, at least one of the component RITs within the SRIT need to fulfil this requirement. |

Table 3 Technical performance requirements and their evaluation method and related test environments

| Related Usage scenario | Sub-items | Evaluation method | Related test environment(s) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | eMBB |  |  |  |  | URLLC |  | mMTC |  |
|  |  |  | Indoor Hotspot – eMBB |  | Dense Urban – eMBB |  | Rural – eMBB | Urban Macro – URLLC |  | Urban Macro – mMTC |  |
| eMBB | Peak data rate | Analytical | Y |  |  |  |  |  |  |  |  |
|  | Peak spectral efficiency | Analytical | Y |  |  |  |  |  |  |  |  |
|  | User experienced data rate | - For single layer: Analytical - For multi-layer: SLS* |  |  | Y |  |  |  |  |  |  |
|  | 5th percentile user spectral efficiency | SLS | Y |  | Y |  | Y |  |  |  |  |
|  | Average spectral efficiency | SLS | Y |  | Y |  | Y |  |  |  |  |
|  | Area traffic capacity | Analytical | Y |  |  |  |  |  |  |  |  |
|  | Energy efficiency | Inspection | Y |  |  |  |  |  |  |  |  |
|  | Mobility | LLS* + SLS | Y |  | Y |  | Y |  |  |  |  |
| eMBB, URLLC | User plane latency | Analytical | Y |  |  |  |  | Y |  |  |  |
|  | Control plane latency | Analytical | Y |  |  |  |  | Y |  |  |  |
|  | Mobility interruption time | Analytical | Y |  |  |  |  | Y |  |  |  |
| URLLC | Reliability | LLS+SLS |  |  |  |  |  | Y |  |  |  |
| mMTC | Connection density | Opt 1: LLS + SLS Opt 2: Full SLS |  |  |  |  |  |  |  | Y |  |
| General | Bandwidth and Scalability | Inspection | Y |  |  |  |  |  |  |  |  |
| Total number of technical performance requirements to be tested to pass the minimum requirement to that test environment |  |  | 11 | 11 |  | 10 |  |  | 5 |  | 2 |

* NOTE: SLS indicates system level simulation. LLS indicates link level simulation.

## IMT-2020 submission templates

According to Document IMT-2020/02 (Rev1), the proponent should include completed templates as defined in Report ITU-R M.[IMT-2020.SUBMISSION], together with self evaluation results to complete the submission, which is Step 3 in the submission and evaluation process.

According to Report ITU-R M.[IMT-2020.SUBMISSION], two templates are defined: description template (see section 5.2.3 in [4]) and compliance template (see section 5.2.4 in [4]). A summary is given in Table 4.

Table 4 Submission template summary

| Submission template |  | Content | Related steps in IMT-2020 process | Remark |
| --- | --- | --- | --- | --- |
| Description template (see section 5.2.3 in [4]) | Characteristics template | Template for description of characteristics of the proposal | - Can be provided in Step 2 for information sharing to ITU-R. - Required in Step 3 to complete the submission. | It should provide sufficient details to facilitate the external evaluation by independent evaluation groups. |
|  | Link budget template | Template for link budget information of the proposal. |  | For 3GPP as a proponent, it would be good that the link budget for the five test environments is provided. |
| Compliance template (see section 5.2.4 in [4]) | For services | Template for service requirement | - Required in Step 3 to complete the submission. | 3GPP provides these templates based on self evaluation through simulation/analytical /inspection results |
|  | For spectrum | Template for spectrum requirements |  |  |
|  | For technical performance | Template for technical performance requirements |  |  |
| Template requirement for SRIT (see section 5.1 in [4]) |  | The completed templates should be provided for each component RIT within the composite SRIT and/or for the composite SRIT. |  |  |

The proponent can provide description template (Characteristics and/or Link Budget template) to ITU-R during its development of the proposal to share the information to ITU-R and independent evaluation groups. However, a complete submission from the proponent should be accomplished in Step 3 with complete submission template including description template and compliance template, as well as self evaluation results. According to [3], Step 3 will be ended at WP 5D#32 meeting in July 2019.

Observation:

3GPP needs to provide description template (including characteristics and link budget templates) and compliance template (including service, spectrum, and technical performance) to complete Step 3 of IMT-2020 submission. Description template might be provided earlier to share information to ITU-R and independent evaluation groups.

## Test environments and Evaluation configurations

The proponent should provide self evaluation to test its proposed RIT/SRIT against the IMT-2020 requirements (including service, spectrum and technical performance), and based on these information, the proponent should provide completed templates.

Self evaluation should test the IMT-2020 requirements under related test environments. There are five test environments for the three usage scenarios (eMBB, mMTC and URLLC) as defined in [6] and are summarized in Table 1 of Section 2.1.

According to Section 8.4 of [6], it is noted that there are multiple evaluation configurations in a specific test environment. Table 5 provides the summary of evaluation configurations under a test environment, and the related technical performance requirements that need simulation efforts.

Table 5 Evaluation configurations and related technical performance requirements

| Usage scenario |  |  | eMBB |  |  | mMTC | URLLC |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test env. | Evaluation configuration | Spectral efficiency (average, 5th percentile user) | User experienced data rate | Mobility | Connection density | Reliability |
| eMBB | Indoor Hotspot – eMBB | Config. A: 4GHz | Simulation |  | Simulation |  |  |
|  |  | Config. B: 30GHz | Simulation |  | Simulation |  |  |
|  |  | Config. C: 70GHz | Simulation |  | Simulation |  |  |
|  | Dense Urban – eMBB | Config. A: 4GHz/Macro layer | Simulation | Analytical | Simulation |  |  |
|  |  | Config. B: 30GHz/Macro layer | Simulation | Analytical | Simulation |  |  |
|  |  | Config. C: Multi-frequency and/or Multi-layer |  | Simulation (for multi-layer) |  |  |  |
|  | Rural - eMBB | Config. A: 700MHz/1732m | Simulation |  | Simulation |  |  |
|  |  | Config. B: 4GHz/1732m | Simulation |  | Simulation |  |  |
|  |  | Config. C (LMLC)*: 700MHz/6000m | Simulation |  |  |  |  |
| mMTC | Urban macro - mMTC | Config. A: 500m/700MHz |  |  |  | Simulation |  |
|  |  | Config. B: 1732m/700MHz |  |  |  | Simulation |  |
| URLLC | Urban macro – URLLC | Config. A: 4GHz |  |  |  |  | Simulation |
|  |  | Config. B: 700MHz |  |  |  |  | Simulation |
| Remark |  | - RIT/SRIT only needs to meet requirement under one of the evaluation configurations. - For average spectral efficiency evaluation in Rural-eMBB, Config. C (LMLC) should be chosen together with either Config. A or B |  |  |  |  |  |

It would be good that 3GPP can provide evaluation results on each of the evaluation configurations in a specific test environment. However, this does not imply a contributor needs to provide evaluation for all evaluation configurations.

On the other hand, there are other alternative parameters in each of evaluation configuration. 3GPP’s self evaluation should consider these parameters and other parameters which are not fully specified in [6], as listed in “Parameters that need further consideration in 3GPP” in Table 6.

Table 6 Other alternative parameters and parameters need further consideration in 3GPP

| Usage | Test env. | Evaluation configuration | Other alternative parameters | Parameters that need further consideration in 3GPP |
| --- | --- | --- | --- | --- |
| eMBB | Indoor Hotspot – eMBB | Config. A: 4GHz | Network layout: - Alt1: 1 TRxP per site - Alt2: 3 TRxPs per site (30UEs per site) | Antenna configuration at TRxP Antenna configuration at UE |
|  |  | Config. B: 30GHz |  |  |
|  |  | Config. C: 70GHz |  |  |
|  | Dense Urban – eMBB | Config. A: 4GHz/Macro layer | - | Antenna configuration at TRxP Antenna configuration at UE |
|  |  | Config. B: 30GHz/Macro layer | - |  |
|  |  | Config. C: Multi-frequency and/or Multi-layer | Network layout (Only for micro layer): - Alt1: 1 TRxP per site - Alt2: 3 TRxPs per site | Detailed drop method for micro layer (if used) and size of cluster for UE dropping (if micro layer exists) |
|  | Rural - eMBB | Config. A: 700MHz/1732m | - | Antenna configuration at TRxP Antenna configuration at UE |
|  |  | Config. B: 4GHz/1732m | - |  |
|  |  | Config. C (LMLC): 700MHz/6000m | - |  |
| mMTC | Urban macro - mMTC | Config. A: 500m/700MHz | Traffic model: - Alt1: 1 message/day /device - Alt2: 1 message/2 hours/device | Antenna configuration at TRxP Antenna configuration at UE Simulation bandwidth |
|  |  | Config. B: 1732m/700MHz |  |  |
| URLLC | Urban macro – URLLC | Config. A: 4GHz | - | Antenna configuration at TRxP Antenna configuration at UE Simulation bandwidth |
|  |  | Config. B: 700MHz | - |  |
| Channel model |  | Two alternatives: Model A/B, essentially for below 6GHz |  |  |

Observation:

There are multiple evaluation configurations in each test environment. It would be good for 3GPP to provide evaluation results for all the evaluation configurations collectively from all contributors.

- This does not imply a contributor needs to provide evaluation for all evaluation configurations.

- 3GPP should review ITU-R evaluation parameters carefully and further consider the alternative parameters and other partially specified parameter provided by ITU-R. 3GPP also needs to define technology related simulation assumptions and parameters.

# Conclusion

This contribution summarizes the ITU-R evaluation criteria for IMT-2020 submission, and the related consideration for self evaluation is provided.

Figure 2 Summary of ITU-R evaluation criteria

As shown in Figure 2,

- 3GPP should provide description templates - including characteristics and link budget, and compliance templates - including service, spectrum, technical performance (see Table 4), together with self evaluation results to complete Step 3 for IMT-2020 submission (see section 2.1).

  - Self evaluation will be based on the candidate RIT/SRIT whose characteristics are described in description template (characteristics).

  - Self evaluation results will be used to develop compliance template, which should demonstrate the proposed RIT/SRIT can pass criteria for entry and acceptance against the IMT-2020 requirements.

  - Description template might be provided earlier to share information to ITU-R and independent evaluation groups.

- 3GPP should evaluate the fulfillment of service requirement and spectrum requirement, and test the technical performance requirements in related test environment(s) using evaluation method as illustrated in Table 2 and Table 3.

- For different test environment, different need of technical performance evaluation is observed (see Table 3).

- Six technical performance requirements will need simulation efforts in related test environments (see Table 3).

- 3GPP should review the evaluation method defined by ITU-R, and might need to develop additional details on evaluation method and/or models as necessary to provide self evaluation results.

- There are multiple evaluation configurations in each test environment (see Table 5). It would be good for 3GPP to provide evaluation results for all the evaluation configurations collectively from all contributors.

- This does not imply a contributor needs to provide evaluation for all evaluation configurations.

- 3GPP should review ITU-R evaluation parameters carefully and further consider the alternative parameters and other partially specified parameter provided by ITU-R. 3GPP also needs to define technology related simulation assumptions and parameters (see Table 6).

# References

- RP-170832, “New Study Item on Self Evaluation towards IMT-2020 submission”, Huawei, Ericsson, March 2017.

- RP-171451, “Revision of SI: Study on self-evaluation towards IMT-2020 submission”, Huawei, Ericsson, Telecom Italia, June 2017.

- Document IMT-2020/02 (Rev. 1), “Submission, evaluation process and consensus building for IMT-2020”, ITU-R WP 5D, Feb 2017, available in RP-170044.

- Report ITU-R M.[IMT-2020.SUBMISSION], “Requirements, evaluation criteria and submission templates for the development of IMT-2020”, ITU-R WP 5D, June 2017, available in RP-171559.

- Report ITU-R M.[IMT-2020.TECH PERF REQ], “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”, ITU-R WP 5D, Feb 2017, available in RP-170044.

- Report ITU-R M.[IMT-2020.EVAL], “Guidelines for evaluation of radio interface technologies for IMT-2020”, ITU-R WP 5D, June 2017, available in RP-171559.

- RP-170044, “Further information on the invitation for submission of proposals for candidate radio interface technologies for the terrestrial components of the radio interface(s) for IMT-2020 and invitation to participate in their subsequent evaluation”, ITU-R WP 5D, March 2017.

- RP-171559, “Availability of Addendum 3 to Circular Letter 5/LCCE/59 related to proposals for candidate radio interface technologies for the terrestrial components of the radio interface(s) for IMT-2020 and their subsequent evaluation”, ITU-R WP 5D September 2017.

As defined in criteria for entry (Step 2), each component RIT of the SRIT needs to still fulfil the minimum requirements of at least two test environments.

A single cell across the three eMBB test environments indicate that the same targets are applied to these test environments.

Different cell for different eMBB test environments indicate that different targets are applied to a specific test environment.