3GPP TSG-RAN #79	RP-180141

Chennai, India, 19 – 22 March 2018

Agenda item:	9.1.1

Source:	Samsung

Title:	Summary of email discussion on NR/LTE MIMO enhancements

Document for:	Discussion

# 1	Introduction

There has been email discussion on NR/LTE MIMO enhancements according to the decision made in RAN#78 about handling of new WI/SI proposals. This contribution is a summary of the email discussion and makes a proposal for further discussion toward RAN#80 meeting.

# 2	Summary of Email Discussion

Companies’ views on Rel-16 work for NR/LTE MIMO enhancements were shared in terms of technology areas as well as detailed technologies for each area. An excel spread sheet is attached to this contribution to show the overall landscape and the level of support for RAN1 work in the form of normative work or study for each technology area. In the following sections, the summary of email discussion is provided.

## NR MIMO enhancements

Twenty companies shared their views. Six major technology areas were identified during the email discussion, among which four have got significantly larger support than the other two. There were also views suggesting detailed technologies that in the moderator’s view were not belonging to any of the major areas. The identified technology areas and corresponding detailed technologies are shown in Table 1.

Table 1: Summary of views on NR MIMO enhancements

|  | Technology Area for Enhancement | No. of companies that showed support for RAN1 work | Detailed Technologies |
| --- | --- | --- | --- |
| N1 | MU-MIMO | 15 | Extension of Type II CSI feedback to rank > 2, L > 4 |
|  |  |  | Overhead reduction (for improved resolution) of Type II CSI feedback |
|  |  |  | Non-linear precoding |
| N2 | Multi-TRP / Multi-panel (introduce support of NC-JT) | 15 | Downlink control signaling enhancement |
|  |  |  | CSI feedback enhancement |
|  |  |  | UL multi-panel transmission |
| N3 | DL/UL reciprocity (including partial reciprocity) | 6 |  |
| N4 | UL MIMO/coverage | 10 | Codebook |
|  |  |  | Frequency selective precoding |
|  |  |  | Transmit diversity |
| N5 | Reference signal | 5 | SRS enhancement |
|  |  |  | CSI-RS enhancement |
|  |  |  | TRS enhancement |
|  |  |  | PTRS enhancement |
|  |  |  | DMRS enhancement |
| N6 | Beam management | 13 | Beam recovery enhancement |
|  |  |  | Beam selection/reporting enhancement |
|  |  |  | Uplink beam management |
|  |  |  | Control channel beam management |
|  |  |  | Others (signaling reduction & mobility enhancement) |
|  | Others | Suggestion from only one company for each bullet on the right | - CW mapping for handling cross-link interference - DL non-transparent TxD - Some leftovers from Rel-15 (A-CSI on PUCCH, CSI feedback for non-uniform antenna array, DMRS-based link adaptation, post-decoding CQI feedback) - Pre-committed CSI acquisition |

## LTE MIMO enhancements

Three companies shared their views. Four major technology areas were identified, all of which showed similar level of support.

Table 2: Summary of views on LTE MIMO enhancements

|  | Technology Area for Enhancement | No. of companies that showed support for RAN1 work | Detailed Technologies |
| --- | --- | --- | --- |
| L1 | CSI enhancement | 2 | Extension of advanced CSI (>2 beams, including Rel-14 leftovers) |
|  |  |  | Others: Aperiodic IMR, CSI feedback for non-uniform antenna array, DMRS-based link adaptation, post-decoding CQI feedback |
| L2 | Transmission mode dynamic switching between CRS-based TM3/4 and DMRS-based TM8/9/10 | 2 |  |
| L3 | Beamforming for control channel | 2 |  |
| L4 | SRS enhancement | 3 | SRS antenna switching among 8 antennas |
|  |  |  | Virtual cell ID for SRS |
|  |  |  | SRS coverage enhancement |
|  |  |  | SRS power control |

# 3	Proposal

On NR MIMO enhancement, the technology areas and detailed technologies proposed over email discussion seem to be beyond what can practically be handled by RAN1. Hence, it may be helpful to reduce the scope of further discussion toward RAN#80 taking into account the level of support for RAN1 work expressed over email discussion.

Proposal 1: On NR MIMO enhancements, it is proposed to continue discussion on the following technology areas.

- N1: MU-MIMO

- N2: Multi-TRP / Multi-panel (introduce support of NC-JT)

- N4: UL MIMO/coverage

- N6: Beam management

On LTE MIMO enhancement, it seems premature to decide upon a direction for further discussion due to the limited participation in email discussion. There seems a need for change in the email discussion structure.

Proposal 2: The moderator proposes to start separate email threads for NR MIMO enhancements and LTE MIMO enhancements after RAN#79.