3GPP TSG RAN Meeting #79	RP-180167

Chennai, India, 19 – 22 March 2017

Agenda item:	9.4.3

Source:	Ericsson

Title:	TP to TR 37.890 EU Regulatory Aspects Europe section

Document for:	Approval

# Introduction

This document is a Text Proposal to the Regulatory Aspects sub-section of 3GPP Techincal Report  “Feasibility Study on 6 GHz for LTE and NR in Licensed and Unlicensed Operations” related to European regulation activities.

# 2	Text proposal

<<< Start of TP>>>

### 4.1.1 	EUROPE

#### 4.1.1.1	Introduction

In Europe, 5.925-6.700 GHz frequency range is allocated on a primary basis to FS-Fixed Service and FSS-Fixed Satellite Service (earth to space), and on a secondary basis to Earth Exploration Satellite Service - EESS (passive); 6.700-7.025 GHz frequency range is allocated on a primary basis to FS and FSS (earth to space, space to earth), and on a secondary basis to EESS (passive); 7.025-7.125 GHz is allocated on a primary basis to FS and on a secondary basis to EESS (passive), but in Russian federation, where, according to RR 5.459, 7.100-7.125 GHz is allocated to space operation service (Earth-to-space) on a primary basis. Generic SRD (UWB), LPR and TLPR also operate in these frequency ranges in no protection and no interference basis. More details are provided in Table 4.1.1.1-1 [20].

Table 4.1.1.1-1: European Allocation in 6 GHz

| Frequency Range [MHz] | Allocation | Applications/Notes |
| --- | --- | --- |
| 5 925 – 6 700 | FIXED FIXED-SATELLITE (EARTH TO SPACE) Earth Exploration Satellite (Passive) 5.149 5.440 5.458 | ESV/Within 5925-6425 MHz FSS Earth stations/Priority for civil networks Fixed/Point-to-point Passive sensors (satellite)/For sea surface temperature, sea surface wind speed and soil moisture measurements Radio astronomy/Spectral line observations, VLBI Radiodetermination applications/Within 4500-7000 MHz for TLRP application and 6000-8500 MHz for LPR applications UWB applications/Generic UWB as well as UWB on-board aircraft regulation within 6.0-8.5 GHz |
| 6 700 – 7 045 | FIXED FIXED-SATELLITE (EARTH TO SPACE) (SPACE TO EARTH) 5.441 Earth Exploration Satellite (Passive) 5.458 5.458A 5.458B | FSS Earth stations/Within 6725-7025 MHz. Priority for civil networks Feeder links Fixed/ Point-to-point PMSE/ portable or mobile wireless video, cordless cameras, temporary P-t-P viedo links in 7-8.5 GHz tuning range Passive sensors (satellite)/For sea surface temperature, sea surface wind speed and soil moisture measurements Radiodetermination applications/Within 4500-7000 MHz for TLRP application and 6000-8500 MHz for LPR applications UWB applications/Generic UWN as well as UWB on-board aircraft regulation within 6.0-8.5 GHz |
| 7 045 – 7 145 | FIXED Earth Exploration Satellite (Passive) 5.458 | Fixed/ Point-to-point PMSE/ portable or mobile wireless video, cordless cameras, temporary P-t-P viedo links in 7-8.5 GHz tuning range Radiodetermination applications/Within 6000-8500 MHz for LPR applications UWB applications/Generic UWB as well as UWB on-board aircraft regulation within 6.0-8.5 GHz |

Allocation for services (ITS, UWB, WIA, non-specific SRDs) in adjacent bands is shown in Table 4.1.1.1-1 and 4.1.1.1-2.

Table 4.1.1.1-2: Adjacent services below 5.925 GHz

| Frequency Range [MHz] | Allocation | Applications/Notes |
| --- | --- | --- |
| 5 850 – 5 925 | FIXED FIXED-SATELLITE (EARTH TO SPACE) MOBILE 5.150 | BFWA/Within 5725-5875 MHz DA2GC/Within 5855-5875 MHz FSS Earth stations/ priority for civil networks ISM/Within 5725-5875 MHz MBR/Within 5852-5872 MHz and 5880-5900 MHz ITS/ Within 5875-5925 MHz and 5855-5875 MHz. Traffic safety applications within 5875-5905 MHz NON-specific SRDs/Within 55725-5875 MHz Radiodetermination applications/ Within 4500-7000 MHz for LPR applications WIA/Within 5725-5875 |

Table 4.1.1.1-3: Adjacent services above 7.145 GHz

| Frequency Range [MHz] | Allocation | Applications/Notes |
| --- | --- | --- |
| 7 145 – 7 190 | FIXED MOBILE SPACE RESEARCH (DEEP SPACE) (EARTH TO SPACE) Space Operation (Earth-to-space) 5.458 | Fixed/ Point-to-point PMSE/ portable or mobile wireless video, cordless camera, temporary viedo links in 7-8.5 GHz tuning range Radiodetermination applications/ within 6.0-8.5 GHz for LPR applications UWB applications/Generic UWB. On-board aircraft regulations within 6.0-8.5 GHz |

#### 4.1.1.2	Licensed operations

Editor’s note: To be further investigated.

#### 4.1.1.3	Unlicensed operations

In December 2017, EC Radio Spectrum Committee finalized the Mandate to CEPT [21] to study and identify harmonised compatibility and sharing technical conditions for a sustainable and efficient use on a shared basis of the frequency range 5.925-6.425 GHz for Wireless Access Systems including Radio Local Area Networks (WAS/RLANs).

CEPT has created two new project teams, SE 45 and FM 57. The Project Team (PT) SE 45 has been tasked to undertake compatibility and sharing studies in the 6 GHz frequency range to support FM 57, and the latter will define the regulatory rules for the band. The scope of the work items in these groups is the introduction of low power wireless access systems (including RLAN) in the frequency range 5.925-6.425 GHz under a non-protected and non-iterference basis, ensuring certainty of continued operation, development and protection of existing services (Fixed Services (FS), Fixed Satellite Service (FSS)) considering RR 5.440 and 5.458.

ETSI has developed the draft TR 103 524 [22] providing information on the technical parameters for WAS/RLANs to support the CEPT Work Items activities covering the frequency range 5.925 GHz to 6.425 GHz. In addition, it contains a request for considering additional frequencies up to 6.725 GHz.

ETSI assumptions/proposals on technical RLAN parameters to be used for compatibility and sharing studies are reported in the following:

- BS maximum TX output power: 30 dBm EIRP;

- Band plan: 5.925-6.725 GHz (guard band towards ITS below 5.925 GHz may be accommodated at the lower edge);

- Channel BW: 20, 40, 80, 160 MHz for the IEEE802 standard; wider channel bandwidth can be supported by 3GPP standards via carrier aggregation, up to 32 carriers (including at least one anchor carrier in a licensed band) for eLAA. (Similar configurations for NR-U mentioned but without details on bandwidths and aggregated carriers).

- TX unwanted emissions in the 6 GHz bands: same transmitter spectrum mask as used in 5 GHz (since same modulation assumed) shown in Figure 4.1.1.3-1;

Figure 4.1.1.3-1: Transmit spectral power mask for RLAN equipment

- TX unwanted emissions outside the 6 GHz bands: as specified in ERC Recommendation 74.01 [23] and reported in Table 4.1.1.3-1.

Table 4.1.1.3-1: Out of band emissions limits

| Frequency range | Limits | Reference Bandwidth |
| --- | --- | --- |
| 30-47 MHz | -36 dBm | 100 kHz |
| 47-74 MHz | -54 dBm | 100 kHz |
| 75-87.5 MHz | -36 dBm | 100 kHz |
| 87.5-118 MHz | -54 dBm | 100 kHz |
| 118-174 MHz | -36 dBm | 100 kHz |
| 174-230 MHz | -54 dBm | 100 kHz |
| 230-470 MHz | -36 dBm | 100 kHz |
| 470-862 MHz | -54 dBm | 100 kHz |
| 0.862-1 GHz | -36 dBm | 100 kHz |
| 1-5.15 GHz | -30 dBm | 1 MHz |
| 5.35-5.5.47 GHz | -30 dBm | 1 MHz |
| 5.725-26 GHz | -30 dBm | 1 MHz |

The Draft ECC Report on compatibility studies related to Wireless Access Systems including Radio Local Area Networks (WAS/RLAN) in the frequency range 5.925 – 6.425 GHz [24], developed by SE45, will use the technical characteristics specified in the ETSI TR 103 524 [22] for RLAN as starting point, and the technical parameters provided from SE19 and SE40 for FS and FSS respectively. It is to be noted that some of the technical parameters (for example max EIRP, TX unwanted emissions, etc. ) for RLAN in the ETSI TR 103 524 may differ from the final rules for the band, depending on the results of the compatibility studies done by CEPT.

##### 4.1.1.3.1 Coexistence aspects

Editor’s note: the following coexistence studies need to be monitored/investigated:

•	In band: 				between RLAN and FS/FSS(Space Stations/ESV)/UWB/Radiodetermination

•	Adjacent band: 	between RLAN and ITS/UWB/WIA/non-specific SRDs

<<< End of TP>>>

# References

[120]	The European Table of Frequency Allocations and applications in the frequency range 8.3 kHz and 3000 GHz (ECA Table), October 2017;

[21]	RSCOM17-53rev1- Mandate to CEPT to study and identigy harmonised compatibility and sharing conditions for wireless access systems including radio local area networks in the band 5925-6425 MHz for the provision of wireless broadband services.

[22]	Draft ETSI TR 103 524 V0.0.10 (2018-01) - ERM System Reference document (SRdoc), “Wireless Access Systems including Radio Local Area Networks (WAS/RLANs) in the band 5 925 MHz to 6 725 MHz”;

[23]	CEPT/ERC/REC 74-01: “Unwanted Emissions in the Spurious Domain”;

[24]	Draft ECC Report, “Compatibility studies related to Wireless Access Systems including Radio Local Area Networks (WAS/RLAN) in the frequency band 5 925 – 6 425 MHz”, December 2017.