3GPP TSG RAN Meeting #79	RP-180184

Chennai, India, 19-23rd March 2018

Source: 	Thales, Nokia

Title:	NR-NTN update on spectrum Ka band HAPS

TDOC Type:	pCR

Agenda Item:	9.3.1  (SID = sFS_NR_nonterr_nw)

Document for:	Approval

Release:	Rel-15

Specification: 	38.811  (SID = sFS_NR_nonterr_nw)

# Abstract

The objective of this document is to add a new band that can be used for NR operating via HAPS.

# Proposed text for approval

It is proposed to add the following texts to TR 38.811 “Study on NR to support Non-Terrestrial Networks”.

* * * Start of changes * * * * (new text)

## 4.8 Spectrum

This clause identifies the frequency bands that can be used for NR via Satellite or HAPS in the different ITU regions which are here below depicted:

Figure 4.8-1: ITU regions and the dividing lines between them.   Region 1 (Yellow), Region 2 (Blue), Region 3 (Purple)

Ka band spectrum for Satellite services via GEO space segment

The following bands are identified for use by GEO based Satellite services (Fixed and/or Mobile) on a worldwide basis

|  | Region 1 | Region 2 | Region 3 |
| --- | --- | --- | --- |
| Downlink (space to earth) | 17.3 – 20.2 GHz | 17.7 – 20.2 GHz | 17.7 – 20.2 GHz |
| Uplink (earth to space) | 27.5 – 30.0 GHz | 27.0 – 30.0 GHz | 27.0 – 30.0 GHz |

Ka band spectrum for Satellite services via Non GEO space segment

The following bands are identified for use by Non GEO based Satellite services (Fixed and/or Mobile) on a worldwide basis, subject to protection of GEO systems.

|  | Region 1 | Region 2 | Region 3 |
| --- | --- | --- | --- |
| Downlink (space to earth) | 17.3 – 20.2 GHz | 17.7 – 20.2 GHz | 17.7 – 20.2 GHz |
| Uplink (earth to space) | 27.5 – 29.1 GHz & 29.5 – 30.0 GHz | 27.0 – 29.1 GHz & 29.5 – 30.0 GHz | 27.0 – 29.1 GHz & 29.5 – 30.0 GHz |

S band spectrum for Satellite services via GEO or Non GEO space segment

The following bands are identified for use by Mobile Satellite services to provide International Mobile Telecommunications (IMT) on a worldwide basis

|  | Region 1 | Region 2 | Region 3 |
| --- | --- | --- | --- |
| Downlink (space to earth) | 2170-2200 MHz | 2160-2200 MHz | 2170-2200 MHz |
| Uplink (earth to space) | 1980-2010 MHz | 1980-2025 MHz | 1980-2010 MHz |

S band spectrum for HAPS based services

The following bands as identified by ITU may be used by HAPS as base stations to provide International Mobile Telecommunications (IMT) on a worldwide basis

In FDD mode

|  | Region 1 | Region 2 | Region 3 |
| --- | --- | --- | --- |
| Downlink (aerial to earth) | 2110-2170 MHz | 2110-2160 MHz | 2110-2170 MHz |
| Uplink (earth to aerial) | 1885-1980 MHz & 2010-2025 MHz | 1885-1980 MHz | 1885-1980 MHz & 2010-2025 MHz |

In TDD mode

|  | Region 1 | Region 2 | Region 3 |
| --- | --- | --- | --- |
| Up and Downlink (aerial to earth and earth to aerial) | 2110-2170 MHz | 2110-2160 MHz | 2110-2170 MHz |

Other bands

The following bands are also identified for use by GEO and in some cases by Non GEO based Satellite services (Fixed and/or Mobile) on a worldwide basis.

The following bands are referred as Q and V bands and is applicable to GEO and Non GEO systems, subject to protection of GEO systems.

|  | Region 1 | Region 2 | Region 3 |
| --- | --- | --- | --- |
| Downlink (space to earth) | 37.5 – 42.5 GHz, 47.5 - 47.9 GHz, 48.2-48.54 GHz & 49.44-50.2 GHz | 37.5 – 42.5 GHz | 37.5 – 42.5 GHz |
| Uplink (earth to space) | 42.5 – 43.5 GHz, 47.2 – 50.2 GHz & 50.4 – 51.4 GHz | 42.5 – 43.5 GHz, 47.2 – 50.2 GHz & 50.4 – 51.4 GHz | 42.5 – 43.5 GHz, 47.2 – 50.2 GHz & 50.4 – 51.4 GHz |

The following bands are referred as Ku bands and is applicable to GEO and Non GEO systems, subject to protection of GEO systems.

|  | Region 1 | Region 2 | Region 3 |
| --- | --- | --- | --- |
| Downlink (space to earth) | 10.7 - 12.75 GHz | 10.7 - 12.7 GHz | 10.7 - 12.75 GHz |
| Uplink (earth to space) | 12.75-13.25 GHz & 13.75-14.5 GHz | 12.75-13.25 GHz & 13.75-14.5 GHz | 12.75-13.25 GHz & 13.75-14.5 GHz |

Note 1: the use of the bands 12.75-13.25 GHz, 10.7-10.95 GHz and 11.2-11.45 GHz by GEO systems is subject to the provisions of ITU Radio Regulation Appendix 30B.

Note2: the use of the bands 11.7-12.5 GHz in Region 1, 12.2-12.7 GHz in Region 2 and 11.7-12.2 GHz in Region 3 by GEO systems is subject to the provisions of ITU Radio Regulation Appendix 30.

The following bands are referred as C band and is applicable to GEO systems. The sub-bands 3700-4200 MHz and 5925-6425 MHz may be used by Non GEO systems, subject to protection of GEO systems.

|  | Region 1 | Region 2 | Region 3 |
| --- | --- | --- | --- |
| Downlink (space to earth) | 3400-4200 MHz and 4500-4800 MHz | 3400-4200 MHz and 4500-4800 MHz | 3400-4200 MHz and 4500-4800 MHz |
| Uplink (earth to space) | 5725-7025 MHz | 5725-7025 MHz | 5725-7025 MHz |

Note: the use of the bands 4500-4800 MHz and  6725-7025 MHz by GEO systems is subject to the provisions of ITU Radio Regulation Appendix 30B.

The following milimeter wave bands are currently identified for HAPS, noting that additional allocations are being studied under WRC-19 Agenda Item 1.14 :

- The frequency bands 47.2-47.5 GHz and 47.9-48.2 GHz:

|  | Region 1 | Region 2 | Region 3 |
| --- | --- | --- | --- |
| Downlink (aerial to earth) | 47.2-47.5 GHz and 47.9-48.2 | 47.2-47.5 GHz and 47.9-48.2 | 47.2-47.5 GHz and 47.9-48.2 |
| Uplink (earth to aerial) | 47.2-47.5 GHz and 47.9-48.2 | 47.2-47.5 GHz and 47.9-48.2 | 47.2-47.5 GHz and 47.9-48.2 |

WRC-97 identified the bands 47.2 - 47.5 GHz and 47.9 GHz - 48.2 GHz for use by HAPS on a global basis in RR 5.552A.

Resolution 122  (WRC-97, revised WRC-00, WRC-03, and WRC-07) provides the regulatory conditions related to the use of HAPS  in the bands 47.2-47.5 GHz and 47.9-48.2 GHz.

- The frequency bands 27.9 - 28.2 GHz and 31-31.3 GHz:

|  | Region 1 | Region 2 | Region 3 |
| --- | --- | --- | --- |
| Downlink (aerial to earth) | 27.9-28.2 GHz |  | 27.9-28.2 GHz |
| Uplink (earth to aerial) | 31-31.3 GHz |  | 31-31.3 GHz |

Note 1: The use in Regions 1 and 3 is limited to certain countries (see below)

WRC-00 identified the bands 27.9 – 28.2 GHz and 31 – 31.3 GHz for use by HAPS in the territory of countries listed in footnotes RR 5.537A and RR 5.543A, on a “no interference, no protection basis”. This designation is limited to the operation in the HAPS-to-ground direction for the band 27.9 – 28.2 GHz and in the ground-to-HAPS direction for the band 31 – 31.3 GHz.

The countries in which such operation is possible are illustrated in the map below:

Countries listed under RR 5.537A and RR 5343A

In addition, the use of the bands 27.9 – 28.2 GHz and 31 – 31.3 GHz is subject to the provisions of Resolution 145 (WRC-03, revised WRC-07 and WRC-12).

- The frequency bands 6440 – 6520 MHz and 6560 – 6640 MHz:

|  | Region 1 | Region 2 | Region 3 |
| --- | --- | --- | --- |
| Downlink (aerial to earth) | 6440-6520 MHz |  | 6440-6520 MHz |
| Uplink (earth to aerial) | 6560-6640 MHz |  | 6560-6640 MHz |

Note 1: The use in Regions 1 and 3 is limited to certain countries (see below)

WRC-12 identified the bands 6440 – 6520 MHz and 6560 – 6640 MHz for use by gateway links for HAPS within the territory of Australia, Burkina Faso, Cote d'Ivoire, Mali and Nigeria in footnote RR 5.457, on a “no interference, no protection basis”. This designation is limited to the operation in the HAPS-to-ground direction for the band 6440 – 6520 MHz and in the ground-to-HAPS direction for the band 6560 – 6640 MHz.
In addition the use of the bands 6440 – 6520 MHz and 6560 – 6640 MHz is subject to the provisions of Resolution 150 (WRC-12).

* * * End of Changes * * * *