3GPP TSG RAN Meeting #81					  	RP-181603

# Gold Coast, Australia, Sept 10th – 13th, 2018

Agenda item:	9.6

Source: 	Samsung

Title: 	UE capability signaling for NR Rel-15

Document for:	Discussion and Decision

Introduction

In RAN1#94 meeting, it was agreed to introduce an UE capability on support of zero slot offset for aperiodic SRS triggering [1]:

Agreement - For SRS for CB PUSCH and antenna switching on FR1, the minimal time internal between the last symbol of the PDCCH triggering the aperiodic SRS transmission and the first symbol of SRS resource is N2 symbols - Support of zero slot offset for aperiodic SRS triggering is UE capability - The time interval in units of OFDM symbols is counted based on the minimum subcarrier spacing between PDCCH and SRS Above agreement does not change the switching time for SUL/UL Above agreement only affects UE capability signalling from RAN2 perspective

Due to time limitation in the last RAN1 meeting and oversight by those involved in the MIMO discussions, it was not reflected in the RAN1 UE feature list. However, it is of general understanding that it would be good have reflect this in RAN so that it can be part of the ASN.1 specification due for approval in September. This contribution provides a possible update on the UE feature list per the highlighted parts above.

Discussion

The following UE feature is recommended to be added in the UE feature list:

| Features | # | Feature group | Components | Prerequisite feature groups (listed in this sheet only) | Need for gNB to know whether the feature is supported by the UE | Consequences if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | RAN5 implication | Note | Responsible WG | RAN WG recommendation | TSG-RAN decision |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. MIMO | [2-54] | Zero slot offset for aperiodic SRS transmission | Support of zero slot offset between aperiodic SRS triggering and transmission | 2-53 | Yes |  | Type 1 | No need | N.A. |  |  |  | Optional with capability signaling |  |

- References

- 3GPP RAN#94 Chairman note