3GPP TSG RAN meeting #81										       RP-181717

Gold Coast, Australia, September 10-13, 2018

## Status Report to TSG

Agenda item:			9.3.4

| WI / SI Name | Study on Non-Orthogonal Multiple Access (NOMA) for NR |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| included in this status report | Core part: |  | Perf. part: |  |  | Testing part: |  |
| Acronym | FS_NR_NOMA |  |  |  |  |  |  |
| Unique ID | 750046 |  |  |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-181403 |  |  |  |  |  |  |
| Target Completion Date | Core part: 12/2018 |  |  |  | Performance part: N/A |  |  |
| Is the WI/SI complete? | Core part: No |  |  |  | Performance Part: N/A |  |  |

Source:

| Leading WG |  | TSG RAN WG1 |
| --- | --- | --- |
| Rapporteur | Name | Yifei Yuan |
|  | Company | ZTE Corporation |
|  | Email | yifei.yuan@ztetx.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

RAN1#94:

- Capture Section 2 and Annex in R1-1809974 into TR38.812.

- The following table for computation complexity analysis of the receiver as the starting point, entries can be updated till RAN1#94bis.

Table I	 Template of Receiver Computation Complexity breakup

| Receiver component | Detailed component | Computation in parametric number of usages, O(.) analysis, [impact factor] |  |  |
| --- | --- | --- | --- | --- |
|  |  | Receiver type 1 | Receiver type 2 | … |
| Detector | UE detection |  |  |  |
|  | Channel estimation |  |  |  |
|  | Rx combining, if any |  |  |  |
|  | Covariance matrix calculation, if any |  |  |  |
|  | Demodulation weight computation, if any |  |  |  |
|  | UE ordering, if any |  |  |  |
|  | Demodulation, if any |  |  |  |
|  | Soft information generation, if any |  |  |  |
|  | Soft symbol reconstruction, if any |  |  |  |
|  | Message passing, if any |  |  |  |
|  | Others |  |  |  |
| Decoder | LDPC decoding |  |  |  |
| Interference cancellation | Symbol reconstruction(Including FFT operations for DFT-S-OFDM waveform), if any |  |  |  |
|  | LLR to probability conversion, if any |  |  |  |
|  | Interference cancellation |  |  |  |
|  | LDPC encoding, if any |  |  |  |
|  | Others |  |  |  |

- The impact factor is to be estimated based on the analysis of computation, memory size, hardware and software implementation, etc.

  - If/How and which entries are to be combined/compared in order to get the total complexity estimate is FFS.

- Companies may provide the impact factor

  - The impact factor is for each cell

- The rows in the above table are subject to potential re-finement, e.g., adding new row(s), merge some rows, etc.

- Note: the numbers may or may not be a function of UL waveform

- FFS whether or not to add row(s) for memory blocks

- Consider mechanism to handle or mitigate the collision on MA signature/RS/resource, if needed

- FFS whether the number of configured MA signature/RS/resource from UE perspective can be 1 or multiple

- FFS whether multiple sets of MA signature/RS/resource can be configured to a UE

- An exemplary list of simulation cases are included in the companion spreadsheet ‘template 1’ in R1-1809789, for initial collection of BLER vs. SNR curves.

  - Companies can select among the list of simulation conditions in templates 1 & 2 when performing initial link level simulations

    - Companies are encouraged to simulate enough cases to support a broad understanding for scenarios under study in NOMA

    - Additional simulation cases may be captured in template 1.

  - For unequal SNR distribution within range [x - a, x + a] (dB), per UE SNR is the average SNR in dB, i.e. x (dB)

- Adopt the companion spreadsheet ‘template 2’ in R1-1809789 as the template for collecting the initial evaluation results of per UE SNR at the target BLER level (in addition to BLER vs. SNR curve).

- CM/PAPR results as proposed for ‘template 3’ in R1-1809789 can be collected

  - It is FFS how the CM/PAPR relates to UE performance tradeoffs, PA backoff, and UE power saving

  - It is FFS how to compare the CM/PAPR results using this template, e.g. modulation order should be aligned or not

  - PAPR is reported as the CCDF of instantaneous power divided by mean power over all the samples.

- Refine the relative SINR and/or I(inter-cell interference)NR values used for link level simulations to reflect those observed in a cell, if needed

- The extent of the refinement, if any, is to be determined according to evaluations

- Further study how many NOMA UEs can be multiplexed in the same PRBs in practical multi-cell deployments by system-level evaluations, taking inter-cell interference and per UE performance into account.

- Adopt the calibration results in R1-1809790, to be captured in TR38.812.

  - To update the legends of the results by replacing companies’ names with the reference indexing

  - Further update is possible

- Further alignment on the decoding algorithm for NOMA evaluations can be considered in the future, if necessary.

  - Note: this is not intended to revise the corresponding calibration results

- Determine the value y for the evaluation with non-zero timing offset (including asynchronous)

  - For Case 1: y = NCP/2

  - For Case 2: y = 1.5*NCP

- For random MA signature (including RS) in LLS, companies report the details of the chosen Option(s):

    - Opt 1: Fixed number of UEs, with each UE randomly selects a MA signature from a pre-configured MA signature pool

      - Number of potential UEs and the pool size should be reported

    - Opt 2: Fixed number of randomly activated UEs, with each potential UE’s MA signature pre-configured.

      - Number of potential UEs and the pool size should be reported

    - Realistic UE/MA signature detection should be performed.

    - DMRS extension, if any

    - FFS whether to align the pool size for performance evaluations

- Adopt the additional assumptions in the following table for SLS calibration:

| Parameters | Assumptions |
| --- | --- |
| Minimum distance between UE and BS | 35m for Case 1, 10m for Case 2 and Case 3 |
| Building penetration loss | Follow the evaluation assumptions |
| TXRU mapping to antenna elements on BS side | One TXRU per vertical dimension per polarization |
| TXRU mapping weights on BS side | TXRU virtualization only in the vertical dimension, i.e., sub-array partition model with 1D virtualization, refer to TR36.897 |
| BS Tx power | 46 dBm for 500m/1732m ISD; 41 dBm for 200m ISD |
| Polarized antenna modeling | Model-2 in TR36.873 |
| Bandwidth | 10MHz |
| UE antenna configuration | 1 (vertical polarization) |
| UT array orientation | uniformly distributed on [0,360] degree |
| UE receiver noise figure | 9dB |
| UE distribution and antenna height | Follow the evaluation assumptions |
| Handover margin | 0dB |
| Wrapping method | Geographical distance based wrapping |
| UE attachment | Based on RSRP |

- The target higher layer system PDR to be used to evaluate the supported system capability in terms of high layer system PAR for mMTC or eMBB scenarios is 1%

- The target percentage of users satisfying reliability and latency requirements to be used to evaluate the supported system capability in terms per UE PAR for URLLC scenario is 95%

- At least for eMBB, companies are encouraged to report the distribution of PDCCH RB utilization (including PDCCH outage) in each slot at each gNB, which can be used for representing the signalling overhead for grants.

  - If used, companies report the system bandwidth and the scheduling mechanism used in simulation.

- The simulation should contain both DL and UL transmissions

- For performance metric of Sum throughput v.s. SNR at given BLER level, for a given pair of {per UE SE, # of UEs}

- Same TBS is assumed for each curve

- Total SNR is used

- Total SNR is simplified as sum of average SNR conducted in dB, i.e. x+10*log10(N) (dB), where N is the number of UEs

- Confirm frequency offset for 4GHz carrier frequency as

    - uniform distribution between -140 and 140 Hz

- For non-zero timing offset (for  asynchronous)

    - For all UEs in Case 1 or all UEs in Case 2, TO values for each UE for each transmission are i.i.d from uniform distribution [0, y], and independent between UEs.

    - For mixed sync and async, X% of UEs with zero TO and (100–X)% with non-zero TO.

      - X = 80

      - Other values are not precluded

    - Note: Companies should provide the details of receiver structure and TO estimation.

- The packet size for NOMA evaluations in eMBB scenario

  - 50~600 bytes Pareto distribution, with shaping parameter alpha = 1.5

- Clarification of ‘‘8 dBi, including 3dB cable loss” for both system-level evaluation and calibration as

  - 8dBi, 0dB cable loss

- In the case of packet segmentation, use 5 bytes packet segmentation overhead for each TB in the SLS evaluation of the NoMA schemes.

  - Company report the details of packet segmentation

#### 2.1.2	Remaining Open issues

- Transmitter side signal processing schemes for non-orthogonal multiple access [RAN1]

- Receivers for non-orthogonal multiple access [RAN1]

- Procedures related to the non-orthogonal multiple access  [RAN1]

- Link and system level performance evaluation [RAN1]

## 2.2	RAN2

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

#### 2.4.2	Remaining Open issues

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

#### 2.5.4	Estimated Level of Completion

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT. Based on the joint RAN-SA#80 session the items relevant for this section are shown in http://www.3gpp.org/ftp/tsg_sa/TSG_SA/TSGS_80/Docs/SP-180561.zip and summarized below:

| Work Area | SA WIDs | Rapporteurs | RAN WIDs | Rapporteurs |
| --- | --- | --- | --- | --- |
| URLLC for 5G | FS_5G_URLLC FS_Vertical_LAN cyberCAV | SA2: Hui.ni@huawei.com SA2: Devaki.chandramouli@nokia.com SA1: joachim.walewski@siemens.com | FS_NR_L1enh_URLLC FS_NR_unlic FS_NR_IIOT | RAN1: chengyan.cheng@huawei.com RAN1: jingsun@qti.qualcomm.com RAN2: dawid.koziol@nokia.com |
| V2X for 5G | FS_eV2XARC | SA2: laeyoung.kim@lge.com SA1: sungduck.chun@lge.com | FS_NR_V2X | RAN1: Hanbyul.seo@lge.com Matthew.webb@huawei.com |
| Positioning | FS_eLCS 5G_HYPOS | SA2: aiming@catt.cn SA1: lionel.ries@esa.int | FS_NR_POS | RAN1:alexey.khoryaev@INTEL.COM asbjorn.grovlen@ERICSSON.COM |
| UE Capabilities | FS_RACS | SA2: harisz@qti.qualcomm.com | FS_RACS_RAN | RAN2: alex.hsu@mediatek.com |
| 5G Satellite Aspects | FS_5GSAT_ARCH 5GSAT | SA2: cyril.michel@thalesaleniaspace.com SA1: cyril.michel@thalesaleniaspace.com | FS_NR_NTN_solutions | RAN3: nicolas.chuberre@thalesaleniaspace.com |

RAN and SA rapporteurs are requested to fill in these clauses jointly.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN1#94:

- R1-1808150	Text proposal to capture agreements made in RAN1#93 to TR 38.812 (NOMA)	ZTE

- R1-1809753 	Discussion on the design of SCMA	Huawei, HiSilicon

- R1-1809148	Transmitter design for uplink NOMA	NTT DOCOMO, INC.

- R1-1809434	Transmitter side signaling processing schemes for NOMA	Qualcomm Incorporated

- R1-1808151	Transmitter side designs for NOMA	ZTE

- R1-1808230	Discussion on NOMA transmitters	vivo

- R1-1808331	Consideration on the transmitter side signal processing for NOMA	Sony

- R1-1808386	NOMA transmitter side signal processing	CATT

- R1-1808499	Transmitter side signal processing schemes for NCMA	LG Electronics

- R1-1808563	Discussion on NOMA Schemes for NR	NEC

- R1-1808677	NOMA transmission scheme	Intel Corporation

- R1-1809499	Transmitter side signal processing schemes for NOMA	Samsung

- R1-1808834	Discussion on NoMA transmitter schemes	CMCC

- R1-1809515	New Coding Scheme for NOMA with Traffic Control	Sequans Communications

- R1-1808955	Discussion on general symbol-to-resource mapping for NR-NOMA	Fraunhofer HHI, Fraunhofer IIS

- R1-1808968	Considerations on NOMA Transmitter	Nokia, Nokia Shanghai Bell

- R1-1808975	TX schemes for NoMA	Ericsson

- R1-1809056	On Transmitter Side signal Processing for NOMA	AT&T

- R1-1809078	Discussion on Transmit Timing Offset for NOMA Evaluation 	InterDigital, Inc

- R1-1808231	Receiver design considerations for NOMA	vivo

- R1-1808969	Receiver considerations for UL NOMA	Nokia, Nokia Shanghai Bell

- R1-1809180	Discussion on Receiver for NOMA	KDDI Corporation

- R1-1808050	Discussion on the design of NoMA receiver	Huawei, HiSilicon

- R1-1809704	Multi-user advanced receivers for NOMA	ZTE

- R1-1808387	Discussion on NOMA receivers	CATT

- R1-1808500	Considerations on the receiver types and NoMA schemes	LG Electronics

- R1-1808678	NOMA receiver structure	Intel Corporation

- R1-1808761	Receivers for NOMA	Samsung

- R1-1808976	Receiver design for NOMA	Ericsson

- R1-1809057	Receiver techniques for NOMA	AT&T

- R1-1809079	Complexity Analysis of IDMA NOMA Reception 	InterDigital, Inc.

- R1-1809496	Receiver for uplink NOMA	NTT DOCOMO, INC.

- R1-1809435	Receivers for NOMA	Qualcomm Incorporated

- R1-1808153	Procedures related to NOMA	ZTE

- R1-1808288	On NOMA procedures for Grant Free	Orange

- R1-1809058	On NOMA Procedures	AT&T

- R1-1808762	Procedures related consideration to NOMA	Samsung

- R1-1808051	Discussion on NoMA related procedures	Huawei, HiSilicon

- R1-1808232	Discussion on NOMA procedures	vivo

- R1-1808290	Considerations on procedures related to NOMA	Fujitsu

- R1-1808332	Considerations on NOMA related procedures	Sony

- R1-1808388	Discussion on NOMA procedures	CATT

- R1-1808501	Discussion on procedures related to NOMA	LG Electronics

- R1-1808564	Considerations on NOMA related procedures	NEC

- R1-1808679	NOMA related procedure	Intel Corporation

- R1-1808835	Discussion on NoMA related procedures	CMCC

- R1-1808915	Consideration on NoMA procedure	Guangdong OPPO Mobile Telecom

- R1-1808970	Procedures to be considered for NOMA operation	Nokia, Nokia Shanghai Bell

- R1-1808977	Procedures for NoMA	Ericsson

- R1-1809080	On NOMA Related Procedures in NR 	InterDigital, Inc.

- R1-1809150	Extended DMRS design and NOMA related procedures	NTT DOCOMO, INC.

- R1-1809181	Discussion on procedures related to NOMA 	KDDI Corporation

- R1-1809190	Discussion on NOMA related procedures	ITRI

- R1-1809283	Considerations on Procedures of Non-orthogonal Multiple Access Schemes	CAICT

- R1-1809316	Considerations on MA Preamble Design for NOMA	KT Corp.

- R1-1809379	Design consideration on NOMA procedure 	Convida Wireless LLC

- R1-1809436	Procedures related to NOMA	Qualcomm Incorporated

- R1-1809487	NOMA Procedure Considerations 	Sierra Wireless, S.A.

- R1-1808052	Further discussion on NoMA evaluations	Huawei, HiSilicon

- R1-1808154	Preliminary system-level simulation results for NOMA	ZTE

- R1-1808204	Link-Level Simulation Results for ACMA with Synchronous, Asynchronous transmission	HUGHES Network Systems Ltd

- R1-1808205	Partially Asynchronous and Multi-stream Transmission of ACMA	HUGHES Network Systems Ltd

- R1-1808233	Evaluations for NOMA	vivo

- R1-1808389	Link level and initial system level evaluation results of NOMA	CATT

- R1-1808502	Further LLS evaluation for NCMA in mMTC	LG Electronics

- R1-1808680	Performance evaluation for NOMA	Intel Corporation

- R1-1809500	Performance evaluation for NoMA	Samsung

- R1-1808971	Evaluations for NOCA	Nokia, Nokia Shanghai Bell

- R1-1808978	Evaluation methodology and metrics for NoMA	Ericsson

- R1-1809059	Link Level Performance for NOMA	AT&T

- R1-1809081	System Level Simulation Methodology in mMTC Scenario 	InterDigital, Inc.

- R1-1809151	Link and system level simulation results and analysis of UL NOMA	NTT DOCOMO, INC.

- R1-1809284	Further LLS results of NOMA schemes	CAICT

- R1-1809437	System level evaluation for NOMA	Qualcomm Incorporated

- R1-1809438	Link level evaluation for NOMA	Qualcomm Incorporated

- R1-1808053	Evaluation results for OMA calibration	Huawei, HiSilicon

- R1-1808054	Evaluation results for mMTC scenario	Huawei, HiSilicon

- R1-1808055	Evaluation results for URLLC scenario	Huawei, HiSilicon

- R1-1808056	Evaluation results for eMBB scenario	Huawei, HiSilicon

- R1-1808155	Link-level simulation results for mMTC scenraio	ZTE

- R1-1808156	Link-level simulation results for URLLC scenraio	ZTE

- R1-1808157	Link-level simulation results for eMBB scenraio	ZTE

- R1-1808158	Physical layer abstraction for NOMA SLS	ZTE

- R1-1808390	Link level evaluation results of OMA	CATT

- R1-1808391	System level calibration of NOMA	CATT

- R1-1808972	Initial Link Level Simulation Results of IDMA	Nokia, Nokia Shanghai Bell

- R1-1808979	DMRS design for NoMA	Ericsson

- R1-1808980	Impairments model for link and system evaluations	Ericsson

- R1-1808981	Link level evaluations for MU-MIMO	Ericsson

- R1-1808982	Observations on NOMA traffic models	Ericsson

- R1-1808983	MU-MIMO operation in NR with configured grants	Ericsson

- R1-1809082	On the impact of timing offset in NOMA Evaluation	InterDigital, Inc.

- R1-1809083	Aspects of IDMA NOMA Transmitter 	InterDigital, Inc.

- R1-1809084	Link Level Simulation Results on IDMA OFDM/DFT-s-OFDM 	InterDigital, Inc.

- R1-1809152	LLS evaluation results of UGMA	NTT DOCOMO, INC.

- R1-1809182	LLS evaluation results for OMA calibration	KDDI Corporation

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template