3GPP TSG RAN Meeting #81	RP-181764

Gold Coast, Australia, Sep. 10 - 13, 2018

Agenda item:		9.3.2

Source:	CATT

Title:	Self-Evaluation on Link Budget for eMBB

Document for:   Discussion and Decision

1	Introduction

According to Report ITU-R M.2411, the proponent of IMT-2020 should provide link budget template to ITU-R as one of the submission templates. One email discussion on link budget was conducted in 3GPP ITU-R AdHoc reflector after RAN1#93 meeting [1] and the parameters and configuration scenarios were offline discussed during RAN1#94 meeting while there was not complete conclusion and results collection yet. In this contribution we provide our considerations on the parameters and configurations focus on eMBB, and preliminary link budget evaluation resuls of NR are also provided.

2.		Considerations on evaluation assumptions

In this contribtion, we are fucus on the three eMBB test environments, Indoor Hotspot-eMBB, Dense Urban-eMBB and Rural-eMBB. The evaluation assumptions are refer to the email discussion and offline discussion, and some proposed evalution configurations for the initial evaluation are summaried in Table 1.

Table 1 Evalution configurations applied for link budget

|  | Configuration | Note |
| --- | --- | --- |
| Test environment | Indoor Hotspot-eMBB Dense Urban-eMBB Rural-eMBB | For the initial evaluation, fucus on the eMBB test environments |
| System configuration | NR 4GHz with 30KHz SCS for Indoor Hotspot-eMBB and Dense Urban-eMBB NR 700MHz with 30KHz SCS for Rural-eMBB For PDCCH, the SCS is 15KHz | For the initial evaluation, fucus on the FR1 and 30KHz SCS |
| Subframe configuration | TDD, DSUUD S slot = (11DL:1GP:2UL) | This is the same as that in 4G evaluation. |
| Physical channel | DL: PDCCH, PDSCH UL: PUCCH, PUSCH |  |
| Transmission bit data for date channel | 3 times of the 4G TDD evaluation assumption | This is aligned with the offline discussion during RAN#94 |
| Spectral efficiency | Calculated based on the transmission bit rate and bandwidth. | Considering the effctive bandwidth based on the subframe configuraion. For guard period (GP), 50% of GP symbols are considered as downlink overhead, and 50% of GP symbols are considered as uplink overhead. It is noted that the required SE is different with different subframe configurations. |
| DL occupied channel bandwidth | 20MHz | It is noted that, for InH environment, the required SE is almost 6 times of that in the 4G evaluation since the evaluated bandwidth is half of that in 4G evaluation, while for other scenarios, the required SE is almost 3 times of that in the 4G evaluation. |
| UL occupied channel bandwidth | 1 PRB for control channel and 4 PRB for data channel (the minimum granularity for scheduling in frequency domain is 4PRB) | It is noted that, for InH environment, the required SE is almost 3 times of that in the 4G evaluation since the actual bandwidth is similar while for DU and Rural enviorment, the evaluated bandwidth is about twice of that in 4G evaluation, i.e. SE (using 30KHz) would be about 1.5 times of 4G in the same scenario. |
| Channel state | NLOS O2I NLOS ( For Dense Urban-eMBB and Rural-eMBB) | For the initial evaluation, fucus on the more challanging cases |
| Cable loss for BS | 0dB or 3dB | Considering different BS antenna types |
| Penetration loss + shadow fading margin | The penetraion margin is the expectation of the penetraion. The impact of the penetration standard deviation (SD) is considered into the calculation of shadow fading margin. The shadow fading margin is calculated by path loss model slope, the integrated SD of shadow fading SD and penetration SD, and target area coverage reliability (ACR). | Refer to [2][3], which are the same evaluation methodologies as those in 4G evaluation. |

3	Evaluation results

In this contribution, we discussed our considerations on evaluation assumptions of link budget and the attached initial evaluation resuls are provided.

For the PDCCH, the evaluation results based on both of the ideal channel estimation and non-ideal channel estimation are provided, and the evalution results base on the non-ideal channel estimation could be futher improved in future. For all other channels, non-ideal channel estimation are applied.

For the cable loss of BS, the values of (8) and (12) in the attached tables can be selected between 0 and 3.

Attachments:

1. link budget template - Channel Model A

2. link budget template - Channel Model B

References

- R1-1808936, “Summary of email discussion on IMT-2020 self evaluation for link budget”, Huawei.

- R1-1808415, “Discussions on calculation of shadow fading margin for link budget”, CATT.

- R1-1808416, “Discussions on penetration margin and SD for link budget”, CATT.