5	Self evaluation of eMBB technical performance

[Editor’s note: The grouping of the technical performance requirements for eMBB usage scenario are based on Report ITU-R M.2410 “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”. However, it does not imply that each of the following technical performance requirements is only related to eMBB. It is just the minimum requirement that shall be evaluated under eMBB. This should also be noted for Section 6 and 7.]

5.3	Average and 5th percentile user spectral efficiency

As defined in Report ITU-R M.2410, average spectral efficiency is the aggregate throughput of all users (the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time) divided by the channel bandwidth of a specific band divided by the number of TRxPs and is measured in bit/s/Hz/TRxP.

The 5th percentile user spectral efficiency is the 5% point of the CDF of the normalized user throughput. The normalized user throughput is defined as the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time, divided by the channel bandwidth and is measured in bit/s/Hz.

As required by Report ITU-R M.2412, average spectral efficiency and 5th percentile user spectral efficiency are assessed jointly using the same simulation.

5.3.1	NR

Average spectral efficiency and 5th percentile user spectral efficiency are evaluated for NR.

5.3.1.1	Indoor Hotspot – eMBB

Evaluation configuration A (carrier frequency = 4 GHz) and evaluation configuration B (carrier frequency = 30 GHz) with either 12TRxP or 36TRxP cases are applied for the evaluations of Indoor Hotspot– eMBB test environment for NR. Both NR FDD and TDD are evaluated. A wide range of antenna configurations and transmission schemes are considered. Detailed evaluation assumptions and results can be found in RP-181816.

5.3.1.1.1	Evaluation configuration A (CF = 4 GHz)

The evaluation results of DL spectral efficiency for NR FDD and NR TDD for evaluation configuration A with 12TRxP are provided in Table 5.3.1.1.1-1.

It is noted that for NR, the component carrier bandwidth can be larger than the simulation bandwidth as defined in Report ITU-R M.2412. For those larger bandwidth cases, the guard band ratio can be further decreased; and the OFDM symbol occupation by PDCCH is expected to be reduced, even to less than one OFDM symbol due to the capability of PDCCH resource sharing with PDSCH.

Such capabilities are evaluated in Indoor Hotspot - eMBB test environment as well as in Dense Urban – eMBB test environment and Rural – eMBB test environment, for DL spectral efficiency. The values of the larger bandwidths are shown in respective tables.

Table 5.3.1.1.1-1 DL spectral efficiency for NR in Indoor Hotspot – eMBB 
(Evaluation configuration A, CF=4 GHz, for 12TRxP)

(a) NR FDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | ITU Requirement |  | Channel model A |  |  |  | Channel model B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Number of samples | BW= 10MHz | BW= 20MHz | BW= 40MHz | Number of samples | BW= 10MHz | BW= 20MHz | BW= 40MHz |
| 32x4 MU-MIMO Type II Codebook (32Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (4,4,2,1,1;4,4) | 15 | Average [bit/s/Hz/TRxP] | 9 | 10 | 10.83 | 12.25 | 13.10 | 5 | 11.20 | 12.67 | 13.54 |
|  |  | 5th-tile [bit/s/Hz] | 0.3 |  | 0.36 | 0.41 | 0.43 |  | 0.38 | 0.42 | 0.45 |
| 32x4 MU-MIMO Type I Codebook (32Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (4,4,2,1,1;4,4) | 15 | Average [bit/s/Hz/TRxP] | 9 | 1 | 11.50 | 13.50 | 13.97 | / | / | / | / |
|  |  | 5th-tile [bit/s/Hz] | 0.3 |  | 0.31 | 0.35 | 0.38 |  | / | / | / |
| 32x4 MU-MIMO Ideal CSI feedback (32Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (4,4,2,1,1;4,4) | 15 | Average [bit/s/Hz/TRxP] | 9 | / | / | / | / | 1 | 12.23 | 13.89 | 14.87 |
|  |  | 5th-tile [bit/s/Hz] | 0.3 |  | / | / | / |  | 0.49 | 0.55 | 0.59 |
| 32x8 MU-MIMO Type II Codebook (32Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (4,4,2,1,1;4,4) | 15 | Average [bit/s/Hz/TRxP] | 9 | 1 | 9.80 | 10.99 | 11.70 | / | / | / | / |
|  |  | 5th-tile [bit/s/Hz] | 0.3 |  | 0.35 | 0.39 | 0.42 |  | / | / | / |

(b) NR TDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement |  | Channel model A |  |  |  | Channel model B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Number of samples | BW= 20MHz | BW= 40MHz | BW= 100MHz | Number of samples | BW= 20MHz | BW= 40MHz | BW= 100MHz |
| 32x4 MU-MIMO, 4T SRS; (32Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (4,4,2,1,1;4,4) | 30 | DDDSU | Average [bit/s/Hz/TRxP] | 9 | 1 | 12.97 | 15.20 | 16.73 | 3 | 12.13 | 14.18 | 15.57 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.3 |  | 0.38 | 0.44 | 0.49 |  | 0.42 | 0.49 | 0.54 |
| 32x4 MU-MIMO, 4T SRS; (32Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (4,4,2,1,1;4,4) | 15 | DDDSU | Average [bit/s/Hz/TRxP] | 9 | 1 | 12.77 | 14.65 | / | 1 | 12.77 | 14.64 | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.3 |  | 0.39 | 0.45 | / |  | 0.40 | 0.46 | / |
| 32x4 MU-MIMO, Type II Codebook (32Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (4,4,2,1,1;4,4) | 15 | DSUU | Average [bit/s/Hz/TRxP] | 9 | 2 | 11.02 | 12.81 | / | 1 | 13.55 | 15.50 | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.3 |  | 0.41 | 0.48 | / |  | 0.50 | 0.58 | / |
| 32x4 MU-MIMO, 4T SRS (32Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (4,4,2,1,1;4,4) | 30 | DSUUD | Average [bit/s/Hz/TRxP] | 9 | 2 | 11.61 | 13.68 | 15.08 | 2 | 11.96 | 14.09 | 15.53 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.3 |  | 0.41 | 0.48 | 0.53 |  | 0.45 | 0.53 | 0.59 |
| 32x4 MU-MIMO, 4T SRS (32Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (4,4,2,1,1;4,4) | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 9 | 3 | 11.73 | 13.46 | / | 3 | 12.34 | 14.04 | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.3 |  | 0.39 | 0.45 | / |  | 0.42 | 0.48 | / |
| 32x4 MU-MIMO, 4T SRS (32Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (4,4,2,1,1;4,4) | 30 | DDDDD DDSUU | Average [bit/s/Hz/TRxP] | 9 | 1 | 9.92 | 11.69 | 12.90 | / | / | / | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.3 |  | 0.31 | 0.37 | 0.41 |  | / | / | / |
| 32x8 MU-MIMO, 8T SRS (32Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (4,4,2,1,1;4,4) | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 9 | 1 | 10.05 | 11.58 | / | / | / | / | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.3 |  | 0.45 | 0.52 | / |  | / | / | / |

The evaluation results of DL spectral efficiency for NR FDD and NR TDD for evaluation configuration A with 36TRxP are provided in Table 5.3.1.1.1-2.

It is observed that both NR FDD and NR TDD fulfill the DL spectral efficiency requirement for all the above configurations in evaluation configuration A.

Table 5.3.1.1.1-2 DL spectral efficiency for NR in Indoor Hotspot – eMBB 
(Evaluation configuration A, CF=4 GHz, for 36TRxP)

(a) NR FDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | ITU Requirement |  | Channel model A |  |  |  | Channel model B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Number of samples | BW= 10MHz | BW= 20MHz | BW= 40MHz | Number of samples | BW= 10MHz | BW= 20MHz | BW= 40MHz |
| 32x4 MU-MIMO Type II Codebook (256Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (8,16,2,1,1;2,8) | 15 | Average [bit/s/Hz/TRxP] | 9 | 7 | 12.06 | 13.64 | / | 4 | 13.12 | 14.83 | / |
|  |  | 5th-tile [bit/s/Hz] | 0.3 |  | 0.35 | 0.40 | / |  | 0.35 | 0.39 | / |
| 32x4 MU-MIMO ideal CSI feedback (128Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;4,4) | 15 | Average [bit/s/Hz/TRxP] | 9 | / | / | / | / | 1 | 14.08 | 15.99 | / |
|  |  | 5th-tile [bit/s/Hz] | 0.3 |  | / | / | / |  | 0.45 | 0.52 | / |
| 32x4MU-MIMO Type I codebook (32Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (4,4,2,1,1;4,4) | 15 | Average [bit/s/Hz/TRxP] | 9 | 1 | 9.51 | 10.67 | / | / | / | / | / |
|  |  | 5th-tile [bit/s/Hz] | 0.3 |  | 0.28 | 0.31 | / |  | / | / | / |

(b) NR TDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement |  | Channel model A |  |  |  | Channel model B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Number of samples | BW= 20MHz | BW= 40MHz | BW= 100MHz | Number of samples | BW= 20MHz | BW= 40MHz | BW= 100MHz |
| 32x4 MU-MIMO, 4T SRS; (256Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (8,16,2,1,1;2,8) | 30 | DDDSU | Average [bit/s/Hz/TRxP] | 9 | 1 | 14.22 | 16.68 | 18.35 | 1 | 14.33 | 16.81 | 18.50 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.3 |  | 0.35 | 0.41 | 0.45 |  | 0.36 | 0.42 | 0.46 |
| 32x4 MU-MIMO, 4T SRS; (256Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (8,16,2,1,1;2,8) | 15 | DDDSU | Average [bit/s/Hz/TRxP] | 9 | 1 | 14.56 | 16.71 | / | 1 | 14.67 | 16.84 | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.3 |  | 0.39 | 0.44 | / |  | 0.39 | 0.45 | / |
| 32x4 MU-MIMO, Type II Codebook (256Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (8,16,2,1,1;2,8) | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 9 |  | 10.38 | 12.35 | / | / | / | / | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.3 | 1 | 0.33 | 0.40 | / |  | / | / | / |
| 32x4 MU-MIMO, Type II Codebook (256Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (8,16,2,1,1;2,8) | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 9 | 2 | 12.16 | 13.88 | / | 2 | 12.37 | 14.12 | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.3 |  | 0.35 | 0.40 | / |  | 0.33 | 0.38 | / |
| 32x4 MU-MIMO, Type II Codebook (256Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (8,16,2,1,1;2,8) | 30 | DSUUD | Average [bit/s/Hz/TRxP] | 9 | 1 | 13.00 | 15.18 | 16.66 | 1 | 13.17 | 15.38 | 16.88 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.3 |  | 0.32 | 0.37 | 0.40 |  | 0.34 | 0.39 | 0.43 |
| 32x4 MU-MIMO, 4T SRS (128Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;2,8) | 30 | DDDSU | Average [bit/s/Hz/TRxP] | 9 | / | / | / | / | 1 | 10.48 | 12.16 | 13.30 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.3 |  | / | / | / |  | 0.31 | 0.36 | 0.40 |

The evaluation results of UL spectral efficiency for NR FDD and TDD for evaluation configuration A with 12TRxP are provided in Table 5.3.1.1.1-3.

Table 5.3.1.1.1-3 UL spectral efficiency for NR in Indoor Hotspot – eMBB 
(Evaluation configuration A, CF=4 GHz, for 12TRxP)

(a) NR FDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | ITU Requirement |  | Channel model A |  | Channel model B |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Number of samples | BW=10MHz | Number of samples | BW=10MHz |
| 2x32 SU-MIMO, OFDMA; (32Rx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (4,4,2,1,1;4,4) | 15 | Average [bit/s/Hz/TRxP] | 6.75 | 3 | 8.75 | 1 | 8.87 |
|  |  | 5th-tile [bit/s/Hz] | 0.21 |  | 0.52 |  | 0.55 |
| 4x32 MU-MIMO, OFDMA; (32Rx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (4,4,2,1,1;4,4) | 15 | Average [bit/s/Hz/TRxP] | 6.75 | 2 | 10.64 | / | / |
|  |  | 5th-tile [bit/s/Hz] | 0.21 |  | 0.49 |  | / |
| 4x32 SU-MIMO, OFDMA(32Rx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (4,4,2,1,1;4,4) | 15 | Average [bit/s/Hz/TRxP] | 6.75 | 3 | 8.88 | 4 | 8.17 |
|  |  | 5th-tile [bit/s/Hz] | 0.21 |  | 0.55 |  | 0.49 |

(b) NR TDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement |  | Channel model A |  | Channel model B |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Number of samples | BW=20MHz | Number of samples | BW=20MHz |
| 2x32 SU-MIMO, Codebook based; (32Rx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (4,4,2,1,1;4,4) | 30 | DDDSU | Average [bit/s/Hz/TRxP] | 6.75 | 1 | 6.95 | 2 | 7.70 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.21 |  | 0.39 |  | 0.44 |
| 2x32 SU-MIMO, Codebook based; (32Rx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (4,4,2,1,1;4,4) | 15 | DDDSU | Average [bit/s/Hz/TRxP] | 6.75 | 1 | 7.17 | 1 | 7.26 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.21 |  | 0.40 |  | 0.41 |
| 4x32 SU-MIMO, Codebook based; (32Rx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (4,4,2,1,1;4,4) | 30 | DDDSU | Average [bit/s/Hz/TRxP] | 6.75 | 1 | 7.58 | 1 | 7.62 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.21 |  | 0.43 |  | 0.43 |
| 4x32 SU-MIMO, Codebook based; (32Rx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (4,4,2,1,1;4,4) | 15 | DDDSU | Average [bit/s/Hz/TRxP] | 6.75 | 1 | 7.87 | 1 | 7.95 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.21 |  | 0.44 |  | 0.45 |
| 4x32 MU-MIMO, Codebook based; (32Rx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (4,4,2,1,1;4,4) | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 6.75 | 1 | 12.91 | / | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.21 |  | 0.55 |  | / |
| 4x32 SU-MIMO, Non-Codebook based; (32Rx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (4,4,2,1,1;4,4) | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 6.75 | 1 | 9.57 | 1 | 9.26 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.21 |  | 0.60 |  | 0.56 |
| 4x32 SU-MIMO, Non-Codebook based; (32Rx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (4,4,2,1,1;4,4) | 30 | DSUUD | Average [bit/s/Hz/TRxP] | 6.75 | 1 | 9.25 | 1 | 8.99 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.21 |  | 0.58 |  | 0.52 |
| 4x32 SU-MIMO, Codebook based; (32Rx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (4,4,2,1,1;4,4) | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 6.75 | 1 | 8.99 | 1 | 8.65 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.21 |  | 0.63 |  | 0.54 |
| 4x32 SU-MIMO, Codebook based; (32Rx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (4,4,2,1,1;4,4) | 30 | DSUUD | Average [bit/s/Hz/TRxP] | 6.75 | 1 | 8.58 | 1 | 8.12 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.21 |  | 0.57 |  | 0.48 |
| 2x32 SU-MIMO, Codebook based; (32Rx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (4,4,2,1,1;4,4) | 30 | DSUUD | Average [bit/s/Hz/TRxP] | 6.75 | 1 | 8.26 | 1 | 9.35 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.21 |  | 0.53 |  | 0.53 |

The evaluation results of UL spectral efficiency for NR FDD and TDD for evaluation configuration A with 36TRxP are provided in Table 5.3.1.1.1-4.

It is observed that both NR FDD and NR TDD fulfill the UL spectral efficiency requirement for all the above configurations in evaluation configuration A.

Table 5.3.1.1.1-4 UL spectral efficiency for NR in Indoor Hotspot – eMBB 
(Evaluation configuration A, CF=4 GHz, for 36TRxP)

(a) NR FDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | ITU Requirement |  | Channel model A |  | Channel model B |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Number of samples | BW=10MHz | Number of samples | BW=10MHz |
| 4x32 MU-MIMO, OFDMA; (256Rx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (8,16,2,1,1;2,8) | 15 | Average [bit/s/Hz/TRxP] | 6.75 | 1 | 15.17 | / | / |
|  |  | 5th-tile [bit/s/Hz] | 0.21 |  | 0.47 |  | / |
| 2x32 MU-MIMO, OFDMA; (32Rx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (4,4,2,1,1;4,4) | 15 | Average [bit/s/Hz/TRxP] | 6.75 | 2 | 7.17 | / | / |
|  |  | 5th-tile [bit/s/Hz] | 0.21 |  | 0.27 |  | / |
| 4x32 SU-MIMO, OFDMA(64Rx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (4,8,2,1,1;2,8) | 15 | Average [bit/s/Hz/TRxP] | 6.75 | 1 | 7.20 | 1 | 7.34 |
|  |  | 5th-tile [bit/s/Hz] | 0.21 |  | 0.40 |  | 0.41 |

(b) NR TDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement |  | Channel model A |  | Channel model B |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Number of samples | BW=20MHz | Number of samples | BW=20MHz |
| 4x32 MU-MIMO, (256Rx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (8,16,2,1,1;2,8) | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 6.75 | 1 | 14.04 | / | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.21 |  | 0.43 |  | / |
| 4x32 SU-MIMO, (64Rx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (4,8,2,1,1;2,8) | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 6.75 | 1 | 6.98 | 1 | 7.59 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.21 |  | 0.40 |  | 0.47 |
| 4x32 SU-MIMO, (64Rx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (4,8,2,1,1;2,8) | 30 | DSUUD | Average [bit/s/Hz/TRxP] | 6.75 | 1 | 7.44 | 1 | 7.48 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.21 |  | 0.42 |  | 0.43 |

5.3.1.1.2	Evaluation configuration B (CF = 30 GHz)

The evaluation results of DL spectral efficiency for NR for evaluation configuration B with 12TRxP are provided in Table 5.3.1.1.2-1.

It is noted that for NR, the component carrier bandwidth can be larger than the simulation bandwidth as defined in Report ITU-R M.2412. For those larger bandwidth cases, the guard band ratio can be further decreased; and the OFDM symbol occupation by PDCCH is expected to be reduced, even to less than one OFDM symbol due to the capability of PDCCH resource sharing with PDSCH.

Such capabilities are evaluated for DL spectral efficiency. The values of the larger bandwidths are shown in respective tables.

Table 5.3.1.1.2-1 DL spectral efficiency for NR in Indoor Hotspot – eMBB 
(Evaluation configuration B, CF=30 GHz, for 12TRxP)

(a) NR FDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | ITU Requirement |  | Number of samples | BW=40 MHz |
| --- | --- | --- | --- | --- | --- |
| 32x32 SU-MIMO (2 panel@UE) gNB: (M,N,P,Mg,Ng; Mp,Np) = (4,8,2,1,1;4,4); UE: (M,N,P,Mg,Ng; Mp,Np) = (2,4,2,1,2; 2,4) | 120 | Average [bit/s/Hz/TRxP] | 9 | 1 | 16.12 |
|  |  | 5th-tile [bit/s/Hz] | 0.3 |  | 0.61 |
| 8x16 SU-MIMO (2 panel@UE) gNB: (M,N,P,Mg,Ng; Mp,Np) = (4,8,2,1,1;2,2); UE: (M,N,P,Mg,Ng; Mp,Np) = (2,4,2,1,2; 1,4) | 120 | Average [bit/s/Hz/TRxP] | 9 | 1 | 9.40 |
|  |  | 5th-tile [bit/s/Hz] | 0.3 |  | 0.47 |
| 8x4 MU-MIMO (2 panel@UE) gNB: (M,N,P,Mg,Ng; Mp,Np) = (4,8,2,1,1;2,2) UE: (M,N,P,Mg,Ng; Mp,Np) = (2,4,2,1,2; 1,1) | 120 | Average [bit/s/Hz/TRxP] | 9 | 1 | 8.50 |
|  |  | 5th-tile [bit/s/Hz] | 0.3 |  | 0.31 |

(b) NR TDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement |  | Number of samples | BW= 80MHz / 100 MHz | BW= 200MHz | BW= 400MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32x8 MU-MIMO, 4T SRS (2 panel@UE) gNB: (M,N,P,Mg,Ng; Mp,Np) = (4,4,2,1,1;4,4); UE: (M,N,P,Mg,Ng; Mp,Np) = (2,4,2,1,2; 1,2) | 60 | DDDSU | Average [bit/s/Hz/TRxP] | 9 | 2 | 12.28 | 14.08 | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.3 |  | 0.31 | 0.36 | / |
| 32x4 MU-MIMO, Type II Codebook (2 panel@UE) gNB: (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;4,4); UE: (M,N,P,Mg,Ng; Mp,Np) = (2,4,2,1,2; 1,1) | 120 | DSUUD | Average [bit/s/Hz/TRxP] | 9 | 1 | 8.47 | 9.80 | 10.46 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.3 |  | 0.34 | 0.40 | 0.42 |
| 32x8 MU-MIMO, 4T SRS (2 panel@UE) gNB: (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;2,8); UE: (M,N,P,Mg,Ng; Mp,Np) = (2,4,2,1,2; 1,2) | 60 | DSUUD | Average [bit/s/Hz/TRxP] | 9 | 1 | 14.88 | 17.08 | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.3 |  | 0.72 | 0.83 | / |
| 32x8 MU-MIMO, 4T SRS (2 panel@UE) gNB: (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;2,8); UE: (M,N,P,Mg,Ng; Mp,Np) = (2,4,2,1,2; 1,2) | 120 | DSUUD | Average [bit/s/Hz/TRxP] | 9 | 1 | 14.93 | 16.95 | 17.65 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.3 |  | 0.61 | 0.69 | 0.72 |
| 64x8 MU-MIMO, 4T SRS (2 panel@UE) gNB: (M,N,P,Mg,Ng; Mp,Np) = (8,16,2,1,1;2,16); UE: (M,N,P,Mg,Ng; Mp,Np) = (2,4,2,1,2; 1,2) | 60 | DSUUD | Average [bit/s/Hz/TRxP] | 9 | 1 | 16.75 | 19.22 | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.3 |  | 1.00 | 1.14 | / |
| 64x8 MU-MIMO, 4T SRS (2 panel@UE) gNB: (M,N,P,Mg,Ng; Mp,Np) = (8,16,2,1,1;2,16); UE: (M,N,P,Mg,Ng; Mp,Np) = (2,4,2,1,2; 1,2) | 120 | DSUUD | Average [bit/s/Hz/TRxP] | 9 | 1 | 16.84 | 19.12 | 19.91 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.3 |  | 1.00 | 1.13 | 1.18 |
| 64x16 MU-MIMO, 4T SRS (2 panel@UE) gNB: (M,N,P,Mg,Ng; Mp,Np) = (4,32,2,1,1;1,32); UE: (M,N,P,Mg,Ng; Mp,Np) = (1,4,2,1,2; 1,4) | 60 | DDDDD | Average [bit/s/Hz/TRxP] | 9 | 1 | 14.80 | 16.55 | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.3 |  | 0.37 | 0.41 | / |
| 8x2 , MU-MIMO, 2T SRS (1 panel at UE) gNB: (M,N,P,Mg,Ng; Mp,Np) = (16,8,2,1,1;2,2); UE: (M,N,P,Mg,Ng; Mp,Np) = (4,4,2,1,1; 1,2) | 120 | DDDU | Average [bit/s/Hz/TRxP] | 9 | 1 | 12.03 | 14.40 | 14.97 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.3 |  | 0.49 | 0.58 | 0.60 |

The evaluation results of DL spectral efficiency for NR for evaluation configuration B with 36TRxP are provided in Table 5.3.1.1.2-2.

It is observed that NR fulfills the DL spectral efficiency requirement for all the above configurations in evaluation configuration B.

Table 5.3.1.1.2-2 DL spectral efficiency for NR in Indoor Hotspot – eMBB 
(Evaluation configuration B, CF=30 GHz, for 36TRxP)

(a) NR TDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement |  | Number of samples | BW= 80MHz / 100MHz | BW= 200MHz | BW= 400MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32x4 MU-MIMO Type II Codebook (2 Panels @ UE) gNB: (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;4,4) UE: (M,N,P,Mg,Ng; Mp,Np) = (2,4,2,1,2; 1,1) | 120 | DSUUD | Average [bit/s/Hz/TRxP] | 9 | 1 | 8.69 | 10.04 | 10.72 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.3 |  | 0.29 | 0.34 | 0.36 |
| 32x8 MU-MIMO 4Tx SRS (2 Panels @ UE) gNB: (M,N,P,Mg,Ng; Mp,Np) = (4,16,2,1,1;1,16) UE: (M,N,P,Mg,Ng; Mp,Np) = (2,4,2,1,2; 1,2) | 60 | DSUUD | Average [bit/s/Hz/TRxP] | 9 | 1 | 12.72 | 14.60 | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.3 |  | 0.38 | 0.43 | / |
| 32x8 MU-MIMO 4Tx SRS (2 Panels @ UE) gNB: (M,N,P,Mg,Ng; Mp,Np) = (4,16,2,1,1;1,16) UE: (M,N,P,Mg,Ng; Mp,Np) = (2,4,2,1,2; 1,2) | 120 | DSUUD | Average [bit/s/Hz/TRxP] | 9 | 1 | 12.11 | 13.75 | 14.32 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.3 |  | 0.31 | 0.35 | 0.36 |
| 64x8 MU-MIMO, 4T SRS (2 panel@UE) gNB: (M,N,P,Mg,Ng; Mp,Np) = (8,16,2,1,1;2,16) UE: (M,N,P,Mg,Ng; Mp,Np) = (2,4,2,1,2; 1,2) | 60 | DSUUD | Average [bit/s/Hz/TRxP] | 9 | 1 | 14.85 | 17.05 | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.3 |  | 0.48 | 0.56 | / |
| 64x8 MU-MIMO, 4T SRS (2 panel@UE) gNB: (M,N,P,Mg,Ng; Mp,Np) = (8,16,2,1,1;2,16); UE: (M,N,P,Mg,Ng; Mp,Np) = (2,4,2,1,2; 1,2) | 60 | DSUUD | Average [bit/s/Hz/TRxP] | 9 | 1 | 14.22 | 16.14 | 16.81 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.3 |  | 0.49 | 0.56 | 0.58 |

The evaluation results of UL spectral efficiency for NR for evaluation configuration B with 12TRxP are provided in Table 5.3.1.1.2-3.

Table 5.3.1.1.2-3 UL spectral efficiency for NR in Indoor Hotspot – eMBB 
(Evaluation configuration B, CF=30 GHz, for 12TRxP)

(a) NR FDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | ITU Requirement |  | Number of samples | BW=40MHz |
| --- | --- | --- | --- | --- | --- |
| 8x8 SU-MIMO, CB based, OFDMA (2 panel@UE) gNB: (M,N,P,Mg,Ng; Mp,Np)= (4,8,2,1,1; 2,2) UE: (M,N,P,Mg,Ng; Mp,Np) = (2,4,2,1,2; 1,2) | 120 | Average [bit/s/Hz/TRxP] | 6.75 | 1 | 7.89 |
|  |  | 5th-tile [bit/s/Hz] | 0.21 |  | 0.48 |
| 4x8 MU-MIMO, CB based, OFDMA (2 panel@UE) ; gNB: (M,N,P,Mg,Ng; Mp,Np)= (4,8,2,1,1; 2,2) UE: (M,N,P,Mg,Ng; Mp,Np) = (2,4,2,1,2; 1,1) | 120 | Average [bit/s/Hz/TRxP] | 6.75 | 1 | 7.66 |
|  |  | 5th-tile [bit/s/Hz] | 0.21 |  | 0.39 |

(b) NR TDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement |  | Number of samples | BW= 80MHz |
| --- | --- | --- | --- | --- | --- | --- |
| 8x16 SU-MIMO, CB based, OFDMA (2 panel@UE) gNB: (M,N,P,Mg,Ng; Mp,Np) = (4,8,2,1,1;2,4); UE: (M,N,P,Mg,Ng; Mp,Np) = (2,4,2,1,2; 1,2) | 60 | DDDSU | Average [bit/s/Hz/TRxP] | 6.75 | 1 | 6.08 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.21 |  | 0.23 |
| 8x32 SU-MIMO, CB based, OFDMA (2 panel@UE); gNB: (M,N,P,Mg,Ng; Mp,Np) = (4,4,2,1,1;4,4); UE: (M,N,P,Mg,Ng; Mp,Np) = (2,4,2,1,2; 1,2) | 60 | DDDSU | Average [bit/s/Hz/TRxP] | 6.75 | 1 | 7.04 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.21 |  | 0.40 |
| 4x32 MU-MIMO, CB based, OFDMA (2 panel@UE); gNB: (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;4,4); UE: (M,N,P,Mg,Ng; Mp,Np) = (2,4,2,1,2; 1,1) | 120 | DSUUD | Average [bit/s/Hz/TRxP] | 6.75 | 1 | 11.44 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.21 |  | 0.43 |
| 8x32 SU-MIMO,Non- CB based, OFDMA (2 panel@UE); gNB: (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;2,8); UE: (M,N,P,Mg,Ng; Mp,Np) = (2,4,2,1,2; 1,2) | 60 | DSUUD | Average [bit/s/Hz/TRxP] | 6.75 | 1 | 7.44 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.21 |  | 0.37 |
| 8x64 MU-MIMO CB based (1 panel@UE); gNB: (M,N,P,Mg,Ng; Mp,Np) = (4,32,2,1,1;1,32) UE: (M,N,P,Mg,Ng; Mp,Np) = (1,4,2,1,1; 1,4) | 60 | UUUUU | Average [bit/s/Hz/TRxP] | 6.75 | 1 | 10.54 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.21 |  | 0.32 |
| 4x2, SU-MIMO, CB based, OFDMA (1 Panel @UE) gNB: (M,N,P,Mg,Ng; Mp,Np) = (16,8,2,1,1;1,1) UE: (M,N,P,Mg,Ng; Mp,Np) = (4,4,2,1,1; 1,2) | 120 | DDDU | Average [bit/s/Hz/TRxP] | 6.75 | 1 | 6.90 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.21 |  | 0.30 |

The evaluation results of UL spectral efficiency for NR for evaluation configuration B with 12TRxP are provided in Table 5.3.1.1.2-4.

It is observed that NR fulfills the UL spectral efficiency requirement for all the above configurations in evaluation configuration B.

Table 5.3.1.1.2-4 UL spectral efficiency for NR in Indoor Hotspot – eMBB 
(Evaluation configuration B, CF=30 GHz, for 36TRxP)

(a) NR TDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement |  | Number of samples | BW=80MHz |
| --- | --- | --- | --- | --- | --- | --- |
| 4x32 MU-MIMO OFDMA, gNB: (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;4,4) UE: (M,N,P,Mg,Ng; Mp,Np) = (2,4,2,1,2; 1,1) | 120 | DSUUD | Average [bit/s/Hz/TRxP] | 6.75 | 1 | 10.38 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.21 |  | 0.31 |
| 8x32 SU-MIMO,Non- CB based, OFDMA (2 Panels @ UE), gNB: (M,N,P,Mg,Ng; Mp,Np) = (8,16,2,1,1;2,8) UE: (M,N,P,Mg,Ng; Mp,Np) = (2,4,2,1,2; 1,2) | 60 | DSUUD | Average [bit/s/Hz/TRxP] | 6.75 | 1 | 7.09 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.21 |  | 0.39 |

5.3.1.2	Dense Urban – eMBB

Evaluation configuration A (carrier frequency = 4 GHz) is applied for the evaluations of Dense Urban – eMBB test environment for NR. Both NR FDD and TDD are evaluated. A wide range of antenna configurations and transmission schemes are considered. Detailed evaluation assumptions and results can be found in RP-181816.

5.3.1.2.1	Evaluation configuration A (CF = 4 GHz)

The evaluation results of DL spectral efficiency for NR FDD and NR TDD for evaluation configuration A are provided in Table 5.3.1.2.1-1.

Similar to Indoor Hotspot – eMBB test environment, the capability of NR in larger bandwidth are evaluated for DL spectral efficiency. The values of the larger bandwidths are shown in respective tables.

It is observed that both NR FDD and TDD fulfill the DL spectral efficiency requirement for these configurations in evaluation configuration A.

Table 5.3.1.2.1-1 DL spectral efficiency for NR in Dense Urban – eMBB 
(Evaluation configuration A, CF=4 GHz)

(a) NR FDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | ITU Requirement |  | Channel model A |  |  |  | Channel model B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Number of samples | BW= 10MHz | BW= 20MHz | BW= 40MHz | Number of samples | BW= 10MHz | BW= 20MHz | BW= 40MHz |
| 32x4 MU-MIMO Type II Codebook (128Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;2,8) | 15 | Average [bit/s/Hz/TRxP] | 7.8 | 8 | 11.34 | 12.98 | 13.88 | 4 | 11.84 | 13.37 | 14.28 |
|  |  | 5th-tile [bit/s/Hz] | 0.225 |  | 0.38 | 0.42 | 0.45 |  | 0.39 | 0.44 | 0.47 |
| 32x4 MU-MIMO Type I Codebook (128Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;2,8) | 15 | Average [bit/s/Hz/TRxP] | 7.8 | 2 | 10.95 | 12.33 | 13.16 | / | / | / | / |
|  |  | 5th-tile [bit/s/Hz] | 0.225 |  | 0.33 | 0.38 | 0.40 |  | / | / | / |
| 32x4 MU-MIMO Type II Codebook (256Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (8,16,2,1,1;1,16) | 15 | Average [bit/s/Hz/TRxP] | 7.8 | 1 | 14.81 | 16.84 | 18.04 | / | / | / | / |
|  |  | 5th-tile [bit/s/Hz] | 0.225 |  | 0.51 | 0.61 | 0.65 |  | / | / | / |
| 32x4 MU-MIMO Type II Codebook (256Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (16,8,2,1,1;2,8) | 15 | Average [bit/s/Hz/TRxP] | 7.8 | 1 | 13.74 | 15.53 | 16.60 | 1 | 13.48 | 15.23 | 16.28 |
|  |  | 5th-tile [bit/s/Hz] | 0.225 |  | 0.48 | 0.54 | 0.57 |  | 0.5 | 0.56 | 0.6 |
| 32x4 MU-MIMO Ideal CSI feedback (128Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;2,8) | 15 | Average [bit/s/Hz/TRxP] | 7.8 | / | / | / | / | 1 | 15.67 | 17.79 | 19.05 |
|  |  | 5th-tile [bit/s/Hz] | 0.225 |  | / | / | / |  | 0.55 | 0.62 | 0.66 |
| 4x4 MU-MIMO Type II Codebook (128Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;2,1) | 15 | Average [bit/s/Hz/TRxP] | 7.8 | 1 | 14.11 | 15.76 | 16.76 | / | / | / | / |
|  |  | 5th-tile [bit/s/Hz] | 0.225 |  | 0.40 | 0.44 | 0.47 |  | / | / | / |
| 32x8 MU-MIMO Type II Codebook (256Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (16,8,2,1,1;2,8) | 15 | Average [bit/s/Hz/TRxP] | 7.8 | 1 | 9.20 | 10.31 | 10.99 | / | / | / | / |
|  |  | 5th-tile [bit/s/Hz] | 0.225 |  | 0.33 | 0.37 | 0.39 |  | / | / | / |

(b) NR TDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement |  | Channel model A |  |  |  | Channel model B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Number of samples | BW= 20MHz | BW= 40MHz | BW= 100MHz | Number of samples | BW= 20MHz | BW= 40MHz | BW= 100MHz |
| 32x4 MU-MIMO, 4T SRS (128Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;2,8) | 30 | DDDSU | Average [bit/s/Hz/TRxP] | 7.8 | 2 | 13.28 | 15.42 | 16.89 | 2 | 12.91 | 15.05 | 16.52 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.225 |  | 0.43 | 0.50 | 0.54 |  | 0.45 | 0.52 | 0.57 |
| 32x4 MU-MIMO, 4T SRS (128Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;2,8) | 15 | DDDSU | Average [bit/s/Hz/TRxP] | 7.8 | 1 | 12.95 | 14.78 | / | 1 | 12.74 | 14.54 | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.225 |  | 0.39 | 0.44 | / |  | 0.4 | 0.46 | / |
| 64x4 MU-MIMO, 4T SRS (192Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (12,8,2,1,1;4,8) | 30 | DDDSU | Average [bit/s/Hz/TRxP] | 7.8 | 1 | 16.10 | 18.84 | 20.70 | 2 | 16.23 | 19.14 | 21.11 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.225 |  | 0.49 | 0.58 | 0.64 |  | 0.56 | 0.86 | 1.06 |
| 64x4 MU-MIMO, 4T SRS (192Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (12,8,2,1,1;4,8) | 15 | DDDSU | Average [bit/s/Hz/TRxP] | 7.8 | 1 | 15.71 | 18.02 | / | 1 | 15.46 | 17.73 | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.225 |  | 0.48 | 0.56 | / |  | 0.49 | 0.56 | / |
| 32x4 MU-MIMO, Type II Codebook (256Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (8,16,2,1,1;1,16) | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 7.8 | 1 | 12.25 | 14.56 | / | / | / | / | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.225 |  | 0.44 | 0.53 | / |  | / | / | / |
| 32x4 MU-MIMO, Type II Codebook (256Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (16,8,2,1,1;2,8) | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 7.8 | 1 | 15.87 | 18.16 | / | 1 | 15.47 | 17.70 | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.225 |  | 0.54 | 0.62 | / |  | 0.54 | 0.62 | / |
| 32x4 MU-MIMO, 4T SRS (128Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;2,8) | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 7.8 | 2 | 12.54 | 14.39 | / | 1 | 14.95 | 17.02 | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.225 |  | 0.36 | 0.41 | / |  | 0.42 | 0.47 | / |
| 32x4 MU-MIMO, Type II Codebook (256Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (16,8,2,1,1;2,8) | 30 | DSUUD | Average [bit/s/Hz/TRxP] | 7.8 | 1 | 15.36 | 17.93 | 19.68 | 1 | 14.94 | 17.44 | 19.14 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.225 |  | 0.51 | 0.60 | 0.66 |  | 0.52 | 0.61 | 0.67 |
| 64x4 MU-MIMO, 4T SRS (192Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (12,8,2,1,1;4,8) | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 7.8 | 1 | 18.28 | 20.95 | / | 1 | 18.12 | 20.76 | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.225 |  | 0.71 | 0.82 | / |  | 0.72 | 0.82 | / |
| 64x4 MU-MIMO, 4T SRS (256Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (16,8,2,1,1;4,8) | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 7.8 | 1 | 18.93 | 21.69 | / | 1 | 18.91 | 21.67 | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.225 |  | 0.76 | 0.87 | / |  | 0.81 | 0.93 | / |
| 64x4 MU-MIMO, 4T SRS (192Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (12,8,2,1,1;4,8) | 30 | DSUUD | Average [bit/s/Hz/TRxP] | 7.8 | 2 | 17.15 | 20.20 | 22.27 | 2 | 16.98 | 20.01 | 22.06 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.225 |  | 0.64 | 0.76 | 0.83 |  | 0.63 | 0.75 | 0.82 |
| 64x4 MU-MIMO, 4T SRS (256Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (16,8,2,1,1;4,8) | 30 | DSUUD | Average [bit/s/Hz/TRxP] | 7.8 | 1 | 18.32 | 21.42 | 23.53 | 1 | 18.34 | 21.44 | 23.55 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.225 |  | 0.75 | 0.87 | 0.96 |  | 0.79 | 0.92 | 1.01 |
| 64x8 MU-MIMO, 4T SRS (256Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (4,32,2,1,1;1,32) | 15 | DDDDD | Average [bit/s/Hz/TRxP] | 7.8 | 1 | 12.99 | 14.71 | / | / | / | / | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.225 |  | 0.32 | 0.36 | / |  | / | / | / |
| 64x4 MU-MIMO, 4T SRS (192Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (12,8,2,1,1;4,8) | 30 | DDDDD DDSUU | Average [bit/s/Hz/TRxP] | 7.8 | 1 | 16.07 | 18.95 | 20.89 | / | / | / | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.225 |  | 0.42 | 0.5 | 0.55 |  | / | / | / |
| 32x8 MU-MIMO, 4T SRS (256Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (16,8,2,1,1;2,8) | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 7.8 | 1 | 7.63 | 8.79 | / | / | / | / | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.225 |  | 0.23 | 0.27 | / |  | / | / | / |
| 128x4, MU-MIMO, 4T SRS (256Tx@gNB); (M,N,P, Mg,Ng,Mp,Np) = (8,16,2,1,1;4,16) | 30 | DDSU | Average [bit/s/Hz/TRxP] | 7.8 | / | / | / | / | 1 | 11.41 | 13.30 | 14.59 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.225 |  | / | / | / |  | 0.38 | 0.45 | 0.49 |

The evaluation results of UL spectral efficiency for NR FDD and NR TDD for evaluation configuration A are provided in Table 5.3.1.2.1-2.

It is observed that both NR FDD and TDD fulfill the UL spectral efficiency requirement for these configurations in evaluation configuration A.

Table 5.3.1.2.1-2 UL spectral efficiency for NR in Dense Urban – eMBB 
(Evaluation configuration A, CF=4 GHz)

(a) NR FDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | ITU Requirement |  | Channel model A |  | Channel model B |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Number of samples | BW=10MHz | Number of samples | BW=10MHz |
| 2x16 SU-MIMO, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;1,8) | 15 | Average [bit/s/Hz/TRxP] | 5.4 | 4 | 7.69 | 1 | 6.67 |
|  |  | 5th-tile [bit/s/Hz] | 0.15 |  | 0.25 |  | 0.22 |
| 2x32 SU-MIMO, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;2,8) | 15 | Average [bit/s/Hz/TRxP] | 5.4 | 2 | 7.31 | 2 | 7.26 |
|  |  | 5th-tile [bit/s/Hz] | 0.15 |  | 0.37 |  | 0.32 |
| 4x32 SU-MIMO, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;2,8) | 15 | Average [bit/s/Hz/TRxP] | 5.4 | 1 | 8.82 | 1 | 8.79 |
|  |  | 5th-tile [bit/s/Hz] | 0.15 |  | 0.45 |  | 0.37 |
| 4x32 MU-MIMO, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (8,16,2,1,1;1,16) | 15 | Average [bit/s/Hz/TRxP] | 5.4 | 1 | 22.48 | / | / |
|  |  | 5th-tile [bit/s/Hz] | 0.15 |  | 0.53 |  | / |
| 4x32 MU-MIMO, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (16,8,2,1,1;2,8) | 15 | Average [bit/s/Hz/TRxP] | 5.4 | 1 | 6.73 | / | / |
|  |  | 5th-tile [bit/s/Hz] | 0.15 |  | 0.28 |  | / |
| 4x16 SU-MIMO, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;1,8) | 15 | Average [bit/s/Hz/TRxP] | 5.4 | 1 | 6.09 | 1 | 6.40 |
|  |  | 5th-tile [bit/s/Hz] | 0.15 |  | 0.33 |  | 0.35 |
| 16x16 SU-MIMO, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;1,8) | 15 | Average [bit/s/Hz/TRxP] | 5.4 | / | / | 1 | 8.67 |
|  |  | 5th-tile [bit/s/Hz] | 0.15 |  | / |  | 0.24 |
| 4x4 SU-MIMO, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;2,1) | 15 | Average [bit/s/Hz/TRxP] | 5.4 | 1 | 9.57 | / | / |
|  |  | 5th-tile [bit/s/Hz] | 0.15 |  | 0.19 |  | / |

(b) NR TDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement |  | Channel model A |  | Channel model B |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Number of samples | BW=20MHz | Number of samples | BW=20MHz |
| 2x32 SU-MIMO, Codebook based, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;2,8) | 30 | DDDSU | Average [bit/s/Hz/TRxP] | 5.4 | 1 | 6.14 | 2 | 6.88 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.15 |  | 0.28 |  | 0.38 |
| 2x32 SU-MIMO, Codebook based, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;2,8) | 15 | DDDSU | Average [bit/s/Hz/TRxP] | 5.4 | 1 | 6.57 | 1 | 6.48 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.15 |  | 0.28 |  | 0.25 |
| 2x64 SU-MIMO, Codebook based, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (12,8,2,1,1;4,8) | 30 | DDDSU | Average [bit/s/Hz/TRxP] | 5.4 | 1 | 7.03 | 1 | 7.00 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.15 |  | 0.36 |  | 0.33 |
| 2x64 SU-MIMO, Codebook based, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (12,8,2,1,1;4,8) | 15 | DDDSU | Average [bit/s/Hz/TRxP] | 5.4 | 1 | 7.49 | 1 | 7.47 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.15 |  | 0.36 |  | 0.29 |
| 4x32 SU-MIMO, Codebook based, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;2,8) | 30 | DDDSU | Average [bit/s/Hz/TRxP] | 5.4 | 1 | 6.73 | 1 | 3.75 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.15 |  | 0.33 |  | 0.31 |
| 4x32 SU-MIMO, Codebook based, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;2,8) | 15 | DDDSU | Average [bit/s/Hz/TRxP] | 5.4 | 1 | 7.20 | 1 | 7.18 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.15 |  | 0.32 |  | 0.29 |
| 4x32 MU-MIMO, Codebook based, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (8,16,2,1,1;1,16) | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 5.4 | 1 | 20.81 | / | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.15 |  | 0.49 |  | / |
| 4x16 SU-MIMO, Non-codebook based,OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;1,8) | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 5.4 | 1 | 6.56 | 1 | 6.90 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.15 |  | 0.27 |  | 0.32 |
| 4x16 SU-MIMO, Non-codebook based,OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;1,8) | 30 | DSUUD | Average [bit/s/Hz/TRxP] | 5.4 | 1 | 6.28 | 1 | 6.34 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.15 |  | 0.26 |  | 0.35 |
| 4x16 SU-MIMO, codebook based,OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;1,8) | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 5.4 | 1 | 6.05 | 1 | 6.43 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.15 |  | 0.25 |  | 0.28 |
| 4x16 SU-MIMO, codebook based,OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;1,8) | 30 | DSUUD | Average [bit/s/Hz/TRxP] | 5.4 | 1 | 5.69 | 1 | 6.02 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.15 |  | 0.23 |  | 0.29 |
| 8x64 MU-MIMO, Codebook based, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (4,32,2,1,1;1,32) | 15 | DDDDD | Average [bit/s/Hz/TRxP] | 5.4 | 1 | 7.53 | / | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.15 |  | 0.17 |  | / |
| 2x32 SU-MIMO, Codebook based, OFDMA, (M,N,P,Mg,Ng; Mp,Np) = (12,8,2,1,1;4,4) | 30 | DSUUD | Average [bit/s/Hz/TRxP] | 5.4 | / | / | 1 | 8.16 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.15 |  | / |  | 0.55 |
| 2x64 SU-MIMO, Codebook based, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (12,8,2,1,1;4,8) | 30 | DSUUD | Average [bit/s/Hz/TRxP] | 5.4 | 1 | 8.28 | 1 | 8.24 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.15 |  | 0.60 |  | 0.58 |
| 4x128, MU-MIMO, Codebook based, OFDMA; (M,N,P, Mg,Ng,Mp,Np) = (8,16,2,1,1;4,16) | 30 | DDSU | Average [bit/s/Hz/TRxP] | 5.4 | / | / | 1 | 5.84 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.15 |  | / |  | 0.18 |

5.3.1.2.2	Evaluation configuration B (CF=30 GHz)

[Editor’s note: to be updated]

5.3.1.3	Rural – eMBB

Evaluation configuration A (carrier frequency = 700 MHz), evaluation configuration B (carrier frequency = 4 GHz), and evaluation configuration C (LMLC) are applied for the evaluations of Rural – eMBB test environment for NR. Both NR FDD and TDD are evaluated. A wide range of antenna configurations and transmission schemes are considered. Detailed evaluation assumptions and results can be found in RP-181816.

5.3.1.3.1	Evaluation configuration A (CF = 700 MHz)

The evaluation results of DL spectral efficiency for NR FDD and NR TDD for evaluation configuration A are provided in Table 5.3.1.3.1-1.

Similar to Indoor Hotspot – eMBB test environment, the capability of NR in larger bandwidth are evaluated for DL spectral efficiency. The values of the larger bandwidths are shown in respective tables.

It is observed that both NR FDD and TDD fulfill the DL spectral efficiency requirement for these configurations in evaluation configuration A.

Table 5.3.1.3.1-1 DL spectral efficiency for NR in Rural – eMBB 
(Evaluation configuration A, CF=700 MHz)

(a) NR FDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | ITU Requirement |  | Channel model A |  |  |  | Channel model B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Number of samples | BW= 10MHz | BW= 20MHz | BW= 40MHz | Number of samples | BW= 10MHz | BW= 20MHz | BW= 40MHz |
| 8x2 MU-MIMO Type II Codebook; (M,N,P,Mg,Ng; Mp,Np) = (8,4,2,1,1;1,4) | 15 | Average [bit/s/Hz/TRxP] | 3.3 | 8 | 5.90 | 6.64 | 7.08 | 4 | 6.37 | 7.14 | 7.61 |
|  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.16 | 0.18 | 0.19 |  | 0.18 | 0.20 | 0.21 |
| 16x2 MU-MIMO Type II Codebook; (M,N,P,Mg,Ng; Mp,Np) = (8,4,2,1,1;2,4) | 15 | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 7.48 | 8.39 | 8.94 | 1 | 7.46 | 8.37 | 8.92 |
|  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.15 | 0.17 | 0.18 |  | 0.16 | 0.18 | 0.19 |
| 8x4 MU-MIMO Type II Codebook; (M,N,P,Mg,Ng; Mp,Np) = (8,4,2,1,1;1,4) | 15 | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 9.42 | 10.66 | 11.40 | 1 | 9.08 | 10.27 | 10.99 |
|  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.16 | 0.18 | 0.19 |  | 0.17 | 0.19 | 0.21 |
| 16x4 MU-MIMO Type II Codebook; (M,N,P,Mg,Ng; Mp,Np) = (4,8,2,1,1;1,8) | 15 | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 15.28 | 17.37 | 18.61 | / | / | / | / |
|  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.47 | 0.54 | 0.57 |  | / | / | / |
| 16x2 MU-MIMO Type II Codebook; (M,N,P,Mg,Ng; Mp,Np) = (4,8,2,1,1;1,8) | 15 | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 9.58 | 10.81 | 11.55 | 1 | 10.02 | 11.23 | 11.96 |
|  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.25 | 0.28 | 0.30 |  | 0.27 | 0.30 | 0.32 |
| 8x2 MU-MIMO Type I codebook; (M,N,P,Mg,Ng; Mp,Np) = (8,4,2,1,1;1,4) | 15 | Average [bit/s/Hz/TRxP] | 3.3 | 2 | 6.27 | 7.04 | 7.5 | 1 | 4.21 | 4.70 | 4.99 |
|  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.16 | 0.18 | 0.19 |  | 0.55 | 0.61 | 0.65 |

(b) NR TDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement |  | Channel model A |  |  |  | Channel model B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Number of samples | BW= 20MHz | BW= 40MHz | BW= 100MHz | Number of samples | BW= 20MHz | BW= 40MHz | BW= 100MHz |
| 8x2 MU-MIMO, 2T SRS; (M,N,P,Mg,Ng; Mp,Np) = (8,4,2,1,1;1,4) | 30 | DDDSU | Average [bit/s/Hz/TRxP] | 3.3 | 2 | 7.79 | 8.96 | 9.76 | 3 | 6.50 | 7.51 | 8.21 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.17 | 0.20 | 0.21 |  | 0.18 | 0.21 | 0.23 |
| 8x2 MU-MIMO, 2T SRS; (M,N,P,Mg,Ng; Mp,Np) = (8,4,2,1,1;1,4) | 15 | DDDSU | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 7.66 | 8.63 | / | 1 | 7.57 | 8.53 | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.16 | 0.18 | / |  | 0.16 | 0.18 | / |
| 16x2 MU-MIMO, 2T SRS; (M,N,P,Mg,Ng; Mp,Np) = (8,4,2,1,1;2,4) | 30 | DDDSU | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 9.21 | 10.64 | 11.61 | 1 | 9.22 | 10.65 | 11.62 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.21 | 0.25 | 0.27 |  | 0.22 | 0.25 | 0.28 |
| 16x4 MU-MIMO, 2T SRS; (M,N,P,Mg,Ng; Mp,Np) = (4,8,2,1,1;1,8) | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 12.61 | 14.99 | / | / | / | / | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.39 | 0.46 | / |  | / | / | / |
| 8x2 MU-MIMO, 2T SRS; (M,N,P,Mg,Ng; Mp,Np) = (8,4,2,1,1;1,4) | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 3.3 | 3 | 8.13 | 9.34 | / | 2 | 9.61 | 11.06 | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.24 | 0.28 | / |  | 0.27 | 0.31 | / |
| 8x2 MU-MIMO, 2T SRS; (M,N,P,Mg,Ng; Mp,Np) = (8,4,2,1,1;1,4) | 30 | DSUUD | Average [bit/s/Hz/TRxP] | 3.3 | 2 | 6.71 | 7.84 | 8.60 | 2 | 6.91 | 8.07 | 8.86 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.19 | 0.22 | 0.24 |  | 0.19 | 0.22 | 0.25 |
| 16x4 MU-MIMO, 4T SRS; (M,N,P,Mg,Ng; Mp,Np) = (4,8,2,1,1;1,8) | 30 | DDSU | Average [bit/s/Hz/TRxP] | 3.3 | / | / | / | / | 1 | 7.00 | 8.16 | 8.94 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.12 |  | / | / | / |  | 0.14 | 0.16 | 0.18 |

The evaluation results of UL spectral efficiency for NR FDD and NR TDD for evaluation configuration A are provided in Table 5.3.1.3.1-2.

It is observed that both NR FDD and TDD fulfill the UL spectral efficiency requirement for these configurations in evaluation configuration A.

Table 5.3.1.3.1-2 UL spectral efficiency for NR in Rural – eMBB 
(Evaluation configuration A, CF=700 MHz)

(a) NR FDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | ITU Requirement |  | Channel model A |  | Channel model B |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Number of samples | BW=10MHz | Number of samples | BW=10MHz |
| 1x8 SU-MIMO, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (8,4,2,1,1;1,4) | 15 | Average [bit/s/Hz/TRxP] | 1.6 | 4 | 4.35 | 2 | 3.61 |
|  |  | 5th-tile [bit/s/Hz] | 0.045 |  | 0.14 |  | 0.13 |
| 4x16 MU-MIMO, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (4,8,2,1,1;1,8) | 15 | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 15.55 | / | / |
|  |  | 5th-tile [bit/s/Hz] | 0.045 |  | 0.63 |  | / |
| 2x8 SU-MIMO, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (8,4,2,1,1;1,4) | 15 | Average [bit/s/Hz/TRxP] | 1.6 | 3 | 6.98 | 2 | 6.01 |
|  |  | 5th-tile [bit/s/Hz] | 0.045 |  | 0.15 |  | 0.18 |
| 2x8 SU-MIMO, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (8,4,2,1,1;1,4) | 15 | Average [bit/s/Hz/TRxP] | 1.6 | / | / | 2 | 4.19 |
|  |  | 5th-tile [bit/s/Hz] | 0.045 |  | / |  | 0.15 |

(b) NR TDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement |  | Channel model A |  | Channel model B |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Number of samples | BW=20MHz | Number of samples | BW=20MHz |
| 2x8 SU-MIMO, Codebook based, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (8,4,2,1,1;1,4) | 30 | DDDSU | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 4.75 | 1 | 4.76 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.045 |  | 0.10 |  | 0.10 |
| 2x8 SU-MIMO, Codebook based, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (8,4,2,1,1;1,4) | 15 | DDDSU | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 5.08 | 1 | 5.05 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.045 |  | 0.10 |  | 0.09 |
| 4x16 MU-MIMO, Codebook based, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (4,8,2,1,1;1,8) | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 14.40 | / | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.045 |  | 0.58 |  | / |
| 2x8 SU-MIMO, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (8,4,2,1,1;1,4) | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 4.44 | 1 | 4.45 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.045 |  | 0.18 |  | 0.16 |
| 2x8 SU-MIMO, Non-codebook based, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (8,4,2,1,1;1,4) | 30 | DSUUD | Average [bit/s/Hz/TRxP] | 1.6 | 2 | 5.08 | 1 | 4.22 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.045 |  | 0.16 |  | 0.17 |
| 2x8 SU-MIMO, codebook based, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (8,4,2,1,1;1,4) | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 4.71 | 1 | 4.74 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.045 |  | 0.20 |  | 0.16 |
| 2x8 SU-MIMO, codebook based, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (8,4,2,1,1;1,4) | 30 | DSUUD | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 4.51 | 1 | 4.56 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.045 |  | 0.18 |  | 0.15 |
| 2x16 MU-MIMO, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (4,8,2,1,1;1,8) | 30 | DDSU | Average [bit/s/Hz/TRxP] | 1.6 | / | / | 1 | 4.44 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.045 |  | / |  | 0.24 |

5.3.1.3.2	Evaluation configuration B (CF = 4 GHz)

The evaluation results of DL spectral efficiency for NR FDD and NR TDD for evaluation configuration B are provided in Table 5.3.1.3.2-1.

Similar to Indoor Hotspot – eMBB test environment, the capability of NR in larger bandwidth are evaluated for DL spectral efficiency. The values of the larger bandwidths are shown in respective tables.

It is observed that both NR FDD and TDD fulfill the DL spectral efficiency requirement for these configurations in evaluation configuration B.

Table 5.3.1.3.2-1 DL spectral efficiency for NR in Rural – eMBB 
(Evaluation configuration B, CF=4 GHz)

(a) NR FDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | ITU Requirement |  | Channel model A |  |  |  | Channel model B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Number of samples | BW= 10MHz | BW= 20MHz | BW= 40MHz | Number of samples | BW= 10MHz | BW= 20MHz | BW= 40MHz |
| 32x4 MU-MIMO Type II Codebook; (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;2,8) | 15 | Average [bit/s/Hz/TRxP] | 3.3 | 6 | 12.33 | 13.92 | 14.87 | 4 | 13.47 | 15.21 | 16.25 |
|  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.41 | 0.47 | 0.50 |  | 0.41 | 0.46 | 0.49 |
| 32x4 MU-MIMO Type II Codebook; (M,N,P,Mg,Ng; Mp,Np) = (8,16,2,1,1;1,16) | 15 | Average [bit/s/Hz/TRxP] | 3.3 | 2 | 17.32 | 19.64 | 21.01 | 1 | 16.63 | 18.79 | 20.09 |
|  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.49 | 0.56 | 0.59 |  | 0.52 | 0.58 | 0.62 |
| 16x4 MU-MIMO Type II Codebook; (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;1,8) | 15 | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 12.58 | 14.23 | 15.22 | 1 | 11.96 | 13.53 | 14.47 |
|  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.12 | 0.14 | 0.15 |  | 0.13 | 0.14 | 0.15 |
| 32x4 MU-MIMO Type I codebook; (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;1,8) | 15 | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 10.76 | 12.07 | 12.86 | 1 | 5.30 | 5.96 | 6.35 |
|  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.22 | 0.25 | 0.26 |  | 1.76 | 1.98 | 2.11 |
| 16x4 MU-MIMO Type I codebook; (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;1,8) | 15 | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 10.21 | 11.43 | 12.16 | / | / | / | / |
|  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.16 | 0.18 | 0.19 |  | / | / | / |
| 16x4 MU-MIMO ideal CSI feedback; (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;1,8) | 15 | Average [bit/s/Hz/TRxP] | 3.3 | / | / | / | / | 1 | 14.23 | 16.11 | 17.23 |
|  |  | 5th-tile [bit/s/Hz] | 0.12 |  | / | / | / |  | 0.38 | 0.43 | 0.45 |
| 32x8 MU-MIMO Type II Codebook; (M,N,P,Mg,Ng; Mp,Np) = (8,16,2,1,1;1,16) | 15 | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 9.70 | 10.88 | 11.58 | / | / | / | / |
|  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.27 | 0.30 | 0.32 |  | / | / | / |

(b) NR TDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement |  | Channel model A |  |  |  | Channel model B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Number of samples | BW= 20MHz | BW= 40MHz | BW= 100MHz | Number of samples | BW= 20MHz | BW= 40MHz | BW= 100MHz |
| 32x4 MU-MIMO, 4T SRS; (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;2,8) | 30 | DDDSU | Average [bit/s/Hz/TRxP] | 3.3 | 2 | 15.35 | 17.83 | 19.52 | 1 | 15.23 | 17.77 | 19.50 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.37 | 0.43 | 0.48 |  | 0.38 | 0.44 | 0.48 |
| 32x4 MU-MIMO, 4T SRS; (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;2,8) | 15 | DDDSU | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 15.31 | 17.47 | / | 1 | 15.18 | 17.33 | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.37 | 0.42 | / |  | 0.37 | 0.42 | / |
| 32x4 MU-MIMO, Type II Codebook; (M,N,P,Mg,Ng; Mp,Np) = (8,16,2,1,1;1,16) | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 14.86 | 17.67 | / | / | / | / | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.39 | 0.46 | / |  | / | / | / |
| 32x4 MU-MIMO, 4T SRS; (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;2,8) | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 3.3 | 2 | 15.04 | 17.46 | / | 1 | 15.17 | 17.84 | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.58 | 0.68 | / |  | 0.49 | 0.58 | / |
| 32x4 MU-MIMO, 4T SRS; (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;2,8) | 30 | DSUUD | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 14.29 | 16.68 | 18.31 | / | / | / | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.47 | 0.55 | 0.60 |  | / | / | / |
| 64x8 MU-MIMO, 8T SRS; (M,N,P,Mg,Ng; Mp,Np) = (4,32,2,1,1;1,32) | 15 | DDDDD | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 18.50 | 20.94 | / | / | / | / | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.46 | 0.51 | / |  | / | / | / |
| 64x4 MU-MIMO, 4T SRS (192Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (12,8,2,1,1;4,8) | 30 | DDDDD DDSUU | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 12.01 | 14.16 | 15.61 | / | / | / | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.29 | 0.35 | 0.38 |  | / | / | / |
| 16x4 MU-MIMO, 4T SRS; (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;1,8) | 30 | DDDSU | Average [bit/s/Hz/TRxP] | 3.3 | / | / | / | / | 1 | 12.67 | 14.74 | 16.16 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.12 |  | / | / | / |  | 0.57 | 0.66 | 0.72 |
| 32x8 MU-MIMO, 8T SRS; (M,N,P,Mg,Ng; Mp,Np) = (8,16,2,1,1;1,16) | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 8.66 | 9.97 | / | / | / | / | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.19 | 0.22 | / |  | / | / | / |
| 64x4 MU-MIMO, 4T SRS (192Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (12,8,2,1,1;4,8) | 30 | DSUUD | Average [bit/s/Hz/TRxP] | 3.3 | 2 | 15.40 | 18.14 | 20.00 | 2 | 15.23 | 17.94 | 19.78 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.58 | 0.69 | 0.76 |  | 0.54 | 0.64 | 0.71 |
| 64x4 MU-MIMO, 4T SRS (192Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (12,8,2,1,1;4,8) | 30 | DDDSU | Average [bit/s/Hz/TRxP] | 3.3 | / | / | / | / | 1 | 14.52 | 16.90 | 18.53 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.12 |  | / | / | / |  | 0.49 | 0.57 | 0.63 |
| 128x4 MU-MIMO, 4T SRS; (M,N,P,Mg,Ng; Mp,Np) = (4,32,2,1,1;2,32) | 30 | DDSU | Average [bit/s/Hz/TRxP] | 3.3 | / | / | / | / | 1 | 10.97 | 12.79 | 14.03 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.12 |  | / | / | / |  | 0.14 | 0.16 | 0.17 |

The evaluation results of UL spectral efficiency for NR FDD and NR TDD for evaluation configuration B are provided in Table 5.3.1.3.2-2.

It is observed that both NR FDD and TDD fulfill the UL spectral efficiency requirement for these configurations in evaluation configuration B.

Table 5.3.1.3.2-2 UL spectral efficiency for NR in Rural – eMBB 
(Evaluation configuration B, CF=4 GHz)

(a) NR FDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | ITU Requirement |  | Channel model A |  | Channel model B |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Number of samples | BW=10MHz | Number of samples | BW=10MHz |
| 1x32 SU-MIMO, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;2,8) | 15 | Average [bit/s/Hz/TRxP] | 1.6 | 4 | 4.28 | 1 | 4.27 |
|  |  | 5th-tile [bit/s/Hz] | 0.045 |  | 0.14 |  | 0.15 |
| 4x32 SU-MIMO, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;2,8) | 15 | Average [bit/s/Hz/TRxP] | 1.6 | 2 | 7.38 | 2 | 6.31 |
|  |  | 5th-tile [bit/s/Hz] | 0.045 |  | 0.29 |  | 0.15 |
| 4x32 MU-MIMO, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (8,16,2,1,1;1,16) | 15 | Average [bit/s/Hz/TRxP] | 1.6 | 2 | 15.96 | / | / |
|  |  | 5th-tile [bit/s/Hz] | 0.045 |  | 0.20 |  | / |
| 2x32 SU-MIMO, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;2,8) | 15 | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 6.27 | 1 | 6.16 |
|  |  | 5th-tile [bit/s/Hz] | 0.045 |  | 0.23 |  | 0.19 |
| 4x16 SU-MIMO, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;1,8) | 15 | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 9.90 | 1 | 6.26 |
|  |  | 5th-tile [bit/s/Hz] | 0.045 |  | 0.02 |  | 0.06 |

(b) NR TDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement |  | Channel model A |  | Channel model B |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Number of samples | BW=20MHz | Number of samples | BW=20MHz |
| 1x32 SU-MIMO, Codebook based, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;2,8) | 30 | DDDSU | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 3.18 | 1 | 3.12 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.045 |  | 0.11 |  | 0.09 |
| 1x32 SU-MIMO, Codebook based, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;2,8) | 15 | DDDSU | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 3.50 | 1 | 3.43 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.045 |  | 0.12 |  | 0.10 |
| 4x32 SU-MIMO, Codebook based, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;2,8) | 30 | DDDSU | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 5.73 | 1 | 5.76 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.045 |  | 0.18 |  | 0.13 |
| 4x32 SU-MIMO, Codebook based, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;2,8) | 15 | DDDSU | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 6.24 | 1 | 5.98 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.045 |  | 0.18 |  | 0.15 |
| 4x32 MU-MIMO, Codebook based, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (8,16,2,1,1;1,16) | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 21.30 | / | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.045 |  | 0.26 |  | / |
| 8x64 SU-MIMO, Codebook based, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (4,32,2,1,1;1,32) | 15 | UUUUU | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 11.10 | / | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.045 |  | 0.08 |  | / |
| 2x64 SU-MIMO, Codebook based, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (12,8,2,1,1;4,8) | 30 | DSUUD | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 7.83 | 1 | 7.66 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.045 |  | 0.31 |  | 0.32 |
| 2x64 SU-MIMO, Codebook based, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (12,8,2,1,1;4,8) | 30 | DDDSU | Average [bit/s/Hz/TRxP] | 1.6 | / | / | 1 | 6.40 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.045 |  | / |  | 0.27 |
| 2x128, MU-MIMO, Codebook-based, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (8,16,2,1,1;4,16) | 30 | DDSU | Average [bit/s/Hz/TRxP] | 1.6 | / | / | 1 | 9.75 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.045 |  | / |  | 0.12 |

5.3.1.3.3	Evaluation configuration C (LMLC)

LMLC (Low mobility large cell) is characterized by the large inter-site distance (ISD=6000m) and the low mobility users in Rural – eMBB test environment.

The evaluation results of DL spectral efficiency for NR FDD and NR TDD for evaluation configuration C are provided in Table 5.3.1.3.3-1.

Similar to Indoor Hotspot – eMBB test environment, the capability of NR in larger bandwidth are evaluated for DL spectral efficiency. The values of the larger bandwidths are shown in respective tables.

It is observed that both NR FDD and TDD fulfill the DL spectral efficiency requirement for these configurations in evaluation configuration C.

Table 5.3.1.3.3-1 DL spectral efficiency for NR in Rural – eMBB 
(Evaluation configuration C, LMLC)

(a) NR FDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | ITU Requirement |  | Channel model A |  |  |  | Channel model B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Number of samples | BW= 10MHz | BW= 20MHz | BW= 40MHz | Number of samples | BW= 10MHz | BW= 20MHz | BW= 40MHz |
| 8x4 MU-MIMO Type II Codebook (M,N,P,Mg,Ng; Mp,Np) = (8,4,2,1,1;1,4) | 15 | Average [bit/s/Hz/TRxP] | 3.3 | 7 | 7.81 | 8.79 | 9.39 | 5 | 8.15 | 9.18 | 9.8 |
|  |  | 5th-tile [bit/s/Hz] | 0.121 |  | 0.2 | 0.22 | 0.24 |  | 0.21 | 0.24 | 0.25 |
| 16x4 MU-MIMO Type II Codebook (M,N,P,Mg,Ng; Mp,Np) = (8,4,2,1,1;2,4) | 15 | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 8.13 | 9.13 | 9.73 | 1 | 8.14 | 9.13 | 9.73 |
|  |  | 5th-tile [bit/s/Hz] | 0.121 |  | 0.2 | 0.23 | 0.24 |  | 0.2 | 0.23 | 0.24 |
| 16x4 MU-MIMO Type II Codebook (M,N,P,Mg,Ng; Mp,Np) = (4,8,2,1,1;1,8) | 15 | Average [bit/s/Hz/TRxP] | 3.3 | 2 | 9.02 | 10.22 | 10.93 | / | / | / | / |
|  |  | 5th-tile [bit/s/Hz] | 0.121 |  | 0.3 | 0.34 | 0.36 |  | / | / | / |
| 8x4 MU-MIMO ideal CSI feedback (M,N,P,Mg,Ng; Mp,Np) = (8,4,2,1,1;1,4) | 15 | Average [bit/s/Hz/TRxP] | 3.3 | / | / | / | / | 1 | 11.26 | 12.73 | 13.61 |
|  |  | 5th-tile [bit/s/Hz] | 0.121 |  | / | / | / |  | 0.27 | 0.30 | 0.32 |
| 8x4 MU-MIMO Type I codebook (M,N,P,Mg,Ng; Mp,Np) = (8,4,2,1,1;1,4) | 15 | Average [bit/s/Hz/TRxP] | 3.3 | 2 | 9.29 | 10.42 | 11.11 | 1 | 3.9 | 4.37 | 4.65 |
|  |  | 5th-tile [bit/s/Hz] | 0.121 |  | 0.29 | 0.33 | 0.35 |  | 0.84 | 0.94 | 1.00 |
| Note 1: According to Report ITU-R M.2410, the 5th percentile user spectral efficiency requriement is not applicable to LMLC. The value shown here is for information only. |  |  |  |  |  |  |  |  |  |  |  |

(b) NR TDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement |  | Channel model A |  |  |  | Channel model B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Number of samples | BW= 20MHz | BW= 40MHz | BW= 100MHz | Number of samples | BW= 20MHz | BW= 40MHz | BW= 100MHz |
| 8x4 MU-MIMO, 4T SRS (M,N,P,Mg,Ng; Mp,Np) = (8,4,2,1,1;1,4); | 30 | DDDSU | Average [bit/s/Hz/TRxP] | 3.3 | 2 | 7.7 | 8.9 | 9.71 | 2 | 8.14 | 9.45 | 10.34 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.121 |  | 0.2 | 0.23 | 0.25 |  | 0.25 | 0.29 | 0.32 |
| 8x4 MU-MIMO, 4T SRS (M,N,P,Mg,Ng; Mp,Np) = (8,4,2,1,1;1,4); | 15 | DDDSU | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 8.01 | 9.02 | / | 1 | 8.02 | 9.04 | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.121 |  | 0.2 | 0.22 | / |  | 0.2 | 0.22 | / |
| 16x4 MU-MIMO, 4T SRS (M,N,P,Mg,Ng; Mp,Np) = (8,4,2,1,1;2,4); | 30 | DDDSU | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 8.61 | 9.98 | 10.91 | 1 | 8.59 | 9.95 | 10.89 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.121 |  | 0.2 | 0.23 | 0.25 |  | 0.19 | 0.22 | 0.25 |
| 16x4 MU-MIMO, 4T SRS (M,N,P,Mg,Ng; Mp,Np) = (8,4,2,1,1;2,4); | 15 | DDDSU | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 8.83 | 9.98 | / | 1 | 8.82 | 9.96 | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.121 |  | 0.22 | 0.25 | / |  | 0.21 | 0.24 | / |
| 16x4 MU-MIMO, Type II Codebook (M,N,P,Mg,Ng; Mp,Np) = (4,8,2,1,1;1,8); | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 10.44 | 12.41 | / | / | / | / | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.121 |  | 0.35 | 0.41 | / |  | / | / | / |
| 64x8 MU-MIMO, 4T SRS (256Tx@gNB) (M,N,P,Mg,Ng; Mp,Np) = (4,32,2,1,1;1,32) | 15 | DDDDD | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 6.4 | 7.25 | / | / | / | / | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.121 |  | / | / | / |  | / | / | / |
| 8x4 MU-MIMO, 4T SRS (M,N,P,Mg,Ng; Mp,Np) = (8,4,2,1,1;1,4); | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 3.3 | 2 | 7.98 | 9.33 | / | 1 | 8.76 | 10.30 | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.121 |  | 0.25 | 0.29 | / |  | 0.23 | 0.27 | / |
| 8x4 MU-MIMO, 4T SRS (M,N,P,Mg,Ng; Mp,Np) = (8,4,2,1,1;1,4) | 30 | DSUUD | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 8.34 | 9.91 | 10.97 | 1 | 8.38 | 9.96 | 11.02 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.121 |  | 0.28 | 0.33 | 0.37 |  | 0.31 | 0.37 | 0.41 |
| 16x4 MU-MIMO, 4T SRS (M,N,P,Mg,Ng; Mp,Np) = (4,8,2,1,1;1,8); | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 5.24 | 6.04 | / | / | / | / | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.121 |  | 0.16 | 0.18 | / |  | / | / | / |
| Note 1: According to Report ITU-R M.2410, the 5th percentile user spectral efficiency requriement is not applicable to LMLC. The value shown here is for information only. |  |  |  |  |  |  |  |  |  |  |  |  |

The evaluation results of UL spectral efficiency for NR FDD and NR TDD for evaluation configuration C are provided in Table 5.3.1.3.3-2.

It is observed that both NR FDD and TDD fulfill the UL spectral efficiency requirement for these configurations in evaluation configuration C.

Table 5.3.1.3.3-2 UL spectral efficiency for NR in Rural – eMBB 
(Evaluation configuration C, LMLC)

(a) NR FDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | ITU Requirement |  | Channel model A |  | Channel model B |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Number of samples | BW=10MHz | Number of samples | BW=10MHz |
| 2x8 SU-MIMO, OFDMA (M,N,P,Mg,Ng; Mp,Np) = (8,4,2,1,1;1,4); | 15 | Average [bit/s/Hz/TRxP] | 1.6 | 2 | 4.22 | 2 | 4.16 |
|  |  | 5th-tile [bit/s/Hz] | 0.0451 |  | 0.14 |  | 0.13 |
| 1x8 SU-MIMO, OFDMA (M,N,P,Mg,Ng; Mp,Np) = (8,4,2,1,1;1,4); | 15 | Average [bit/s/Hz/TRxP] | 1.6 | 2 | 4.54 | / | / |
|  |  | 5th-tile [bit/s/Hz] | 0.0451 |  | 0.09 |  | / |
| 4x8 SU-MIMO, OFDMA (M,N,P,Mg,Ng; Mp,Np) = (8,4,2,1,1;1,4); | 15 | Average [bit/s/Hz/TRxP] | 1.6 | 2 | 7.04 | 1 | 4.80 |
|  |  | 5th-tile [bit/s/Hz] | 0.0451 |  | 0.07 |  | 0.09 |
| 4x16 MU-MIMO, OFDMA (M,N,P,Mg,Ng; Mp,Np) = (4,8,2,1,1;1,8); | 15 | Average [bit/s/Hz/TRxP] | 1.6 | 2 | 8.10 | / | / |
|  |  | 5th-tile [bit/s/Hz] | 0.0451 |  | 0.10 |  | / |
| 8x8 SU-MIMO (M,N,P,Mg,Ng; Mp,Np) = (8,4,2,1,1;1,4); | 15 | Average [bit/s/Hz/TRxP] | 1.6 | / | / | 1 | 5.58 |
|  |  | 5th-tile [bit/s/Hz] | 0.0451 |  | / |  | 0.17 |
| Note 1: According to Report ITU-R M.2410, the 5th percentile user spectral efficiency requriement is not applicable to LMLC. The value shown here is for information only. |  |  |  |  |  |  |  |

(b) NR TDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement |  | Channel model A |  | Channel model B |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Number of samples | BW=20MHz | Number of samples | BW=20MHz |
| 2x8 SU-MIMO, Codebook based, OFDMA (M,N,P,Mg,Ng; Mp,Np) = (8,4,2,1,1;1,4); | 30 | DDDSU | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 3.33 | 1 | 3.31 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.0451 |  | 0.06 |  | 0.05 |
| 2x8 SU-MIMO, Codebook based, OFDMA (M,N,P,Mg,Ng; Mp,Np) = (8,4,2,1,1;1,4); | 15 | DDDSU | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 3.53 | 1 | 3.53 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.0451 |  | 0.05 |  | 0.04 |
| 4x8 SU-MIMO, Codebook based, OFDMA (M,N,P,Mg,Ng; Mp,Np) = (8,4,2,1,1;1,4); | 30 | DDDSU | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 3.84 | 2 | 3.53 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.0451 |  | 0.07 |  | 0.07 |
| 4x8 SU-MIMO, Codebook based, OFDMA (M,N,P,Mg,Ng; Mp,Np) = (8,4,2,1,1;1,4); | 15 | DDDSU | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 4.03 | 1 | 4.04 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.0451 |  | 0.06 |  | 0.05 |
| 4x16 SU-MIMO, Codebook based, OFDMA (M,N,P,Mg,Ng; Mp,Np) = (4,8,2,1,1;1,8); | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 10.59 | / | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.0451 |  | 0.08 |  | / |
| 8x64 MU-MIMO, Codebook based, OFDMA (M,N,P,Mg,Ng; Mp,Np) = (4,32,2,1,1;1,32) | 15 | UUUUU | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 4.99 | / | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.0451 |  | / |  | / |
| 4x8 SU-MIMO, Codebook based, OFDMA (M,N,P,Mg,Ng; Mp,Np) = (8,4,2,1,1;1,4); | 30 | DSUUD | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 4.41 | 1 | 3.56 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.0451 |  | / |  | / |
| Note 1: According to Report ITU-R M.2410, the 5th percentile user spectral efficiency requriement is not applicable to LMLC. The value shown here is for information only. |  |  |  |  |  |  |  |  |

5.3.2	LTE

Average spectral efficiency and 5th percentile user spectral efficiency are evaluated for LTE.

5.3.2.3	Rural – eMBB

Evaluation configuration A (carrier frequency = 700 MHz), evaluation configuration B (carrier frequency = 4 GHz), and evaluation configuration C (LMLC) are applied for the evaluations of Rural – eMBB test environment for LTE. Both LTE FDD and TDD are evaluated. Detailed evaluation assumptions and results can be found in RP-181816.

5.3.2.3.1	Evaluation configuration A (CF = 700 MHz)

The evaluation results of DL spectral efficiency for LTE FDD and TDD for evaluation configuration A are provided in Table 5.3.2.3.1-1.

It is observed that both LTE FDD and TDD fulfill the DL spectral efficiency requirement for these configurations in evaluation configuration A.

Table 5.3.1.3.1-1 DL spectral efficiency for LTE in Rural – eMBB 
(Evaluation configuration A, CF=700 MHz)

(a) LTE FDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | ITU Requirement |  | Channel model A |  | Channel model B |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Number of samples | BW=10MHz | Number of samples | BW=10MHz |
| 8x2 MU-MIMO Type II Codebook; gNB: (M,N,P,Mg,Ng; Mp,Np) = (8,4,2,1,1;1,4) | 15 | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 4.66 | 1 | 4.63 |
|  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.12 |  | 0.12 |
| 16x2 MU-MIMO Type II Codebook; (M,N,P,Mg,Ng; Mp,Np) = (4,8,2,1,1;1,8) | 15 | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 6.45 | 1 | 6.46 |
|  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.16 |  | 0.15 |

(b) LTE TDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement |  | Channel model A |  | Channel model B |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Number of samples | BW=20MHz | Number of samples | BW=20MHz |
| 16x2 MU-MIMO, 2T SRS; (M,N,P,Mg,Ng; Mp,Np) = (4,8,2,1,1;1,8) | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 11.05 | 1 | 11.22 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.29 |  | 0.29 |

The evaluation results of UL spectral efficiency for LTE FDD and TDD for evaluation configuration A are provided in Table 5.3.2.3.1-2.

It is observed that both LTE FDD and TDD fulfill the UL spectral efficiency requirement for these configurations in evaluation configuration A.

Table 5.3.1.3.1-1 UL spectral efficiency for LTE in Rural – eMBB 
(Evaluation configuration A, CF=700 MHz)

(a) LTE FDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | ITU Requirement |  | Channel model A |  | Channel model B |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Number of samples | BW=10MHz | Number of samples | BW=10MHz |
| 1x8 SU-MIMO, Codebook based, DFT-S-OFDM; (M,N,P,Mg,Ng; Mp,Np) = (8,4,2,1,1;1,4) | 15 | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 3.59 | 1 | 3.59 |
|  |  | 5th-tile [bit/s/Hz] | 0.045 |  | 0.10 |  | 0.10 |
| 2x8 SU-MIMO, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (8,4,2,1,1;1,4) | 15 | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 4.30 | 1 | 4.29 |
|  |  | 5th-tile [bit/s/Hz] | 0.045 |  | 0.22 |  | 0.20 |

(b) LTE TDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement |  | Channel model A |  | Channel model B |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Number of samples | BW=20MHz | Number of samples | BW=20MHz |
| 2x8 SU-MIMO, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (8,4,2,1,1;1,4) | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 3.78 | 1 | 3.80 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.045 |  | 0.15 |  | 0.13 |

5.3.2.3.2	Evaluation configuration B (CF = 4 GHz)

The evaluation results of DL spectral efficiency for LTE FDD and TDD for evaluation configuration B are provided in Table 5.3.2.3.2-1.

It is observed that both LTE FDD and TDD fulfill the DL spectral efficiency requirement for these configurations in evaluation configuration B.

Table 5.3.2.3.2-1 DL spectral efficiency for LTE in Rural – eMBB 
(Evaluation configuration B, CF=4 GHz)

(a) LTE FDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | ITU Requirement |  | Channel model A |  | Channel model B |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Number of samples | BW=10MHz | Number of samples | BW=10MHz |
| 32x4 MU-MIMO Type II Codebook; (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;2,8) | 15 | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 9.65 | 1 | 9.63 |
|  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.28 |  | 0.28 |

(b) LTE TDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement |  | Channel model A |  | Channel model B |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Number of samples | BW=20MHz | Number of samples | BW=20MHz |
| 32x4 MU-MIMO, 4T SRS; (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;2,8) | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 3.3 | / | / | 1 | 12.25 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.12 |  | / |  | 0.43 |
| 64x8 MU-MIMO, 8T SRS; (M,N,P,Mg,Ng; Mp,Np) = (4,32,2,1,1;1,32) | 15 | DDDDD | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 15.70 | / | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.38 |  | / |

The evaluation results of UL spectral efficiency for LTE for evaluation configuration B are provided in Table 5.3.2.3.2-2.

It is observed that both LTE fulfill the UL spectral efficiency requirement for these configurations in evaluation configuration B.

Table 5.3.2.3.2-2 UL spectral efficiency for LTE in Rural – eMBB 
(Evaluation configuration B, CF=4 GHz)

(a) LTE TDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement |  | Channel model A |  | Channel model B |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Number of samples | BW=20MHz | Number of samples | BW=20MHz |
| 8x64 SU-MIMO, Codebook based, OFDMA; (M,N,P,Mg,Ng; Mp,Np) = (4,32,2,1,1;1,32) | 15 | UUUUU | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 10.50 | / | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.045 |  | 0.07 |  | / |

5.3.2.3.3	Evaluation configuration C (LMLC)

LMLC (Low mobility large cell) is characterized by the large inter-site distance (ISD=6000m) and the low mobility users in Rural – eMBB test environment.

The evaluation results of DL spectral efficiency for LTE for evaluation configuration C are provided in Table 5.3.2.3.3-1.

It is observed that both LTE fulfills the DL spectral efficiency requirement for these configurations in evaluation configuration C.

Table 5.3.2.3.3-1 DL spectral efficiency for LTE in Rural – eMBB 
(Evaluation configuration C, LMLC)

(a) LTE FDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | ITU Requirement |  | Channel model A |  | Channel model B |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Number of samples | BW=10MHz | Number of samples | BW=10MHz |
| 8x4 MU-MIMO Type II Codebook (M,N,P,Mg,Ng; Mp,Np) = (8,4,2,1,1;1,4) | 15 | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 5.96 | 1 | 5.97 |
|  |  | 5th-tile [bit/s/Hz] | 0.121 |  | 0.15 |  | 0.15 |
| Note 1: According to Report ITU-R M.2410, the 5th percentile user spectral efficiency requriement is not applicable to LMLC. The value shown here is for information only. |  |  |  |  |  |  |  |

The evaluation results of UL spectral efficiency for LTE for evaluation configuration C are provided in Table 5.3.2.3.3-2.

It is observed that both LTE fulfills the UL spectral efficiency requirement for these configurations in evaluation configuration C.

Table 5.3.2.3.3-2 UL spectral efficiency for LTE in Rural – eMBB 
(Evaluation configuration C, LMLC)

(a) LTE FDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | ITU Requirement |  | Channel model A |  | Channel model B |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Number of samples | BW=10MHz | Number of samples | BW=10MHz |
| 2x8 SU-MIMO, Codebook based, DFT-S-OFDM (M,N,P,Mg,Ng; Mp,Np) = (8,4,2,1,1;1,4); | 15 | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 3.36 | 1 | 3.31 |
|  |  | 5th-tile [bit/s/Hz] | 0.0451 |  | 0.07 |  | 0.06 |
| Note 1: According to Report ITU-R M.2410, the 5th percentile user spectral efficiency requriement is not applicable to LMLC. The value shown here is for information only. |  |  |  |  |  |  |  |