5	Self evaluation of eMBB technical performance

[Editor’s note: The grouping of the technical performance requirements for eMBB usage scenario are based on Report ITU-R M.2410 “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”. However, it does not imply that each of the following technical performance requirements is only related to eMBB. It is just the minimum requirement that shall be evaluated under eMBB. This should also be noted for Section 6 and 7.]

5.6	Area traffic capacity

As defined in Report ITU-R M.2410, area traffic capacity is the total traffic throughput served per geographic area (in Mbit/s/m2). The throughput is the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time.

5.6.1	NR

NR evaluation for area traffic capacity is conducted for Indoor Hotspot – eMBB test environment.

5.6.1.1	Indoor Hotspot – eMBB

The area traffic capacity of NR is evaluated using analytical way based on the downlink average spectral efficiency evaluation for NR in Indoor Hotspot – eMBB test environment. The analytical way is defined in Report ITU-R M.2412. A wide range of antenna configurations and transmission schemes are considered. Detailed evaluation assumptions and results can be found in RP-181816.

5.6.1.1.1	Evaluation configuration A (CF = 4 GHz)

The evaluation results of area traffic capacity for NR FDD and NR TDD for evaluation configuration A with 12TRxP are provided in Table 5.6.1.1.1-1.

It is assumed that for FDD and TDD with 15 kHz SCS, a component carrier with 40 MHz is used; and for TDD with 30 kHz SCS, a component carrier with 100 MHz is used. Multiple component carriers are aggregated to achieve the target area traffic capacity. The assumed aggregated system bandwidth is given in Table 5.6.1.1.1-1.

Table 5.6.1.1.1-1 Area traffic capacity for NR in Indoor Hotspot – eMBB 
(Evaluation configuration A, CF=4 GHz, for 12TRxP)

(a) NR FDD

| Scheme and antenna configuration | Sub-carrier spacing | ITU Requirement [Mbps/m2] | Channel model A |  |  | Channel model B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Number of samples | Assumed system bandwidth [MHz] | Area traffic capacity [Mbps/m2] | Number of samples | Assumed system bandwidth [MHz] | Area traffic capacity [Mbps/m2] |
| 32x4 MU-MIMO Type II Codebook (32Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (4,4,2,1,1;4,4) | 15 | 10 | 10 | 400 | 10.48 | 5 | 400 | 10.83 |
| 32x4 MU-MIMO Type I Codebook (32Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (4,4,2,1,1;4,4) | 15 | 10 | 1 | 360 | 10.06 | / | / | / |
| 32x4 MU-MIMO Ideal CSI feedback (32Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (4,4,2,1,1;4,4) | 15 | 10 | / | / | / | 1 | 360 | 10.70 |
| 32x8 MU-MIMO Type II Codebook (32Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (4,4,2,1,1;4,4) | 15 | 10 | 1 | 440 | 10.30 | / | / | / |

(b) NR TDD

| Scheme and antenna configuration | Sub-carrier spacing | Frame structure | ITU Requirement [Mbps/m2] | Channel model A |  |  | Channel model B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Number of samples | Assumed system bandwidth [MHz] | Area traffic capacity [Mbps/m2] | Number of samples | Assumed system bandwidth [MHz] | Area traffic capacity [Mbps/m2] |
| 32x4 MU-MIMO, 4T SRS; (32Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (4,4,2,1,1;4,4) | 30 | DDDSU; S (10D,2G,2U) | 10 | 1 | 400 | 10.13 | 3 | 500 | 11.79 |
| 32x4 MU-MIMO, 4T SRS gNB: (M,N,P,Mg,Ng; Mp,Np) = (4,4,2,1,1;4,4) | 15 | DDDSU; S (10D,2G,2U) | 10 | 1 | 480 | 10.65 | 1 | 480 | 10.64 |
| 32x4 MU-MIMO, Type II Codebook, gNB: (M,N,P,Mg,Ng; Mp,Np) = (4,4,2,1,1;4,4) | 15 | DSUUD; S (11D,1G,2U) | 10 | 1 | 600 | 10.28 | 1 | 600 | 10.50 |
| 32x4 MU-MIMO, 4T SRS, gNB: (M,N,P,Mg,Ng; Mp,Np) = (4,4,2,1,1;4,4) | 30 | DSUUD; S (11D,1G,2U) | 10 | 1 | 600 | 10.71 | 1 | 600 | 10.91 |
| 32x4 MU-MIMO, 4T SRS, gNB: (M,N,P,Mg,Ng; Mp,Np) = (4,4,2,1,1;4,4) | 15 | DSUUD; S (11D,1G,2U) | 10 | / | / | / | 3 | 640 | 10.14 |

The evaluation results of area traffic capacity for NR FDD and NR TDD for evaluation configuration A with 36TRxP are provided in Table 5.6.1.1.1-2.

It is assumed that for FDD and TDD with 15 kHz SCS, a component carrier with 40 MHz is used; and for TDD with 30 kHz SCS, a component carrier with 100 MHz is used. Multiple component carriers are aggregated to achieve the DL target user experienced data rate. The assumed aggregated system bandwidth is given in Table 5.5.1.1.1-2.

It is observed that both NR FDD and TDD fulfill the area traffic capacity requirement for all the above configurations in evaluation configuration A.

Table 5.6.1.1.1-2 Area traffic capacity for NR in Indoor Hotspot – eMBB 
(Evaluation configuration A, CF=4 GHz, for 36TRxP)

(a) NR FDD

| Scheme and antenna configuration | Sub-carrier spacing | ITU Requirement [Mbps/m2] | Channel model A |  |  | Channel model B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Number of samples | Assumed system bandwidth [MHz] | Area traffic capacity [Mbps/m2] | Number of samples | Assumed system bandwidth [MHz] | Area traffic capacity [Mbps/m2] |
| 32x4 MU-MIMO Type II Codebook (256Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (8,16,2,1,1;2,8) | 15 | 10 | 7 | 160 | 13.10 | 4 | 120 | 10.68 |
| 32x4 MU-MIMO ideal CSI feedback (128Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;4,4) | 15 | 10 | / | / | / | 1 | 120 | 11.51 |
| 32x4MU-MIMO Type I codebook (32Tx@gNB); (M,N,P,Mg,Ng; Mp,Np) = (4,4,2,1,1;4,4) | 15 | 10 | 1 | 160 | 10.24 | / | / | / |

(b) NR TDD

| Scheme and antenna configuration | Sub-carrier spacing | Frame structure | ITU Requirement [Mbps/m2] | Channel model A |  |  | Channel model B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Number of samples | Assumed system bandwidth [MHz] | Area traffic capacity [Mbps/m2] | Number of samples | Assumed system bandwidth [MHz] | Area traffic capacity [Mbps/m2] |
| 32x4 MU-MIMO, 4T SRS (256Tx@gNB), gNB: (M,N,P,Mg,Ng; Mp,Np) = (8,16,2,1,1;2,8) | 30 | DDDSU; S (10D,2G,2U) | 10 | 1 | 200 | 16.67 | 1 | 200 | 16.81 |
| 32x4 MU-MIMO, 4T SRS (256Tx@gNB), gNB: (M,N,P,Mg,Ng; Mp,Np) = (8,16,2,1,1;2,8) | 15 | DDDSU; S (10D,2G,2U) | 10 | 1 | 160 | 12.15 | 1 | 160 | 12.24 |
| 32x4 MU-MIMO, Type II Codebook (256Tx@gNB), gNB: (M,N,P,Mg,Ng; Mp,Np) = (8,16,2,1,1;2,8) | 15 | DSUUD; S (6D,2G,6U) | 10 | 1 | 280 | 10.37 | / | / | / |
| 32x4 MU-MIMO, Type II Codebook (256Tx@gNB), gNB: (M,N,P,Mg,Ng; Mp,Np) = (8,16,2,1,1;2,8) | 15 | DSUUD; S (11D,1G,2U) | 10 | 2 | 240 | 11.28 | 2 | 240 | 11.47 |
| 32x4 MU-MIMO, Type II Codebook (256Tx@gNB), gNB: (M,N,P,Mg,Ng; Mp,Np) = (8,16,2,1,1;2,8) | 30 | DSUUD; S (11D,1G,2U) | 10 | 1 | 200 | 11.28 | 1 | 200 | 11.43 |

5.6.1.1.2	Evaluation configuration B (CF = 30 GHz)

The evaluation results of area traffic capacity for NR for evaluation configuration B with 12TRxP are provided in Table 5.6.1.1.2-1.

It is assumed that for TDD with 60 kHz SCS, a component carrier with 200 MHz is used; and for TDD with 120 kHz SCS, a component carrier with 400 MHz is used. Multiple component carriers are aggregated to achieve the target area traffic capacity. The assumed aggregated system bandwidth is given in Table 5.6.1.1.2-1.

Table 5.6.1.1.2-1 Area traffic capacity for NR in Indoor Hotspot – eMBB 
(Evaluation configuration B, CF=30 GHz, for 12TRxP)

(a) NR TDD

| Scheme and antenna configuration | Sub-carrier spacing | Frame structure | ITU Requirement [Mbps/m2] | Number of samples | Assumed system bandwidth [MHz] | Area traffic capacity [Mbps/m2] |
| --- | --- | --- | --- | --- | --- | --- |
| 32x8 MU-MIMO, 4T SRS (2 panel@UE), gNB: (M,N,P,Mg,Ng; Mp,Np) = (4,4,2,1,1;4,4); UE: (M,N,P,Mg,Ng; Mp,Np) = (2,4,2,1,2; 1,2) | 60 | DDDSU; S (10D,2G,2U) | 10 | 2 | 600 | 12.79 |
| 32x8 MU-MIMO, 4T SRS (2 panel@UE), gNB: (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;2,8); UE: (M,N,P,Mg,Ng; Mp,Np) = (2,4,2,1,2; 1,2) | 60 | DSUUD; S (11D,1G,2U) | 10 | 1 | 600 | 11.57 |
| 32x8 MU-MIMO, 4T SRS (2 panel@UE), gNB: (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;2,8); UE: (M,N,P,Mg,Ng; Mp,Np) = (2,4,2,1,2; 1,2) | 120 | DSUUD; S (11D,1G,2U) | 10 | 1 | 800 | 15.93 |
| 64x8 MU-MIMO, 4T SRS (2 panel@UE), gNB: (M,N,P,Mg,Ng; Mp,Np) = (8,16,2,1,1;2,16); UE: (M,N,P,Mg,Ng; Mp,Np) = (2,4,2,1,2; 1,2) | 60 | DSUUD; S (11D,1G,2U) | 10 | 1 | 600 | 13.02 |
| 64x8 MU-MIMO, 4T SRS (2 panel@UE), gNB: (M,N,P,Mg,Ng; Mp,Np) = (8,16,2,1,1;2,16); UE: (M,N,P,Mg,Ng; Mp,Np) = (2,4,2,1,2; 1,2) | 120 | DSUUD; S (11D,1G,2U) | 10 | 1 | 800 | 17.98 |
| 8x4 , MU-MIMO, 2T SRS (1 panel at UE), gNB: (M,N,P,Mg,Ng; Mp,Np) = (16,8,2,1,1;2,2); UE: (M,N,P,Mg,Ng; Mp,Np) = (4,4,2,1,1; 1,2) | 120 | DDDU | 10 | 1 | 800 | 17.96 |

The evaluation results of area traffic capacity for NR for evaluation configuration B with 36TRxP are provided in Table 5.6.1.1.2-2.

It is assumed that for TDD with 60 kHz SCS, a component carrier with 200 MHz is used; and for TDD with 120 kHz SCS, a component carrier with 400 MHz is used. Multiple component carriers are aggregated to achieve the target area traffic capacity. The assumed aggregated system bandwidth is given in Table 5.6.1.1.2-2.

It is observed that both NR FDD and TDD fulfill the area traffic capacity requirement for all the above configurations in evaluation configuration B.

Table 5.6.1.1.2-2 Area traffic capacity for NR in Indoor Hotspot – eMBB 
(Evaluation configuration B, CF=30 GHz, for 36TRxP)

(a) NR TDD

| Scheme and antenna configuration | Sub-carrier spacing | Frame structure | ITU Requirement [Mbps/m2] | Number of samples | Assumed system bandwidth [MHz] | Area traffic capacity [Mbps/m2] |
| --- | --- | --- | --- | --- | --- | --- |
| 32x8 MU-MIMO, 4T SRS (2 panel@UE), gNB: (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;2,8); UE: (M,N,P,Mg,Ng; Mp,Np) = (2,4,2,1,2; 1,2) | 60 | DSUUD; S (11D,1G,2U) | 10 | 1 | 400 | 19.77 |
| 32x8 MU-MIMO, 4T SRS (2 panel@UE), gNB: (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;2,8); UE: (M,N,P,Mg,Ng; Mp,Np) = (2,4,2,1,2; 1,2) | 120 | DSUUD; S (11D,1G,2U) | 10 | 1 | 400 | 19.39 |
| 64x8 MU-MIMO, 4T SRS (2 panel@UE), gNB: (M,N,P,Mg,Ng; Mp,Np) = (8,16,2,1,1;2,16); UE: (M,N,P,Mg,Ng; Mp,Np) = (2,4,2,1,2; 1,2) | 60 | DSUUD; S (11D,1G,2U) | 10 | 1 | 200 | 11.54 |
| 64x8 MU-MIMO, 4T SRS (2 panel@UE), gNB: (M,N,P,Mg,Ng; Mp,Np) = (8,16,2,1,1;2,16); UE: (M,N,P,Mg,Ng; Mp,Np) = (2,4,2,1,2; 1,2) | 120 | DSUUD; S (11D,1G,2U) | 10 | 1 | 400 | 22.76 |